3GPP TSG RAN Meeting #109 								                   RP-251969

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	OPPO

Title:	Discussion on requirements of 6G new and existing services

Document for:	Discussion

Introduction

The objective part of the approved RAN WG SID [1] states:

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services.

Rather than providing end-to-end services on its own, RAN formulates air-interface relevant capabilities to enable the commercial deployment of various services ranging from basic voice to multi-modality data streams of XR. We believe it is very important to have a discussion on requirements of new services as well as evolution of existing services, from RAN perspective, to reach consensus at RAN plenary level and to provide guidance to future RAN WG studies.

Discussion

Traditionally, telecommunication systems mainly focus on the transportation of information streams with various quantitative and qualitative requirements (e.g. to support service classes like interactive, background, streaming and conversational), together with supplementary services like location/positioning. The 5G radio access system supports various deployment scenarios, and its performance are evaluated against a list of key performance indicators [2]. Hundreds of 5G networks have been deployed since 2019 and billions of end users have subscribed 5G services world-wide.

For the commercial success of a new generation of system, it will be natural to expect improvements from two aspects based on the experience of 5G:

1. The reduction of cost under similar service scenarios

2. The creation of new value by fostering new types of services

These two aspects, which are well reflected in the RAN WG SID [1], are discussed in detail in the following parts.

Requirements of evolution of existing services in 6G

5G RAN offers support to various services including mainstream data services (e.g., interactive, background, streaming and conversational), supplementary services like Multimedia Broadcast/Multicast Service, location/positioning Service, Critical Communications services (e.g., public safety communications, emergency communications and public warning/emergency alert systems) and V2X communication. In later phases of 5G, various technical solutions like RedCap/eRedCap and Ambient IoT were developed towards scenario-oriented services a.k.a., “vertical” use cases like Cellular IoT and short-range logistics/asset management. Additionally, the 5G-NTN technology was introduced targeting the coverage of areas not technically feasible or economically viable to terrestrial networks.

However, some services offered by 5G e.g., URLLC and V2X, have not been actually deployed in noticeable scale so far due to reasons like cost and complexity, as well as mismatch with evolution cycles of “vertical” industries. RAN technologies introduced to support these services often involve modifications of existing specifications, complex compatibility checks and sometimes confusing UE feature subgroupings, all of which significantly burden the RAN working groups and have negative impacts on the evolution of essential 5G features. Thus, the continued support or evolution of these “unused” existing services (e.g., URLLC) shall be well justified e.g., by strong and clear market demand, and de-prioritized in the first 6G release.

For 6G, it could be confidently predicted that eMBB types of services like voice and data will continue to be fundamental and thus, 6G eMBB shall be continuously enhanced for performance and cost “with acceptable performance/complexity trade-off” [1]. Cellular IoT based on eMBB has also been widely deployed in 5G. These services should be supported in 6G with high priority.

Just like previous generations, for eMBB-type and other existing services, the ability to provide large and seamless coverage and sufficient capacity with reasonable cost and engineering efforts is critical to the proliferation of 6G services and consequently, the commercial success. If we further breakdown the composition of cost, it may be found out that both CAPEX (capital expenditures, e.g., the cost to build up a network) and OPEX (Operating Expenditure, e.g., the cost to maintain operation of a network) are important. 6G RAN should provide better cost control mechanisms than previous generations e.g., by incorporating joint network and UE energy saving from the very first release.

6G NTN may continue to evolve on the basis of 5G NTN while at the same time share the technical framework of 6G eMBB to gain economy of scale.

- Proposal 1	The 6G RAN shall support eMBB types of services like voice, data, and cellular IoT with high priority and better CAPEX and OPEX control mechanisms.

- Proposal 2	The continued support or evolution of “unused” existing services shall be well justified and de-prioritized in the 6G RAN first release.

- Proposal 3	The 6G NTN may continue to evolve on the basis of 5G NTN and at the same time share the technical framework with 6G eMBB.

Requirements of new services in 6G

6G should aim to provide substantial values to end users and the industry by fostering new value-adding services. 6G new service may be categorized into 1. communication oriented services; 2. non-communication-oriented services. 6GR should be able to support both communication oriented and non-communication oriented new services with minimum impact to existing services and reasonable cost and complexity.

For the first category of new services, lessons from previous generations show that technologies introduced in later stages are more prone to difficulties brought by backward compatibility if the benefits provided by the new technologies are not big enough. It is understood that since the cost of upgrading the infrastructure, replacing gigantic volume of existing devices and re-integrating operational systems may be prohibitively high, many users may choose to stick to “older” technologies. For example, from device perspective, instead of reusing a lower capability 5G modem design, the RedCap/eRedCap need a redesign of modem chipset, which introduces a new cost, rather than directly saving the cost. So, if the quantity of deployed devices has not passed a certain threshold, the deployment of RedCap/eRedCap would not be economically beneficial.

One possible solution is to design the 6G RAN in a “bottom-up” way i.e., to firstly define a “minimum” functionality set for both the network and the UE, and to add-up additional capabilities as needed from different usage scenarios. A “functionality set” may contain PHY parameters like supported bandwidth, number of antenna ports/MIMO layers, HARQ related parameters (process number, soft buffer...), as well as MAC and RRC procedure related functionalities e.g., different level of mobility support. One example of “bottom-up” 6G RAN system design is shown in Figure 1.

For new scenario-oriented services a.k.a., “verticals”, the most important improvement in 6G may be the capability to share the technical framework with eMBB from Day One, so they will be able to share the economy of scale brought by the vast number of devices and network equipment of eMBB. Scenario-specific features/enhancements could be developed as “add-ons” or “plug-ins” to eMBB technical base without introducing issues to backward compatibility.

Figure 1. functionality-set based modular system design for 6G

There could be many different kinds of non-communication-oriented services to be supported by 6G RAN. Potential new services like AI agents, data frame work and compute are been heatedly discussed among SA working groups, and RAN should take sufficient precautions to accommodate these potential new services e.g., architecture support of potential new services from the RAN side to fulfil performance requirements with reasonable complexity.

One example of potential new services for RAN to support is the error tolerant communication [4]. In this use case, the information source like real-time VR streaming may be divided into a non-error-tolerant class (e.g., contents of eye-tracking and fast-moving objects, audio, haptic information for both DL and UL) and an error-tolerant class (e.g., background or slow-moving objects). When the radio channel propagation conditions turn worse (e.g., the user takes a high-speed vehicle or goes into insufficiently covered rural areas), the transmitter recognizes this change and reduce the bits (down to zero) allocated to the error-tolerant parts to guarantee the transmission of non-error-tolerant parts, and the receiver recovers the background or slow-moving objects using an on-board AI model and a (pre-installed) semantic library. The overall end-to-end latency from the source to the receiver should be minimized to provide a satisfactory experience to the end user. Figure 2 shows the procedure.

Figure 2. Error-tolerant data transmission for real-time immersive service

- Proposal 4	The 6G RAN shall support both communication-oriented and non-communication-oriented new services in a future-proof way with reasonable cost and complexity.

- Proposal 5	New scenario-oriented services shall strive to have common technical framework with eMBB through modular design, and scenario-specific features/enhancements could be developed as “add-ons” or “plug-ins” without impacting backward and forward compatibility

Conclusion

In this contribution requirements of evolution of existing services and requirements of new services in 6G RAN are discussed. The conclusion and proposals to the TR38.914 are summarized as follows.

- Proposal 1	The 6G RAN shall support eMBB types of services like voice, data, and cellular IoT with high priority and better CAPEX and OPEX control mechanisms.

- Proposal 2	The continued support or evolution of “unused” existing services shall be well justified and de-prioritized in the 6G RAN first release.

- Proposal 3	The 6G NTN may continue to evolve on the basis of 5G NTN and at the same time share the technical framework with 6G eMBB.

- Proposal 4	The 6G RAN shall support both communication-oriented and non-communication-oriented new services in a future-proof way with reasonable cost and complexity.

- Proposal 5	New scenario-oriented services shall strive to have common technical framework with eMBB through modular design, and scenario-specific features/enhancements could be developed as “add-ons” or “plug-ins” without impacting backward and forward compatibility.

Reference

- RP-251881	New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- 3GPP TR38.913 V18.0.0, Study on Scenarios and Requirements for Next Generation Access Technologies, 2024-03

- RP-251873	3GPP TR38.914 0.1.0, Study on 6G Scenarios and Requirements, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- S1-253521	Update of use case on real-time VR live service with deterministic user experience, OPPO, Huawei, 3GPP TSG SA WG1 Meeting #111,	25-29 August 2025, Goteborg, Sweden

Text Proposal to TR38.914 v0.1.0

TP Start

5.4	Requirements of New and Existing Services

5.4.1 Requirements of evolution of existing services in 6G

The 6G RAN shall support eMBB types of services like voice, data, and cellular IoT with high priority and better CAPEX and OPEX control mechanisms.

The continued support or evolution of “unused” existing services shall be well justified and de-prioritized in the 6G RAN first release.

The 6G NTN may continue to evolve on the basis of 5G NTN and at the same time share the technical framework with 6G eMBB.

5.4.2 Requirements of new services in 6G

The 6G RAN shall support both communication-oriented and non-communication-oriented new services in a future-proof way with reasonable cost and complexity.

New scenario-oriented services shall strive to have common technical framework with eMBB through modular design, and scenario-specific features/enhancements could be developed as “add-ons” or “plug-ins” without impacting backward and forward compatibility.

End of TP