3GPP TSG RAN Meeting #109	                                                   RP-251975

Beijing, China, September 15-18, 2025

## Status Report to TSG

Agenda item:			10.5.3

| WI / SI Name | New WID on LTE-based 5G Broadcast Phase 2 |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | LTE_terr_bcast_Ph2 |  |  |  |
| Unique ID | 1060081 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250794 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 12/2025 | Testing part: N/A |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 25% | Testing part: N/A |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | De Vita, Assunta |
|  | Company | EBU |
|  | Email | assunta.devita@rai.it |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements from RAN1#122

### 2.1.1.1 Time-frequency interleaver design for MBMS dedicated cells

Agreement

Adopt the following text proposal for TS 36.211:

- Reason for change: In some cases, the value of d in the frequency interleaver may be negative or zero, in which case the row permutation is incorrect;

- Summary of changes: Clarify that row permutation is only applied if RK-K+1>0;

- Consequence if not approved: Specification is not implementable for cases of RK-K+1≤0.

******** start of TP *************

6.5.2 Frequency Domain interleaving

The input to the interleaver is the block of complex-valued symbols yp0, …, yp(Msymbap-1)  defined in clause 6.3.4.1 and the output from the interleaver is a block of complex-valued symbols yp0, …, yp(Msymbap-1).

<<unchanged text>>

Permute the rows , if RK-K+1>0, such that row ri is moved to row i where

ri=ri-1+dri-1≤R-d-1idRotherwise
d=RRRK-K+1

******** end of TP *************

Note: it can be discussed in a future meeting whether an essential correction is needed for the case where RK-K+1≤0.

Agreement

The contents to be included in the MBMS-ROM-Info-19 are as follows:

MBMS-ROM-Info-r19 ::= SEQUENCE { mbms-ROM-Freq-r19 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r19 ENUMERATED {kHz15, kHz7dot5, kHz2dot5, kHz1dot25}, mbms-Bandwidth-r19 ENUMERATED {n6, n15, n25, n30, n35, n40, n50, n75, n100}

Agreement

To maintain backwards compatibility with legacy non-time-frequency-interleaved PMCHs associated an MCCH, frequency interleaving is not applied to MCCH and MSI.

Agreement

Additional contents to be included in the MBMS-ROM-Info-19 is as follows:

β and Nsoftref as described in TS 36.212, for MCH with time-interleaving

Agreement

Adopt the following text proposal for TS 36.211, with a separate UE capability for the newly added alternative:

- Reason for change: In some cases, it is observed that an alternative value of the cyclic shift may provide performance improvement;

- Summary of changes: Add two new methods for cyclic shift;

- Consequence if not approved: New values of cyclic shift are not supported.

6.5.1  Cyclic Shift for PMCH

The input to the cyclic shift is the block of bits b0, …, bMbit-1 defined in clause 6.3.1 and the ouput is a block of bits b0, …, bMbit-1.

The block of bits b0, …, bMbit-1 shall be cyclically shifted by Xi bits to form the block b0, …, bMbit-1 according to

bn=b(n-Xi mod Mbit)

where

-	Xi is given by

Xi=r=C-SiC-1Er

Si=iα mod C

-	i∈0, 1, …, N-1 denotes the ith subframe to which the transport block is mapped

-	N is the number of subframes to which the transport block is mapped

-	Er is the number of bits in the rth codeblock within a subframe as defined in TS 36.212 [3]

-	C is the number of codeblocks in the transport block

-	α is given by

-	if the higher-layer parameter XXX is set to alphaOne

α=1

-	if the higher-layer parameter XXX is set to alphaOther

α=CNL

- if the higher-layer parameter XXX is set to alphaOther2

Xi= Ainf ,nSF×floor(NSC ×QmN), 	Ainf ,nSF ∈{0,…N-1}

where Ainf ,nSF= m=07c8nf mod 12810+nSF+m2mmod N where nf is the system frame number and nSF is subframe number within the frame. The pseudo-random sequence ci is defined by clause 7.2 and shall be initialized with cinit=NIDMBSFN at the beginning of each radio frame for which nf mod 128=0, where  NSC  is the number of subcarriers available in 1 OFDM symbol for PMCH,  NSC =Mbit/(QmL), Qm is the modulation order.

- where L is the number of OFDM symbols used for the PMCH transmission.

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements from RAN2#131

### 2.2.1.1 Signalling impacts to support time-frequency interleavers

Agreements

Agreements on RRC open issues

- RRC-4: RRC capability CR will be merged into overall RRC CR

- For open issue RRC-1: Introduce Rel-19 specific MBS Interest Indication IE (MBMSInterestIndication-v19xy-IEs) taking the TP from R2-2505413 as baseline. We will consider RAN1 agreements. Consider also whether procedural changes, e.g. as in R2-2505557 are needed.

- RRC-2: Capture the RAN1 agreement in stage-2 running CR.

- For open issue RRC-3: Wait for RAN1 further update, if any, and finalize during RRC CR review.

Agreements on MAC open issues

- MAC-1: RAN2 to specify the HARQ handling for Rel-19 time-interleaved MCH transmission in MAC specification. (Adopt TP 1 as baseline, can attempt to simplify)

- MAC-2: Mirror the same changes from 6.1.3.7 to 6.1.3.7a in MAC CR.

- Following RAN1 agreement, RAN2 should clarify in MAC that the number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

#### 2.2.2	Remaining Open issues

None.

## 2.3	RAN3

#### 2.3.1	Agreements from RAN3#129

2.3.1.1   Signalling Support

Agreements

- R3-255811 Introduction of LTE-based 5G Broadcast Phase 2

#### 2.3.2	Remaining Open issues

None

## 2.4	RAN4

#### 2.4.1	Agreements from RAN4#116

### 2.4.1.1    UE demodulation for time/frequency interleaving

Agreements

- R4-2512620  WF for [116][334] 5G_Broadcast_Ph2

#### 2.4.2	Remaining Open issues

Start discussion on evaluation results and finalize requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#122

- R1-2505232	Maintenance for LTE based 5G broadcast	Huawei, HiSilicon

- R1-2505505	Discussion on Time-frequency interleaver design for MBMS dedicated cells	ZTE Corporation, Sanechips

- R1-2505559	Time-frequency interleaver design for MBMS dedicated cells	Samsung

- R1-2505645	Maintenance on LTE-based 5G Broadcast	Xiaomi

- R1-2506194	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

RAN2#131

- R2-2505022	LS on RAN2 aspects for LTE-based 5G Broadcast Phase 2 (R1-2504922; contact: EBU)	RAN1	LS in	Rel-19	LTE_terr_bcast_Ph2	To:RAN2

- R2-2505411	Introduction of LTE-based 5G Broadcast Phase 2	Qualcomm Incorporated, EBU	CR	Rel-19	36.331	18.6.0	5143	-	B	LTE_terr_bcast_Ph2-Core

- R2-2505412	[POST130][510][LTE Broadcast] RRC Open Issues	Qualcomm Incorporated, EBU	report	Rel-19	LTE_terr_bcast_Ph2-Core

- R2-2505554	[POST130][511][LTE Broadcast] MAC Open Issues	Samsung	report

- R2-2505556	Introduction of LTE-based 5G Broadcast Phase 2	Samsung	CR	Rel-19	36.321	18.4.0	1593	-	B	LTE_terr_bcast_Ph2-Core

- R2-2505740	Introduction of LTE-based 5G Broadcast Phase 2	Huawei, HiSilicon	CR	Rel-19	36.306	18.5.0	1920	-	B	LTE_terr_bcast_Ph2

- R2-2505741	Introduction of LTE-based 5G Broadcast Phase 2	Huawei, HiSilicon, Qualcomm Incorporated	CR	Rel-19	36.331	18.6.0	5144	-	B	LTE_terr_bcast_Ph2

- R2-2505799	Introduction of LTE-based 5G Broadcast Phase 2	ZTE Corporation, Sanechips	CR	Rel-19	36.300	18.5.0	1428	-	B	LTE_terr_bcast_Ph2-Core

RAN3#129

- R3-255674 Consideration on LTE-based 5G Broadcast Phase 2  Huawei, Qualcomm Incorporated, EBU

- R3-255675 Introduction of LTE-based 5G Broadcast Phase 2  Huawei, Qualcomm Incorporated, EBU

- R3-255739 Discussion on LTE-based 5G Broadcast  ZTE Corporation

- R3-255740 Correction for LTE-based 5G Broadcast  ZTE Corporation

RAN4#116

- R4-2509231 Work plan for LTE-based 5G Broadcast Phase 2 (RAN4 Performance Part)   EBU

- R4-2509030 On demodulation test cases for time-frequency interleaving for 5G Broadcast  Qualcomm Technologies Ireland