3GPP TSG RAN Meeting #109 								                   RP-251998

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.2

Source:	OPPO

Title:	Discussion on UE-type based feature sets

Document for:	Discussion

Introduction

UE types and related features/capabilities has always been a profound topic ever since 3G times and this is not only a technical issue, but also in many cases is directly related to the UE availability, time-to-market, etc. and eventually affects the market success of relevant services. This observation reflects the complex nature of real-world usage scenarios and the strong demands on suitable UE formfactors and radio features/capabilities. This document discusses the pros and cons of different options and the benefits of UE-type based feature sets and capability definition.

Discussion

Retrospect on 4G and 5G UE capability definition and reporting

A glance back into the history reveals that when LTE specifications were drafted, UE categories (including DL-category and UL-category) were defined primarily focusing on peak data rate of both DL and UL utilizing parameters like maximum number of DL/UL-SCH transport block bits received within a TTI and maximum number of supported layers for spatial multiplexing [1]. Additionally, features not directly related to data rate are reported using UE Capability report. Later on, the “feature group indicator (FGI)” was introduced to indicate mandatory features together with their implementation and IoDT status [3, Clause B1].

This arrangement seemed reasonable at the time since most LTE UEs were smartphones and the types of expected services are mainly mobile Internet content delivery, so the peak data rate, which in many cases could be more or less proportional to the achievable user throughput, to some extent reflects the level of services end users could enjoy e.g., the time to load a complex web page and whether the user could watch an online HD video smoothly on their smartphones. Thus, the UE category concept was able to provide some guidance to the end users when they were choosing their next smartphone.

In the 5G era, as the number of target usage scenarios vastly increased (eMBB, mMTC and URLLC; various “vertical” scenarios etc.) and number of features surged, it became increasingly difficult and less meaningful to continue using peak data rate-oriented UE-categories. Thus, 5G NR uses the universal method for the UE to report all capabilities using RRC signalling. However, new problems emerged e.g., with the introduction of new UE-capabilities every release, the size of RRC messages also increased. And in cases e.g., when CA is configured and handover is happening, the RRC message size exceeds the limit of a PDCP SDU/PDCP Control PDU i.e., 9K Bytes. Thus, segmentation of RRC messages was necessary, which made the operations of both gNB and UE more complex.

From implementation and deployment perspectives, too many UE capabilities without meaningful categorization have brought complexity and additional costs to the whole industry, at least on the following aspects:

- For both chipset and infra-vendors, it is hard to decide which feature(s) should be developed, IoDTed and finally commercialized

- For UE vendors and operators, it is hard to decide on mandatory /optional capability categorization.

- Only “globally mandatory for all device types” kinds of features are actually deployed in the market. Optional features are very much not likely deployed, even if they are “truly mandatory” for certain device types. And this practice seriously limits the room of performance and cost optimization for many scenario-oriented applications e.g., NES, FWA, while introduces unnecessary redundancies across various devices targeting different usage scenarios since many “globally mandatory” features are actually NOT required in these scenarios but have to be implemented

As a summary, we may deduce the following observation for the handling of device features in 4G and 5G:

Observation: the handling of device features/capabilities in 4G and 5G has serious limitations and cannot meet the requirement of performance and cost optimization for various application scenarios.

Potential UE-type based feature sets definition in 6G and relevant capability categorization

The 6G targets at even wider range of usage scenarios - “anticipated diverse service classes ranging from immersive multimedia and massive IoT to integrated communications and sensing” [4], thus the definition of UE types and relevant capability reporting are becoming imminently important for economically viable network roll-out and large-scale device penetration.

The 6G WG SID [4] demands the study to “target scalable and forward compatible design for diverse device types”, while RAN1 #122 started discussion on this topic and reached the following agreements as a first step [5]:

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

Agreement

- Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

We believe one of the best possible ways to achieve optimized performance for specific application scenarios while at the same time to enjoy the economy of scale from a single-origin “unified” ecosystem, is to define different UE-types optimized for different scenarios, and at the same time keeping a common technology base. Here the different UE-types may own different feature sets, and the feature set of one UE-type is not necessarily the super-set or sub-set of the feature set of another UE-type. And the contents of this common technology base are up to further study, where some possible components could be listed like unified initial access scheme including SSB design.

One candidate solution is to adopt device-type specific UE features as the base of UE capability categorization:

- Define a lean “global” mandatory feature set for all device types, where each feature in the list shall be well justified to be truly mandatory

  - eMBB based low-end IoT could be a starting point/template of the “global” mandatory feature set

- Define different feature lists (with a mandatory/optional tag for each feature) on top of the “global” mandatory feature set for different device types, e.g., eMBB, Wearable, NTN, FWA, Sensing

  - Here the “mandatory” features are actually device-type specifically mandatory, i.e., conditionally mandatory

  - The device type specific features cover both data-rate related and non-data-rate related features e.g., energy efficiency, mobility, sensing

A possible format of device-type specific feature list is shown in Table 1.

| Mandatory Capability | Device type 1 | Device type 2 | Device type 3 | … | … |
| --- | --- | --- | --- | --- | --- |
| Feature 1 |  |  |  |  |  |
| Feature 2 |  |  |  |  |  |
| Feature 3 |  |  |  |  |  |
| … |  |  |  |  |  |

- Table 1. a candidate format of the device-type specific feature list

- Here we list some questions for further study:

Question 1: What device types should be categorized in the 6G first release? Should NTN and Sensing be included?

Question 2: What precautions should be taken for device types introduced in later releases e.g., compatibility issues?

Question 3: What criteria should be used to identify device-type-specific features?

Question 4: How to identify the values or value ranges for quantitative device-type-specific features?

Question 5: How to define the “global” mandatory feature set for all device types? Should the features of the lowest device type be mandatory for all device types?

Question 6: Should the Base Station also define mandatory feature sets in correspondence to the UE types in order to support usage scenarios with full capability and high efficiency?

- One example of a device type based feature set is shown in Table 2:

| Mandatory Capability | Device type |  |  |  |
| --- | --- | --- | --- | --- |
|  | Low-end IoT UE | High-end IoT UE (e.g., Wearable) | Handheld eMBB/Immersive Communication UE | Non-handheld large-size UE (e.g., FWA/CPE, HMD) |
| Max. CBW | [5MHz] | [20MHz] | [200MHz] | [400MHz] |
| Max. number of MIMO layers | [1T1R] | [1T2R] | [3T6R] | [4T8R] |
| Data rate related Feature 3 |  |  |  |  |
| Data rate related Feature 4 |  |  |  |  |
| … |  |  |  |  |
| Non-data rate related feature 1 | xxx |  | N/A |  |
| Non-data rate related feature 2 |  |  |  |  |
| … |  |  |  |  |

- Table 2. example of a device-type specific feature list

Note: The device types listed in the table are only for example. Additional types are not precluded, e.g., NTN, sensing, vehicles.

Note: The features listed in the table are only for example. Other features are not precluded, e.g., duplex, sensing, mobility, energy-efficient mode, enhanced coverage.

- As a summary, we have the following proposals:

- Proposal 1:	6G RAN should define device types according to usage scenarios for mandatory/optional UE capability categorization.

      - Taking the categorization similar as in Table 2 as a starting point.

- Proposal 2:	Each device type associates with a mandatory feature list which optimizes performance and cost for the target usage scenario.

Global mandatory feature set for all device types

To achieve the “scalable and forward compatible design for diverse device types” as required in 6GR SID [4], a scalable feature system can be adopted for 6GR air interface, as shown in Figure 1. A low-cost and energy-efficient “Basic feature set” can be defined for 6GR air interface, which provides “global” mandatory feature for various device types. For a specific device type, a device-type specific feature set can be defined which “grows up” from the Basic feature set.

  - Beyond the Basic feature set, device type specific feature sets can be defined for 6G MBB (Smart phone, Wearable IoT), low-end IoT and other verticals.

  - The Basic feature set is a lean “global” mandatory feature list for all device types, where each feature in the list shall be well justified to be truly mandatory, and provides basic energy-efficient features for various device types.

Figure 1. Scalable feature system for 6GR

- Proposal 3:	Define a lean “global” mandatory feature set list for all device types, where each feature in the list shall be well justified to be truly mandatory, and provides basic energy-efficient features for various device types.

- Proposal 4:	Define add-on device type specific feature lists (with mandatory/optional marked for each feature) on top of the “global” mandatory feature set for different device types, e.g., eMBB, Wearable, NTN, FWA

- Proposal 5:	The device type specific features cover both data-rate related and non-data-rate related features e.g., energy efficiency, mobility, coverage extension, and sensing.

Low-end IoT feature set and global mandatory feature set

The next question to be answered is whether a specific device type can be firstly identified which provides a template for the global mandatory feature set.

Low-end IoT is the lowest end device type in 6G on most of aspects, which is very suitable to provide the template for identifying the global mandatory feature set. And from our perspective, eMBB and low-end IoT are most important device types for the 1st release of 6G system. And each 6G MBB device are naturally a 6G low-end IoT device in “low-data rate” mode. For example, when a smartphone is in a “screen off” status or communicates with IoT devices, only the low-rate “background data” are received and transmitted, and only “IoT-like” traffic is conveyed for the smartphone. Therefore, the 6G low-end IoT provides a proper design template for 6G Basic feature set. Starting from the template, some features specially optimized for low-end IoT devices and not fit to other device types (e.g., extreme coverage enhancement for MTC sensors) can be cut off.

Therefore, a proper working approach is to firstly define the mandatory feature set for 6G low-end IoT, then use the low-end IoT feature set as a template to form the global mandatory feature set, where existing features unjustified for other candidate usage scenarios should be taken out, and necessary “outside” features could be added in.

Figure 2. Low-end IoT provides a good template for defining Basic feature set

- Proposal 6:	Use the low-end IoT feature set as a template/starting point for definition of the “global” mandatory feature set for 6GR.

      - Features specially optimized for low-end IoT (e.g., stronger coverage enhancement) does not have to be included in the “global” mandatory feature set.

Conclusion

In this contribution, the necessity and methodology of device-type based feature set/capability categorization are discussed. Based on 4G/5G experience, the following observation can be made:

Observation: the handling of device features/capabilities in 4G and 5G has serious limitations and cannot meet the requirement of performance and cost optimization for various application scenarios.

The following are proposed for further discussion:

- Proposal 1:	6G RAN should define device types according to usage scenarios for mandatory/optional UE capability categorization.

      - Taking the categorization similar as in Table 2 as a starting point.

- Proposal 2:	Each device type associates with a mandatory feature list which optimizes performance and cost for the target usage scenario.

- Proposal 3:	Define a lean “global” mandatory feature set list for all device types, where each feature in the list shall be well justified to be truly mandatory, and provides basic energy-efficient features for various device types.

- Proposal 4:	Define add-on device type specific feature lists (with mandatory/optional marked for each feature) on top of the “global” mandatory feature set for different device types, e.g., eMBB, Wearable, NTN, FWA

- Proposal 5:	The device type specific features cover both data-rate related and non-data-rate related features e.g., energy efficiency, mobility, coverage extension, and sensing.

- Proposal 6:	Use the low-end IoT feature set as a template/starting point for definition of the “global” mandatory feature set for 6GR.

      - Features specially optimized for low-end IoT (e.g., stronger coverage enhancement) does not have to be included in the “global” mandatory feature set.

References

- 3GPP TS36.306 V18.5.0 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities, 2025.6

- 3GPP TR38.913 V18.0.0, Study on Scenarios and Requirements for Next Generation Access Technologies, 2024-03

- 3GPP TS36.331 V18.6.0,

- RP-251881	New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- Chairman notes of RAN1 #122, End of meeting, Bengaluru, India, Aug 25th – 29th, 2025