3GPP TSG RAN Meeting #109				RP-251999

Beijing, China, September 15-18, 2025

Source:	OPPO

Title:	Discussion on 6G deployment scenarios

Agenda Item:	8.2.1.1

Document for:	Discussion

Introduction

In RAN#108, the 6G deployment scenarios were discussed, and the first TP was agreed in [1]. But some items were left TBD for further study. In this paper, our views on the remaining issues on 6G deployment scenarios are presented.

Discussions on 6G deployment scenarios

Carrier Frequency, Layout and ISD

In RAN#108, operators required to consider the carrier frequency combinations in real 5G deployment, which include more carrier frequencies than in TR 38.913. For example, a layout with 3 or 4 carrier frequencies (e.g., among 30GHz, 7GHz, 4GHz, 2GHz, 700MHz). We think that the operators’ requirements on reflecting the real deployment reality should be respected. Meanwhile the complexity of evaluation based on the deployment scenarios should also be considered. Our proposals for Carrier Frequency, Layout and ISD are provided in Table 1.

Table 1: Proposed carrier frequency, layout and ISD in 6G deployment scenarios

| Deployment scenario | Remaining issue from RAN#108 on Carrier Frequency, Layout and ISD | Proposals for Carrier Frequency, Layout and ISD |
| --- | --- | --- |
| Indoor hotspot (Section 4.1) | Layout: 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) | Use the same layout for other carrier frequencies, i.e., the same set of indoor sites are shared by TRxPs for different carrier frequencies. TP for Layout: 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| Dense urban (Section 4.2) | Carrier frequency: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] Layout: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD ISD: Micro: [100]m | TP for Carrier frequency: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] If 7GHz is used for independent networking, the layout should be Hex Grid. When it is used together with lower bands (e.g., 4GHz, 2GHz, 700MHz), it should be used for hotspot in micro layer. TP for Layout: Two layers for Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz: - Macro layer: Hex. Grid, 4GHz + 2GHz + 700MHz (FFS joint or separate BS scheduler for different frequencies) - Micro layer: Random drop, 7GHz ISD: No need to define ISD for micro cells because they are randomly dropped. Micro: [100]m |
| Rural (Section 4.3) | ISD: ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] | 5000m is for 700MHz. 7500m is too large even for 700MHz. TP for ISD: ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| Urban macro (Section 4.4) | Carrier frequency: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] Layout: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD ISD: Macro: 500m [TBD on micro layout] | TP for Carrier frequency: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] If 7GHz is used for independent networking, the layout should be Hex Grid. When it is used together with lower bands (e.g., 4GHz, 2GHz, 700MHz), it should be used for hotspot in micro layer. TP for Layout: Two layers for Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz: - Macro layer: Hex. Grid, 4GHz + 2GHz + 700MHz (FFS joint or separate BS scheduler for different frequencies) - Micro layer: Random drop, 7GHz Although lower band (e.g., 2GHz, 700MHz) can support larger ISD, the typical ISD in urban macro scenario is still 500m, in which the cells are capacity-limited. TP for ISD: Macro: 500m [TBD on micro layout] |
| Sub-Urban macro (Section 4.5) | Carrier frequency: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] Layout: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD | TP for Carrier frequency: [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] If 7GHz is used for independent networking, the layout should be Hex Grid. When it is used together with lower bands (e.g., 4GHz, 2GHz, 700MHz), it should be used for hotspot in micro layer. TP for Layout: Two layers for Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz: - Macro layer: Hex. Grid, 4GHz + 2GHz + 700MHz (FFS joint or separate BS scheduler for different frequencies) - Micro layer: Random drop, 7GHz |
| Urban coverage for massive connection (Section 4.8) | TBD | For this scenario, the urban macro layout and ISD can be reused, but only considering low-frequency band, e.g., 2GHz and 700MHz. TP for Carrier frequency: Around 2GHz+Around 700MHz TP for Layout: One layers for Around 2GHz+Around 700MHz: - Macro layer: Hex. Grid, 2GHz + 700MHz (FFS joint or separate BS scheduler for different frequencies) TP for ISD: Macro: 500m |

Proposal 1: For defining the carrier frequency, layout and ISD for 6G deployment scenarios, consider operators’ requirements on reflecting the deployment reality and the complexity of evaluation based on the deployment scenarios.

- Consider the proposals in Row 3 of Table 1 for Section 4.1 ~ 4.5 and 4.8 of TR38.914.

Number of UE antenna elements

Regarding the number of UE antenna elements defined in the 6G deployment scenarios, two principles should be considered:

Firstly, different device types should be considered for different deployment scenarios. At least 3 types of devices can be considered for 6G:

- Low-end IoT UE, e.g., MTC devices in macro coverage.

- Handheld MBB UE, e.g., smart phone, XR devices in macro/micro coverage.

- FWA UE, e.g., CPE in macro coverage.

Secondly, the real requirements from market should be considered. In 5G RAN1 specification, number of BS antennas are defined based on real market demands. For example, 32T×32R antenna ports were defined in Rel-15 to Rel-18 RAN1 specification. In the typical commercial 5G deployment, 32T×32R has been implemented with large scale. Then defining 128T×128R in Rel-19 is aligned with the market demand in near future. Thus, increase the number of antenna ports of 6G BS in 7GHz band to 256T×256R is a reasonable configuration (as shown in Table 2), which may lead to a typical number of BS antenna elements of 1024T×1024R. However, on UE side, a big gap between number of antenna elements in RAN1 specification and in typical commercial deployment can be observed. As shown in Table 2, 4T×8R antenna ports are defined in Rel-15 to Rel-17 RAN1 specification. And the number was increased to 8T×8R in Rel-18. However, the typical commercial deployment (e.g., high-end 5G smartphones) still stays on up to 2T×4R. Therefore, we suggest that the maximum UE antenna elements stays at 8T×8R. And the number of UE antenna elements in the deployment scenarios should follow the typical commercial deployment for each device type, e.g.,

- For low-end IoT UE,

  - 1T×1R.

- For handheld/wearable MBB UE,

  - 2T×4R will be the main stream.

  - For some high-end handheld UE 3T×6R is possible.

- For FWA UE,

  - 3T×6R might be more popular.

  - 4T×8R might be more high-end implementation.

Table 2: Maximum number of antenna ports in RAN1 specs and in typical commercial deployment

|  |  | Max. number of antenna ports defined in RAN1 specs | Max. number of antenna ports in typical commercial deployment |
| --- | --- | --- | --- |
| BS side | 5G | 32T×32R@R15-R18; 128T×128R@R19 | Up to 32T×32R |
|  | 6G R21 | 256T×256R (@7GHz) | 256T×256R (@7GHz) |
| UE side | 5G | 4T×8R@R15-R17; 8T×8R@R18 | Up to 2T×4R |
|  | 6G R21 | 8T×8R | For low-end IoT UE, - 1T×1R. For handheld/wearable MBB UE, - 2T×4R will be the main stream. - For some high-end handheld UE 3T×6R is possible. For FWA UE, - 3T×6R might be more popular. - 4T×8R might be more high-end implementation. |

Considering the typical device type in each deployment scenario, the corresponding number of UE antenna elements is proposed in Table 3 for FR1 and FR2.

Table 3: Proposed number of UE antenna elements in 6G deployment scenarios

| Deployment scenario | OPPO’s proposal for UE antenna elements |
| --- | --- |
| Indoor hotspot (Section 4.1) | Handheld/wearable MBB UE: 3T×6R. |
| Dense urban (Section 4.2) | Handheld/wearable MBB UE: 3T×6R. |
| Rural (Section 4.3) | Handheld/wearable MBB UE: 3T×6R. FWA UE: 4T×8R. |
| Urban macro (Section 4.4) | Handheld/wearable MBB UE: 3T×6R. |
| Sub-Urban macro (Section 4.5) | Handheld/wearable MBB UE: 3T×6R. |
| Urban coverage for massive connection (Section 4.8) | Low-end IoT UE: 1T×1R. |

Proposal 2: For defining the number of UE antenna elements in 6G deployment scenarios, consider the typical commercial deployment in market and typical device type in each deployment scenario.

- Consider the proposals in Table 3 for Section 4.1 ~ 4.5 and 4.8 of TR38.914 for FR1 and FR2.

User distribution and UE speed

Regarding the user distribution and UE speed defined in the 6G deployment scenarios, the 5G NR setting can be reused as baseline for the same deployment scenarios, e.g., 10 users per TRxP. For rural scenario, the option of user distribution reflecting more realistic commercial deployment can be considered. For the medium UE speed, it is suggested to consider a unified typical speed, e.g., 30km/h. 40km/h makes small impact to evaluation results.

Table 4: Proposed user distribution and UE speed in 6G deployment scenarios

| Deployment scenario | OPPO’s TP for UE antenna elements |
| --- | --- |
| Indoor hotspot (Section 4.1) | [10] users per TRxP NOTE4 |
| Dense urban (Section 4.2) | [10] users per TRxP with single-layer only |
| Rural (Section 4.3) | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| Urban macro (Section 4.4) | TBD20% outdoor vehicles (30km/h) and 80% indoor (3km/h) [10] users per TRxP |
| Sub-Urban macro (Section 4.5) | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 4030km/h, 80% Indoor in houses: 3km/h] [10] users per TRxP |

Proposal 3: For defining the user distribution and UE speed in 6G deployment scenarios, consider the typical commercial deployment in market in each deployment scenario.

- Consider the proposals in Table 4 for Section 4.1 ~ 4.5 of TR38.914.

Deployment scenario for 6G Sensing

6G sensing targets a framework of joint sensing and communication. Therefore, it can be a straight-forward logic that the deployment scenarios for 6G sensing do not need to go beyond the deployment scenarios for 6G communication. However, the equipment involving in a wireless sensing is generally a pair of sensing transmitter (STX) and sensing receiver (SRX), which are not necessarily a pair of base station and UE that are major aspects in consideration of the communication scenarios, such as inter-BS distance and UE distribution. In other words, while 6G sensing is assumed to be applicable within 6G communication scenarios, it should not preclude for 6G sensing scenario to follow different scenario parameters from the corresponding communication scenario.

One outstanding example is a sensing setup with both STX and SRX being UE’s, which decouples the sensing deployment from the cellular communication deployment requirements and relaxes the need of deploying a base station in case such BS deployment is not desirable, including but not limited to:

- Sensing in every senior apartment room or nursery room, where multiple such rooms are built side-by-side in a row which makes it infeasible or not cost-effective to have one indoor hotspot per room.

- Sensing in certain corner areas of indoor public facilities (e.g., corridors in hotel, cinema, gym and etc), which are reachable by NLOS propagation for communication, but maybe difficult to be reached by LOS sensing propagation.

- Mobile sensing functions/applications that are only based on portable devices such as traditional handsets and IoT units.

It should be noted that the UE-to-UE sensing does not necessarily require side-link specification as a prerequisite, given it only needs an opportunity of STX transmission and an opportunity of SRX reception on the same set of radio resources, which is already feasible in 5G UE-dedicated TDD. It is also noted that the UE-to-UE sensing channel models for various scenarios are already supported in existing TR 38.901.

Proposal 4: Add “indoor localized sensing scenario” in TR38.914 Section 4, as either an additional deployment scenario or a sub-scenario of “indoor hotspot”, where

- It targets to UE-UE sensing only.

- It assumes a system coexistence between BS-UE communication and UE-UE sensing, following the same radio assumptions as in existing “indoor hotspot” scenario for carrier frequency, UE RF parameters and radio resources (power, time-frequency resource, interference etc).

- In case of a new scenario, a new deployment map layout can be considered where no BS is located in the concerned indoor room but may be located outdoor with O2I propagation.

- In case of a sub-scenario under “indoor hotspot”, the same deployment map layout as in existing “indoor hotspot” applies.

- Note: It does not require sidelink as a prerequisite.

On UE Bandwidths

In TR 38.913 and the draft TR 38.914 [1], the deployment scenarios are defined with System bandwidth and Aggregated system bandwidth. UE bandwidths are not defined in the deployment scenarios. For most of deployment scenarios, the maximum UE bandwidth can be defined as same as system bandwidth. The exception may be the “Urban coverage for massive connection” deployment scenario in which only the low-end IoT UEs are considered.

6G network will support different bandwidths for different device types. The First is Low-end IoT UE, e.g., MTC devices.  The RF/BB capability is limited for that type of devices. The second is the higher capable, e.g., 6G smart phone for immersive broadband. The other type like XR devices & FWA UE would share similar bandwidth capability as smart phone, although more middle bandwidths may be introduced based on the further discussion.

In NR, FR1 RedCap, and eRedCap UEs support 20 MHz UE RF bandwidth. That is much higher than the LTE IoT like NB-IoT and eMTC. The NR backward compatibility actually result quit high cost for RedCap enhancement. This gives a lot restriction of Energy Efficiency and Lower Complexity. Taken this lesson into account, we consider the maximum required UE bandwidth supported by the lowest-end UE should be much smaller.

However, too small value will restrict performance of other channels. Thus, we prefer some middle range as [3-5] MHz, for sub-carrier spacing 15Hz. It can be further discussed if that requirement can be scalable according subcarrier spacing.

Therefore, in the deployment scenarios, two types maximum UE maximum bandwidths can be considered:

- For the deployment scenario only considering low-end IoT UE,

  - [3-5] MHz

- For the deployment scenario considering handheld/wearable MBB UE, and/or FWA UEs,

  - System bandwidth.

Considering the typical device type in each deployment scenario (as should in Table 3), the corresponding maximum UE bandwidths are proposed in Table 5.

Table 5: Maximum UE bandwidth for 6G deployment scenarios

| Deployment scenario | Maximum UE bandwidths |
| --- | --- |
| Indoor hotspot (Section 4.1) Dense urban (Section 4.2) Rural (Section 4.3) Urban macro (Section 4.4) Sub-Urban macro (Section 4.5) | System bandwidth. (with Handheld/wearable MBB UE and/or FWA UE) |
| Urban coverage for massive connection (Section 4.8) | [3-5] MHz (only with Low-end IoT UE) |

Proposal 5: For the 6G deployment scenarios, the maximum UE bandwidths list in Table5 can be considered.

Conclusion

It is suggested to consider the following proposals/TPs for TR 38.914 Section 4 Deployment scenarios:

Proposal 1: For defining the carrier frequency, layout and ISD for 6G deployment scenarios, consider operators’ requirements on reflecting the deployment reality and the complexity of evaluation based on the deployment scenarios.

- Consider the proposals in Row 3 of Table 1 for Section 4.1 ~ 4.5 and 4.8 of TR38.914.

Proposal 2: For defining the number of UE antenna elements in 6G deployment scenarios, consider the typical commercial deployment in market and typical device type in each deployment scenario.

- Consider the proposals in Table 3 for Section 4.1 ~ 4.5 and 4.8 of TR38.914.

Proposal 3: For defining the user distribution and UE speed in 6G deployment scenarios, consider the typical commercial deployment in market in each deployment scenario.

- Consider the proposals in Table 4 for Section 4.1 ~ 4.5 of TR38.914.

Proposal 4: Add “indoor localized sensing scenario” in TR38.914 Section 4, as either an additional deployment scenario or a sub-scenario of “indoor hotspot”, where

- It targets to UE-UE sensing only.

- It assumes a system coexistence between BS-UE communication and UE-UE sensing, following the same radio assumptions as in existing “indoor hotspot” scenario for carrier frequency, UE RF parameters and radio resources (power, time-frequency resource, interference etc).

- In case of a new scenario, a new deployment map layout can be considered where no BS is located in the concerned indoor room but may be located outdoor with O2I propagation.

- In case of a sub-scenario under “indoor hotspot”, the same deployment map layout as in existing “indoor hotspot” applies.

- Note: It does not require sidelink as a prerequisite.

Proposal 5: For the 6G deployment scenarios, the maximum UE bandwidths list in Table5 can be considered.

References

- RP-251873, TR 38.914 v0.1.0 for Study on 6G Scenarios and requirements, CMCC, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.