3GPP TSG RAN Meeting #109                                                       RP-252002

Beijing, China, September 15-18, 2025

Source:	vivo

Title:	Discussion on 6G key performance indicators

Agenda Item:	8.2.1.2

Document for:	Discussion and Decision

Introduction

In RAN#108, TR 38.914 V0.1.0 [1] was endorsed which captures the agreed definition and requirement values of key performance indicators. However, many items still are TBD. In this contribution, we provide the requirement values and high-level evaluation methodology. For energy efficiency and sensing, we provide separate definitions or additional performance metrics for ITU IMT-2030 and 3GPP internal usage.

Discussion

KPI definition, values and evaluation methodology

In this section, we provide our views on candidate KPIs, including the target values, the high-level evaluation methodology and applicable usage scenarios in Table 1. For the traditional KPIs and the composite requirements KPI, we suggest same definition and target values for ITU IMT2030 and 3GPP internal usage.

Table 1 View on KPIs

| Candidate KPIs |  | Usage scenarios | View on target value(s) | Enhancement times to IMT-2020 | Evaluation methodology |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | IC | DL: 80 Gbit/s UL: 40 Gbit/s | 4x | Analytical |
| User experienced data rate (Mbit/s) |  | IC | Dense Urban-IC: DL: 400~600 Mbit/s UL: TBD | DL: 4~6x UL: TBD | DL: Analytical in case of single layer. Simulation in case of multiple layers UL: Simulation |
| Spectral efficiency | Peak spectral efficiency (bit/s/Hz) | IC | DL: 60 Mbit/s UL: 30 Mbit/s | 2x | Analytical |
|  | Average spectral efficiency (bit/s/Hz/TRxP) | IC | TE; DL; UL. Indoor Hotspot-IC; 27; 16.875. Dense Urban-IC; 23.4; 13.5. Rural-IC; 9.9; 4. | DL: 3x UL: 2.5x | Simulation |
|  | 5th percentile user spectral efficiency (bit/s/Hz) | IC | TE; DL; UL. Indoor Hotspot-IC; 0.9; 0.525. Dense Urban-IC; 0.675; 0.375. Rural-IC; 0.36; 0.1125. | DL: 3x UL: 2.5x | Simulation |
| Area traffic capacity (Mbit/s/m²) |  | IC | Indoor Hotspot-IC: 40~60 Mbit/s/m2 | 4~6x | Analytical |
| Connection density (devices/km²) |  | MC | Same as agreement in RAN #108 106 |  | Simulation |
| Mobility (bit/s/Hz @ km/h) |  | IC | Same as agreement in RAN #108 |  | Simulation |
| Mobility interruption time (ms) |  | IC and HRLLC | TBD | TBD |  |
| Latency | User plane latency (ms) | IC and HRLLC | Same as agreement in RAN #108 – 4ms for IC – 1ms for HRLLC |  | Analytical |
|  | Control plane latency (ms) |  | Same as agreement in RAN #108 20ms |  | Analytical |
| Reliability |  | HRLLC | Same as agreement in RAN #108 1-10-5 |  | Simulation |
| Composite requirement |  | IC | Data rate: DL: 30Mbps UL: 10Mbps |  | Simulation |
|  |  |  | Latency: DL: 10ms UL: 30ms |  |  |
|  |  |  | Reliability: DL: 99% UL: 99% |  |  |
|  |  |  | Capacity: number of users per km2 Dense urban-IC: 1000 |  |  |
| Coverage |  |  |  |  |  |
| Positioning | Positioning accuracy | IC and ISAC | [10cm] |  | Simulation |
| Bandwidth |  |  | Same as agreement in RAN #108 400MHz |  | Inspection |
| Sensing-related capability | Detection/False alarm probability | ISAC | TBD |  | Simulation |
|  | Localization accuracy |  | TBD |  |  |
|  | Velocity accuracy |  | TBD |  |  |
|  | Micro-Doppler accuracy |  | TBD |  |  |
| AI-related accuracy |  | AIAC | Qualitative |  | Inspection |
| Energy efficiency | Network EE | IC | TBD |  | Analytical and simulation |
|  | Device EE (definition of ITU IMT-2030) |  | Test environment(s); Selected load case(s); The reference case; Relative energy saving (%). N. A; empty load; Fully loaded case; 90%. Dense urban-IC; <5%; 90%. |  |
| Security |  |  | Same as agreement in RAN #108 |  |  |
| Resilience |  |  | TBD |  |  |
| Interoperability |  |  | Same as agreement in RAN #108 |  |  |

Sensing

In ITU-R WP5D #49 meeting, the following Sensing-related capabilities are agreed:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

•	Sensing Resolution: TBD

It is observed that these requirements are the primary performance metrics concerning detection and tracking sensing use cases. Other use cases, such as motion detection and vital sign monitoring, and sensing-assisted communication should also be considered in 6G sensing study.

Regarding the use case of motion detection and vital sign monitoring, such as UAV identification, bird identification, human respiratory/sleep monitoring, sports monitoring, etc., the movement characterization of targets need to be identified and measured to support these sensing services, and the velocity accuracy which is more applicable to linear motion cannot characterize this type of motion. Therefore, we propose the following sensing-related metrics:

Micro-Doppler accuracy - It is defined as the difference between the estimated doppler and the actual doppler due to the sensing object motion.

Regarding the use case of sensing-assisted communication, the sensing results, such as the environmental information and UE state (e.g., position and height of UE) can be used to assist communication (e.g., beam tracking and mobility) and enhance communication performance, some traditional communication performance metrics can be considered, such as spectrum efficiency, user perceived throughput (UPT), HOF, RLF etc.

- For sensing, consider following key performance metrics:

  - Detection Probability and False Alarm Probability

  - Horizontal/Vertical Localization Accuracy

  - Velocity Accuracy

  - Micro-doppler accuracy

The performance requirements, which are highly related to the use cases and deployment scenarios, can refer to the discussion in our companion contribution [2].

Energy efficiency

The 3GPP internal definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

For unloaded case, the energy efficiency could be defined taking both the impact on paging/control signaling latency and energy consumption into account, two options can be considered.

- Option 1, define EE=VP, where v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] latency deterioration.

For loaded case, the energy efficiency could be defined by taking both the data rate and power consumption into account, two options can be considered.

- Option 1, define EE=VP  or EE=PV ,  where P is the power consumption, v is the user perceived throughput (UPT), which is widely used in 3GPP evaluations in all SI/WIs and reflects the user's experience data rate, and P=EC/T, where EC is the energy consumption during an evaluation time T, EC= Pon ×Ton + Psleep ×Tsleep+ Ptransition ×Ttransition .

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] data rate deterioration.

- Both unloaded case and loaded case should be considered for energy efficiency.

- The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

- For unloaded case, two options can be considered:

- - Option 1:  EE=VP  , where P is the power consumption, v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- -  Option 2:  EE is the relative power consumption with tolerable/less than [TBD] latency deterioration.

- For loaded case, two options can be considered:

- - Option 1:  EE=VP  or E=PV , where P is the power consumption, v is the user perceived throughput (UPT).

- - Option 2:  EE is the relative power consumption with tolerable/less than [TBD] data rate deterioration.

Conclusion

In this contribution, we provide our considerations on 6G KPIs and the proposals are as following:

- For sensing, consider following key performance metrics:

  - Detection Probability and False Alarm Probability

  - Horizontal/Vertical Localization Accuracy

  - Velocity Accuracy

  - Micro-doppler accuracy

- Both unloaded case and loaded case should be considered for energy efficiency.

- The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

- For unloaded case, two options can be considered:

- - Option 1:  EE=VP  , where P is the power consumption, v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- -  Option 2:  EE is the relative power consumption with tolerable/less than [TBD] latency deterioration.

- For loaded case, two options can be considered:

- - Option 1:  EE=VP  or E=PV , where P is the power consumption, v is the user perceived throughput (UPT).

- - Option 2:  EE is the relative power consumption with tolerable/less than [TBD] data rate deterioration.

- Based on discussion in section 2, it is proposed to agree on the TPs below.

TPs for 38.914

=====================Start of the changes======================

5.1.1	Peak data rate

[Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. This requirement is evaluated by analytical approach.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is 80 Gbit/s.

–	Uplink peak data rate is 40 Gbit/s.]

5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. This requirement is evaluated by analytical approach.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is 60 bit/s/Hz.

–	Uplink peak spectral efficiency is 30 bit/s/Hz.

These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink.

5.1.3	User experienced data rate

Editor note: name of this KPI may need to be changed

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For the evaluation of uplink user experienced data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account.

For the evaluation of downlink user experienced data rate, in case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency,  let W denote the channel bandwidth and SEuser denote the 5th percentile user  spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. The downlink user experienced data rate is evaluated by analytical approach in case of single layer and by simulation approach in case of multiple layers. The uplink user experienced data rate is evaluated by simulation approach.

The minimum requirements for the user experienced data rate are as follows in the Dense Urban – IC [test environment]:

–	Downlink user experienced data rate is 400~600 Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

5.1.4	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. This requirement is evaluated by simulation approach.

The minimum requirements for 5th percentile user spectral efficiency for various [test environments] are summarized in Table 5.1.4-1.

Table 5.1.4-1: 5th percentile user spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – IC | 0.9 | 0.525 |
| Dense Urban – IC | 0.675 | 0.375 |
| Rural – IC | 0.36 | 0.1125 |

5.1.5	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. This requirement is evaluated by simulation approach.

The minimum requirements for average spectral efficiency for various [test environments] are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – IC | 27 | 16.875 |
| Dense Urban – IC | 23.4 | 13.5 |
| Rural – IC | 9.9 | 4 |

5.1.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. This requirement is evaluated by analytical approach.

The minimum requirement for area traffic capacity in downlink is 40~60 Mbit/s/m² in the Indoor Hotspot – IC [test environment].

5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios. This requirement is evaluated by analytical approach.

The minimum requirements for user plane latency are:

–	4ms for IC

–	1ms for HRLLC

The above assumes unloaded conditions (i.e. a single user) for small IP packets (e.g. 0 byte payload + IP header), for both downlink and uplink.

5.1.8	Control plane latency

Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE).

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios. This requirement is evaluated by analytical approach.

The minimum requirement for control plane latency is 20 ms.

Note: Proponents are encouraged to consider lower control plane latency, e.g. 10 ms.

5.1.9	Connection density

Connection density is the number of connections fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario. This requirement is evaluated by simulation approach.

The minimum requirement for connection density is 106 connections per km2.

5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario. This requirement is evaluated by simulation approach.

The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Urban Macro-HRLLC [test environment], assuming small application data (e.g. 20 bytes application data + protocol overhead).

Proponents are encouraged to consider larger packet sizes, e.g. layer 2 PDU size of up to 100 bytes.

5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

–	Stationary: 0 km/h

–	Pedestrian: 0 km/h to 10 km/h

–	Vehicular: 10 km/h to 120 km/h

–	High speed vehicular: 120 km/h to 500 km/h

–	Airplane: 500 km/h to 1000/1200 km/h

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. This requirement is evaluated by simulation approach.

Table 5.1.11-1 defines the mobility classes that shall be supported in the respective [test environments].

Table 5.1.11-1: Mobility classes

|  | [Test environments] for IC |  |  |
| --- | --- | --- | --- |
|  | Indoor Hotspot – IC | Dense Urban – IC | Rural – IC |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 5.1.11-2. This assumes the user is moving at the maximum speed in that mobility class in each of the [test environments].

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| [Test environment] | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC | 2.25 | 10 |
| Dense Urban – IC | 1.68 | 30 |
| Rural – IC | [0.8~1.2] | 120 |
|  | [0.45~0.675] | 500 |

These values were defined assuming an antenna configuration as described in Report ITU-R M.[IMT-2030.EVAL].

5.1.12	Mobility interruption time

[Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for mobility interruption time is TBD.

5.1.13	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is at least 400 MHz. This requirement is evaluated by inspection approach.

5.1.14	Positioning

Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy describes the closeness of the measured position of the device to its true position value. The accuracy can describe the accuracy either of an absolute position or of a relative position. It can be further broken into a horizontal position accuracy – referring to the position error in a 2D reference or horizontal plane, and into a vertical position accuracy – referring to the position error on the vertical axis or altitude.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Integrated Sensing and Communication usage scenarios. This requirement is evaluated by simulation approach.

The preliminary requirements could be [10cm].

5.1.15	Energy efficiency

5.1.15.1	Energy efficiency for ITU IMT-2030

Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device power saving in relation to the traffic characteristics.

The requirement is defined as the relative energy savings (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. For unloaded case, this requirement is evaluated by analytical approach, for loaded case, this requirement is evaluated by simulation approach.

Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

The minimum requirements for Energy efficiency are summarized in Tables below.

Table: The minimum requirements for Network Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy saving (%) |
| --- | --- | --- | --- |
| N. A | Empty load | Fully loaded case* | TBD |
| Dense urban-IC | TBD |  | TBD |
| Rural-IC | TBD |  | TBD |

Table: The minimum requirements for Device Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy saving (%) |
| --- | --- | --- | --- |
| N. A | Empty load | Fully loaded case | [90%] |
| Dense urban-IC | [<5%] NOTE |  | [90%] |

NOTE: assume one active UE in one TRxP with FTP3 traffic (such as 0.5MB packet size, 200ms mean inter-arrival time)

5.1.15.2	Energy efficiency for 3GPP internal

Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device power saving in relation to the traffic characteristics.

For unloaded case, the energy efficiency could be defined taking both the impact on paging/control signaling latency and energy consumption into account, two options can be considered.

- Option 1, define EE=VP, where v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] latency deterioration.

For loaded case, the energy efficiency could be defined by taking both the data rate and power consumption into account, two options can be considered.

- Option 1, define EE=VP  or EE=PV ,  where P is the power consumption, v is the user perceived throughput (UPT), which is widely used in 3GPP evaluations in all SI/WIs and reflects the user's experience data rate, and P=EC/T, where EC is the energy consumption during an evaluation time T, EC= Pon ×Ton + Psleep ×Tsleep+ Ptransition ×Ttransition .

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] data rate deterioration.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. This requirement is evaluated by analytical and simulation approach.

5.1.16	Composite requirement

Editor note: KPI related to data rate, latency, reliability, capacity. Name of this KPI may need to be changed

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per km2.

This requirement is defined as the number of users per km2 of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. This requirement is evaluated by simulation approach.

The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1.

Table 5.1.16-1: Composite requirement

| Test environment | Data rate (Mbits/s) | Latency (ms) | Packet success probability | Number of users per km2 |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | DL: 30 UL: 10 | DL: 10 UL: 30 | 99% | 1000 |

5.1.17	Sensing

5.1.17.1	Sensing for ITU IMT-2030

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario. This requirement is evaluated by simulation approach.

5.1.17.2	Sensing for 3GPP internal

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

•	Micro-Doppler accuracy: It is defined as the difference between the estimated doppler and the actual doppler due to the sensing object motion. The required value of micro-doppler accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the micro-doppler estimation error.

This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario. This requirement is evaluated by simulation approach.

5.1.18	AI

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These capabilities include:

-	Data collection

-	Data/Model exchange

-	AI model training

-	AI model inference

-	AI model monitoring

-	AI computing resource management

This requirement is defined for the purpose of evaluation in the Artificial Intelligence and Communication usage scenario. This requirement is evaluated by inspection approach.

======================End of the changes======================

Reference

[1]    RP-251873-3GPP TR 38.914 V0.1.0

[2]	 RP-252006, “Discussion on requirements for 6G ISAC”, vivo.

Average spectral efficiency corresponds to “spectrum efficiency” in Recommendation ITU-R M.2160.