3GPP TSG RAN meeting #109		      RP-252003

Beijing, China, 15th Sep – 18th Sep 2025

Source:	vivo

Title:	Discussion on 6G requirements for data services

Agenda Item:	8.2.1.3

Document for:	Discussion and Decision

- Introduction

The SID on “Study on 6G scenarios and requirements” has been approved at RANP#107 meeting [1], which includes the following objective regarding RAN architecture.

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

In the corresponding TR 38.914 [2], the “Requirements for architecture and migration” is captured in section 5.2. In this contribution, we will discuss 6G radio access network (RAN) architecture requirements for data services driven by new service data.

- Data requirements of new services

  - Sensing data requirements

Twenty sensing use cases (UCs) are captured in the SA1 requirements TR22. 870 [3], such as low-altitude unmanned aerial vehicle (UAV) supervision, environment object reconstruction, sensing assisted communication in industrial park, etc. There are six potential sensing modes as shown in Figure 1.

Figure 1: potential sensing modes

In TS 22.137 [4], the basic procedure of sensing has been described, including capability reporting, data collection, data transmission, data processing, and capability exposure. Therefore, the sensing data requirements can be listed as follows.

- As shown in Figure 1, in TRP monostatic, TRP-TRP bistatic, or UE-TRP bistatic sensing mode, base station (BS) acts as a sensing measurement node. In TRP-UE bistatic, UE monostatic, or UE-UE bistatic sensing mode, the user equipment (UE) acts as a sensing measurement node. When the UE or the BS acts as a sensing measurement node, the UE or the BS generates sensing data. Thus, the RAN architecture shall support the generation of sensing data to meet the sensing requirements.

- Based on sensing modes and sensing use cases, the collected sensing data needs to be transmitted among different entities within the 3GPP system.

  - In case of TRP monostatic sensing mode and a low-altitude UAV supervision UC, sensing data is generated by BSs and then transmitted from BSs to core network (CN) nodes. Thus, the RAN architecture shall support the generation and transport of sensing data to meet the sensing requirements.

  - In case of TRP-UE bistatic sensing mode and environment object reconstruction UC, sensing data is generated by UEs and then transmitted from UEs to CN nodes. In the latter case, RAN nodes transparently transport the sensing data. Thus, the RAN architecture shall support the transport of sensing data to meet the sensing requirements.

  - In case of TRP-UE bistatic sensing mode and sensing assisted communication in industrial park UC, sensing data is transmitted from UEs to BSs. These sensing results could be used by 6G system to enhance the communication performance, e.g., to optimize the beam management procedure and other procedures. Thus, the RAN architecture shall support the visibility of sensing data.

  - For multi-static sensing, there are more than one sensing measurement node. For example, multi-static sensing consists of BS monostatic and BS-UE bistatic. The sensing data collected by the UE (such as delay Doppler) needs to be integrated with the sensing data collected by the BS to form point cloud data. Then the point cloud data is transmitted to the target node (e.g., CN node). Thus, the RAN shall support the on-path processing of the received sensing data and forwarding it to the data consumer.

For the evaluation of sensing data volume, the sensing data transmitted by BSs or UEs requires further study and discussion. From our understanding, the potential sensing parameters can be time/frequency-domain samples of original received signals from each antenna port, channel impulse response obtained by channel estimation results, delay, Doppler, angle, signal strength, etc. In addition to the measured parameters, the volume of sensing data also depends on the UC. Some evaluations in [3] and [5], the volume of sensing data can reach the order of magnitude of 10 Mbps ~ Gbps.

  - AI/ML data requirements

In the Rel-19 AI/ML for air interface item, the UE side data collection scheme was discussed, including the following options.

1a. UE collects and directly transfers training data to the data collection entity outside the mobile network operator (MNO) (e.g., Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

Among them, Option 3 requires the BS to first acquire the data and then transfer it to the OAM, with the requirement of full controllability and visibility:

- Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of service level agreement (SLA). This includes initiating, terminating, and fully managing data transfer.

- Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA.

We think the requirement is also valid for 6G, i.e., the 6G RAN architecture shall support the transport and visibility of training data for AI/ML service. In addition, to enable two-sided models, the sharing of dataset/model parameters from NW-side to UE or UE-side training entity was discussed for the use case of AI/ML CSI compression. For two-sided models, the BS shall support the generation of dataset and then provide it to the training entity.

Regarding the data volume of dataset/model parameters sharing, taking CSI compression as an example, the typical data volume for different options is shown in Table 1.

Table 1: Data size for dataset/parameter sharing

| Option | Typical data size |
| --- | --- |
| 1. Dataset sharing consisting of {(Target CSI, CSI feedback)} | 225 MB |
| 2. Encoder parameter sharing | 11.6 MB |
| 3. Encoder parameter sharing + dataset sharing consisting of {target CSI} | 161.6 MB |

  - Summary of new service data requirements

Based on the above analysis, the RAN architecture requirements for data services driven by new service data are shown in Figure 2.

Figure 2: the requirements of 6G RAN architecture driven by new service data

We have the following proposal.

Proposal 1: The design of the 6G RAN architecture shall support the generation, transport and/or visibility of new service data (e.g., sensing data, AI/ML data) efficiently to fulfil new service requirements.

- Generation: RAN nodes can directly generate data.

- Transport: RAN nodes can transparently transport data to the target node. Or RAN nodes can on-path process the received data packets and then forward to the data consumer.

- Visibility: Specific data content can be visible to RAN nodes. RAN nodes may use data for various purpose, e.g., performing data integration or to enhance the communication performance.

- TP for 38.914

----------------------------------------------------start of TP--------------------------------------------------------------

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The following requirements shall be considered:

–	The design of the 6G RAN architecture shall support the generation, transport, and/or visibility of new service data (e.g., AIML data, sensing data) efficiently to fulfil new service requirements.

----------------------------------------------------end of TP--------------------------------------------------------------

- Conclusion

In this paper, we discuss 6G RAN architecture requirements for data services driven by new service data, such as sensing and AI/ML. We have the following proposals:

Proposal 1: The design of the 6G RAN architecture shall support the generation, transport and/or visibility of new service data (e.g., sensing data, AI/ML data) efficiently to fulfil new service requirements.

- Generation: RAN nodes can directly generate data.

- Transport: RAN nodes can transparently transport data to the target node. Or RAN nodes can on-path process the received data packets and then forward to the target node.

- Visibility: Specific data content can be visible to RAN nodes. RAN nodes may use data for various purpose, e.g., performing data integration or to enhance the communication performance.

Proposal 2: Capture the above TP into TR 38.914.

- Reference

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

- RP-251873 draft TR38.914

- 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements.

- 3GPP TS 22.137, Service requirements for Integrated Sensing and Communication.

- R1-2506093 Discussion on Measurement and Evaluation for ISAC.