3GPP TSG RAN Meeting #109	RP-252021

Beijing, China, September 15-18, 2025

Source:	ZTE Corporation, Sanechips

Title:	General views on 6G deployment scenarios

Agenda Item:	8.2.1.1

Document for:   Discussion and Decision

- Introduction

In RAN#107, the new 6G RAN SID was updated[1], where new services and the corresponding scenarios and requirements should be a crucial part of study. Then, in RAN#108, progress on details of 6G deployment scenarios was made in [2], and then the left-over issues were highlighted accordingly. So, in this contribution, we elaborate our general views on 6G deployment scenarios, especially based on the analysis from coverage and energy efficiency perspective.

- Principles of deployment scenario

As shown in the SID, the goal of 6G includes the following two bullets:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

As the frequency goes up, e.g. from 3.5GHz to 7 GHz, both propagation loss and penetration loss increase significantly. It results in an estimated 10.87~14.9 dB coverage gap between 3.5GHz and 7 GHz. Compensating for this gap is crucial to ensure successful 6G deployment. In order to enhance overall cell-edge performance, and meanwhile to ensure energy efficiency and energy saving for both network and UE side, the following directions with impacts on deployment scenario and requirements should be considered.

- Spatial domain, such as increasing the number of antennas/RF chains at the BS and/or UE, and introducing coherent joint transmission multi-TRP (CJT-mTRP) and developing a new architecture of DL-TRP and UL-TRP decoupling, i.e., asymmetric DL sTRP/UL mTRP.

- Multi-carrier operation, allowing UEs to flexibly operate across both 7 GHz and lower frequency bands with low UE complexity. Especially, for UL, by configuring a UE with both low frequency and high frequency carriers, it can solve the coverage issue while keep good system capacity.

2.1 Antenna modeling on BS and UE

6GR is expected to reuse the existing 5G mid-band (~3.5GHz) site grid for 6G deployments at approximately 7GHz, targeting coverage comparable to 5G mid-band. For example, compared to 3.5GHz, 7GHz experiences an additional 6.02dB of path loss and 4.85dB of penetration loss (under high-loss model). Adding the 4.0dB extra loss measured in the field yields a total compensation requirement of about 14.9dB.

- Ultra-massive MIMO can be considered a key technology to ensure sufficient link budget, and BS equipped with 1536 or 2048 antenna elements should be evaluated. Furthermore, the uniform planar array denoted as (M, N, P, Mg, Ng; Mp, Np), along with the radiation patterns defined in NR, can be reused in 6G discussions. Figure 2-1-1 illustrates an example of a candidate antenna array at 7GHz.

Figure 2-1-1 Typical antenna array for 3.5GHz and 7GHz

In Rel-19, the new UE antenna models for handheld UTs and CPEs are introduced for emulating actual UE device types. In the new handheld UT models, the candidate antenna positions better reflect actual product dimensions, with significant variations in orientations. It should be noticed that, the new model has non-negligible impacts on the evaluation of transmission schemes:

- Regarding DL/UL transmission, in addition to single polarization per position, each antenna element has own boresight of directional RF antenna pattern, and then distance among neighboring antenna ports is larger than the half-wavelength. It means that the correlation of antenna port(s) is much lower than normal ULA/UPA arrays assumed in 5G-NR.

- Besides, for UL transmission, the potential antenna imbalance among antenna port(s), e.g., 0~6 dB or even more, may severely affect the estimation accuracy of different UL antenna ports, which has certain impacts on uplink CSI acquisition.

Figure 2-1-2 Illustration of new UE types: handheld UT (left) and CPE (right)

For CPEs, only antenna topology is specified in TR 38.901, while other critical parameters such as operating frequency are missing. For example, the current CPE antenna element spacing is 10cm, which corresponds to exactly half wavelength when assuming an operating frequency of 1.5GHz. However, the element spacing is much larger than half wavelength for 15GHz, leading to channel correlation reduction. Therefore, we propose that the legacy 8-TX/RX UE antenna model discussed in NR (e.g., R18-MIMO UL 8-Tx) can be used to replace CPE, i.e., (M, N, P, Mg, Ng; Mp, Np) = (2, 2, 2, 1, 1; 2, 2) with omnidirectional antenna elements and (dH, dV) = (0.5, 0.5) λ.

Proposal 1-1: To ensure overall coverage, focus on cell-edge performance and UL coverage (especially for co-site deployment of around 7GHz), large antenna array with more than 1024 BS antenna elements (e.g. up to 1536 or 2048 antenna elements) and 8 or more Tx and Rx UE antenna elements (e.g., up to 16) should be considered.

2.2 Multi-TRP operation including asymmetric DL sTRP/UL mTRP

To address the issue of coverage and/or higher downlink spectrum efficiency, multi-TRP operation (coherent joint transmission, CJT, and non-coherent joint transmission, NCJT) and new architecture of DL-TRP and UL-TRP decoupling have an increased interest in the industry, partly due to new available frequency bands. Then, in this section, we elaborate the 6G deployment scenarios with multi-TRP operation schemes.

- Regarding FR1/U6GHz, coherent joint transmission (CJT) can be used to improve coverage and average throughput in commercial deployments with high-performance backhaul and synchronization.

- Then, regarding FR2, with low NW implementation complexity, NCJT can be considered as a starting point for FR2 deployment to accommodate the challenge of unpredicted blockage.

The deployment scenarios of multi-TRP operation (CJT/NCJT) can be divided into the following three categories: a) intra-site, b) inter-site, and c) intra-&inter-site as shown in Figure 2-2-1.

Figure 2-2-1 Deployment scenarios for CJT: (a) intra-site, (b) inter-site, (c) intra-&inter-site

Proposal 1-2-1: To improve transmission performance, especially for cell-edge performance, in addition to legacy single-TRP operation, multi-TRP operation should be considered for 6G deployment: CJT (targeting for FR1/around-7GHz) and NCJT transmission (targeting FR2/around 30GHz).

Current transmission schemes assume that UL and DL services are provided by the same physical serving site. However, in heterogeneous networks, while considering that there may be different optimal serving TRPs for DL and UL and potentially NW-DL energy saving, DL and UL site decoupling can distinctly improve UL transmission performance (especially for cell-edge UE) and has been specified in Rel-19 MIMO, as shown in Figure 2-2-2.

- Case #1: On the single-layer deployment scenarios, e.g., indoor hotspot, dense urban, rural, urban macro, regarding TRP operation, a mixed mode of {DL+UL TRP, UL-only TRP} should be supported.

- Case #2: On the two-layer deployment scenarios, e.g., dense urban (two-layer), micro layer can further consider the candidate deployment scenario of UL-only TRP, in addition to DL+UL-TRP.

Figure 2-2-2 Asymmetric DL single TRP / UL multi-TRP scenarios

Proposal 1-2-2: To reduce deployment cost, ensure UL coverage and save NW energy cost, the new architecture of DL-TRP and UL-TRP decoupling, i.e., asymmetric DL sTRP/ UL mTRP

- Case #1: On the single-layer deployment scenarios, a mixed mode of {DL+UL TRP, UL-only TRP} should be supported.

- Case #2: On the two-layer deployment scenarios, e.g., dense urban (two-layer), micro layer can further consider the candidate deployment scenario of UL-only TRP, in addition to a mixed mode of {DL+UL TRP, UL-only TRP} for macro/micro.

2.3 Multi-carrier based deployment scenario

In some scenarios, compensating for the significant coverage gap between 3.5 GHz and 7 GHz via single-carrier operation can be highly challenging, especially for UL because of limited UE transmission power. Moreover, some single-carrier-based enhancements may remain optional, meaning 6G deployments in Day 1 cannot rely on these non-mandatory features. For operators with access to multiple spectrum bands, a practical solution is to leverage flexible spectrum utilization—combining high-frequency bands for capacity for cell-center UEs and low-frequency bands for coverage for cell-edge UEs.

In Figure 2-3-1, it presents a comparison of ‘UE distance -UL throughput’ among different frequency bands. It can be observed that the UE at lower frequency band typically have better UL throughput when the UE distance to base station is larger than 100 meters.

Figure 2-3-1 Comparison of ‘UE distance - UL throughput’ in different frequency bands

In 5G, CA is an optional feature requiring a UE to simultaneously receive and transmit among the multiple aggregated carriers. This leads to the situation that CA is not widely commercialized in early 5G deployment. In 6G, more flexible spectrum utilization meanwhile with low UE complexity should be considered, e.g., carrier-switch based operation. Therefore, multi-carrier based deployment scenario should be supported for 6G with taking flexible spectrum utilization and UE complexity into account.

Proposal 1-3: To achieve comparable UL coverage and better capacity in 6G deployment scenarios, especially around 7GHz, support multi-carrier based deployment scenario for 6G with taking flexible spectrum utilization and UE complexity into account.

- Discussion on 6G deployment scenarios

In RAN#108, twelve 6GR deployment scenario(s) have been captured in TR 38.914: indoor hotspot, dense urban, urban macro, rural, sub-urban macro, high speed, extreme long-distance coverage in low density area, urban coverage for massive connection, air-to-ground scenario, non-terrestrial network, [urban grid] and [highway scenario]. Regarding two pending deployment scenario, we are open to consider high-way scenario, but fail to identify the necessity of introducing urban grid while considering that that deployment scenario is much relevant to sidelink service which is not considered in 6G Day-1. Then, in this contribution, we elaborate our views on the 6G deployment scenarios for each deployment scenarios.

Per overall 6G vision on network performance (e.g., coverage and cell-edge performance) along with the overall energy consumption, another critical scenario of ‘multi-layer heterogeneous network with assisting node’ is identified and then the corresponding evaluation assumption is elaborated accordingly.

Then, for ISAC, we propose to introduce two more deployment scenarios, i.e., indoor factory and nearshore water, for sensing humans or automated guided vehicles (AGVs) inside a factory and ships traveling on nearshore waters or rivers, respectively.

3.1 Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

In our views, modification on its attributes is listed in Table 3.1 in red.

Table 3.1 Modification on attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: [1] GHz (DL+UL) |
| Layout | Single layer: - Indoor floor (Open office) Note: Co-site & single-layer deployment for Around 4GHz+Around 7GHz is considered. |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | Around 4GHz: Up to 256 Tx and Rx antenna elements Around 7GHz: Up to 1024 Tx and Rx antenna elements Around 30GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 4GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| Multi-TRP operation | Around 4GHz and 7GHz: single TRP, or coherent joint transmission multi-TRP Around 30GHz: single TRP, or non-coherent joint transmission multi-TRP Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Proposal 2-1: Regarding indoor hotspot deployment scenarios for 6GR, the modification on attributes for indoor hotspot listed in Table 3.1 is supported.

3.2 Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

In our views, modification on its attributes is listed in Table 3.2 in red.

- Note: From 6GR evaluation on deployment scenario(s), up to two-layer deployment is sufficient. Then, the mixed frequency case of ‘Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz’ is NOT needed.

- Regarding two-layer deployment, for random dropping, the minimum distance between micro TRP centres is at least 57.9m (referring to Table A 2.1-9 In TR 38.802).

Table 3.2 Modification on attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz/7GHz + Around 30GHz Around 4 GHz/7GHz + Around 4GHz/7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: [1] GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz/7GHz + Around 4GHz/7GHz/30 GHz: - Macro layer: Hex. Grid, Around 4GHz/7GHz - Micro layer: Random/Fixed drop, Around 4GHz/7GHz/30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD Micro-layer: (a) Random drop, (b) Fixed drop |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100] At least 57.9 m between micro nodes All micro TRxPs are all outdoor |
| BS antenna elements | Around 4GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements Around 30GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements | Around 4GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| Multi-TRP operation | Around 700MHz, 2GHz, 4GHz and 7GHz: single TRP, or coherent joint transmission multi-TRP Around 30GHz: single TRP, or non-coherent joint transmission multi-TRP Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Sensing target | Sensing target type: Human, or UAV Distribution: - 100% outdoor - Horizontal for human or UAV: uniformly distributed within a sensing area (e.g., center cell) - Vertical for UAV: Uniformly distributed between 25m and 300m Mobility: - Human: 3km/h - UAV: horizontal - uniform distribution between 0km/h and 180km/h; vertical 0km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Proposal 2-2: Regarding dense urban deployment scenarios for 6GR, the modification on attributes for dense urban listed in Table 3.2 is supported.

3.3 Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

In our views, the modification on its attributes is listed in Table 3.3 in red.

- Regarding ISD 3: 7500m assuming 700MHz, in our views, that is not a typical deployment for now. Then, the ISD 2 of 5000m is sufficient.

- Regarding user distribution and UE speed, the typical case of 50% outdoor vehicles (120km/h), 50% indoor (3 km/h) is recommended, while considering the normal cases but also simplifying the subsequent evaluation.

Table 3.3 Modification on attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7GHz: Up to [400] MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Note: Co-site & single-layer deployment for Around 700MHz+Around 2GHz/7GHz is considered. |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700MHz: Up to 64 Tx and Rx antenna elements Around 4GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements Around 30GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements | Around 700MHz: Up to 4 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| Multi-TRP operation | Around 700MHz, 2GHz, 4GHz and 7GHz: single TRP, or coherent joint transmission multi-TRP Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| Sensing target | Sensing target type: Human or UAV Distribution: - 100% outdoor - Horizontal for human or UAV: uniformly distributed within a sensing area (e.g., center cell) - Vertical for UAV: Uniformly distributed between 25m and 300m Mobility: - Human: 3km/h - UAV: horizontal - uniform distribution between 0km/h and 180km/h; vertical 0km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Proposal 2-3: Regarding rural deployment scenarios for 6GR, the modification on attributes for rural listed in Table 3.3 is supported.

3.4 Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

In our views, the modification on its attributes is listed in Table 3.4 in red.

- Note: From 6GR evaluation on deployment scenario(s), up to two-layer deployment is sufficient. Then, the mixed frequency case of ‘Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz’ is NOT needed.

- Regarding two-layer deployment, for random dropping, while considering that the ISD for Macro is extended to 500m from 200m over dense-urban, the minimum distance between micro TRP centres can be extended to at least 144.8m (referring to Table A 2.1-9 In TR 38.802).

Table 3.4 Modification on attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz/7GHz + Around 30GHz Around 4 GHz/7GHz + Around 4GHz/7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: [1] GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz/7GHz + Around 4GHz/7GHz/30 GHz: - Macro layer: Hex. Grid, Around 4GHz/7GHz - Micro layer: Random/Fixed drop, Around 4GHz/7GHz/30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD Micro-layer: (a) Random drop, (b) Fixed drop |
| ISD | Macro: 500m Micro: 3 micro TRxPs per macro TRxP Micro: At least 144.8 m All micro TRxPs are all outdoor |
| BS antenna elements | Around 700MHz: Up to 64 Tx and Rx antenna elements Around 4GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements Around 30GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements | Around 700MHz: Up to 4 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | [10] users per TRxP Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP 10 users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Sensing target | Sensing target type: Human, or UAV Distribution: - 100% outdoor - Horizontal for human or UAV: uniformly distributed within a sensing area (e.g., center cell) - Vertical for UAV: Uniformly distributed between 25m and 300m Mobility: - Human: 3km/h UAV: horizontal - uniform distribution between 0km/h and 180km/h; vertical 0km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Proposal 2-4: Regarding urban macro deployment scenarios for 6GR, the modification on attributes for urban macro listed in Table 3.4 is supported.

3.5 Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. This scenario will be noise-limited and/or interference-limited, using macro TRxPs. Then, regarding the pending part on the key characteristics, the original suggestions: ‘[The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.]’ is correct in our views.

Proposal 2-5-1: Regarding sub-urban macro, the following description on the key characteristics of this scenario is recommended:

- The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.

In our views, the modification on its attributes is listed in Table 3.5 in red.

- Note: From 6GR evaluation on deployment scenario(s), the mixed frequency case of ‘Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz’ is NOT needed. Then, one-layer deployment assumption is sufficient herein.

- Then, regarding user distribution and UE speed, we still prefer to use the typical assumption: 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h.

Table 3.5 Modification on attributes for sub-urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| System bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) |
| Layout | Single layer - Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD Note: Co-site & single-layer deployment for Around 4GHz+Around 7GHz and Around 2GHz+ Around 700 MHz is considered. |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| TRP antenna elements | Around 700MHz: Up to 64 Tx and Rx antenna elements Around 4GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements Around 30GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements | Around 700MHz: Up to 4 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| Multi-TRP operation | Around 700MHz, 4GHz and 7GHz: single TRP, or coherent joint transmission multi-TRP Around 30GHz: single TRP, or non-coherent joint transmission multi-TRP Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| User distribution and UE speed | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h] 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Proposal 2-5-2: Regarding sub-urban macro deployment scenarios for 6GR, the modification on attributes for sub-urban macro listed in Table 3.5 is supported.

3.6 High speed

In RAN#108, whether/how to handle high-speed scenario, e.g., merging high speed and high way together, is pending. In our views, the deployment scenario on high speed is focus on the high-speed train scenario as described in TS 38.913. Then, we still recommend to have separate scenarios for high-speed (train) and high-way. Regarding the description on this deployment scenario, we may consider that in TR 38.913 as a starting point.

Proposal 2-6-1: Regarding high-speed, the following description on this scenario is recommended (from TR 38.913):

- The high speed deployment scenario focuses on continuous coverage along track in high speed trains. The key characteristics of this scenario are consistent passenger user experience and critical train communication reliability with very high mobility. In this deployment scenario, dedicated linear deployment along railway line and the deployments including SFN scenarios captured in Section 6.2 of 3GPP TR 36.878 [6] are considered, and passenger UEs are located in train carriages. For the passenger UEs, if the antenna of relay node for eNB-to-Relay is located at top of one carriage of the train, the antenna of relay node for Relay-to-UE could be distributed to all carriages.

Then, in our views, the recommendation on its attributes is listed in Table 3.6. The update over the corresponding legacy table in TR 38.913 (i.e., Table 6.1.5-1) is marked in red.

Table 3.6 Recommendation on attributes for high speed

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro only: Around 4GHz or Around 7GHz Macro relay nodes: 1) For BS to relay: Around 4 GHz or Around 7GHz - For relay to UE: Around 30 GHz or Around 7GHz or Around 70 GH or Around 4 GHz 2) For BS to relay: Around 30 GHz - For relay to UE: Around 30 GHz or Around 7GHz or Around 70 GHz or Around 4 GHz |
| Aggregated system bandwidth | Around 4GHz: Up to 200 MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL) Around 30GHz or Around 70GHz: Up to 1GHz (DL+UL) |
| Layout | Macro only: - Around 4GHz or Around 7GHz: Dedicated linear deployment along the railway line as following. RRH site to railway track distance: 100m Macro + relay nodes: - Around 4GHz or Around 7GHz: Dedicated linear deployment along the railway line as following. RRH site to railway track distance: 100m - Around 30GHz: Dedicated linear deployment along the railway line as in following. RRH site to railway track distance: 5m. |
| ISD | - Around 4GHz or Around 7GHz: ISD 1732m between RRH sites, two TRxPs per RRH site. - Around 30GHz: 1732m between BBU sites, 3 RRH sites connected to 1 BBU, one TRxP per RRH site, inter RRH site distance (580m, 580m, 572m). - Small cell within carriages: ISD = 25m. |
| BS antenna elements | Around 4GHz: Up to 256512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements Around 30GHz: Up to 2562048 Tx and Rx antenna elements |
| UE antenna elements | Relay Tx: Up to 256 antenna elements Relay Rx: Up to 256 antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements |
| Multi-TRP operation | Around 4GHz and 7GHz: single TRP, or coherent joint transmission multi-TRP Around 30GHz: single TRP, or non-coherent joint transmission multi-TRP Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| User distribution and UE speed | 100% of users in train For non-full buffer, 300 UEs per macro cell (assuming 1000 passengers per high-speed train and at least 10% activity ratio) Maximum mobility speed: 500km/h |
| Service profile | Alt 1: Full buffer Alt 2: FTP model 1/2/3 with packet size 0.5 Mbytes, 0.1 Mbytes (other value is not precluded) Other traffic models are not precluded, e.g., for critical train communications. |

Proposal 2-6-2: Regarding the high speed deployment scenarios for 6GR, the recommended attributes listed in Table 3.6 is supported.

3.7 Extreme long distance coverage in low density area

For the scenario of extreme long distance coverage in low density area, it is applicable for both sensing and communication. For example, macro base station can be used for detecting the presence of ships on the sea or river. The coverage distance be be reached to more than 10kms even at 7GHz or 30GHz.

Proposal 2-7: Regarding the extreme long distance coverage deployment scenario for 6GR, the corresponding modification on the attributes listed in Table 3.7 is supported.

Here is our text proposal for the existing TR 38.914.

-----------------------------------------------------------Text proposal --------------------------------------------------------------------

The extreme long range deployment scenario is defined to allow for the provision of services for very large areas with low density of users or sensing targets whether they are humans, ships and machines (e.g. wilderness, river or water, remote village or island areas, etc). The key characteristics of this scenario are Macro cells with very large area coverage supporting [at least] basic data speeds and voice services, with low to moderate user throughput and low user or sensing target density.

Table 3.7 Attributes for extreme long distance coverage

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 7GHz Around 30GHz |
| System Bandwidth | [20] 40 MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells |
| Cell range | 10km range (Isolated cell) for sensing 100 km range (Isolated cell) to be evaluated through system level simulations for communication. Feasibility of Higher Range shall be evaluated through Link level evaluation (for example in some scenarios ranges up to 150-300km may be required). |
| User density and UE speed | User density: TBD Speed up to 160 km/h |
| Sensing target | Ship |
| Traffic model | Average data throughput at busy hours/user: 30 kbps User experienced data rate: up to 2 Mbps DL while stationary and 384 kbps DL while moving. |

-----------------------------------------------------------The End --------------------------------------------------------------------

3.8 Urban coverage for massive connection

The urban coverage for massive connection scenario focuses on large cells and continuous coverage to provide massive MTC. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas, with very high connection density of massive MTC devices. This deployment scenario is for the evaluation of the KPI of connection density.

Regarding the attributes on this deployment scenario, we may consider the legacy Table 6.1.7-1 in TR 38.913 as a starting point, and the details can be found in the Table 3.8. The update over the corresponding legacy table is marked in red.

Table 3.8 Attributes for urban coverage for massive connection

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700MHz, 2100 MHz as an option |
| Network deployment including ISD | Macro only, ISD = 1732m, 500m |
| Device deployment | Indoor, and outdoor in-car devices |
| Maximum mobility speed | 20% of users are outdoor in cars (100km/h) or 20% of users are outdoors (3km/h) 80% of users are indoor (3km/h) Users dropped uniformly in entire cell |
| Service profile | Non-full buffer with small packets |
| BS antenna elements | 2 and 4 Rx ports (8 Rx ports as optional) 4 and 8 Rx ports (16/32 Rx ports as optional) |
| UE antenna elements | 1Tx |

Proposal 2-8: Regarding the urban coverage for massive connection scenario for 6GR, the recommended attributes listed in Table 3.8 are supported.

3.9 Air-to-Ground Scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay).

In our views, the modification on its attributes is listed in Table 3.9 in red.

- Regarding traffic mode, the end user experienced data rate of 10Mbps is recommended.

Table 3.9 Attributes for air-to-ground scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro + relay: for BS to relay: [4] GHz |
| System Bandwidth | [100] MHz (DL+UL) |
| Layout | Macro + relay nodes |
| Cell range | Macro cell: 100 km range to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation. Relay: up to 80 m |
| User density and UE speed | End user density per Macro: UE speed: Up to 1000 km/h Altitude: Up to 15 km |
| Traffic model | End User experienced data rate: 10Mbps |

Proposal 2-9: Regarding the air-to-ground scenario for 6GR, the corresponding modification on the attributes listed in Table 3.9 is supported.

3.10 Non-Terrestrial Network

This deployment scenario is defined to allow for the provision of services for those areas where the terrestrial service is not available and also for those services that can be more efficiently supported by the satellite systems such as broadcasting service. Satellite acts as a fill-in especially on roadways and rural areas where the terrestrial service isn’t available. The supported services via the Satellite system are not limited to just data and voice, but also for others such as machine type communications, broadcast and other delay tolerant services.

For 6G NTN, more scenarios are considered in addition to 5G NTN to enable flexible NTN deployment. The following aspects can be considered for simulation:

- Regarding the layout of NTN, in addition to the single-orbit system considered in 5G NTN, multi-orbit system should be considered in 6G to enable a flexible topology and refarming of space-borne resource. Moreover, TN-NTN integrated system should also be considered according to regulation.

- Regarding the payload type, transparent and regenerative payload are already considered in 5G NTN and need to be also supported in 6G NTN. Additionally, the case where a transparent payload has limited onboard processing capability, e.g., MT onboard which enables gNB on the ground to directly control the satellite beam, is also preferred to be supported, which guarantees joint operation between satellite controlling and scheduling with reduced cost at satellite.

- Regarding the satellite assumption, in addition to the basic link level parameters assumed in 5G NTN, the parameters for systematic assessment should also be considered in 6G NTN. Due to limited satellite RF capability, beam hopping is needed to improve the coverage area in system level. To evaluate the system level coverage performance, beam hopping related parameter assumptions need to be considered, e.g., number of planned footprints for a satellite and number of simultaneous active beams.

Regarding the attributes on this deployment scenario, in addition to examples listed in [6], more inputs in Table 3.10 as follows are considered as a starting point. The update over the corresponding legacy table is marked in red.

Table 3.10: Examples for Satellite Deployment

| Attributes | Deployment-1 | Deployment-2 | Deployment-3 |
| --- | --- | --- | --- |
| Carrier Frequency NOTE1 | Around 1.5 or 2 GHz for both DL and UL | Around 20 GHz for DL Around 30 GHz for UL | Around 40 or 50 GHz Around 10 GHz for both DL and UL |
| Duplexing | FDD/TDD/HD-FDD | FDD/HD-FDD | FDD/HD-FDD |
| Satellite architecture NOTE 2 | Bent-pipe On-Board processing | Bent-pipe, On-Board Processing | Bent-pipe, On-Board Processing |
| Beam hopping parameter | Number of simultaneous active beams: 32/16; Number of planned footprints: 1058 | - | - |
| Typical satellite system positioning in the 6G architecture | Access network | Backhaul network Access network | Backhaul network Access network |
| System Bandwidth (DL + UL) | Up to 2*10 MHz | Up to 2*250 MHz | Up to 2 * 1000 MHz |
| Satellite Orbit | GEO, MEO, LEO, Mixed NOTE 3 | LEO, MEO, GEO | LEO, MEO, GEO |
| TN-NTN topology NOTE4 | Overlapped coverage, non-overlapped coverage |  |  |
| UE Distribution | 100% Outdoors | 100% Outdoors | 100% Outdoors |
| UE Mobility NOTE5 | Fixed, Portable, Mobile | Fixed, Portable, Mobile | Fixed, Portable, Mobile |

NOTE1:	The carrier frequencies noted here are for evaluation purpose only, satellites are deployed in wide range of frequency bands including L band (1-2GHz), S band (2-4GHz), C band (3.4-6.725 GHz), Ku band (10.7-14.8 GHz), Ka band (17.3-21.2 GHz, 27.0-31.0 GHz) and Q/V bands (37.5-43.5 GHz, 47.2-50.2 GHz and 50.4-51.4 GHz) and more. Here the around xGHz is used to denote the carrier frequency close to xGHz rather than used as band proxy to denote a band range.

NOTE2:	Bent pipe refers to the architecure where the satellite transponders are transparent—only amplify and change frequency but preserve the waveform, which may be controlled by the Gateway via 6GR or other interface. On Board Processing satellite transponders incorperate regeneration — including modulating and coding the waveform.

NOTE3:	Mixed orbits refers to the deployment with joint service from different orbits.

NOTE4:	The overlapped coverage refers to the deployment that TN and NTN provide the service to the UE in target area. The non-overlapped coverage refers to the deployment that the only the service from either TN or NTN is available for the in target area at given time instant.

NOTE5:	Mobile consitutes of both hand-helds and other moving platform receivers such as automobiles, ships, planes etc. Currently the hand-helds are limited to L and S bands but the research is ongoing to support higher bands.

Proposal 2-10: Regarding Non-Terrestrial Network (NTN) scenario for 6GR, the recommended attributes listed in Table 3.10 are supported.

3.11 Highway Scenario

The highway deployment scenario focuses on scenario of vehicles placed in highways with high speeds. The main KPIs evaluated under this scenario would be reliability/availability under high speeds/mobility (and thus frequent handover operations).

Regarding the attributes on this deployment scenario, we may consider the legacy Table 6.1.8-1 in TR 38.913 as a starting point, and the details can be found in the Table 3.11. The update over the corresponding legacy table is marked in red.

Table 3.11 Attributes of Highway

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro only: Below 6 GHz (around 6 GHz) Around 4GHz, Around 7GHz Macro + RSUs NOTE2: 1) For BS to RSU: Below 6 GHz (around 6 GHz) NOTE3 2) RSU to vehicles or among vehicles: below 6 GHz |
| Aggregated system bandwidth | Up to 200MHz (DL+UL) Up to 100MHz (SL) |
| Layout | Option 1: Macro only Option 2: Macro + RSUs NOTE2 |
| ISD | Macro cell: ISD = 1732m, 500m(Optional) Inter-RSU distance = 50m or 100m |
| BS antenna elements | Tx: Up to 2562048 Tx Rx: Up to 2562048 Rx |
| UE antenna elements | RSU Tx: Up to 8 Tx RSU Rx: Up to 8 Rx Vehicle Tx: Up to 8 Tx Vehicle Rx: Up to 8 Rx |
| User distribution and UE speed | 100% in vehicles Average inter-vehicle distance (between two vehicles’ center) in the same lane is 0.5sec or 1sec * average vehicle speed (average speed: 100-300km/h) |
| Traffic model | 50 messages per 1 second with absolute average speed of either - 100-250 km/h (relative speed: 200 – 500km/h), or - 30 km/h |
| Sensing target | Sensing target type: vehicle, road for infrastructure collapse monitoring, corner reflector for structural health monitoring Distribution: - 100% outdoor - Vehicle: The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. Mobility: - vehicle: 140km/h - road: 0km/h - corner reflector: 0km/h |

Proposal 2-11: Regarding the highway scenario for 6GR, the recommended attributes listed in Table 3.11 are supported.

3.12 New scenario on multi-layer heterogeneous network with assisting node

To address the issue of coverage and cell-edge performance along with the overall energy consumption, the deployment with more sustainable assisting nodes, including passive/semi-passive nodes, e.g., Reconfigurable intelligent surfaces (RIS), Network-controlled repeater (NCR) can be considered[3][4][5]. For example, the low consumption can be assumed for RIS to enable the deployment with different needs.

The detailed attributes on multi-layer heterogeneous network with assisting node are listed in Table 3.12.

Table 3.12 Recommended attributes for the new scenario of multi-layer heterogeneous network with assisting node

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Carrier Frequency |  | Around 4 GHz Around 7 GHz Around 30 GHz |
| Aggregated system bandwidth |  | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1 GHz (DL+UL) |
| Layout |  | Two layers: - Macro or micro TRxPs layers, Hex. Grid Assisting node layer, dropped assisting nodes randomly or at a fixed distance range from the TRxP as shown in the following. |
| Configuration of the assisting node | Distribution of assisting nodes | - For dense urban scenarios: uniform/macro TRxPs, e.g., 4 assisting nodes per sector; - For indoor hotspot scenarios: 100% indoor, e.g., 2 assisting nodes per sector; |
|  | Antenna configuration of assisting nodes | 2D antenna array with the number up to companies’ report |
|  | Power consumption Per Element | For RIS with adapative beam: 0.4 mw is baseline[4]. For RIS with fixed beam: 0 mw. Other values are not precluded, which is up to companies’ report. |
| Other attributes, e.g., attributes related to BS and UE |  | Same as the value for basic deployment scenarios |

More specifically, we have the following analysis:

- Regarding the layout, the assisting node layer includes multiple assisting nodes (e.g., RIS nodes), which can be dropped randomly or at a fixed distance range from the TRxP as shown in Figure 3-13. Then the distance range value can be determined according to the specific evaluation requirements and the restriction of ISD as well as cell radius. And by deploying such two-layer network with RIS nodes, the increase in energy efficiency can be achieved compared to single-layer network with only TRxPs.

- Regarding the configuration of assisting nodes, similar as the configuration attributes of BS and UE, at least the distribution and antenna elements of assisting nodes shall be considered, and the specific value of parameter highly depends on the deployment scenarios, carrier frequency, and target performance objectives.

Proposal 2-12: For 6GR deployment scenario, a new deployment scenario of multi-layer heterogeneous network with assisting node should be introduced, in order to address the issue of coverage and cell-edge performance along with the overall energy consumption.

- In such case, the recommended attributes assumptions listed in Table 3.12 should be supported.

3.13 New scenario on indoor factory

The indoor factory deployment scenario can refer to TR 38.901 and TR 38.857. In indoor factory scenarios, the sensing targets are humans or automated guided vehicles (AGVs) inside a factory. Mono-static or bi-static sensing can be performed using TRPs and/or UEs selected from the corresponding communication scenario.

ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed in Table 3.13.

Table 3.13 Attributes for indoor factory

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz Around 30 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1 GHz (DL+UL) |
| Layout | Single layer: - Indoor floor |
| ISD | ISD = 50m (Equivalent to 18TRxPs per 300m x 150m) |
| BS antenna elements | Around 4GHz: Up to 256 Tx and Rx antenna elements Around 7GHz: Up to 1024 Tx and Rx antenna elements Around 30GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 4GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP |
| Sensing target | Sensing target type: Human, or AGV Distribution: - 100% indoor - Uniformly distributed over the horizontal area of the convex hull of the TRP deployment Mobility: - Human: 3km/h - AGV: 30km/h |

Proposal 2-13: For 6GR deployment scenario, a new deployment scenario of indoor factory should be introduced, in order to consider the a new ISAC deployment sensing humans or automated guided vehicles (AGVs) inside a factory

- In such case, the recommended attributes for indoor factory listed in Table 3.13 should be supported.

3.14 New scenario on nearshore water

In nearshore water scenarios, the sensing targets are ships traveling on nearshore waters or rivers, BSs are primarily land-based as shown in Figure 3-14. Mono-static or bi-static sensing can be performed using TRPs selected from the corresponding communication scenario.

Figure 3-14 Deployment scenario of nearshore waters

ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table 3.14.

Table 3.14 Attributes for nearshore water

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 7 GHz Around 30 GHz |  |
| Layout | Macro cell: ISD = at least 500m |  |
| BS antenna elements | Up to 2048 Tx and Rx antenna elements |  |
| Sensing transmitters and sensing receivers | TRP mono-static, TRP-TRP bi-static |  |
| Sensing targets | Sensing target type | Ship |
|  | Mobility | Uniform distribution between 10km/h and 60km/h |
|  | Distribution | 100% Outdoor, uniformly distributed within a sensing area |

Proposal 2-14: For 6GR deployment scenario, a new deployment scenario of nearshore water should be introduced, in order to consider the a new ISAC deployment sensing ships traveling on nearshore waters or rivers

- In such case, the recommended attributes for nearshore water listed in Table 3.14 should be supported.

- Conclusion

In this contribution, we elaborate our general views on 6G deployment scenarios, based on the principle analysis from antenna modelling, multi-TRP operation and multi-carrier aspects. We have the following proposals:

Proposal 1-1: To ensure overall coverage, focus on cell-edge performance and UL coverage (especially for co-site deployment of around 7GHz), large antenna array with more than 1024 BS antenna elements (e.g. up to 1536 or 2048 antenna elements) and 8 or more Tx and Rx UE antenna elements (e.g., up to 16) should be considered.

Proposal 1-2-1: To improve transmission performance, especially for cell-edge performance, in addition to legacy single-TRP operation, multi-TRP operation should be considered for 6G deployment: CJT (targeting for FR1/around-7GHz) and NCJT transmission (targeting FR2/around 30GHz).

Proposal 1-2-2: To reduce deployment cost, ensure UL coverage and save NW energy cost, the new architecture of DL-TRP and UL-TRP decoupling, i.e., asymmetric DL sTRP/ UL mTRP

- Case #1: On the single-layer deployment scenarios, a mixed mode of {DL+UL TRP, UL-only TRP} should be supported.

- Case #2: On the two-layer deployment scenarios, e.g., dense urban (two-layer), micro layer can further consider the candidate deployment scenario of UL-only TRP, in addition to a mixed mode of {DL+UL TRP, UL-only TRP} for macro/micro.

Proposal 1-3: To achieve comparable UL coverage and better capacity in 6G deployment scenarios, especially around 7GHz, support multi-carrier based deployment scenario for 6G with taking flexible spectrum utilization and minimal UE complexity into account.

Proposal 2-1: Regarding indoor hotspot deployment scenarios for 6GR, the modification on attributes for indoor hotspot listed in Table 3.1 is supported.

Proposal 2-2: Regarding dense urban deployment scenarios for 6GR, the modification on attributes for dense urban listed in Table 3.2 is supported.

Proposal 2-3: Regarding rural deployment scenarios for 6GR, the modification on attributes for rural listed in Table 3.3 is supported.

Proposal 2-4: Regarding urban macro deployment scenarios for 6GR, the modification on attributes for urban macro listed in Table 3.4 is supported.

Proposal 2-5-1: Regarding sub-urban macro, the following description on the key characteristics of this scenario is recommended:

- The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.

Proposal 2-5-2: Regarding sub-urban macro deployment scenarios for 6GR, the modification on attributes for sub-urban macro listed in Table 3.5 is supported.

Proposal 2-6-1: Regarding high-speed, the following description on this scenario is recommended (from TR 38.913):

- The high speed deployment scenario focuses on continuous coverage along track in high speed trains. The key characteristics of this scenario are consistent passenger user experience and critical train communication reliability with very high mobility. In this deployment scenario, dedicated linear deployment along railway line and the deployments including SFN scenarios captured in Section 6.2 of 3GPP TR 36.878 [6] are considered, and passenger UEs are located in train carriages. For the passenger UEs, if the antenna of relay node for eNB-to-Relay is located at top of one carriage of the train, the antenna of relay node for Relay-to-UE could be distributed to all carriages.

Proposal 2-6-2: Regarding the high speed deployment scenarios for 6GR, the recommended attributes listed in Table 3.6 is supported.

Proposal 2-7: Regarding the extreme long distance coverage deployment scenario for 6GR, the corresponding modification on the attributes listed in Table 3.7 is supported.

Proposal 2-8: Regarding the urban coverage for massive connection scenario for 6GR, the recommended attributes listed in Table 3.8 are supported.

Proposal 2-9: Regarding the air-to-ground scenario for 6GR, the corresponding modification on the attributes listed in Table 3.9 is supported.

Proposal 2-10: Regarding Non-Terrestrial Network (NTN) scenario for 6GR, the recommended attributes listed in Table 3.10 are supported.

Proposal 2-11: Regarding the highway scenario for 6GR, the recommended attributes listed in Table 3.11 are supported.

Proposal 2-12: For 6GR deployment scenario, a new deployment scenario of multi-layer heterogeneous network with assisting node should be introduced, in order to address the issue of coverage and cell-edge performance along with the overall energy consumption.

- In such case, the recommended attributes assumptions listed in Table 3.12 should be supported.

Proposal 2-13: For 6GR deployment scenario, a new deployment scenario of indoor factory should be introduced, in order to consider the a new ISAC deployment sensing humans or automated guided vehicles (AGVs) inside a factory

- In such case, the recommended attributes for indoor factory listed in Table 3.13 should be supported.

Proposal 2-14: For 6GR deployment scenario, a new deployment scenario of nearshore water should be introduced, in order to consider the a new ISAC deployment sensing ships traveling on nearshore waters or rivers

- In such case, the recommended attributes for nearshore water listed in Table 3.14 should be supported.

- References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., 	Deutsche Telekom

- RP-251872, Text proposal for TR 38.914 for deployment scenarios, CMCC

- China Telecom, Trial of RIS deployment at 3.5 GH, URL: http://www.chinatelecom.com.cn/news/xinyidaixinxitongxin/202504/t20250402_87145.html.

- Cao, X., Deng, C., Yin, Y. et al. Two-bit dual-polarized reconfigurable intelligent surface with low power consumption for 6G near-field communication. Front Inform Technol Electron Eng 25, 1695–1707 (2024). https://doi.org/10.1631/FITEE.2400379

- Y. Hao, C. Deng, X. Cao, Y. Yin and K. Sarabandi, "A High Aperture Efficiency 1-bit Reconfigurable Reflectarray Antenna With Extremely Low Power Consumption," in IEEE Transactions on Antennas and Propagation, vol. 72, no. 1, pp. 1015-1020, Jan. 2024, doi: 10.1109/TAP.2023.3310579.

- 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies (Rel-15).