3GPP TSG RAN Meeting #109	   RP-252024

Beijing, China, September 15-18, 2025

Source:	 ZTE Corporation, Sanechips

Title:	 Views on RAN KPIs for IMT-2030

Agenda Item:	8.2.1.2

Document for:    Discussion and Decision

- Introduction

In RAN#103 meeting, the following RAN plenary work plan was agreed for IMT-2030 aspects [1] where the RAN study includes two phases corresponding to ITU IMT-2030 TPR and 6G general aspects respectively.

RAN plenary work is split into an ITU focused SI (study item) and General RAN SI - IMT-2030 discussion is expected in RAN from 09/24 to 12/24 - RP SI Rel-20 focusing on ITU– IMT-2030: approval 12/24 until 06/25 - RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In RAN#106 meeting, the new SID was endorsed for Study on 6G scenarios/use cases and requirements where the first phase on ITU TPR part is included.

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

In RAN#107, the moderator submitted a document comprising of all the proposals received from the meeting in RP-250819 [6].

In RAN#108 meeting, some preliminary agreements have been made regarding the TPRs and their values, as captured in the reply LS to ITU-R WP 5D RP-251825 [9]. In this contribution, we provide our views on some TPR / KPI values that are still pending.

- TPR candidates and values

In ITU-R WP5D#48 in February 2025, input documents including candidate TPR list provided by 3GPP RAN were discussed in SWG radio aspects. A preliminary list of Minimum Technical Performance Requirements (MTPRs) [4] was agreed and sent to SWG Evaluation from SWG radio aspects as shown below:

Preliminary list of Minimum Technical Performance Requirements

(Note: Square brackets indicate further discussion required)

| Minimum Technical Performance Requirement | Method |
| --- | --- |
| AI-related capabilities | (to be determined) |
| Area traffic capacity | (Quantitative) (Note 1) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) (Note 2) |
| [Joint requirement on data rate, latency, reliability and capacity] | To be determined |
| [Coverage] | To be determined |
| [Security] | To be determined |
| [Resilience] | To be determined |
| [Sustainability] | To be determined |
| [Interoperability] | To be determined |

Note 1: The evaluation technique for quantitative requirements is determined by SWG Eval (e.g., analysis, simulation, etc.).

Note 2: The reliability requirement may be replaced by the “Joint Requirement” (if agreed).

Coverage refers to the ability to provide access to communication services for users in a desired service area. In TR 38.913, coverage and extreme coverage are already defined as a KPI in section 7.10 and 7.10.1 which are evaluated by calculating MaxCL. We believe in IMT-2030, this definition can be re-used.

Proposal 1: Do not define new KPIs for coverage, i.e. re-use the definition in section 7.10 in TR 38.913.

It can also be noted that 3GPP NR technologies passed the ITU-R IMT-2020 requirements with huge margins for some TPRs, for instance, DL peak data rate can be as high as 171.2 Gbps (in FR2), which is more than 8 times higher than IMT-2020 TPR on DL peak data rate. NR can also achieve 64.6 Gbps (in FR2) UL peak data rate, which is already 6 times higher than IMT-2020 TPR. Thus, even if we propose to set the TPR of peak data rate to be 5 times higher than IMT-2020, it can be seen as a moderately advanced goal as NR can already satisfy this goal.

Observation 1: According to TR 37.910 “Study on self evaluation towards IMT-2020 submission”, many of the proposed IMT-2020 TPR target values are met by 3GPP NR technologies with a huge margin.

During the discussions happening in the ITU-R WP 5D, when discussing the TPR values, members also discuss the related configurations. For instance, to achieve 5 times higher peak data rate, it may imply at least 2 times larger bandwidth. To achieve 2.5 times higher peak spectral efficiency, 16 MIMO layers may be supported compared to 8 MIMO layers assumed for IMT-2020. We want to provide some observations on how to correctly interpret the relationship between ITU IMT-2030 TPRs and 3GPP configurations, as well as how a candidate technology can be seen as satisfying IMT-2030 evaluations. From ITU-R M.2412 [11],

8.4 Evaluation configurations Evaluation configurations are defined for the selected test environments. The configuration parameters shall be applied in analytical and simulation assessments of candidate RITs/SRITs. For the cases when there are multiple evaluation configurations under the selected test environment, one of the evaluation configurations under that test environment can be used to test the candidate RITs/SRITs. The technical performance requirement corresponding to that test environment is fulfilled if this requirement is met for one of the evaluation configurations under that specific test environment.

It is clear that from ITU perspective, a technology proposal meets the requirement if any of a supported configuration meets the requirement.

Observation 2: From ITU evaluation perspective, a technology proposal meets the requirement if any of a supported configuration meets the requirement.

As reflected by some previous discussions, the phrase ‘minimum’ TPR by ITU can cause misunderstanding. Some may worry that a ‘minimum’ TPR means that all configurations of a technology must all meet such requirements, while this is certainly not the case.

Observation 3: The ITU ‘minimum’ TPR can be met by using any of the configuration supported by a technology, or, equivalently, the configuration resulting the best performance.

Continuing this discussion, as long as one part of the technology can meet the requirements, the whole technology is judged to pass the evaluation. This means, as long as the best performance of 3GPP 6G meets the IMT-2030 TPRs, the whole 3GPP 6G specifications will be recognized as a valid IMT-2030 technology. Devices and network equipments that operate with any possible configurations will be recognized as part of the 6G system. Take the number of antennas as an example, if the peak data rate is 5 times higher than IMT-2020, which may imply 16 UE Rx antennas at 7 GHz, it means that up to 16 Rx antennas are supported by evaluations (1, 2, 4, 8, 16 are all possible configurations). 3GPP can meet the ITU requirements on peak data rate using 16 Rx configuration at 7 GHz, and it is totally fine if the peak data rate cannot be achieved with 1/2/4/8 Rx as this is how ITU evaluations work and is true for almost all technology proposals.

In short, from ITU evaluation perspective, as long as a part (the best part) of the specifications meet the TPRs, the whole technology proposal meets the IMT-2030 requirements. From commercialization perspective, operators and vendors are free to choose a subset of the specifications / configurations from 3GPP 6G, as all these subsets / configurations are recognized as qualified 6G technologies. From 3GPP perspective, supporting up to 16 antennas at device side for evaluations is an effective way to allow for a higher maximum data rate to be more future proof, as well as making 3GPP terrestrial components more competitive among all the candidate technologies. From both ITU and 3GPP perspectives, the evaluation assumptions do not force vendors or operators to choose any particular implementation, as is clearly stated in the ITU evaluation guidelines,

8.4 Evaluation configurations …… (omitted parts) The configuration parameters (and also the propagation and channel models in Annex 1 of this Report) are solely for the purpose of consistent evaluation of the candidate RITs/SRITs and relate only to specific test environments designed for these evaluations. Therefore, the configuration parameters should not be considered as those that must be used in any deployment of any IMT-2020 system nor should they be taken as the default values for any other or subsequent study in ITU or elsewhere. They do not necessarily themselves constitute any requirements on the implementation of the system.

Thus, supporting more antennas at the device side for the purpose of evaluation has the benefits of being more future proof and demonstrating the advantages of 3GPP terrestrial components, while not forcing implementation choices.

Proposal 2: Consider up to 16 UE Rx for device types such as CPEs at 7 GHz for evaluations, which will be future proof and make 3GPP terrestrial specifications competitive enough for IMT-2030.

On the number of antennas in configurations, we also observe that 8 Rx will likely remain as a configuration under 4 GHz carrier frequency.

Observation 4: According to the ITU discussions, no one proposed to remove Configuration A (4 GHz and 8 UE Rx) from the list of configurations, and thus, 8 UE Rx will likely remain a valid configuration under 4 GHz.

For the new KPI, likely to be named as “5th percentile user data rate”, a new evaluation approach is used. Before in IMT-2020, the simulation was done on 20 MHz, then an associated data rate was acquired. As a next step, bandwidth scaling was done on the data rate. For instance, if the data rate was 10 Mbps on 20 MHz, and the total system bandwidth was 100 MHz, the user experienced data rate in UL was 50 Mbps. Now, we need to do simulations directly using 100 MHz, since for UE, the transmit power is limited and not per bandwidth, so bandwidth scaling might not be applicable. Under this new simulation approach, we re-run our simulations using 100 MHz bandwidth, 26 dBm for UE Tx power, 1024 antennas at the BS side, 4 Tx antenna at UE side, enabling MU-MIMO scheme, the simulation results show that UL 5th percentile user data rate can achieve 50 Mbps.

Proposal 3: Define the UL 5th percentile user data rate as 50 Mbps.

For composite requirement, the ITU-R WP 5D has finalized the definition, the next step is to study proper values for it. As an emerging use case for IC is XR services, and the composite requirement is also meant to characterize XR-like services, we suggest to distinguish between XR and traditional video services by demonstrating the multi-modality nature of XR.

Observation 5: Multi-modality (mixed traffic model of video, haptic, etc) is a nature of XR services that distinguishes them from traditional video services.

Based on the observation, we propose to define the traffic model as below

| Traffic model | DL Video: 45Mbps@60FPS, reliability = 99%, PDB = 10ms Haptic: fixed 100 Byte/s, reliability = 99.9%, PDB=5ms, pareto distribution arrival UL Video: 20Mbps@60FPS, PER=99%, PDB = 10ms |
| --- | --- |

Proposal 4: Define a multi-modal traffic model (corresponding to data rate, latency and reliability) for composite requirement as in the table above, which comprises of video and haptic data.

For ISAC, we also need to define new KPIs to characterize the system performance. We suggest to define detection probability and false alarm rate, localization accuracy and velocity estimation accuracy as three KPIs, with their proposed definition listed below.

Sensing-related capabilities are measured in terms of the following KPIs: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Estimation Accuracy: Velocity estimation accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

Considering SA1 requirements and achievable performance, we suggest the following sensing performance requirements: detection probability 95%, false alarm probability 5%, horizontal localization accuracy 2m, vertical localization accuracy 5m, velocity accuracy 1m/s.

Proposal 5: Define detection probability and false alarm probability, localization accuracy and velocity estimation accuracy as sensing KPIs.

- Detection probability: 95%

- False alarm probability: 5%

- Horizontal localization accuracy: 2m

- Vertical localization accuracy: 5m

- Velocity accuracy: 1m/s

Positioning is supported by NR and is expected to be supported by 6GR as well. The KPI for positioning is naturally the positioning accuracy, and we believe it is critical to consider both horizontal and vertical position accuracy at the same time. There are some use cases demanding vertical positioning accuracy, such as emergency (911) services and positioning for UAVs. We propose the definition for positioning KPI as shown below.

Positioning is the ability to estimate the position of connected devices. The positioning accuracy is the 90% point of the CDF of the device positioning error for both horizontal and vertical directions. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude). The minimum requirements for the device positioning accuracy are TBD.

Based on ITU-R M.2160 and achievable positioning accuracy captured in TR38.855 and TR38.857, the target of the positioning accuracy is proposed as below

Proposal 6: Define both horizontal and vertical positioning accuracy.

  - Horizontal positioning accuracy: 20cm-1m (indoor), 3m (outdoor) for 90% of UEs

  - Vertical positioning accuracy: 5m for 90% of UEs

With regards to the mobility interruption time, it should be noted that the basic mobility comes in two flavours: UE controlled and network controlled. In case of 5G, in addition to the basic mobility features, a number of mobility enhancements were specified in successive releases and this resulted in features such as LTM, CLTM, CHO, etc. In 6G the mobility framework should consider all these features and main focus should be on a unified and simplified framework for mobility. The final detailed design for mobility depends partly on the RAN level security framework and also RAN architecture.

Furthermore, one of the key considerations for the mobility framework is the overall design for spectrum aggregation (including support for CA and/or DC etc) and the general design to minimize the throughput fluctuation during the handover in case CA/DC is configured. Furthermore, applications such as autonomous driving require that there is no interruption time during mobility. Thus, a target of 0 ms mobility interruption time should be included as part of the 6G system KPI. The target of 0 ms interruption time should be achievable in a simple and realistic mobility framework for 6G. As noted in 3GPP TR 37.910, 0 ms interruption time can be achieved in 5G for instance by using beam mobility and/or CA based mobility framework. Thus, in 6G this can be a realistic target using beam mobility and/or CA based mobility combined with features such as cell-free and/or fast role change between SCell and PCell.

Proposal 7: The mobility interruption time of 0 ms in 6G should be targeted in typical scenarios, aiming for a unified mobility framework for network-controlled mobility.

We implement the above suggested points in a text proposal submitted to this meeting, RP-252025.

- Summary

Proposal 1: Do not define new KPIs for coverage, i.e. re-use the definition in section 7.10 in TR 38.913.

Observation 1: According to TR 37.910 “Study on self evaluation towards IMT-2020 submission”, many of the proposed IMT-2020 TPR target values are met by 3GPP NR technologies with a huge margin.

Observation 2: From ITU evaluation perspective, a technology proposal meets the requirement if any of a supported configuration meets the requirement.

Observation 3: The ITU ‘minimum’ TPR can be met by using any of the configuration supported by a technology, or, equivalently, the configuration resulting the best performance.

Proposal 2: Consider up to 16 UE Rx for device types such as CPEs at 7 GHz for evaluations, which will be future proof and make 3GPP terrestrial specifications competitive enough for IMT-2030.

Observation 4: According to the ITU discussions, no one proposed to remove Configuration A (4 GHz and 8 UE Rx) from the list of configurations, and thus, 8 UE Rx will likely remain a valid configuration under 4 GHz.

Proposal 3: Define the UL 5th percentile user data rate as 50 Mbps.

Observation 5: Multi-modality (mixed traffic model of video, haptic, etc) is a nature of XR services that distinguishes them from traditional video services.

Proposal 4: Define a multi-modal traffic model (corresponding to data rate, latency and reliability) for composite requirement as in the table above, which comprises of video and haptic data.

Proposal 5: Define detection probability and false alarm probability, localization accuracy and velocity estimation accuracy as sensing KPIs.

- Detection probability: 95%

- False alarm probability: 5%

- Horizontal localization accuracy: 2m

- Vertical localization accuracy: 5m

- Velocity accuracy: 1m/s

Proposal 6: Define both horizontal and vertical positioning accuracy.

  - Horizontal positioning accuracy: 20cm-1m (indoor), 3m (outdoor) for 90% of UEs

  - Vertical positioning accuracy: 5m for 90% of UEs

Proposal 7: The mobility interruption time of 0 ms in 6G should be targeted in typical scenarios, aiming for a unified mobility framework for network-controlled mobility.

- References

- RP-240823 Additional Considerations for 6G Timeline Source: TSG Chairs

- RP-250810 “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-243202	Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP TSG#106, December 2024

- ITU-R WP5D 563 Report on the 48th meeting of Working Party 5D (Geneva, 4-13 February 2025) Chair, WP 5D

- 3GPP RAN, TR 37.910, “Study on self evaluation towards IMT-2020 submission”

- RP-250819 Moderator's summary for IMT-2030 TPRs, Moderator (CMCC)

- Report ITU-R M.2412-0 “Guidelines for evaluation of radio interface technologies for IMT-2020”

- 3GPP SA1, TR 22.870, “Study on 6G Use Cases and Service Requirements”

- RP-251825 Reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN

- ITU-R Document 5D/TEMP/345-R1: Working Document Towards A Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]

- Appendix: preliminary simulation results

## Composite requirement

For the value of composite requirement for immersive communication, we provide some candidate values shown as follows:

- For test environment of Dense Urban-IC, the DL capacity can be 70Gbits/s/km2.

- For test environment of Urban Macro-IC, the DL capacity is 10G bits/s/km2

  - Traffic model: Video: 45Mbps@60FPS, reliability=99%, PDB = 10ms; Haptic: fixed 100 Byte/s, reliability = 99.9%, PDB=5ms, pareto distribution arrival

For DL composite requirement in test environment UMa-IC, we provide the following statistics of transmitted XR packet sizes and service time for each XR packet, respectively.

| Figure a) transmitted XR packet sizes | Figure b) service time for each packet |
| --- | --- |

The candidate value of capacity in composite requirement can be calculated by:

(Average transmitted XR packet size / average service time for each packet / A) * UE density,

where average transmitted XR packet size can be obtained in Figure a), average service time for each packet can be obtained in Figure b). UE density is average 5 UE per cell

## Sensing

| Sensing mode | TRP monostatic |
| --- | --- |
| Carrier frequency | 6GHz |
| bandwidth | 100 MHz |
| SCS | 30kHz |
| BS Tx power | 49 dBm |
| BS antenna configuration | (M, N, P, Mg, Ng) = (16, 16, 2, 1, 1), dH = 0.5 λ，dV = 0.5 λ |
| BS height | 35m |
| BS noise figure | 5 dBi |
| Thermal noise | -174dBm/Hz |
| UAV height (sensing target) | 200m |
| UAV velocity | 0-60km/h uniform in horizontal |
| UAV distribution | 5 targets in center sector |
| Number of Drops | 200 |

(a) Velocity

(b) Pos_H_Cluster                          (c) Pos_V_Cluster

Figure: TRP monostatic (coordinated sensing) sensing CDF with (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1)

Table: TRP monostatic (coordinated sensing) sensing results

| Horizontal localization accuracy | Vertical localization accuracy | Velocity accuracy | Detection probability |
| --- | --- | --- | --- |
| 0.850 | 0.565 | 0.332m/s | 98.0% |