3GPP TSG RAN Meeting #109                                         RP-252027

Beijing, China, September 15-18, 2025

Source:	 ZTE Corporation, Sanechips

Title:	 Requirements for 6GR services

Agenda Item:	8.2.1.4

Document for:    Discussion and Decision

- Introduction

In RAN#107 meeting, the revised SID for study on 6G scenarios and requirements was endorsed as below [1].

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

Further, in RAN#108 meeting, the initial version of TR 38.914 towards to outcome of the above 6G RAN study was agreed in [2].

In this contribution, we provide our views on the requirements of new and existing services including positioning service, sensing, AI services, HRLLC, Immersive communication, etc. The details can be found in section 2.

- Discussion

For 6G, ITU-R has defined usage scenarios and overarching aspects of IMT-2030 as shown in Figure 1. The requirement for services includes immersive communication, sensing, AI, HRLLC, etc.

Figure 1 IMT-2030 scenarios from ITU-R

In 3GPP, new services and corresponding requirements are being studied in SA1 for 6G. After the SA1 discussions for four meetings, 3GPP 6G use case TR 22.870 [3] has reached 77% completion covering new use cases and requirements on System and Operational Aspects, AI, ISAC, Ubiquitous Connectivity, Massive Communication and Verticals, etc. For the upcoming 6G RAN study, it’s recommended to consider the typically identified services.

  - AI services

According to discussion in the latest ITU-R WP 5D meeting, the AI-related capabilities which refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications have been discussed, including“Network for AI” and “AI for network”. The RIT should support AI-related capabilities such as Data collection, AI model training/inference/execution, Distributed processing/learning, AI computing, which refers to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to above AI functionalities.

Until now, 54 AI-related use cases have been agreed in AI chapter of TR 22.870 [3], which include requirements about AI/computing capabilities, AI agents, AI-related collaborative tasks and so on. Some key scenario and requirements include:

- AI capabilities

AI capabilities include the following aspects: 1) AI/ML model training, inference and AI model management; 2) AI service QoS (e.g. latency, inference accuracy) guarantee and service continuity; 3) AI/computing resource allocation and monitoring.

- AI agents

AI agents can be divided into two types: 1) AI agents on applications of UE or application server; 2) AI agents on 6G network. For the AI agents on applications, it may invoke 3GPP service (e.g. communication, sensing, AI) or other AI agents via intent. For AI agents on 6G network, it may provide the 3GPP service towards UE or third party based on the received request via intent.

- AI-related collaborative tasks

The AI-related collaborative task may include: 1) collaborative AI task via AI agents; 2) collaborative AI task on multiple UEs; 3) AI services to enable collaboration.

Base on these discussions, certain scenario requirements deserve the attention of the RAN side. RAN needs to consider which AI-related capabilities need to be supported and how these capabilities can be adapted to align with the requirements of these scenarios, like providing services through user intent/AI agent, collaborative task between AI applications via multiple UE/AI agents. In these use case RAN plays a pivotal role in enabling system efficiency, better user experience and real-time adaptation.

-  For the AI service traffic, the round-trip latency for data transmission and the latency for AI inference should be considered jointly. New traffic transmission characteristics (e.g. burst data rate, token, etc.) for GenAI may be considered as well which pends to SA2 discussion on QoS framework.

- Intent can provide a novel way for users to interact with the network. The RAN side need to acquire assistance information provided by the application layer to ensure the consistency in understanding of intent from user. If the intent involves multiple devices/applications, it’s equally important to ensure coordination and consistency across devices. 6G network needs to fully understand and interprets the intent, translating it into specific service and resource allocation for each participating device, monitoring task progress, and dynamically adjusting resources as needed with involvement of RAN.

- With AI service introduced by 6G network, the network can leverage AI-related capabilities to empower UEs and applications to collaboratively accomplish more complex task, like based on the different capabilities of data collection devices, may first undergo AI inference at the device level before transmitting results to the 6G network for further inference and model updates, or directly transmit raw data to the 6G network for comprehensive multi-source data fusion or complex model training.

6G design should support AI services including:

- Cross-layer information exchange needs to be supported, including the assistance application layer information sharing to RAN directly or via CN.

- RAN should support a uniform mechanism to transfer AI related data/information to CN.

- RAN should support AI related data collection and management in an efficient way.

- RAN should support and guarantee the performance of intent-driven intelligent cooperation among devices.

  - Sensing

In Rel-19 ISAC channel modeling study, the use cases of detection and tracking of UAV, pedestrian, Vehicle, and AGV had been justified, and the corresponding channel modeling had been completed. Therefore, the support of these use cases should be considered in 6GR RAN1 study.

Furthermore, some new use cases have been endorsed in SA1 for 6G ISAC [3]. Notably, ship detection, infrastructure collapse monitoring, and structural health monitoring have garnered substantial support from operators[4]. The commercial outlook and public safety benefits of these use cases are highly positive.

- Ship detection           (b) Infrastructure Collapse Monitoring    (c) Structural health monitoring on a bridge

Figure 2 Examples of SA1 defined new use cases on 6G ISAC

In addition to 5G-A use cases (UAV, Pedestrian, Vehicle, AGV), 6G RAN design shall support the following sensing use cases.

- Ship detection

- Infrastructure collapse monitoring

- Structural health monitoring

  - Immersive communication

In 5G NR, traffic for eXtended reliability (XR) is introduced in Rel-17 [6], which is characterized by quasi-periodic, large and variable packet size, stringent latency requirement and high reliability requirement. For 6GR, according to [3][7], immersive communication provides service of connecting remote user, remote robot, AI agent, generative AI and even the virtual digital world, which contains new features compared to 5G NR with more diverse traffic type, e.g., haptic data or kinaesthetic senses [8]. Additionally, as shown in Figure 3, when the number of sensors increases, haptic packets are aggregated and generate a haptic frame, where the frame sizes vary.

Figure 3 Typical scenario and haptic steam for immersive communication

For immersive communication, the QoS requirements are different among data streams. According to [7], the haptic stream has the QoS requirement in terms of 99.9999% reliability and less than 5ms latency, while video stream has the QoS requirement in terms of 99% reliability and around 10ms latency. For 6G immersive communication, good immersive experience lies on not only all streams meet their own QoS requirements, but also the synchronization among different streams associated with same immersive application e.g. for haptic stream and video stream [8]. Therefore, composite requirement of data rate, latency, reliability, capacity should be considered [2].

6G RAN design shall provide mechanisms to support Immersive communication service and consider the combination with positioning, sensing, computing and AI capabilities.

  - Positioning

In the endorsed SA1 TR 22.870 [3], numerous use cases for 6G positioning have been captured, e.g. use case on Immersive gaming, seamless Immersive Reality in Education, high-precision Industrial control, and Remote and Automatic construction as shown in the Figure 4.

Figure 4 Examples of SA1 defined use cases on 6G positioning

As described below in TR 22.870 [3], 6G system shall be able to support location and positioning services.

5.4.2 Support of legacy services The 6G system shall be able to support the following services: - Mission Critical Services, i.e., MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56], - Message service, ref TS 22.262 [57], - SMS, ref TS 22.101 [58], - Multimedia communication services, ref TS 22.101 [5f], TS 22.261 [14], TS 22.173 [59], - IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59], - Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14], - Location and positioning services, ref TS 22.261 [14], TS 22.071 [61], - Broadcast and Multicast Services, ref TS 22.261 [14],

In addition, ITU-R has decided to define a quantitative TPR for 6G positioning, the corresponding test environment scenarios may include both indoor and outdoor scenarios. The corresponding evaluation methodology is also under discussion in ITU-R WP5D where the wireless-dependent positioning method is supposed to be used. It is expected the TPR in ITU-R will be completed in February 2026, and the requirements on positioning can be reused in RAN1 then.

From RAN perspective, we see a lot of commonality between sensing and positioning, for example, reference signal design, and physical layer procedure design. Thus, we think the support of 6GR wireless-dependent positioning can be studied with sensing together. Considering the lessons learned from NR positioning, we suggest to make 6GR design simpler than NR including framework, procedure, RS design, etc.

6G RAN design shall provide mechanism to enable positioning techniques.

  - HRLLC

In NR, research on physical layer enhancements for URLLC was performed [5], with use cases mainly focus on industry and verticals. To achieve high reliability and low latency, NR supports numerous features from Rel-15 to Rel-19, such as CG-PUSCH, PDSCH/PUSCH mapping type B, low-SE CQI/MCS table, self-slot A/N feedback, processing capability 2, higher PDCCH aggregation level, PDSCH/PUSCH repetition, pre-emption/cancellation transmission, span-based PDCCH monitoring, intra-UE prioritization, SPS HARQ-ACK deferral, retransmission of cancelled HARQ-ACK, PUCCH cell switching, sub-band full duplex, etc.

For 6G, ITU has defined usage scenarios and overarching aspects of IMT-2030, including one vertex of the hexagon dedicated to HRLLC.

Meanwhile, in the endorsed SA1 new use cases for 6G [3], 25 use cases are captured for industry and verticals which are typical HRLLC scenarios. For example, 3D factory model based AR guided task operation, Spatial Computing enabled Dynamic Material Management, independent 6G Local Network for Factory, network slice for Industry IoT etc.. The communication requirements for high data rate, lower latency and higher reliability for multiple connected machines are needed. Beyond these, network resilience, cooperation with positioning, sensing, computing and AI capabilities are also important for industry and vertical scenarios.

Thus, it is critical to meet HRLLC service requirements from 6G day 1. The typical set of PHY techniques from NR can be the starting point. Scalable designs and different sets of mandatory features for different IoT requirements/device types are essential.

6G RAN design shall provide mechanisms to support HRLLC service, and consider combination with positioning, sensing, computing and AI capabilities

  - Security requirements for RAN

Overall security framework at RAN level strongly depends again on SA study and the outcome of this study. At RAN level, security for both UP and CP needs to be considered and lower layer security is important for 6G. Compared to 5G, it is likely that the main differences in security framework for 6G may come from the requirement to support lower layer security and also to support secure system information. These aspects need to be considered along with SA decisions for overall security framework and should be studied in conjunction with overall RAN/Core network architecture. So, a separate section should be added in the TR for security requirements at RAN level as proposed below.

RAN security requirements for 6G should include lower layer security and secure system information broadcast taking into consideration the overall network architecture in RAN and core.

  - Other services

As the basic service for the cellular network, the voice service over 6GR is considered as one essential to enable the widely deployment of 6G. To achieve it, the corresponding study with consideration on the traffic requirement, codec should be considered, e.g., to ensure the coverage.

Additionally, FWA service is also critical for 6G with clear commercial requirement(s). For extending these services in 6GR, exploring new spectrum, improving spectral efficiency/coverage and achieving native energy saving should be taken into account in the study.

6G RAN design shall provide mechanism to support others services, e.g., voice, FWA.

- TPs for suggested services

Based on the proposals in section 2 for the suggested services, the corresponding TPs are proposed in this section.

----------------------------------------------- Start of Text Proposals for TR 38.914 ---------------------------------------------------

5.4	Requirements of New and Existing Services

5.4.a AI services

- 6GR design should support AI services including:

- Cross-layer information exchange needs to be supported, including the assistance application layer information sharing to RAN directly or via CN.

- RAN should support a uniform mechanism to transfer AI related data/information to CN.

- RAN should support AI related data collection and management in an efficient way.

- RAN should support and guarantee the performance of intent-driven intelligent cooperation among devices.

5.4.b	Sensing services

The 6GR should enable, and improve if suitable, state-of-art sensing techniques, such as RAN-embedded (based on RAN wireless sensing signal) and RAN-external (e.g. Radar, Lidar, Camara, IEEE802.11 station, UWB, etc.).

The 6GR sensing shall exploit high bandwidth, massive antenna systems, network architecture/ functionalities. 6GR sensing shall support indoors/outdoors scenarios and horizontal/vertical dimensions.

6GR shall fulfill regulatory sensing requirements.

6GR shall at least support sensing use cases include the detection and tracking of UAV, Pedestrian, Vehicle, AGV and Ship, infrastructure collapse monitoring, and structural health monitoring.

6GR design targets for sensing service include:

1.	Support accuracy and latency as defined in TS22.137, and TR 22.870 for some use cases

3.	Reduced network complexity

3.	Reduced device cost

4.	Reduced device power consumption

5.	Efficient signalling over the air interface and in the network

6.	Support for hybrid sensing methods

7.	Scalability (support for large number of sensing objects)

8.	High security

9.	High availability

10. Sensing data collection, transmission, processing in an efficient and secured way

5.4.c	Location/Positioning services

The 6GR should enable positioning techniques.

The 6GR positioning shall exploit high bandwidth, massive antenna systems, network architecture/ functionalities and deployment of massive number of devices. 6GR positioning shall support indoors/outdoors usage scenarios and horizontal/vertical dimensions.

6GR shall fulfill regulatory positioning requirements.

6GR shall provide location/positioning services to meet the positioning requirements of the typical use cases in TR 22.870, considering integration sensing, positioning and communication.

5.4.d Immersive communication

6GR RAN design shall provide mechanism to support Immersive communication service.

6GR RAN design shall meet performance requirements for typical use cases on Immersive communication in TR 22.870.

6GR RAN shall provide mechanisms to combine Immersive Communication service with positioning, sensing, computing and AI capabilities to support Immersive type use cases in TR22.870.

5.4.e HRLLC

6GR RAN design shall support Hyper Reliable and Low Latency Communication services to meet performance requirements for typical use cases on Industry and Verticals in TR 22.870.

6GR RAN design shall support network resilience to enhance the redundancy and availability of HRLLC service.

6GR RAN shall provide mechanisms to combine HRLLC with positioning, sensing, computing and AI capabilities for Industry and Vertical scenarios in TR22.870.

5.4.f	Security and Privacy related requirements relevant for Radio Access

The RAN design shall ensure support for integrity and confidentiality protection of radio signalling messages, including messages between RAN and Core network nodes and lower layer signalling messages in RAN.

The RAN design shall ensure the ability to support integrity and confidentiality protection of user plane messages, including messages between RAN and Core network nodes, with the use of such security to be configurable during security set-up.

The RAN design shall ensure the ability to support integrity protection of system information messages.

The RAN design shall ensure support for the allocation and use of identities to provide user privacy, e.g. reduce the need for sending any permanent identities in the clear.

The RAN design shall ensure the efficient establishment of RAN security mechanisms.

The RAN design shall ensure resilience against jamming.

5.4.g Other services

6GR RAN design shall provide mechanism to support other services, e.g. voice and FWA.

----------------------------------------------- End of Text Proposals for TR 38.914 ---------------------------------------------------

- References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements  CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-251873 3GPP TR 38.914 V0.1.0 Study on 6G Scenarios and Requirements (Release 20)

- TR 22.870 (V0.3.1), Study on 6G Use Cases and Service Requirements

- RP-25xxxx Views on 6G ISAC use cases and requirements  ZTE Corporation, Turk Telekom, Verizon, China Telecom, China Unicom, Telecom Italia, CAICT, ETRI, Queens University Belfast, Pengcheng Laboratory, Shanghai Jiao Tong University, Sanechips, CATT

- TR 38.824, Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)

- TR 38.838, Study on XR (Extended Reality) evaluations for NR

- TR 22.856, Study on Localized Mobile Metaverse Services

- TR 22.261, Service requirements for the 5G system