3GPP TSG RAN meeting #109                                                 RP-252036

Beijing, China, Sep 15-18, 2025

Title: 	Views on Rel-20 RRM topics for 5G-A

Source: 	ZTE Corporation,Sanechips

Agenda item:	9.1.4.4

Document for:	Discussion

Introduction

In the RAN#108 meeting, there were lots of discussions on Rel-20 RAN4 package and the discussion outcomes are endorsed in the way forward on RAN4 R20 package [1]. In this contribution, we would like to share some further views on Rel-20 RRM topics for 5G-A.

Discussion

- 2.1. RRM enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ Rel-20 UE RRM proposals so far are summarized in the following table.

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - (e)RedCap UE enhancement - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI - This also include example bands, UE RF scope including potential complexity reduction, UE demod - Fast SCell activation - Further discussion on the possible down scoping in RAN#109 based on the following two objectives - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation - No RAN1/2 scope is expected - Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

In the last RAN meeting, it was agreed to have the down-selection for fast SCell activation, from our commercial deployment interest for fast CA and also from the workload perspective, we propose to keep the SCell activation delay reduction without CSI reporting;

Proposal 1: propose to keep the SCell activation delay reduction without CSI reporting with the detailed objective  as following

    - SCell activation delay reduction by scheduling SCell prior to valid CSI report for intra-band co-located CA, including SSB-less cases

      - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026.

- 2.1.3 WID objective

- Core part:

- (e)RedCap UE enhancement

  - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI

  - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 less than 5MHz WI

    - This also include example bands, UE RF scope including potential complexity reduction, UE demod

- Fast SCell activation

  - SCell activation delay reduction by scheduling Scell prior to valid CSI report for intra-band co-located CA, including SSB-less cases

    - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026.

- Perf part:

- (e)RedCap UE enhancement

  - To specify the test case for RLM/BFD relaxation for (e)Redcap UE

  - To specify the test case for RLM/BFD relaxation for Redcap UE with less than 5MHz;

  - To specify the demod requirement for Redcap UE with less than 5MHz;

- Fast SCell activation

  - To specify the test case for Fast Scell activation

Conclusions

In this contribution, we shared some further views on Rel-20 RRM enhancement based on RAN#108 moderator’s way forward.

Proposal 1: propose to keep the SCell activation delay reduction without CSI reporting with the detailed objective  as following

    - SCell activation delay reduction by scheduling SCell prior to valid CSI report for intra-band co-located CA, including SSB-less cases

      - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026.

Proposal 2: propose to rephrase the objective in the following way:

- Core part:

- (e)RedCap UE enhancement

  - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI

  - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 less than 5MHz WI

    - This also include example bands, UE RF scope including potential complexity reduction, UE demod

- Fast SCell activation

  - SCell activation delay reduction by scheduling Scell prior to valid CSI report for intra-band co-located CA, including SSB-less cases

    - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026.

- Perf part:

- (e)RedCap UE enhancement

  - To specify the test case for RLM/BFD relaxation for (e)Redcap UE

  - To specify the test case for RLM/BFD relaxation for Redcap UE with less than 5MHz;

  - To specify the demod requirement for Redcap UE with less than 5MHz;

- Fast SCell activation

  - To specify the test case for Fast Scell activation

References

[1] RP-251879,Way forward on RAN4 R20 package, RAN4 Chair (Apple), Approved.