3GPP TSG-RAN Meeting #109	RP-252053

Beijing, China, 15 – 18 September 2025

Agenda Item:	8.2.1.3

Source:	NEC

Title:	Requirements for 5G-6G Multi-RAT Spectrum Sharing for migration

Document for:	Discussion and Decision

1	Introduction

The study for migration in 6G stems from the discussions in RAN #108 meeting [1], where requirements were laid out for 5G-6G migration in the following objectives:

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

(7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]
NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Given these objectives from the previous plenary discussion, a number of aspects need to be considered for the 6G study.

2	Discussion

To reduce the CAPEX costs of 6G for operators, MRSS needs to support several key functionalities:

The first of these functionalities is an open interface that allows for a standardized solution to enable inter-vendor operations between 5G and 6G for MRSS. This allows operators to maintain flexible vendor selection as they transition to the next generation of communications, as opposed to being locked into a single vendor for the next generation of radio equipment. MRSS is meant to be a baseline feature of 6G and should be designed in a way that allows for flexible deployment for the operators. This would require the 5G network to provide the necessary signals and resource allocations, including, but not limited to: RS resource allocation, RACH information, uplink/downlink resource allocation.

Proposal 1: MRSS to support an open interface between 5G and 6G to allow for inter-vendor collaboration across communications generations.

The second of these functionalities is the ability to change allocations of 5G and 6G resources within the network. In DSS, a problem that occurred during real-world deployment is that one generation was allocated too few resources. This resulted in service outages that affected the customers of the said operator in the generation with too little resources. To mitigate this problem for MRSS, requirements must be set to allow for the ease of resource reconfiguration between the generations. While the exact speed at which the resource allocation between the two radio types can be up to further discussion within the working groups, plenary should reach a common understanding that this flexibility is required to have successful MRSS deployment.

Proposal 2: MRSS to support flexible resource allocation between generations 5G and 6G.

The third functionality is a simplified multiplexing scheme for MRSS. While the current proposals in RAN1 have been focused on puncturing/rate matching around 5G resources when the network is operating in 6G, this is a very archaic approach to solving the MRSS issues. The original DSS has rate matching/puncturing for 4G signals because 4G signals were configured for specific locations within the resource grid that could not be removed. This is no longer the case for 5G, as the resources can be allocated to any location within the resource grid. This flexibility, afforded to us by 5G, can lead the way for simplified methods of 5G or 6G deployment where the 5G and 6G networks can operate in an FDM or TDM mode. This allows for simplified deployment where 5G and 6G may use their own dedicated resources that do not need to be punctured or rate matched around the specific signals of the other generation.

Proposal 3: MRSS to support multiplexing types that are easy to implement, like FDM or TDM.

The fourth functionality is a flexible yet separate configuration of a 6G TDD pattern with respect to 5G. Any sort of MRSS needs to take into account the preexisting 5G allocations, particularly around existing TDD patterns of TDD 5G networks. A method needs to be established in which the existing TDD pattern of the NR is not to be disturbed, while allowing 6G flexibility to change its uplink and downlink scheduling to suit its needs. Given the growth in the streaming industry within the last few years, the current resources dedicated to uplink may not be sufficient for the designs of tomorrow. A common understanding for 6G may be that not all operators will want new spectrum for 6G to save on CAPEX. This will lead to the 6G enhanced uplink being used in the same spectrum as legacy downlink heavy NR designs. MRSS will need to identify solutions that will allow the TDD pattern of MRSS to be the same as legacy NR while allowing additional uplinks to be used for 6G as needed. Some methods that can be used to solve this issue include dynamic TDD and SBFD.

Proposal 4: MRSS to support separate 5G and 6G TDD patterns to fit the traffic type needs of each radio generation.

3	Conclusion

In this paper, we give our overview of requirements for architecture and migration:

Proposal 1: MRSS to support an open interface between 5G and 6G to allow for inter-vendor collaboration across communications generations.

Proposal 2: MRSS to support flexible resource allocation between generations 5G and 6G.

Proposal 3: MRSS to support multiplexing types that are easy to implement, like FDM or TDM.

Proposal 4: MRSS to support separate 5G and 6G TDD patterns to fit the traffic type needs of each radio generation.

References

[1]	RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108, June 9-13, 2025.