3GPP TSG-RAN Meeting #109	RP-252055

Beijing, China, 15 – 18 September 2025

Agenda Item:	8.2.2

Source:	NEC

Title:	Study on 6G radio

Document for:	Discussion and Decision

1	Introduction

During RAN1's first meeting for 6G (RAN1 #122) in Bengaluru, the chairman’s notes [1] captured several points that are to be discussed in plenary or to study and identify the lessons learned from NR.

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Identify the high-level aspects which impact on the NR-6GR MRSS support

  - Including the lessons learned from LTE-NR DSS

Agreement

- Study and identify the lessons learned from NR BWP framework

Agreement

- Study and identify the lessons learned from NR spectrum utilization and aggregation framework

  - DC is subject to RANP decision in June 2026

  - Note: MRSS aspects are separate discussion

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

Agreement

- For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

This document presents our views and discussions regarding these topics.

2	Discussion

2.1	MRSS

In 4G-5G deployment of DSS, a major issue during deployment was the lack of flexibility for DSS to reconfigure its associated resources between the generations of communications technology. This has led to service outages when an operator used this DSS technology, and the network was unable to allocate sufficient resources for one radio technology.

Proposal 1: MRSS to support flexible resource allocation between generations 5G and 6G.

For additional discussions on MRSS, please refer to our Tdoc RP-252053.

2.2	Spectrum utilization and aggregation

Coming out of 5G Carrier Aggregation, a number of issues and pain points need to be addressed to allow for a smooth transition for such an important feature.

The first issue that comes with the current CA framework is the multiple options for similar functions. Supplementary Uplink was introduced in NR to help solve the issue of insufficient uplink. The idea of this feature is good, but it offers many of the same functionalities as carrier aggregation (albeit without the ability for additional downlink). Supplementary Uplink and CA share many of the same purposes, so there should be no need to support two separate functions going into 6G. In the spirit of simplification of features, the SUL functionalities can be incorporated into the CA framework for 6G.

On the matter of SUL and CA, another issue that was introduced was the additional BW combinations for SUL. This was already on top of the large amount of band combinations for CA, which had made implementation a nightmare as the tables needed to be stored within the UE. A simplified CA feature that incorporates SUL characteristics may also be beneficial in reducing total band combinations.

Proposal 2: Overlapping features within CA framework should be condensed into a unified framework that limits band combination complexity.

Another issue with the current CA framework would be the idea of PCells. The original intention of PCells is to offer a cell that is in charge of controlling the CA by allowing for the broadcast of control and PRACH information to be located within the same cell. This can lead to an issue where the PCell is overburdened with control signaling of all the associated SCells. To remedy this issue, CA should consider giving control back to SCells to configure its own control or PRACH signals.

Proposal 3: Reduce the burden on PCell by offloading some control functionalities to SCell.

A third problem lies in the independent configurations of the component carriers within the CA framework. The current CA framework with two Component Carriers has independent scheduling of PDSCH and PDCCH. While there has been work for scheduling multiple CC with the same DCI, the limitation is that the PDCCH is always within the same CC. Enhancements can be made so that any CC can be used as the CC that carries the PDCCH for multiple cells and to reduce further the overburdening of the primary cell.

Proposal 4: Reduce the burden on PCell by blurring the lines between SCell and PCell so that any cell can serve as the “PCell” of the aggregated carriers.

2.2	Duplexing

The NR design of duplexing revolved around semi-static configurations of SubBand Full Duplex. This scope has an area of issue and an area of improvement that can be explored for 6G.

The issue of the current framework for SBFD lies in the fact that it is not really semi-static in the current specifications. There have not been sufficient specifications regarding the issues of configured grants and what to do with them after the semi-static reconfiguration of the SBFD BW that leaves the uplink out of the SBFD uplink bandwidth. Traditionally, these have been considered a dynamic SBFD problem, but these issues also persist within semi-static configurations of SBFD. It is essential to address these issues for the deployment of SBFD, as the SBFD Uplink may be configured to be either too large or too small, potentially causing service disruptions.

Proposal 5: Study methods to ensure the functionality of SBFD devices after the BW has been reconfigured.

The proposed improvement for 6G SBFD is Overlapped Subband Full Duplex, or simply Full Duplex. At the gNB side, this can be explored within the study item of 6G as a step forward for greater spectrum efficiency and uplink coverage. It is understandable that some have doubts concerning the CLI issues that are associated with full duplex, so this can serve as a stepping stone to study its feasibility going into 6G. If the study deems that OSFD cannot be done under the current constraints, then it can be left out of the 6G WID.

Proposal 6: Study gNB side overlapped full duplex for 6G.

2.4	Harmonization of Terrestrial and Non-Terrestrial Networks

The support for NTN in 5G relies on numerous add-on features that are irrelevant to TN, such as ephemeris data and GNSS-based initial access. This piecemeal approach to NTN integration has hindered its commercialization throughout the 5G evolution.

For 6G, a fundamental harmonization between TN and NTN is essential. The core physical layer design should be unified from the outset to ensure rapid commercial deployment without the need for costly, low-level hardware redesigns.

To achieve this harmonization without sacrificing performance or time-to-market, NTN characteristics must be embedded into every agenda of the 6G standardization process. Key areas requiring integrated design include frame structure, waveform, channel coding, modulation, energy efficiency, AI/ML, initial access, MIMO, physical layer control, data scheduling, HARQ operation, and duplexing.

Proposal 7: All 6G study agendas should explicitly consider a checklist of NTN characteristics to confirm performance enhancements and to help foster the commercial success of 6G in both terrestrial and non-terrestrial domains.

3	Conclusion

In this paper, we provided our views on 6G radio from the RAN1 perspective.

Proposal 1: MRSS to support flexible resource allocation between generations 5G and 6G.

Proposal 2: Overlapping features within CA framework should be condensed into a unified framework that limits band combination complexity.

Proposal 3: Reduce the burden on PCell by offloading some control functionalities to SCell.

Proposal 4: Reduce the burden on PCell by blurring the lines between SCell and PCell so that any cell can serve as the “PCell” of the aggregated carriers.

Proposal 5: Study methods to ensure the functionality of SBFD devices after the BW has been reconfigured.

Proposal 6: Study gNB side overlapped full duplex for 6G.

Proposal 7: All 6G study agendas should explicitly consider a checklist of NTN characteristics to confirm performance enhancements and to help foster the commercial success of 6G in both terrestrial and non-terrestrial domains.

References

[1] Chair notes RAN1#122 v13

[2] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[3] R1-2506618, FL summary#3 on overview of 6GR air interface, NTT DOCOMO (Moderator)