3GPP TSG-RAN Meeting #109	RP-252057

Beijing, China, 15 – 18 September 2025

Agenda item:		9.2.2	Study on GNSS resilient NR-NTN operation

Title:		Discussion on GNSS resilient NR-NTN operation

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#108, the following objectives in the SID have been approved [1]:

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

In this contribution, we further discuss the direction of this SID based on the recent RAN1#122 discussion [2].

# 2. Discussion

The objective of the SID emphasizes that “This study will initially focus on 1/ (where GNSS information is temporarily unavailable)”.

From the UE's perspective, two situations may occur:

- The UE acquires GNSS information at time T0, before subsequent updates are lost.

- The UE regains GNSS information at time T1, after a period during which no updated GNSS information was available.

T0

T1

Thus, the duration of temporary GNSS unavailability can be defined as T1-T0.

Based on the justification, such unavailability may result from the following causes:

- Potential occurrence of GNSS jamming:

  - The duration of jamming is beyond the control of both the UE and the network.

- Potential occurrence of GNSS spoofing:

  - The duration of spoofing is beyond the control of both the UE and the network.

- Potential occurrence of other effects (e.g. poor geometry, local blockages, solar storm activity or simply increased GNSS measurement period to save UE power):

  - A UE may mitigate poor geometry or local blockages through movement or antenna adjustment; however, the duration of GNSS unavailability depends on the physical environment and UE velocity, which can only be partially influenced by the UE.

  - The duration of a solar storm is beyond the control of both the UE and the network.

  - The GNSS measurement period can be controlled by either the UE or the network.

- UE possibly being in an adverse environment where NTN access is possible but GNSS signal is too weak, due to the difference in link budget margin between both systems:

  - The link budget of the GNSS system is constrained by the non-3GPP ecosystem; therefore, the duration of GNSS unavailability cannot be controlled by either the UE or the network.

Observation 1: From the cause perspective, only under the intentionally increased GNSS measurement period circumstance, the GNSS unavailable duration can be clearly defined. If the study is limited to quantifiable temporary unavailability, it would effectively address only this specific scenario.

If we want to solve all the GNSS-unavailable scenarios, there is no proper way to model the temporarily unavailable duration within a pre-assumed value.

Observation 2: Assuming a predefined duration of GNSS unavailability allows the study to cover only one of many possible GNSS-unavailability scenarios in the justification of the SID.

Proposal 1: The study on GNSS resilience should prioritize solving substantive problems related to GNSS unavailability when dealing with temporary unavailability.

Proposal 2: Adopt the following TP for the SID.

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario - Note: This study should try to provide resilient solutions to all the occurrences described in the justification. Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

# 3. Conclusion

In this contribution, we further discuss the direction of this study on GNSS resilience based on the recent RAN1#122 discussion and have the following observations and proposals.

Observation 1: From the cause perspective, only under the intentionally increased GNSS measurement period circumstance, the GNSS unavailable duration can be clearly defined. If the study is limited to quantifiable temporary unavailability, it would effectively address only this specific scenario.

Observation 2: Assuming a predefined duration of GNSS unavailability allows the study to cover only one of many possible GNSS-unavailability scenarios in the justification of the SID.

Proposal 1: The study on GNSS resilience should prioritize solving substantive problems related to GNSS unavailability when dealing with temporary unavailability.

Proposal 2: Adopt the following TP for the SID.

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario - Note: This study should try to provide resilient solutions to all the occurrences described in the justification. Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

# References

[1] RP-251863	New SID: Study GNSS resilient NR-NTN operation, June 9–13, 2025

[2] Chairman Note, RAN1#122

# Relevant agreements from the reference [2]

Agreement

For the study on GNSS resilient operation, scenarios should be considered where:

- Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

- Scenario 2: There is a previously acquired GNSS based position

  - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

- Note: Two scenarios above belong to description as in SID.

Agreement:

For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes:

| Satellite orbit | GSO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Satellite scenario parameters | At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered. |  |  |
| Beam size (note 3) | At least values captured in Table 6.1.1.1-1/2 are considered, |  |  |
| Minimum elevation angle | 30° (LEO), 12.5° (GSO) |  |  |
| Frequency ranges/bands | FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz) |  |  |
| UE type | Handheld (L/S band), VSAT (Ku/Ka) |  |  |
| UE speed (note 1) | L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2) |  |  |
| Beam/cell type (note 4) | (Quasi)-Earth-fixed beams/cells and Earth-moving |  |  |
| Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies. |  |  |  |

Note: The cyan highlighted parameters are prioritized for evaluation purposes.

Agreement:

For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area

- Case a: the location uncertainty area is the area served by the cell or beam.

- Case b: the location uncertainty area is a circle of radius X km

  - X to be reported

UE altitude error should be taken into account.

- UE altitude error to be reported.

For both Case a and Case b above, RAN1 to evaluate at least:

- Differential delay values

- Differential frequency offset/Doppler values