3GPP TSG RAN Meeting #109								RP-252075

Beijing, China, September 15-18, 2025

Agenda Item:		9.6.4.7

Source:			Samsung, Xiaomi

Title:				Summary for WI Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

WI code(s):			NR_IoT_NTN_req_test_enh

leading WG:		RAN4

Release:			Rel-19

1	Introduction

The Rel-19 WI of Enhanced requirements and conductive test methodology for NR NTN and IoT NTN is targeted on two main aspects for further enhancement on NR-NTN and IoT-NTN in RAN4. One is to enable both NR-NTN and IoT-NTN high power UE (HPUE) to transmit with higher power in FR1-NTN bands, by conducting and concluding co-existence studies and specifying UE RF requirements. The other one is to enable NR-NTN to operate with less than 5MHz channel bandwidth, by specifying corresponding system parameters, UE RF requirements, RRM requirements and satellite-access node (SAN) requirements for these channel bandwidths.

2	Description

Core: High power UE (HPUE) for NTN

In this WI, in order to protect the operators in adjacent channel from this HPUE operation, the co-existence studies were performed and concluded for between NR-NTN/IoT-NTN and NR-TN network for the UEs transmitting in 26dBm for Power class (PC) 2, 29dBm for PC1.5 and 31dBm in PC1 in FR1-NTN bands. The co-ex studies were performed by system level simulations from different sourcing companies, and the simulations followed the same methodologies and deployment assumptions as Rel-17 NR-NTN co-ex studies in same FR1-NTN bands.

With the observations and conclusions from the co-ex studies, after discussion, the adjacent channel leakage ratio (ACLR) for NR-NTN HPUE were concluded as 31dBc and 34dBc for PC2 and PC1 respectively; and ACLR for NB-IoT based IoT-NTN were concluded as 37 dBc for both PC2 and PC1; and ALCR for eMTC based IoT-NTN was concluded as 31 dBc for PC2.

For the NR-NTN HPUE RF requirements, the power class 2 and power class 1 for NR-NTN band n256 and n255 are introduced. For the IoT-NTN HPUE RF requirements, the power class 2 and power class 1 for NB-IoT based IoT-NTN band 256 and 255 are introduced; and power class 2 for eMTC based IoT-NTN band 256 and 255 are introduced in this release. The relevant HPUE requirements for these power classes and bands are specified, including maximum output power (MOP) and associated tolerances, maximum output power reduction (MPR) to meet emission requirements and ACLR for these power classes; additional maximum output power (A-MPR) to meet additional emission requirements and reference sensitivity degradation (RSD) for these newly introduced bands.

Core: Less than 5MHz for NTN

In this WI, in order to enable less than 5MHz channel bandwidth operations for NR-NTN, the system parameters, UE RF requirements, RRM requirements and satellite-access node (SAN) requirements are enhanced from existing FR1-NTN requirements.

For the system parameters, 3MHz transmission bandwidth is introduced to the maximum transmission bandwidth configuration; minimum guard band is specified to 3MHz UE channel bandwidth; GSCN and sync raster are enhanced with 3MHz channel bandwidth; and this UE channel bandwidth 3MHz is introduced to FR1-NTN bands n256, n255 and n254 in this release.

For UE RF requirements, the enhancements are made to almost all Tx and Rx requirements that related to UE channel bandwidth. The impacts include transmitter power, output power dynamics, output RF spectrum emissions in Tx parts, and reference sensitivity, maximum input level, adjacent channel selectivity, blocking characteristics and spurious emissions in Rx parts. And the changes are made to introduce relevant requirements to 3MHz channel bandwidth.

For RRM requirements, the enhancements are made to handover, conditional handover, satellite switching with re-sync, link recovery, intra-frequency measurements, inter-frequency measurements procedures. The key enhancements of these changes are made to incorporate the different SSB for less than 5MHz channel bandwidth.

For SAN RF requirements, the enhancements are also made to introduce missing requirements for 3MHz channel bandwidths in most of Tx and Rx requirements. The impacts include the reference sensitivity level, dynamic range, in-band sensitivity and blocking, in-channel selectivity, OTA output power dynamics, OTA transmitted signal quality, OTA unwanted emissions, OTA sensitivity, OTA reference sensitivity level, OTA dynamic range, OTA in-band selectivity and blocking, OTA in-channel selectivity and associated reference channels for Rx requirements and dynamic ranges.

Performance parts:

This WI also includes the RF, RRM, demodulation performance requirements work by introducing new NGSO channel model, and also the SAN conformance and RRM conformance work by introducing less than 5MHz channel bandwidth. These works are still under discussion, and will be further completed in the performance phase of this WI.

3	References

[1] RP-252076, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

[2] RP-252074, Status Report to TSG: Enhanced requirements and conductive test methodology for NR NTN and IoT NTN