3GPP TSG RAN Meeting #109		                                      RP-252094

Beijing, China, September 15 – 18, 2025

Source:	CMCC

Title:	Requirements for 6G Network Topology

Agenda Item:	8.2.1.3

Document for:	Discussion

Introduction

In this contribution, we provide overviews on 6G network topology with network-controlled intermediate nodes, such as repeaters, reconfigurable intelligent surfaces (RIS), to improve the coverage or capacity. The coverage performance of RIS-aided network is also evaluated via system level simulations.

Network Topology

Key Performance Requirement

Coverage has always been a fundamental performance indicator in wireless networks, directly impacting user experience, service availability, and network capacity. With the introduction of 5G, the use of higher frequency bands—such as millimeter-wave (mmWave) spectra—has become essential to achieve multi-Gbps data rates and ultra-low latency. However, these high-frequency signals suffer from inherent propagation limitations, including severe path loss, sensitivity to blockages, and lack of diffraction. As a result, achieving seamless coverage in 5G networks, especially in indoor environments, urban canyons, and rural areas, remains challenging.

As the industry looks toward 6G, coverage requirements will become even more pressing due to the expected use of higher spectrum ranges. These bands offer enormous bandwidth but would also face more severe propagation losses. As captured in 6G SID [1], it is required to support re-using existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. That means even in the band around 7GHz, coverage would face some issues. Moreover, 6G networks are envisioned to support ubiquitous connectivity in three-dimensional space—including aerial, maritime, and deep indoor environments—while also integrating sensing and communication capabilities. Such expanded scope necessitates more efficient, scalable, and intelligent coverage extension mechanisms. In addition to the spectral expansion, 6G systems are expected to exhibit significantly higher energy efficiency and reduced deployment costs. Sustainable network growth will require solutions that minimize the power consumption while maximizing the reuse of existing infrastructure.

Network Topology with Evolved Nodes

To achieve seamless coverage while simultaneously optimizing for both energy efficiency and spectral efficiency, the existing heterogeneous network (HetNet) topology can be continued and evolved. This architecture moves beyond the traditional homogeneous network of macro-cell towers, e.g., to strategically deploy a multi-layered ecosystem of cells of varying sizes and capabilities. This typically includes a basic layer of high-power/high tower macro-cells for wide-area coverage, which is then underlaid with a dense layer of low-power nodes such as micro, pico, femto, distributed antenna systems (DAS), to boost capacity in high-demand hotspots.

Figure 1 Heterogeneous network (HetNet) topology [1]

The micro layer of nodes within the HetNet architecture can be significantly enriched and expanded beyond traditional small cells. This layer can now incorporate a sophisticated array of network-controlled nodes. Key examples include network-controlled repeaters (NCR), which offer a managed solution for extending coverage and enhancing signal quality in challenging areas like outdoor to indoor coverage or at cell edges [2]. Furthermore, the integration of passive nodes, such as reconfigurable intelligent surfaces (RIS), presents a revolutionary approach. These programmable surfaces can dynamically manipulate the wireless environment by intelligently reflecting and directing electromagnetic waves towards desired users, thereby boosting the cell coverage and spectral efficiency without generating any active transmission signals or consuming significant amount of power. The strategic deployment of these advanced nodes creates a more intelligent, flexible, and highly efficient radio access network under the overarching macro-cell layer.

Figure 2 Cooperative transmission with the reflection array of RIS

- To provide flexible and efficient coverage, 6G network topology requires even more adaptive and sustainable solutions.

- 6G networks can effectively reuse and expand the existing multi-layer HetNet topology. To holistically balance seamless coverage and deployment costs, the micro layer should prioritize intelligently orchestrated, easily deployable nodes.

Coverage Performance Evaluation

This section presents the impact of different RIS deployment strategies on the coverage performance and the near-field effect of RIS-UE links. More details of the simulation set up, parameters and results can be referred to [3].

Impact of RIS deployments

Figure 3 shows the cumulative distribution functions (CDF) of the RSRP. The UE and RIS panels are distributed near the cell edges, with the consideration that the RSRP is a coverage-oriented metric and that it is likely that the mobiles near the cell edges would have coverage issue. When each RIS panel has 16×16 elements, the beam-width of the main lobe is about 7.4º without special codebooks, which is wide enough to cover the area served by the RIS panel. A heat map illustrating the geo-distribution of the RSRP gains within the considered 7-cell (21-sector) network is provided, which shows the coverage area of each RIS panel as well. The red dots on the map denote the locations of the RIS panels. The small blue clusters represent areas where the deployment of RIS panels results in RSRP gains compared to the baseline system (without RIS).

Figure 3 RSRP gain heatmap (left); CDF of the RSRP (right)

Seven CDF curves of the SINR are illustrated in Figure 4, as a function of the locations, and number and size of the RIS panels. As for the setting with 24 RIS panels each having 16×16 elements per sector (solid green line for a uniform distribution of RIS panels and dashed magenta line for cell edge RIS panels) and 4 RIS panels each having 40×40 elements (solid red line for a uniform distribution of RIS panels and dashed cyan line for cell edge RIS), the total number of RIS elements per sector is very close, i.e., 6144 vs. 6400. It is observed that, in both cases of uniform distribution of RIS panels and cell-edge deployment of RIS panels, deploying fewer RIS panels per sector of larger size is more preferrable than deploying more RIS panels per sector of smaller size. The SINR performance of uniform and cell edge RIS deployments is not very different, except that the SINR spreads more widely when the RIS panels are uniformly distributed. As the number of 40×40 RIS panels per sector is increased from 4 to 8 (solid blue line for cell edge RIS and dashed blue line for uniform distributed RIS), the gain in terms of average SINR is increased from 4 dB to 7 dB.

Figure 4 CDF of the SINR

- The effectiveness of an RIS panel is fundamentally determined by the total aperture or elements of the panel. Either deployments with more panels of smaller size or less panels of larger size can be dependent on the network layout and propagation characteristics, etc.

- In consideration of typical 3GPP setting, deployments with large size RIS panels but with limited node number provide better SINR performance, compared with small size RIS but larger node numbers. It means that the RIS with large number elements or larger size should be focused in future studies.

Near-field analysis

Figure 5 shows the CDFs of the distance between an RIS panel and the served UEs. The RIS panels are deployed either at the cell edges (solid blue line for 16×16 RIS and solid red line for 40×40 RIS) or uniformly distributed (dashed blue line for 16×16 RIS and dashed red line for 40×40 RIS). The UEs are uniformly distributed. The criterion utilized for the RIS-UE pairing is that the path gain of the BS-RIS-UE cascaded link is no less than 3 dB than the path gain of the BS-UE direct link. When 16×16 element RIS panels are deployed, the aperture is 1.0 m. The corresponding Rayleigh distance is roughly 19 m (dashed black line), which is much shorter than the typical distance between a UE and its serving RIS panel as seen in Figure 5. In another words, the propagation occurs predominantly in the far field. When 40×40 element RIS panels are deployed, however, the aperture becomes 2.6 m. The corresponding Rayleigh distance is 118 m (dashed magenta line). According to Figure 5 there is roughly 50% of probability that a UE would be within 118 m from its serving RIS panel, i.e., the propagation occurs in the near field.

Figure 5 CDFs of the distance between a RIS panel and its served UE

- As the aperture of the RIS panel increases, near field propagation becomes prevailing in the RIS-UE link.

- The codebooks utilized for far field propagation would be sub-optimal for the RIS-UE link. The new codebook which utilizes the near field characteristics can be considered, including the phase shifts of the RIS elements, channel state information (CSI) estimation and feedback.

Conclusion

In this contribution, we provide overviews on 6G network topology with network-controlled intermediate nodes, such as repeaters, reconfigurable intelligent surfaces (RIS), to improve the coverage or capacity. The observations and proposals are as follows.

- To provide flexible and efficient coverage, 6G network topology requires even more adaptive and sustainable solutions.

- The effectiveness of an RIS panel is fundamentally determined by the total aperture or elements of the panel. Either deployments with more panels of smaller size or less panels of larger size can be dependent on the network layout and propagation characteristics, etc.

- As the aperture of the RIS panel increases, near field propagation becomes prevailing in the RIS-UE link.

- 6G networks can effectively reuse and expand the existing multi-layer HetNet topology. To holistically balance seamless coverage and deployment costs, the micro layer should prioritize intelligently orchestrated, easily deployable nodes.

- In consideration of typical 3GPP setting, deployments with large size RIS panels but with limited node number provide better SINR performance, compared with small size RIS but larger node numbers. It means that the RIS with large number elements or larger size should be focused in future studies.

- The codebooks utilized for far field propagation would be sub-optimal for the RIS-UE link. The new codebook which utilizes the near field characteristics can be considered, including the phase shifts of the RIS elements, channel state information (CSI) estimation and feedback.

References

- RP-251881, New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- 3GPP TS 38.300 V18.6.0, “NR; NR and NG-RAN Overall Description (Release 18)”, Jul., 2025.

- Y. Yuan, Y. Huang, X. Su, B. Duan, N. Hu and M. D. Renzo, "Reconfigurable intelligent surface system level simulations for industry standards," in IEEE Communications Magazine, doi: 10.1109/MCOM.005.2400500.