3GPP TSG RAN Meeting #109                                RP-252095

Beijing, China, September 15-18, 2025

Source: 	CMCC

Title:	Requirements for massive communication in 6G

Agenda item:	8.2.1.4

Document for:	Discussion & Decision

Introduction

Through 4G and 5G, multiple 3GPP cellular IoT networks have been built for divergent services and scenarios, each of which supports a device type with specific capability. Those 3GPP IoT technologies are summarized in the following table, from the perspective of typical use cases and key technical parameters (e.g., peak data rate, latency, coverage, and supported channel bandwidth).

Table 1. Summary of IoT technologies in 3GPP

| Technology | Characteristics | Typical use cases | Peak data rate | Latency | Coverage | Supported channel bandwidth | Rx power consumption |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT | Ultra-low device power consumption to support battery-less operation | Logistics, assets tracking, battery-less or disposable sensor | 10s/100s Kbps | ≤10 seconds | Indoor for Device 1, Similar to MBB @ outdoor for Device C | 100s kHz | 1 μW for Device 1, ~1 mW for Device C |
| NB-IoT | Low UE power consumption / cost, enhanced coverage | Metering, sensor | A few 100 Kbps |  | 20 dB higher than MBB |  | A few 10 mW |
| eMTC |  |  | ~1 Mbps |  | 15 dB higher than MBB | A few MHz |  |
| eRedCap | Reduced UE power consumption / cost | Wearables, industrial sensor, controller | 5~10 Mbps | <1 second | Same as MBB |  | ~100 mW |
| LTE Cat.1/1bis |  |  |  |  |  | 20 MHz |  |
| LTE Cat.4 | Reduced UE power consumption / cost | Standard/High-definition video monitoring | 50~150 Mbps |  |  |  |  |
| RedCap |  |  |  |  |  |  |  |
| Industry IoT, including URLLC | Ultra-high data rate | Super-definition video monitoring, AR/VR | 1~10 Gbps | 10s milli-second |  | 100 MHz | 100s mW |
|  | 10-6 reliability, milli-second level latency | Industrial fieldbus | - | Milli-seconds |  |  |  |

For the technical study on 6G radio (6GR), one objective is to use a common design to “meet mobile broadband service requirements as high priority, to also meet vertical needs”. Massive communication is an important part of the vertical needs for 6G. However, its target services and requirements are still unclear. A proper scope for massive communication is important for not only the commercial success of 6G, but also the feasibility of the objective of common design for various device types in 6GR.

According to the objective of “develop 3GPP requirements for 6G Radio for improvement of existing services and for new services” [1], this contribution discusses and provides input on the requirements for massive communication in 6G.

Discussion

Minimum requirements for 6G Massive Communication

Long battery-life for IoT UE

Actually, the life-cycle cost of a cellular IoT UE can highly depend on its battery-life. A large number of IoT UEs are deployed to operate for several years or even longer. For example, the lifetime of an electricity or water meter is usually required to be no shorter than five years, during which battery replacement is probably needed.

As the ordinary coin-cell or AA-battery can only support a IoT UE with Rx/Tx power consumption of 10s/100s mW to work for months or few years, dedicated high-capacity (e.g., several Ampere-Hour) battery is usually used. However, such battery is usually expensive, whose cost reaches a few dollars, which is similar to the low-end cellular IoT module itself. Considering the additional labour cost for battery replacement, the maintenance cost dominates the life-cycle cost of those IoT UEs.

Based on the above, it is desirable to support at least one type of low-end IoT UE with sufficiently low power consumption for long battery-life in 6GR. Referring to the existing 3GPP LPWA technologies, such as NB-IoT and eMTC, the following aspects paly important role in achieving the target long battery-life.

- The required minimum number of UE Tx/Rx antenna determines not only the number of RF chain, but also the baseband processing complexity. It further impacts the UE power consumption heavily. An antenna number of 1 is recommended for both UE transmitter and receiver.

- The clock frequency of baseband processing depends on the supported channel bandwidth and peak data rate. The sampling clock frequency is proportional to the channel bandwidth, considering proper oversampling rate for good link performance. Furthermore, the clock frequency of a certain blocks (e.g., encoder and decoder) is determined by peak data rate. To achieve comparable power consumption to the existing LPWA technologies, a maximum channel bandwidth of e.g. 5 MHz and peak data rate of e.g. a few Mbps is suggested.

- Energy saving mechanism significantly reduce the average power consumption by allowing IoT UE to enter idle or sleep mode, which consumes only milli-watt or even micro-watt power. The mature mechanisms in the existing LPWA technologies can be referred to or reused for at least the lowest-tier UE in 6GR. It is also not precluded to introduce other new techniques, such as the LP-WUS feature studied and specified in 5G-A.

To cover the majority of the IoT markets, IoT UE with long battery-life should be supported in 6GR. The battery-life and power consumption of existing LPWA technologies (e.g., NB-IoT and eMTC) can be referred to for the corresponding requirements.

Coverage enhancement for cellular IoT

The deployment of IoT UE is distinct from cell-phone or FWA UE. The distinction makes coverage enhancement as an essential requirement for low-end cellular IoT technologies.

First, a large portion of low-end IoT UEs, such as electricity or water meters, are usually deployed at indoor corner or even basement. The penetration loss significantly increases the propagation loss of the wireless link. Furthermore, the required service availability or coverage rate can be as high as 99% for enterprise IoT applications, which is much higher than the typical value of e.g. 95% for MBB service.

Second, most low-end IoT UEs are deployed with restrictions on both form factor and installation location. It is not only impractical to use larger antenna for an IoT UE, but also hard to flexibly adjust its antenna for better link performance. In other words, more link budget margin is needed for massive communication than normal MBB service.

Third, IoT UEs are widely deployed all over both urban and rural areas. It is uneconomic to deploy dedicated relay nodes to cover the large number of scattered blind spots. To enhance the direct link between base station and IoT UE is desired.

To some extent massive communication relies on ubiquitous coverage more than normal MBB service. It is important to consider the coverage enhancement requirement at the very beginning of 6GR design.

Latency, mobility, connection density

Except the requirements on UE battery-life and coverage, the other important requirements include the following aspects.

- Latency: Comparing to MBB service, the latency requirement can be relaxed for a large portion of use cases for massive communication, especially those low-end UE can be applied to. The relaxed latency requirement not only allows low-end IoT UE to use lower clock frequency for reduced power consumption, but also enables coverage enhancement by repetitions or very low code rate. For the services using the lowest-tier IoT UE, a maximum end-to-end latency of e.g. 10 seconds should be acceptable.

- Mobility: the lowest-tier 6GR UE can be used by some wearable device for the sake of long battery-life. Voice communication is usually an important function for those wearable devices. Consequently, a certain handover scheme needs to be supported for even the lowest-tier 6GR UE.

- Connection density: The corresponding requirement proposed in [2] need to be satisfied.

Considerations on the evolution of LPWA technologies

Among the 3GPP IoT technologies listed in Table 1, the two LPWA technologies (NB-IoT and eMTC) were specified in 4G, which have been deployed since ~10 years ago. Actually, only these two technologies clearly target at a UE battery-life of 10 years and coverage enhancement of 10+ dB comparing to normal MBB service. Considering regular technology iteration and the potential refarming of 4G networks in the 6G era, 6GR should support a device type with similar battery-life and cost to the existing LPWA technologies from Day 1, to avoid introducing an independent RAT for LPWA IoT in a later stage.

Text proposal

## 5.4.x	Minimum requirements for Massive Communication

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication:

- Massive Communication shall be supported in FR1. UE half-duplex FDD operation is supported in FDD band at least for the lowest-tier UE.

- The minimum number of UE receive antennas is 1.

- The required UE bandwidth for the lowest-tier UE is [5] MHz. The UE can support operation in a carrier bandwidth different from the UE bandwidth.

- The minimum PHY peak data rate is [1~2] Mbps in DL and [1~2] Mbps in UL.

- Battery-life of [10] years is supported at least for the lowest-tier UE, with the traffic model of e.g. hour-level periodical report.

- Coverage enhancement of [10] dB is supported at least for the lowest-tier UE, comparing to immersive communication.

- The end-to-end latency is relaxed to e.g. [10] seconds for the lowest-tier UE.

- Handover is supported by all the 6GR UEs for voice communication.

Conclusions

In this contribution we propose adding the text proposal in section 3 to TR 38.914 clause 5.4.x.

References

- RP-251395, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#108, June 2025.

- RP-251761, “Draft reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN#108, June 2025.