# 3GPP TSG RAN meeting #109                                         RP-252097

Beijing, China, September 15-18th, 2025

Source: 			CMCC

Title:  		    Discussion on 6G Voice

Agenda item:	8.2.1.4

Document for: 	Discussion

Introduction

Voice services are the basic services of each generation of mobile communications. Meanwhile, in the evolution of voice technology, it could be continuously integrated with other technologies, such as AI. As operator, China Mobile provides users with a new generation of call products named “new call” based on intelligent network. Leveraging AI algorithms and big data, this service upgrades traditional voice and video call services, bringing the UEs a new experience characterized by stability, high definition, visual interaction, and intelligent efficiency. In this contribution, we will share our views on voice in 6G.

Discussion

At 6G day1, we should still start by providing voice services, building a low-latency, high-definition calling experience and evolving toward immersive voice and 3D calls. To this end, 6G should make carrier-grade voice a native, fundamental capability, integrate regulatory functions such as emergency call and lawful interception (LI) from day one, and proactively introduce new legal requirements like digital identity, trusted positioning, and immersive emergency communications.

Proposal 1: Support Voice over 6G (Vo6G) from Day 1 together with emergency call and lawful interception (LI).

Constrained by terminal capabilities and the initial immaturity of SA deployments, operators widely adopted EPS Fallback during 5G’s commercial launch, routing 5G voice traffic back to 4G. The fallback mechanisms are implemented with redirection-based or handover-based fallback. For the redirection-based fallback, the gNB release the UE to LTE frequency with RRCRelease message. The UE selects target LTE cell and performs random access. For the handover-based fallback, the gNB sends handover command with RRCReconfiguration message with LTE frequency and PCI. The UE performs random access towards the target cell. Even after VoNR became fully available, the large installed base of VoLTE-only devices compelled operators to keep EPS Fallback as a safety net. As a result, today’s networks operate a dual-track voice architecture—VoNR alongside EPS Fallback—with both paths anchored in a unified IMS.

Looking ahead to the early days of 6G, voice services will inevitably face the same challenge. Given the extensive roll-out of 5G networks, we recommend introducing a “5G Fallback” mechanism for 6G: whenever a UE or the network does not yet support native 6G voice, the call can seamlessly fall back to 5G/VoNR. At the same time, to mitigate the service interruption and latency inherent in traditional redirection-based fallback, an inter-RAT handover procedure can be employed to deliver millisecond-level, user-transparent migration, ensuring a superior voice experience.

Proposal 2: Support of voice fallback and handover to previous generation, i.e., 5G.

Furthermore, to support the real-time communication requirement anywhere, it is also important to support voice via NTN including both GEO and LEO to provide voice service when there is less TN coverage. In addition, LPWA devices should support voice to provide voice service for diverse IoTs.

Proposal 3: 6G NTN and LPWA support voice.

A text proposal is also provided in the annex and suggested to be captured to the TR 38.914.

Proposal 4: Capture the text proposal in section 4 to TR 38.914.

Conclusion

In this contribution we share our views on the requirement of 6G voice. Proposals are as follows:

Proposal 1: Support Voice over 6G (Vo6G) from Day 1 together with emergency call and lawful interception (LI).

Proposal 2: Support of voice fallback and handover to previous generation, i.e., 5G.

Proposal 3: 6G NTN and LPWA support voice.

Proposal 4: Capture the text proposal in section 4 to TR 38.914.

Text proposal

## 5.4.x	Voice in 6G

The 6G radio and 6G RAN architecture shall support Voice in 6G with the necessary support to ensure that emergency services like emergency call and lawful interception (LI). Voice fallback and handover are supported for Voice in 6G. Furthermore, NTN and LPWA support Voice in 6G.