3GPP TSG-RAN WG Meeting #109       	                      RP-252098

Beijing, China, 15th-18th Sep 2025

Agenda item:	8.2.1.4

Source:	CMCC

Title:	Requirements of AI for 6G RAN

WID/SID:	FS_6G_RAN_Scen_Req

Document for:	Discussion

- Introduction

In last meeting, RAN plenary approved the 6G SID [1], wherein it can be observed that AI/ML is a crucial component of 6G.

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

This contribution will focus on the requirements of AI/ML for 6G RAN.

- Discussion

## 2.1 Requirements of AI for 6G RAN

During the study on 5G NR and AI integration, a unified Life Cycle Management (LCM) for AI/ML for NR air interface illustrated in the following Figure [2] is defined. Furthermore, RAN1/RAN2/RAN3 have studied or standardized multiple AI/ML for RAN use cases, such as AI/ML based beam management, CSI prediction, CSI compression, AI/ML for Mobility, AI/ML enabled network energy saving, load balancing, slicing, etc..

Figure 1: Functional framework for AI/ML for NR Air Interface

However, the existing 5G-A network architecture remains unchanged to support the enhancements on AI/ML, which has limitations on AI data collection and model transfer/delivery.

- Data Collection

In 5G-A, the data collection procedure is triggered by OAM, and cannot be initiated by gNB autonomously which is not flexible especially for the RAN-centric use cases.

For the transmission of collected data, there are some restrictions using the legacy CP/UP solution as follows:

- The existing SRB cannot transmit the large volumes of data for model training (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes).

- The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the large volumes of data collected by the UE cannot be transmitted to the gNB directly via DRB.

On the other hand, the data for model training is collected per use case for now, which may result in duplicated data collection and reporting, especially for basic measurement results. For example, L1 beam measurement results can be used for both AI/ML based beam management and AI/ML for mobility use cases.

- Model transfer/delivery

For model transfer/delivery from gNB to UE, the similar challenges to data transmission needs to be resolved:

- The existing SRB cannot transmit the large model (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes).

- The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the model trained at gNB cannot be transferred to the UE directly via DRB, especially for use cases with two-sided model.

Observation 1: There are some challenges for AI data collection and AI/ML model transfer/delivery, including the transmission of large volume of data and AI/ML model, as well as duplicated data collection and reporting.

## 2.2 Data Plane

As we analyzed above, there are some issues on AI data collection and model transfer/delivery, such as limitations on CP/UP solution for large volumes of data and model transfer, duplicated data collection and reporting. Therefore, we propose to introduce a RAN data plane to support the large volumes of AI data collection and transmission, especially for model training, and AI/ML model transfer/delivery.

Proposal 1: It is proposed to introduce a RAN data plane to support AI data collection/transmission and AI/ML model transfer/delivery.

## 2.3 Potential use cases

In our contribution [3], we propose to introduce a centralized AI node in 6G RAN to satisfy the requirements on RAN for AI use cases, wherein the centralized AI node can support AI computing resource management, online model training and service exposure.

We believe that the centralized AI node can be also used to better support AI for RAN use cases by the flexible and efficient AI computing resource management. On the other hand, the duplicated AI data collection and reporting we mentioned above will be avoided, since the centralized AI node can coordinate the duplicated requests for the same data, store and process the collected data for different use cases, e.g. the L1 measurement results used for beam management and mobility.

Observation 2: AI for RAN use cases can be greater supported by the centralized AI node, based on the flexible and efficient AI computing resource management. Furthermore, the duplicated AI data collection and reporting can be avoided.

In 5G-A, RAN1/RAN2/RAN3 led to study and specify various uses cases separately as follows:

- AI/ML for Air Interface (RAN1-led)

  - CSI compression

  - CSI prediction

  - Beam management

  - Positioning accuracy enhancement

- AI/ML for Mobility (RAN2-led)

  - RRM measurement prediction

  - Measurement event prediction

  - Note: Focus on network triggered L3-based handover

- AI/ML for NG-RAN (RAN3-led)

  - Network energy saving

  - Load balance

  - Mobility optimization

  - Coverage and Capacity Optimization

  - Network Slicing

  - NR-DC

For 6GR AI/ML, we could take 5G-A LCM as starting point, and consider 5G-A use cases as starting point if the corresponding non-AI technology is supported in 6G. Furthermore, other potential 6G new use cases could be identified and investigated, for example:

- Joint source/channel coding (JSCC)

- DMRS design or AI receiver

- Mobility management

  - AI/ML based LTM

  - AI/ML based CHO

  - AI/ML based RLF/HOF

Proposal 2: It is proposed to take 5G-A LCM as starting point, and consider 5G-A use cases as starting point if the corresponding non-AI technology is supported in 6G. Furthermore, other potential 6G new use cases could be identified and investigated, e.g. Joint source/channel coding (JSCC), DMRS design or AI receiver, AI/ML based mobility.

# 3	Conclusion

Here are the proposals for requirements of AI for 6G RAN.

Observation 1: There are some challenges for AI data collection and AI/ML model transfer/delivery, including the transmission of large volume of data and AI/ML model, as well as duplicated data collection and reporting.

Proposal 1: It is proposed to introduce a RAN data plane to support AI data collection/transmission and AI/ML model transfer/delivery.

Observation 2: AI for RAN use cases can be greater supported by the centralized AI node, based on the flexible and efficient AI computing resource management. Furthermore, the duplicated AI data collection and reporting can be avoided.

Proposal 2: It is proposed to take 5G-A LCM as starting point, and consider 5G-A use cases as starting point if the corresponding non-AI technology is supported in 6G. Furthermore, other potential 6G new use cases could be identified and investigated, e.g. Joint source/channel coding (JSCC), DMRS design or AI receiver, AI/ML based mobility.

# 4	Text Proposal to TR 38.914

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

#### 5.2.X Data Plane

A RAN data plane is introduced to support AI data collection, especially large volumes of data for model training, and AI/ML model transfer/delivery.

## 5.4	Requirements of New and Existing Services

#### 5.4.X	AI/ML for RAN

6G AI/ML LCM takes 5G-A LCM as starting point.

6G AI/ML considers 5G-A use cases as starting point if the corresponding non-AI technology is supported in 6G. Furthermore, other potential 6G new use cases could be identified and investigated, e.g. Joint source/channel coding (JSCC), DMRS design or AI receiver, AI/ML based mobility.

# 5	Reference

- RP-251881 New SID: Study on 6G Radio

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-252092 Views on architecture of 6G RAN for AI