3GPP TSG RAN Meeting #109	RP-252100

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	CMCC

Title:	Scenarios and Requirements for 6G NTN

Document for:	Discussion, Decision

Introduction

As in the 6G RAN SID [1] agreed in RAN #108, the following objective is supported:

j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

In this contribution, we would like to provide some preliminary considerations on the requirements and scenarios for 6G NTN.

Discussion

2.1 Scenarios

Orbital and cell coverage type

In 5G, both GSO (Geo Synchronous Orbit) and NGSO (Non Geo Synchronous Orbit) are supported, with NGSO including Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). Further 5G NTN can be deployed to provide coverage with earth-moving cell, quasi-earth-fixed cell and earth-fixed cell. For 6G NTN, all the Orbital and cell type mentioned above could be supported as well. Besides, multi-orbit satellites system deployment scenarios should also be considered (e.g., interaction between GEO and LEO, MEO and LEO)

Payload type

Both transparent and regenerative payload are supported in 5G NTN, and it is reasonable to also support both the two payload type in 6G NTN.

Duplex mode

Regarding the duplex mode, only FDD mode has been supported in 5G. However, considering the limited NTN spectrum resources and the wider bandwidth requirement especially for LEO satellites, TDD frequency band can be deployed to provide wider band for the application of NTN. Therefore, the design of TDD frame structure should settle the longer round trip time between UE and satellites. The application of TDD spectrum in NTN scenarios can be considered in 6G Day one, including the design of TDD frame structure, HARQ mechanism etc..

Proposal 1: The following scenarios could be considered for 6G NTN:

- Orbital type: GSO and NGSO orbital satellite. Earth-moving cell, quasi-earth-fixed cell and earth-fixed cell

- Payload type: Transparent and regenerative payload

- Duplex mode: TDD and FDD

2.2 Requirements

6G network is expected to provide ubiquitous connectivity through TN and NTN. Further in the 6G SID [1], harmonized 6G Radio design for TN and NTN, including their integration is needed.

Coverage Enhancement for UL & DL

All satellites for 6G satellite networks (covering both GSO and NGSO constellations) to be deployed are expected to be designed for providing connectivity for all user terminals served across the satellite footprints. However, it can be observed that in the next few years, the current limitation or implementation difficulties on the satellite payload may still exist. Some types of satellites with limited number of simultaneous Tx/Rx beams may also exist and will provide services in 6G. Due to the poor link budget and the restriction of the number of active beams per satellite, the satellite payload may be unable to have all its beams actives with the nominal EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth. The situation that one satellite only has 16 simultaneously transmitted beams covering 1024 or 1058 beam footprints should be considered in 6G. The beam hopping related technologies still need to be considered in 6G. Since there is a strong need to implement coverage enhancement techniques (including both link level enhancements and the enhancements of coverage ratio) to optimize CAPEX and OPEX for a given targeted coverage. Hence, coverage enhancements are needed to ensure that all user terminals can be served across the satellite footprint while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS via SSB periodicity extension for beam hopping, low PAPR waveform introduction etc.

On the other hand, satellites beam control by base station on the ground can be considered to improve the flexibility and operation efficiency.

Observation 1: It can be observed that in the next few years, the current limitation or implementation difficulties on the satellite payload may still exist. The satellites with limited number of simultaneous Tx/Rx beams which are required to serve 1024 or 1058 beam footprints may also exist and should be considered in 6G.

Proposal 2: Coverage enhancements (including both link level enhancements and the enhancements of coverage ratio) are needed to ensure that all user terminals can be served or allowed to be accessed across the satellite footprint while maximizing the overall satellite throughput via SSB periodicity extension, low PAPR wave introduction, etc..

Enhanced GNSS Operation

In 5G Rel-17/18/19 NR-NTN assumes 100% GNSS service availability, where the UE uses its GNSS coordinates i.e., UE position together with the ephemeris of the serving satellite to compensate the timing and frequency shift. However, in real network, 100% GNSS service availability cannot be guaranteed due to interference and spoofing. For example, a GNSS performance always tend to be de-graded during up to 14.4 minutes per day (=1% * 24 x 60 mn) and per GNSS system. Hence, in 5GA R20, a new SI: Study on GNSS resilient NR-NTN operation is agreed in RAN #108.Meanwhile, for 6G NTN, reducing the reliance on GNSS such as GNSS resilient or even GNSS free needs to be considered as well. And the outcome of the 5GA R20 SI can be taken into account.

Proposal 3: 6G NTN may need to consider GNSS resilient or even GNSS free cases, and the outcome of the 5GA R20 SID can be taken into account.

Support of voice in NTN

In RAN #108, a new WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4 is agreed, which is mainly to specify the enhancement of NB-IoT NTN for supporting IMS voice call over GSO. For 6G system, considering the real-time communication requirements anywhere, it is also important to support voice via NTN including both GEO and LEO for the scenarios where conventional TN communication infrastructure is destroyed or the area without TN coverage.

Proposal 4: 6G NTN can consider to support voice via both GEO and LEO satellites.

Seamless mobility for intra-NTN and NTN-TN

Towards seamless service continuity between NTN-NTN, NTN-TN, TN-NTN. For NTN-NTN mobility, the interruption time and the signalling overhead are mainly motivation towards an efficient seamless service continuity. 5G introduces in particular RACH-less HO, satellite switch with re-sync procedures, inter-RAT (TN-)NTN mobility (such as mobility from E-UTRAN TN to NR NTN). Therefore for 6G NTN, seamless mobility for intra-NTN and NTN-TN could be considered.

Further for intra-NTN case, multi-orbit coordination (e.g. communication coordination based on the characteristics of different satellite orbits) could also be considered.

Proposal 5: 6G NTN could consider to support seamless service continuity between NTN-NTN, NTN-TN, TN-NTN.

NTN-TN integration

Regarding NTN-TN integration, the following aspects could be discussed:

- Unified radio design. For example, unified design of waveform, frame structure, RACH, HARQ operation, etc., which needs to consider the NTN specific characters such as large propagation delay, Doppler and high-speed movement.

- NTN-TN coordination. From the perspective of operational efficiency, for operators who are potentially with both terrestrial and non-terrestrial networks, a tight interaction between TN and NTN can be considered to maximize the system efficiency. The interference issue should be considered for the areas that NTN dominated coverage and TN dominated coverage are proximal or even overlapped. Leveraging the NTN network and existing TN is a cost-effective way to deliver a ubiquitous network.

Proposal 6: For NTN-TN integration, the following aspects could be considered:

- Unified radio design: Unified design of waveform, frame structure, RACH, HARQ operation, etc., which needs to consider the NTN specific characters such as large propagation delay, Doppler and high-speed movement.

- NTN-TN coordination: Coordination between TN and NTN can be considered to maximize the system efficiency as well as interference alleviation.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some preliminary considerations on scenarios and requirements for 6G NTN and have the following observations and proposals:

Observation 1: It can be observed that in the next few years, the current limitation or implementation difficulties on the satellite payload may still exist. The satellites with limited number of simultaneous Tx/Rx beams which are required to serve 1024 or 1058 beam footprints may also exist and should be considered in 6G.

Proposal 1: The following scenarios could be considered for 6G NTN:

- Orbital type: GSO and NGSO orbital satellite.

- Cell coverage type: Earth-moving cell, quasi-earth-fixed cell and earth-fixed cell

- Payload type: Transparent and regenerative payload

- Duplex mode: TDD and FDD

Proposal 2: Coverage enhancements (including both link level enhancements and the enhancements of coverage ratio) are needed to ensure that all user terminals can be served or allowed to be accessed across the satellite footprint while maximizing the overall satellite throughput via SSB periodicity extension, low PAPR wave introduction, etc..

Proposal 3: 6G NTN may need to consider GNSS resilient or even GNSS free cases, and the outcome of the 5GA R20 SID could be the baseline.

Proposal 4: 6G NTN can consider to support voice via both GEO and LEO satellites.

Proposal 5: 6G NTN could consider to support seamless service continuity between NTN-NTN, NTN-TN, TN-NTN.

Proposal 6: For NTN-TN integration, the following aspects could be considered:

- Unified radio design: Unified design of waveform, frame structure, RACH, HARQ operation, etc., which needs to consider the NTN specific characters such as large propagation delay, Doppler and high-speed movement.

- NTN-TN coordination: Coordination between TN and NTN can be considered to maximize the system efficiency as well as interference alleviation.

Reference

- RP-251881_6G_WG_SID

Text proposal

Based on the discussion above we propose the following text proposal for TR 38.914.

## 5.4	Requirements of New and Existing Services

  - 5.4.x	Requirements for 6G NTN

  - 5.4.x.x	Features inherited from 5G

The features supported in 5G such as random access, HARQ, idle/inactive and connected mobility, etc. should be supported in 6G system .

  - 5.4.x.x	Coverage enhancement

The 6G system should support coverage enhancements for the non-terrestrial network.

  - 5.4.x.x	GNSS resilient/free

The 6G system should support GNSS resilient/free for the non-terrestrial network.

  - 5.4.x.x	Support of Voice

The 6G system should support voice for the non-terrestrial network.

  - 5.4.x.x	Seamless service continuity

The 6G system should support seamless service continuity for intra-TN, between TN and NTN.

  - 5.4.x.x	NTN-TN integration

The 6G system should support TN-NTN harmonized radio design including waveform, frame structure, RACH, HARQ operation, etc., which needs to consider the NTN specific characters such as large propagation delay, Doppler and high speed movement. And the 6G system should support TN-NTN coordination.