3GPP TSG RAN Meeting #109					RP-252119

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Sony, Nordic Semiconductor ASA

Title:	6G massive IoT Requirements

Document for:	Discussion and decision

# 1	Background

Massive communication is among the six usage scenarios which are targeted by IMT-2030 [1] for 6G communications. Meanwhile, from the 3GPP perspective, it has been agreed to study 6GR so that it can support diverse device types, including massive IoT devices [2].

In this contribution we share our view on the key design considerations and requirements for 6G massive IoT.

# 2	Design Consideration on 6G massive IoT

Cellular IoT devices (cat-M1/NB-IoT, (e)Redcap) have been successfully deployed and have supported various use cases over the years. Those devices are expected to last for decades and continuously provide service even in the 6G era. Meanwhile, new 6G massive IoT devices are expected to be designed and introduced in 6G-day1, which can support existing use cases with a smooth migration path to 6G in the future and can also support new use cases that bring additional revenue streams.

Based on the reflections from legacy IoT services and demand from potential new use cases, the following aspects shall be considered when design the 6G massive IoT:

- Adding new features in later stages pose challenges (technical and market). Therefore, target a 6GR that supports from day-1 diverse device types scaling up from the lowest capability device (e.g., 6G massive IoT) to higher capability devices .

- The multiple 3GPP cellular IoT solutions create some market confusion and fragmentation and have increased the implementation effort and complexity, and thus a single massive IoT solution as part of 6GR would be preferred.

- Considering the longevity of IoT services in the field, it is important to “plan ahead” and make critical decisions in the initial design (e.g. minimum bandwidth), to ensure the future proofness of 6G massive IoT design.

- Based on experience from legacy IoT, there are areas where complexity reduction can be improved.  for example, overhead reduction for small payload communication, SAW-less frontend implementation, mobility relaxation, etc. The complexity reduction on IoT device implementation can benefit the growth of the cellular IoT ecosystem and unlock new markets.

# 3	6G massive IoT design considerations and KPIs

Based on the design directions discussed above, the following KPIs are proposed for 6G massive IoT devices, which can also be seen as the lowest capability devices of 6G:

| Parameters | Values |
| --- | --- |
| UE channel bandwidth | 3 MHz |
| Duplex mode | HD-FDD, TDD/FDD |
| Downlink/Uplink peak data rate | [5/5] Mbps |
| Number of RF chain | 1T/1R |
| Tx power | 23 dBm |
| Mobility | Idle mode mobility as baseline |
| MCL | >= 6G mBB (support deep indoor services, e.g., 154 dB) |

- UE Channel Bandwidth: Considering that the massive IoT device is likely to be the lowest capability/category device in 6GR, the minimum bandwidth of 6G massive IoT would also be the  minimum bandwidth of 6GR and help determine the initial access design.

- 3MHz CBW was introduced in the later stages of 5G NR due to the need of certain operators and deployment scenarios. It can be expected that such a demand would be needed in the 6G era as well. Therefore, 3MHz CBW needs to be supported to facilitate the flexibility needed to offer IoT deployments in various spectrum allocations.

- Meanwhile, from the device perspective, a 3MHz bandwidth can effectively enable a fully SAW-less implementation for IoT devices in HD-FDD duplex mode. We further elaborate the importance of fully SAW-less at the end of this section.

- Duplex mode: HD-FDD must be supported for 6G IoT, and TDD/FD-FDD can be considered. A duplexer, not only increases the front-end size, but also attenuates by ~3dB, as consequence twice as much power is required to achieve the same transmit power. Therefore, HD-FDD is the most important duplex mode for IoT applications, it can effectively reduce the device power consumption (at least by half) and heat dissipation while ensuring a similar network throughput compared to FD-FDD.  Combining with a 3MHz CBW mentioned above, a fully SAW-less HD-FDD implementation for IoT devices allows for a small-form single SKU design for global operation.

- Downlink (DL)/uplink (UL) data rate: 5/5 data rate is identified as sufficient for typical IoT-based use cases. However, a higher data rate (exceeding 10/10 Mbps) can be supported as an optional feature if required by specific use cases.

- Mobility: The measurement effort for mobility has been identified as one of the main burdens for IoT operation. However, connected mode mobility is not needed for stationary IoT use cases; having it as an optional feature provides the benefit of reduced device memory footprint.

Maximum coupling loss (MCL): based on the lessons learned from previous generations, depending on the native MCL design of 6G, either a reasonable coverage extension (CE) value, e.g., 10 dB, for 6G IoT devices, and/or a native design with improved coverage should be provided as part of the 6G design for all devices.

As mentioned in the bandwidth and duplex mode discussion above, SAW-less HD-FDD design is a key consideration for RF domain complexity reduction in comparison to (e)Redcap in 5G NR. With a fully SAW-less HD-FDD design, a true single stock-keeping unit (SKU) design for global operation is possible, allowing for true economies of scale. The SAW-less design allows for multiple bands to be supported without additional SAW filters for the additional bands, meaning that there is little to no incremental complexity to supporting additional bands. In addition, a fully SAW-less HD-FDD design can achieve a reduced footprint of the chipset and a smaller IoT module. Last but not least, the insertion loss on Tx and Rx chain can be reduced and the power efficiency of the system can be enhanced.

A similar design principle has been widely adopted in 4G IoT, e.g., cat-M1 and NB-IoT, in commercial production thanks to 4G-IoT’s even smaller CBW. However, it is not the case in 5G NR as larger BW needs to be supported, even for (e)Redcap devices, which is one of the factors that creates a significant implementation feasibility difference between 4G IoT and 5G (e)Redcap devices. The UL CBW is particularly critical here as SAW filters are usually needed to suppress the high order intermodulation distortion (IMD) products to meet the emission limits that are defined either in 3GPP or by regulators. Therefore, a slightly reduced BW compared to 5G eRedcap, e.g., 3MHz, or allowable output power backoff in challenged scenario, would be required in the 6G design to enable a fully SAW-less HD-FDD implementation.

# 4	Co-existence between 4G IoT and 6G

In addition to the design of 6G massive IoT, protecting the longevity of legacy IoT devices, e.g., 4G IoT devices, is also important. Such a consideration needs to be taken into account when designing the overall 6G system. It is necessary to address the need of operators running successful 4G IoT businesses that would like to continue providing (4G IoT-based) services in the 6G era, with at least basic co-existence between 4G IoT and 6G.

# 5	Conclusion

In this contribution we propose to capture the following text in TR38.914:

## 5.4.x	Minimum requirements for massive Communication

The 6GR and 6G RAN architecture shall support the following minimum requirements for massive Communication:

- The minimum number of UE receive antennas shall be 1, at least for FR1 bands.

- The supported UE channel bandwidth shall be 3 MHz, at least for low FR1 bands.

- Half-duplex FDD operation shall be supported.

- Support global operation in multiple bands with minimum incremental device complexity, e.g., device with single stock-keeping unit (SKU) design.

- The coexistence between 4G IoT and 6G shall be supported.

# References

Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU-R, November 2023.

RP-251881, “Study on 6G Radio,” 3GPP TSG RAN Meeting #108, Prague, June 9-13, 2025.