3GPP TSG RAN Meeting #109   											   RP-252121

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	CMCC

Title:	Discussion on 6GR deployment scenarios for 38.914

Document for:	Approval

Introduction

In RAN#107, the Revised SID: “Study on 6G Scenarios and requirements” has been approved [1]. In this contribution, we provide our views on 6G Scenarios and Requirements based on TR 38.913 [2] and current version of TR 38.914 [3].

Deployment scenarios for 6G

In this section, we discuss the deployment scenarios for 6G, including indoor hotspot, dense urban, rural, urban macro, high speed, urban coverage for massive connection, air-to-ground scenario, non-terrestrial network and unmanned aerial vehicle. The specific attributes for each essential scenario are listed in each subsection.

Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 2.1.

Table 2.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 4GHz + Around 7GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m (Equivalent to 12TRxPs per 120m x 50m) |
| BS antenna elements NOTE4 | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 2048 Tx and Rx antenna elements Around 30 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the aggregated system bandwidth used for either DL or UL via switching in time-domain.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 2.2.

Table 2.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz (single layer) Around 4 GHz (single layer) Around 7 GHz (single layer) Around 4 GHz + Around 7GHz (two layers) Around 4 GHz + Around 30GHz (two layers) |
| Aggregated system bandwidth NOTE2 | Around 2 GHz: Up to 120MHz (DL+UL) Around 4 GHz: Up to 200MHz (DL+UL) Around 7 GHz: Up to 400MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer for 2GHz or 4 GHz or 7 GHz: - Hex. Grid Two layers for 4 GHz + 7 GHz or 4 GHz + 30 GHz: - Macro layer: Hex. Grid, 4 GHz - Micro layer: Random drop, 7 GHz or 30 GHz Step 1 NOTE3: Around 4 GHz in Macro layer (including macro cell only) Step 2 NOTE3: Both Around 4 GHz & Around 7 GHz or 30 GHz may be available in Macro & Micro layers (including 1 macro layer, macro cell only) |
| ISD | Single layer: 200m Macro layer: 200m Micro layer: 100m, 3micro TRxPs per macro TRxP NOTE4, All micro TRxPs are all outdoor |
| BS antenna elements NOTE5 | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 2048 Tx and Rx antenna elements Around 30 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | Two layers: Step1: NOTE3: Uniform/macro TRxP, 10 users per TRxP NOTE6, NOTE7 Step2: NOTE3: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP NoTE6, Single layer: 10 users per TRxP 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	Step 1 shall be used for the evaluation of spectral efficiency KPIs. Step2 shall be used for the evaluation of the other deployment scenario dependant KPIs.

NOTE4:	This value is the baseline and other number of micro TRxPs per macro TRxP (e.g., 6 or 10) is not precluded.

NOTE5:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

NOTE6:	10 users per TRxP is the baseline with full buffer traffic. 20 users per macro TRxP with full buffer traffic is not precluded.

Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 2.3.

Table 2.3: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz (single layer) Around 4 GHz (single layer) Around 7 GHz (single layer) Around 4 GHz + Around 7 GHz (two layers) Around 4 GHz + Around 30 GHz (two layers) |
| Aggregated system bandwidth NOTE2 | Around 2 GHz: Up to 40 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for 4 GHz + 7 GHz or 4 GHz + 30 GHz: - Macro layer: Hex. Grid, 4 GHz - Micro layer: Random drop, 7 GHz or 30 GHz Step 1 NOTE3: Around 4 GHz in Macro layer (including macro cell only) Step 2 NOTE3: Both Around 4 GHz & Around 7 GHz or 30 GHz may be available in Macro & Micro layers (including 1 macro layer, macro cell only) |
| ISD | Single layer: 500m Macro layer: 500m Micro layer: 250m, 3micro TRxPs per macro TRxP NOTE4, All micro TRxPs are all outdoor |
| BS antenna elements NOTE3 | Around 2 GHz or Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 2048 Tx and Rx antenna elements Around 30 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | Around 2 GHz or Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRxP NOTE5 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

NOTE4:	This value is the baseline and other number of micro TRxPs per macro TRxP (e.g., 6 or 10) is not precluded.

NOTE5:	10 users per TRxP is the baseline with full buffer traffic. 20 users per TRxP with full buffer traffic is not precluded.

Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 2.4.

Table 2.4: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz (for ISD 1 and ISD2) Around 4 GHz (for ISD 1) Around 7 GHz (for ISD 1) Around 700 MHz and Around 2 GHz combined (for ISD 2) Around 700 MHz and Around 7 GHz combined (for ISD 1) |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 20 MHz(DL+UL) NOTE3 Around 4 GHz: Up to 120 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m |
| BS antenna elements NOTE4 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | 50% outdoor vehicles (120km/h) and 50% indoor (3km/h), 10 users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450MHz – 960MHz identified for WRC-15 are currently being considered and around 700MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	Consider larger aggregated system bandwidth if 20MHz cannot meet requirement.

NOTE4: 	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

High speed

The high speed deployment scenario focuses on continuous coverage along track in high speed trains. The key characteristics of this scenario are consistent passenger user experience and critical train communication reliability with very high mobility. In this deployment scenario, dedicated linear deployment along railway line and the deployments including SFN scenarios captured in Section 6.2 of 3GPP TR 36.878 [4] are considered, and passenger UEs are located in train carriages.

Figure 1 illustrates a typical layout of high speed deployment scenario. In this deployment scenario, multiple RRHs connect to one BBU with fiber and share the same cell ID. For each RRH, usually there are two TRPs which orient opposite directions along the track, and 8 RRHs are connected to one BBU with fiber and share the same cell ID to form one large macro cell.

Figure 1. Layout illustration of high speed deployment scenario

Some of its attributes are listed in Table 2.5.

Table 2.5: High Speed

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro NOTE2 only: Around 4 GHz or Around 7 GHz |
| Aggregated system bandwidth NOTE3 | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Macro only: - Around 4 GHz or 7 GHz: Dedicated linear deployment along the railway line as in Figure 1. - RRH site to railway track distance: 100m |
| ISD | - Around 4 GHz or 7 GHz: ISD 500m or 1732m between RRH sites, two TRxPs per RRH site. See Figure 1. |
| BS antenna elements NOTE4 | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | 100% of users in train For non-full buffer, 300 UEs per macro cell (assuming 1000 passengers per high-speed train and at least 10% activity ratio) Maximum mobility speed: 500km/h |
| Service profile | Alt 1: Full buffer Alt 2: FTP model 1/2/3 with packet size 0.5 Mbytes, 0.1 Mbytes (other value is not precluded) Other traffic models are not precluded, e.g., for critical train communications. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range.

NOTE2:	For Macro, it is assumed RRH sharing the same cell ID or having different cell ID.

NOTE3:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

Urban coverage for massive connection

The urban coverage for massive connection scenario focuses on large cells and continuous coverage to provide mMTC. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas, with very high connection density of mMTC devices. This deployment scenario is for the evaluation of the KPI of connection density.

Some of its attributes are listed in Table 2.6.

Table 2.6: Attributes of urban coverage for massive connection

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700 MHz, 2 GHz as an option |
| Network deployment including ISD | Macro only, ISD = 1732m, 500m |
| Device deployment | Indoor, and outdoor in-car devices |
| Maximum mobility speed | 20% of users are outdoor in cars (100km/h) or 20% of users are outdoors (3km/h) 80% of users are indoor (3km/h) Users dropped uniformly in entire cell |
| Service profile | Non-full buffer with small packets |
| BS antenna elements | 2 and 4 Rx ports (8 Rx ports as optional) |
| UE antenna elements | 1Tx |

Air-to-Ground scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay).

Some of its attributes are listed in Table 2.7.

Table 2.7: Attributes for Air-to-Ground Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro + relay: for BS to relay: 4 GHz, 7GHz |
| System Bandwidth | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Macro + relay nodes NOTE1 |
| Cell range | Macro cell: 100 km range to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation. Relay: up to 80 m |
| User density and UE speed | End user density per Macro: NOTE2 UE speed: Up to 1200 km/h Altitude: Up to 15 km |
| Traffic model | End User experienced data rate: 384 kbps DL. NOTE3 |

NOTE1:	BS to relay link should be the priority for study compared to relay to UE link.

NOTE2:	Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.

NOTE3:	Target values for UL are lower than DL, 1/3 of DL is desirable.

Non-Terrestrial Network

The Non-Terrestrial Network deployment scenario is defined to allow for the provision of services for those areas where the terrestrial service is not available and also for those services that can be more efficiently supported by the satellite systems such as broadcasting service. Satellite acts as a fill-in especially on roadways and rural areas where the terrestrial service isn’t available. The supported services via the Satellite system are not limited to just data and voice, but also for others such as machine type communications, broadcast and other delay tolerant services.

Some of its attributes are listed in Table 2.8.

Table 2.8: Attributes for Non-Terrestrial Network

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | <6 GHz for both DL and UL |
| Duplexing | FDD, TDD |
| Satellite architecture | Bent-pipe, On-Board processing NOTE2 |
| Typical satellite system positioning in the 6G architecture | Access network |
| System Bandwidth (DL + UL) | FDD: Up to 2*[20 MHz], TDD: 30MHz |
| Satellite Orbit | GEO, LEO, MEO |
| UE Distribution | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile NOTE3 |

NOTE1:	The carrier frequencies noted here are for evaluation purpose only, satellites are deployed in wide range of frequency bands including L band (1-2GHz), S band (2-4GHz), C band (3.4-6.725 GHz), Ku band (10.7-14.8 GHz), Ka band (17.3-21.2 GHz, 27.0-31.0 GHz) and Q/V bands (37.5-43.5 GHz, 47.2-50.2 GHz and 50.4-51.4 GHz) and more. Here the around xGHz is used to denote the carrier frequency close to xGHz rather than used as band proxy to denote a band range.

NOTE2:	Bent pipe refers to the architecure where the satellite transponders are transparent—only amplify and change frequency but preserve the waveform. On Board Processing satellite transponders incorperate regeneration — including modulating and coding the waveform. Regenerative payload refers to the architecture where satellite carries partial or full base station functions on board.

NOTE3:	Mobile consitutes of both hand-helds and other moving platform receivers such as automobiles, ships, planes etc. Currently the hand-helds are limited to L and S bands but the research is ongoing to support higher bands.

Unmanned Aerial Vehicle

The unmanned aerial vehicle (UAV) deployment scenario focuses on continuous coverage for UAVs with low altitude. The key characteristics of this scenario are continuous wide area coverage in low altitude and critical UAV communication reliability with high mobility. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 2.9.

Table 2.9: Attributes for Unmanned Aerial Vehicle

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 GHz Around 4 GHz Around 7 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 20 MHz(DL+UL) NOTE3 Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Hex |
| ISD | 500 m 1000m |
| BS antenna elements NOTE4 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | Uniformly distributed Maximum mobility speed: 150km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450MHz – 960MHz identified for WRC-15 are currently being considered and around 700MHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	Consider larger aggregated system bandwidth if 20MHz cannot meet requirement.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

Proposal

We propose to agree on related text for the above to be captured in the new TR 38.914 during this meeting.

References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom

- 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements

- 3GPP TR 36.878, Study on performance enhancements for high speed scenario in LTE