3GPP TSG RAN Meeting #109												RP-252126

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	Huawei, HiSilicon

Title:	Discussion on 6G deployment scenarios

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the scenario and relevant key assumptions in the scenario, the objective of this SID includes:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the suTRP equent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- …...

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure suTRP tantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

…..

In this contribution, the considerations and views on the 6G deployment scenarios and assumptions are discussed and corresponding text proposal is provided.

Requirements of deployment scenario consideration

General configurations

The layers described in deployment scenarios usually refer to Macro layer and Micro layer, which is relevant to the deployment topology. Single layer usually refers to the case that only macro layer is deployed, it can be with either single carrier or multiple carriers. Two layers refer to the case that both macro layer and micro layer are deployed, where micro layer can be used to compensate macro layer coverage and usually multi carriers are considered in this case. In initial phase of 6G, the macro layer performance will be fundamentally important for operators to plan the network deployment, therefore evaluation for single macro layer shall be prioritized. Another aspect is about single carrier and multiple carriers. During the LTE-A and NR stage, the system level evaluation method uses single carrier in a single layer to evaluate spectrum efficiency and user experience data rates. This is seen useful to also apply to 6G, considering the case that operators usually would have one major band for initial 6G standalone deployment, with wide and continuous coverage. Moreover, the energy saving performance needs also to be evaluated for single carrier single layer case, considering the initial deployment situation. Therefore, 6G shall consider single carrier in single layer for evaluation.

Proposal 1: Single carrier in single layer (i.e. macro layer) shall be considered for evaluation.

Multiple carrier single layer deployment is also possible in some regions, to utilize aggregated data rate from multi-bands for macro deployment. This could also be considered for evaluation, e.g. user experience data rate, energy saving etc.

Proposal 2: Multiple carriers in single layer (i.e. macro layer) shall also be considered for evaluation.

Regarding aggregated bandwidth, according to [2], the LS to ITU-R, the 7GHz with at least channel bandwidth of 200MHz from system perspective has already been agreed upon in the RAN#106 meeting. 3GPP document for ITU-R IMT-2030 with at least 400MHz system bandwidth was also approved in RAN#107 [3]. Besides 7 GHz, TR 38.914 [4] has also given the aggregated bandwidth for existing bands, and the principle is to also utilize the aggregated spectrum resources for these bands. In this case the proposed value in the last meeting can be accepted for 6G evaluation.

Proposal 3: Adopt aggregated bandwidth proposed in TR 38.914 [4].

User distribution and density

The major evaluation assumption is 10 users per cell in the technology study stage of LTE-A and NR. However, it is expected that 6G will support more services, e.g. various AI applications, more advanced immersive communications etc. The number of users using these emerging services is also expected to be increased, once these applications become more and more popular in the coming 10 years. Therefore reusing 10 users per cell may not be sufficient for future, and it is suggested to consider adding other options, e.g. 20, 30 and 50 users per cell for 6G evaluation. Such additional user distribution values need to be considered for indoor case and outdoor cases like dense urban, where new services are expected to be widely used by consumers.

Proposal 4: Multi-level of user number per cell shall be considered for 6G evaluation [10, 20, 30, 50].

Traffic model

In RAN1#122 meeting [5], the study on traffic model was agreed as below:

Agreement - Study which of the following traffic models are to be used for 6G evaluations, e.g., - Full buffer - FTP Model 1 (in TR 36.814) - FTP Model 2 (in TR 36.814) - FTP Model 3 (in TR 36.872) - XR Traffic models (in TR 38.838) - VoIP model (as in TR 36.814) - Instant message (as in TR 38.840) - Study whether to introduce the following traffic models for 6G evaluations considering, e.g., - FTP-3 variant with packet delay budget requirement - Details FFS - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget) - Details FFS - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc. - Details FFS - Study whether to introduce new/additional approaches that can reflect the impact of bidirectional traffic flows on performance metrics (e.g., impact of UL TCP ACK latency on DL throughput/latency) Note: Whether/how to consider the combination of traffic model and loading level will be studied under individual agendas.

TR 38.914 [4] has considered both full buffer and non-full-buffer traffic for 6G evaluation. For non-full-buffer traffic, the FTP-3 model with packet size and arrival rate is widely used for communication traffic less sensitive to latency. For traffic with latency requirement, such as XR, immersive communication, AI/ML, etc., it is important to consider how latency requirements are included in traffic models. The XR traffic models in TR 38.838 [5] have captured packet delay budget requirements, which can be used for XR composite requirement evaluation. For the evaluation of AI/ML related capability, new traffic models should be introduced to incorporate the features of tokens, such as token arrival rate, token size, token success rate requirement, and token delay budget. Moreover, latency requirements should also be considered in evaluation of energy saving performance based on QoS satisfaction criteria [7]. A variant of FTP-3 with packet delay budget requirements can be used to define the 6GR traffic with potentially different QoS requirements.

Proposal 5: The traffic model should be based on the requirement of the service to consider with/without latency requirement, e.g. packet delay budget. For non-full-buffer traffic with QoS requirement,

- Use XR traffic models in TR 38.838 for XR composite requirement evaluation

- Use FTP-3 variant with packet delay budget requirement for energy efficiency evaluation

- Introduce new AI/ML traffic models for AI/ML related capability evaluation

TRP antenna elements and UE antenna elements

From service perspective, the emerging use cases in 6G, such as applications of Extended Reality (XR), sophisticated Digital Twin (DT), and rapid advancement of AI require, necessitate data rates far exceeding those supported by current 5G networks. Advanced XR and immersive metaverse experiences will require network capacities 100 times greater than current capabilities, with data rates up to 10 Gbps per user [7]. Even higher data rates requirements than tens of Gbps are forecast for services like Digital Twins and haptic communication [8-10]. The rapid development of AI also brings higher data rates to produce synthesized data timely far exceeding human-generated data. These high-performance applications require more advanced capabilities from both UE and network sides.

On the other hand, capabilities from network and UE sides are kept evolving, from 4G to 5G, large array massive MIMO has been introduced which brings 5G higher spectral efficiency, higher data rate and coverage performance than 4G. In the upcoming 5 years, the techniques will continue to evolve and the practical parameters for future deployment shall take such development into account.

Having the above in mind, 6G shall consider a higher number of antenna ports. Note that in 5G Rel-19 128 ports have already been supported, to support 256 or 512 ports is a reasonable expectation for 6G especially for those bands with larger bandwidth, e.g. ~7GHz, which will have 2048 ~ 3072 antenna elements. Therefore, it is proposed to consider at least 2048 elements as the maximum value for 6G techniques evaluation. It is also worth mentioning this is the maximum value proposed for evaluation, which can bring benefits to encourage companies to have thorough evaluation of solutions, while for each specific deployment scenario the antenna elements can be adjusted.

Proposal 6: The max number of TRP antenna elements is at least 2048 for ~7GHz, and the specific number used for evaluation can be different according to specific deployment scenarios.

From UE perspective, the UE antenna elements in sub-3GHz band (e.g. 700MHz, 2GHz) and 5G C-band are defined as up to 4 Tx/Rx and 8 Tx/Rx in TR 38.913 [11], respectively. Also, in Rel-19, 3GPP studies the channel model in 7-24GHz FR3 spectrum, and the conclusions related to UE antenna element are agreed as below:

Agreement in RAN1#121 Agreement in RAN1#120b For CPE devices adopt the following device dimensions for UE antenna modeling: - (x cm × y cm × z cm) = (0 cm × 20 cm × 20 cm) - Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device. - Note: a candidate antenna location (e.g., 9) can be used for multiple antennas

Based on the channel model and evaluation assumptions in the current 3GPP release, the UE supported antenna elements may vary per different bands or according to different device type capabilities. For ~7GHz the bandwidth is large, to well support massive MIMO, the UE with more advanced capabilities needs to support higher antenna elements, e.g.  16 or 32 antenna elements; while for UEs with lower capabilities, 4, 8 can also be considered. It should be noted that the number of UE Tx or Rx ports is usually smaller than the number of antenna elements, and can be further discussed in the evaluation stage, e.g. from 2T4R to 32T32R considering different device types and the way of antenna distribution within the devices. Additionally considering there are different UE implementations, other distribution of UE antenna elements may need to be further considered.

Proposal 7: The following UE antenna element numbers are considered for evaluation:

- ~7GHz: Up to 16 antenna elements

- ~4GHz: Up to 8 antenna elements

- sub 3GHz (e.g. 700MHz, 2GHz): Up to 4 antenna elements

Specific UE antenna element number used in evaluation can be different according to device types.

ISAC evaluation

ISAC is one new defined usage scenario in IMT-2030. In practical deployment it is cost-effective to utilize communication network to provide ISAC capabilities. Therefore, the evaluation of ISAC techniques shall be considered in those deployment scenarios which were adopted in 6G at last plenary, such as indoor, dense urban, rural, urban macro and so on. The major parameters to be taken into account are the sensing target distribution and corresponding speed. The specific value can be different for different deployment scenarios, e.g. less sensing targets should be considered in indoor scenarios compared to outdoor scenarios.

Proposal 8: Sensing target distribution and speed shall be taken into account in 6G deployment scenarios, including indoor hotspot, dense urban, rural, urban macro, and sub-urban macro scenarios.

NTN evaluation

Non-Terrestrial-Network (e.g. Satellite) is an essential complement to TN and therefore its deployment scenario shall also be considered. Different kinds of satellite orbits need to be addressed considering different regions requirements, and evaluation on FDD/HD-FDD MSS bands already defined in FR1 and FR2 needs to be supported as well.

Proposal 9: NTN shall be supported as one deployment scenario for evaluation.

UAV scenario

In TR 36.777 [12], deployment scenarios were defined for aerial vehicles (AV) to investigate the coverage of LTE network deployments. In TR 38.913 [11], a light aircraft scenario was defined to allow for the provision of services for general aviation aircrafts to initiate and receive mobile services. 3GPP has provided connectivity support for UAV in both 4G and 5G. With the thriving growth of low-altitude economy, the UAV scenario should also be supported in 6G evaluation.

Proposal 10: UAV scenario shall be supported as one deployment scenario for evaluation.

Summary

We have discussed the principles of deployment scenarios. The proposals are as below:

Proposal 1: Single carrier in single layer (i.e. macro layer) shall be considered for evaluation.

Proposal 2: Multiple carriers in single layer (i.e. macro layer) shall also be considered for evaluation.

Proposal 3: Adopt aggregated bandwidth proposed in TR 38.914 [4].

Proposal 4: Multi-level of user number per cell shall be considered for 6G evaluation [10, 20, 30, 50].

Proposal 5: The traffic model should be based on the requirement of the service to consider with/without latency requirement, e.g. packet delay budget. For non-full-buffer traffic with QoS requirement,

- Use XR traffic models in TR 38.838 for XR composite requirement evaluation

- Use FTP-3 variant with packet delay budget requirement for energy efficiency evaluation

- Introduce new AI/ML traffic models for AI/ML related capability evaluation

Proposal 6: The max number of TRP antenna elements is at least 2048 for ~7GHz, and the specific number used for evaluation can be different according to specific deployment scenarios.

Proposal 7: The following UE antenna element numbers are considered for evaluation:

- ~7GHz: Up to 16 antenna elements

- ~4GHz: Up to 8 antenna elements

- sub 3GHz (e.g. 700MHz, 2GHz): Up to 4 antenna elements

Specific UE antenna element number used in evaluation can be different according to device types.

Proposal 8: Sensing target distribution and speed shall be taken into account in 6G deployment scenarios, including indoor hotspot, dense urban, rural, urban macro, and sub-urban macro scenarios.

Proposal 9: NTN shall also be supported as one deployment scenario for evaluation.

Proposal 10: UAV scenario shall be supported as one deployment scenario for evaluation.

Proposal 11: Adopt the corresponding text proposal described in section 5.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- RP-243325, “DRAFT LTI Response to LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN#106, December 2024.

- RP-250819, “Moderator's summary for IMT-2030 TPRs”, RAN#107, March 2025.

- RP-251873, 3GPP TR 38.914 V0.1.0, “Study on 6G Scenarios and Requirements”

- 3GPP TSG RAN WG1 #122, Chair notes RAN1#122, Bengaluru, India, Aug 25th – 29th, 2025.

- 3GPP TR 38.838, Study on XR (Extended Reality) evaluations for NR

- RP-252129, Consideration on 3GPP requirements and KPIs in addition to ITU-R IMT-2030 TPRs, Huawei, HiSilicon

- Qualcomm, "Why and what you need to know about 6G in 2022," [Online]. Available: https://www.qualcomm.com/content/dam/qcomm-martech/dm-assets/documents/web-why-and-what-you-need-to-know-about-6G-in-2022.pdf

- Ericsson: 6G - Follow the journey to the next generation networks, Available: https://www.ericsson.com/en/6g

- S. Tarkoma et al., "Exploring the 6G Potentials: Immersive, Hyper-Reliable, and Low-Latency Communication," arXiv preprint arXiv:2310.06425, 2023.

- "The Digital Twin Technology Applied To 6G Communication," Available: https://semiengineering.com/the-digital-twin-technology-applied-to-6g-communication/

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

Text proposal for the TR

Proposed wording based on the conclusion of this document.

< Unchanged parts are omitted >

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz |
| Aggregated system bandwidth NOTE1,2 | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1GHz (DL+UL) |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements NOTE4 | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP NOTE4 |
| Sensing target distribution and Sensing target speed | Sensing target distribution and Sensing target speed 100% Indoor, 3km/h [1] sensing target per TRxP. NOTE5 Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI NOTE6. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.  The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g. 20, 30, 50) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [1] sensing target per TRxP are starting point, other number of sensing targets per TRxP (e.g., 5 or others) is not precluded.

NOTE6: For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 120 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400MHz (DL+UL) Around 30 GHz: 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements NOTE4 | Around 700 MHz: Up to 32 Tx and Rx antenna elements Around 2 GHz: Up to 128 Tx and Rx antenna elements Around 4 GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10, or 30 users per TRxP [10, 30] users per TRxP with single-layer only NOTE5 80% indoor (3km/h), 20% outdoor (30km/h) |
| Sensing target distribution and Sensing target speed | [10] sensing targets per TRxP NOTE6 UAV is up to 160km/h, Vehicle is up to 60km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI NOTE7. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	ISD 200m is the baseline, other number of ISD (e.g., 250 m) is not precluded.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE5:	[10, 30] users per TRxP is the baseline, other number of users per TRxP (e.g., 20, 50) is not precluded.

NOTE6:	Sensing target number per TRxP depends on the use cases and sensing target type, [10] sensing targets per TRxP are starting point, other number of sensing targets per TRxP (e.g. 20 or others) is not precluded.

NOTE7: For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 120 MHz (DL+UL) Around 4 GHz: Up to 200]MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements NOTE3 | Around 700 MHz: Up to 32 Tx and Rx antenna elements Around 2GHz: Up to 128 Tx and Rx antenna elements Around 4 GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP NOTE4 |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP NOTE5 UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. NOTE6 |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [5] sensing targets per TRxP are starting point, other number of sensing targets per TRxP (e.g., 10 or others) is not precluded.

NOTE6: For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth NOTE1,2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 120 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements NOTE3 | Around 700 MHz: Up to 32 Tx and Rx antenna elements Around 2GHz: Up to 128 Tx and Rx antenna elements Around 4 GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements |
| User distribution and UE speed | TBD [10] users per TRxP NOTE4 |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP NOTE5 UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI NOTE6. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [5] sensing targets per TRxP are the baseline, other number of sensing targets per TRxP (e.g., 10 or others) is not precluded.

NOTE7: For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| System bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 120MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400MHz (DL+UL) |
| Layout | Single layer - Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| TRP antenna elements NOTE3 | Around 700 MHz: Up to 32 Tx and Rx antenna elements Around 2 GHz: Up to 128 Tx and Rx antenna elements Around 4 GHz: Up to 512 Tx and Rx antenna elements Around 7GHz: Up to 2048 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements |
| User distribution and UE speed | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h] [10] users per TRxP NOTE4 |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP NOTE5 UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. NOTE6 |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [5] sensing targets per TRxP are the baseline, other number of sensing targets per TRxP (e.g., 10 or others) is not precluded.

NOTE6: For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

< Unchanged parts are omitted >

…

< Unchanged parts are omitted >

## 4.10	Non-Terrestrial Network

Editor note: orbit type and payload type will be captured in section 4.10

This deployment scenario is defined for non-Terrestrial network. Satellite acts as a fill-in especially on roadways and rural/disaster areas where the terrestrial service isn’t available.

Some of its attributes are listed in Table 4.10.

Table 4.10: Attributes for Non-Terrestrial Network

| Attributes | Deployment-1 | Deployment-2 |
| --- | --- | --- |
| Carrier Frequency | Around 2 GHz for both DL and UL | Around 20 GHz for DL Around 30 GHz for UL |
| Duplexing | FDD | FDD/HD-FDD |
| Satellite architecture | Bent-pipe On-board Processing | Bent-pipe, On-Board Processing |
| Typical satellite system positioning in the 6G architecture | Access network | Access network Backhaul network |
| System Bandwidth (DL + UL) | Up to 2*30 MHz | Up to 2*400 MHz |
| Satellite Orbit | GEO, MEO, LEO | GEO, MEO, LEO |
| UE Distribution | 100% Outdoors | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile | Fixed, Portable, Mobile |

< Unchanged parts are omitted >

…

< Unchanged parts are omitted >

## 4.XX	UAV

The Unmanned Aerial Vehicle (UAV) scenario is defined to allow for the provision of services for general UAV and Urban Air Mobility (UAMs) to initiate and receive mobile services. The key characteristics of this scenario are deploying dedicated or shared (i.e, for both aerial and terrestrial users) network to support data services for aerial users that might be travelling at high speeds.

Some of its attributes are listed in Table 4.XX.

Table 4.XX: Attributes for Low-altitude Airspace

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz |
| System bandwidth | Around 4 GHz: Up to 200 MHz (DL+UL) |
| Layout | Single layer |
| ISD | ISD1: 200m. ISD2: 500m ISD3: 1732m |
| User distribution and UE speed | Aerial UE altitude: up to [1000] m NOTE1 Aerial UE speed: up to 250kmph Number of total aerial UEs per TRxP is up to [5] NOTE2 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The altitude of aerial UEs up to [1000] m is the baseline, other altitude of aerial UEs (e.g., 300 or 600) is not precluded.

NOTE2:	[5] aerial UEs per TRxP are the baseline, other number of aerial UEs per TRxP (e.g., 15 or others) is not precluded.

< Unchanged parts are omitted >