3GPP TSG RAN Meeting #109	RP-252136

Beijing, China, September 15 - 19, 2025

Agenda Item:	8.2.1.4

Source:	Huawei, HiSilicon

Title:	Consideration on Support of Diverse Device Types for Various Services

Document for:	Discussion and Decision

Introduction

In the RAN#108 meeting, a New SID: Study on 6G Radio was approved [1]. Its objectives include the following:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In this contribution, the considerations and views for 6GR device types are discussed.

6GR Device

Background

Both 5G NR and LTE support a wide range of device types, from high-performance eMBB to mid-performance reduced-complexity devices and low-power IoT devices. Examples include NR Rel-15/16 eMBB, NR Rel-17/18 RedCap, as well as LTE Cat-1, Cat-1 bis, Cat-4, NB-IoT, and Cat-M. These device categories differ in bandwidth capability, peak data rate, antenna configuration, coverage requirements, cost, battery life, and other factors, thereby meeting different application scenarios. Figure 1 shows the landscape of diverse 5G and LTE device types.

Figure 1. The landscape of diverse 5G and LTE device types

For 6G Day One design, which device types that exist in 5G need to be supported and whether new device types need to be introduced should be discussed. The following section is trying to analyze this aspect.

Consideration on diverse device types

As mentioned in RAN#108 SID, 6G targets a scalable and forward-compatible design that can support various device types from Day-1, while minimizing operators’ cost and implementation complexity. The device diversity in 6G will be richer than 5G, and several typical categories should be taken into considerations.

First, eMBB remains as the mainstream. Smartphones will continue to dominate, and to highlight the performance and experience advantages of 6G over 5G, high-end devices should support large bandwidth capability, e.g., 200 MHz.

Second, IoT devices need to be considered for massive connections. From 4G to 5G, various types of IoT devices have been defined for IoT services, and it is worth considering whether all of them are essential and feasible to be supported in 6G Day One radio design. This will be addressed in Sec 2.3.

Last but not least, advanced device types also need to be considered. For new 6G applications such as collaborative robots, AI agents, immersive communications, etc., there is a need to satisfy both high data rate and low latency requirements for good user experience. Devices such as embodied robots, CPE, and FWA terminals, with fewer form-factors constraints, can better support these more advanced services through higher-order modulation, more Tx/Rx chains, and larger bandwidth (e.g. 400MHz) capabilities.

Proposal 1: Multiple device types shall be studied in 6G, addressing requirements from both existing and emerging services:

- IoT

- eMBB

- new services, e.g., AI agents, collaborative robots with higher performance requirements

Proposal 2: Capabilities for each device type shall be studied with consideration on whether form-factor limitation applies.

Consideration on design target for IoT devices

IoT devices are usually designed for the market with low data rate, better coverage and massive connectivity of things. As shown in Figure 1:

- 5G (e)RedCap has similar design targets as Cat 4/1(bis), which typically has 20 MHz bandwidth, and addresses IoT applications such as the wearables, monitoring cameras, and smart grid sensors. These applications usually do not require coverage extension

- eMTC has a smaller bandwidth of 1.4 MHz and provides up to 10dB coverage extension, which addresses IoT applications such as smart metering and smart home scenarios in the indoor environments.

- NB-IoT has an even smaller bandwidth of 180KHz but provides up to 20dB coverage extension, which addresses IoT applications such as electricity metering and water metering that require extreme deep coverage due to basement installations or deployments at depth under water. The radio design of NB-IoT is different from that of NR/LTE radio.

It can be seen that the major design targets for IoT devices are the maximum channel bandwidth and coverage. These two aspects have a fundamental impact on initial access design as well. Therefore, a detailed analysis of these two aspects is provided below.

- Bandwidth Capability of IoT Devices

In 5G discussion, RedCap has been introduced in Rel-17, with 20 MHz bandwidth design. eRedCap has been introduced in Rel-18 to reduce the baseband bandwidth to 5 MHz while keeping 20 MHz for RF bandwidth. During the (e)RedCap study, evaluations of complexity reduction regarding 100 MHz, 20 MHz and 5 MHz was summarized in Table 1 below from TR 38.865 [2].

Note that the reference Rel-15 NR UE has the following features:

- FDD: 100 MHz, 2 Rx, 2 layers, DL 256QAM, FD-FDD

- TDD: 100 MHz, 4 Rx, 4 layers, DL 256QAM

The main UE bandwidth reduction options considered for FR1 are as follows:

- Option BW1: Both RF and BB bandwidths are 5 MHz for UL and DL.

- Option BW2: 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL.

- Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

Table 1: Average relative UE complexity (RF+BB) achieved by BW reduction

| Options | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| FD-FDD 1Rx | ## 100% | ## 44.74% | ## 39.44% | ## 40.65% | ## 41.15% |
| TDD 1Rx | ## 100% | ## 31.41% | ## 27.88% | ## 28.88% | ## 29.01% |
| HD-FDD 1Rx | ## 100% | ## 39.07% | ## 33.58% | ## 34.41% | ## 35.60% |
| FD-FDD 2Rx | ## 100% | ## 65.24% | ## 55.90% | ## 57.76% | ## 59.55% |
| TDD 2Rx | ## 100% | ## 43.99% | ## 38.08% | ## 40.12% | ## 40.61% |
| HD-FDD 2Rx | ## 100% | ## 58.99% | ## 50.26% | ## 51.79% | ## 53.57% |

It can be seen from Table 1 that when the bandwidth is reduced from 100 MHz to 20 MHz, the complexity can be reduced by about 35%~70% depending on the specific duplex and antenna options, while further reduction of bandwidth from 20 MHz to 5 MHz only gives about ~10% more relative complexity reduction: e.g., in option HD-FDD 1Rx, ~61% complexity reduction can be achieved from 100 MHz to 20 MHz, but only ~66% complexity reduction from 100 MHz to 5 MHz.

On the other hand, compared with 20 MHz, UEs with 5 MHz bandwidth need to support more repetitions to reach the same coverage target. For example, as analyzed in [2], for SIB1 reception, transmitting over 48RBs (17.28 MHz with 30kHz SCS) versus being limited to 11RBs (3.96 MHz with 30kHz SCS) shows a performance gap of about 8~9dB. To achieve the same coverage, the latter requires 8~10 times more repetitions, which impacts overall system efficiency, especially energy efficiency from both network side and UE side. Consequently, there seems no significant gains to support 5 MHz bandwidth compared with 20 MHz.

Regarding bandwidth even smaller than 5 MHz, e.g., 3 MHz, 1.4 MHz, similar analysis as the above applies. The relative complexity reduction will become even smaller compared with 5 MHz, while the additional repetitions required for coverage would further degrades system performance, especially the energy efficiency from both network and UE sides. Therefore, supporting IoT devices with bandwidth capability below 20 MHz is not considered as a good option.

In [3], statistics on the global shipment volume of IoT modules are presented as Figure 2 below. It can be seen that LTE Cat-1 bis has occupied a significant ratio of the market and is expected to maintain its position for a long time, while Rel-17 RedCap is expected to experience rapid growth in the coming years, gradually increasing its market share. Both LTE Cat-1 bis and Rel-17 RedCap has a bandwidth capability of 20 MHz.

Figure 2. statistics on the global shipment volume of IoT modules

In conclusion, considering trade-off among device cost and complexity, energy consumption and market demands, it is proposed that IoT devices in 6G support a channel bandwidth capability of 20 MHz. It is worth mentioning that devices with 20 MHz bandwidth capability can still operate on channels with narrower bandwidth (e.g., 3 MHz, 5 MHz), thus accommodating different spectrum requirements. To be more specific, this means that for IoT devices, 20MHz should be supported as a common max channel bandwidth. In this case, on bands where there are spectrum resources larger than 20MHz, the network can configure at least 20MHz for initial access for better efficiency, and there is no problem that IoT devices can access to the network; on bands where spectrum resources are smaller than 20MHz, e.g., 5MHz, these IoT devices are still capable of accessing to the network. In short, the initial access can avoid fragmented design as there is no need to support various small bandwidth for IoT devices.

Proposal 3: 6G IoT devices support 20 MHz in maximum as the channel bandwidth for both RF and baseband.

- Coverage Target of IoT devices

For IoT devices, the need to support coverage extension depends on the application requirements and the actual deployment scenarios. In many cases, applications require coverage extension due to indoor deployment; however, the specific targets may vary across different applications.

In 5G, NB-IoT supports 164dB MCL, which corresponds to 20dB coverage extension. Achieving a 20dB coverage extension requires hundreds or even thousands of repetitions (in NB-IoT, up to 2048 repetitions are defined to meet the 164dB MCL target)..

In the Day One design of 6GR, the performance of eMBB still plays a very important role in enabling the wide deployment of a new generation. If such extreme coverage requirements need to be considered, the design of common channels and signal would have to consider a huge number of repetitions, which would significantly impact the performance of eMBB services in terms of access latency, experienced data rate, energy consumption and so on. Therefore, for applications requiring extreme coverage extension, it is suggested that they should not be included in the Day One design of 6GR and can continue to be supported by NB-IoT. Consequently, co-existence between NB-IoT and 6G deployment may need to be considered.

Proposal 4: Extreme coverage extension, e.g., 164dB MCL, is not considered in 6G Day One design, and the corresponding applications can continue to be supported by NB-IoT.

Other than such extreme coverage extension, there are also applications which require moderate coverage extension, e.g., up to 154dB MCL. The number of repetitions for a 5~10dB coverage extension will be reduced a lot compared with 20dB extension (only about 3%~10% repetitions of 20dB extension are required), and the resulting impact on eMBB performance may be acceptable. Therefore, it is worth studying a reasonable coverage extension range (5~10dB) to satisfy market needs while avoiding significant performance degradation for eMBB services.

Another aspect to consider for coverage target is the deployment bands. If the bands are in sub-3 GHz FDD spectrum, the propagation loss is relatively small, and existing macro cells operating on these bands can already provide good coverage performance. However, for bands around 7GHz, due to additional propagation loss, there may already be a coverage gap considering co-site deployment with C-Band. In such cases, the coverage improvement needs to consider both IoT device capability and also different pathloss characteristics for each band. It is desirable to consider some common techniques for coverage improvement to address both aspects to avoid solution fragmentation.

Proposal 5: Moderate coverage extension (e.g., around 10dB) for IoT devices can be studied to address market demands, with performance impact on eMBB services to be evaluated.

Conclusion

According to the above discussions, we have the following proposals:

Proposal 1: Multiple device types shall be studied in 6G, addressing requirements from both existing and emerging services:

- IoT

- eMBB

- new services, e.g., AI agents, collaborative robots with higher performance requirements

Proposal 2: Capabilities for each device type shall be studied with consideration on whether form-factor limitation applies.

Proposal 3: 6G IoT devices support 20 MHz in maximum as the channel bandwidth for both RF and baseband.

Proposal 4: Extreme coverage extension, e.g., 164dB MCL, is not considered in 6G Day One design, and the corresponding applications can continue to be supported by NB-IoT.

Proposal 5: Moderate coverage extension (e.g., around 10dB) for IoT devices can be studied to address market demands, with performance impact on eMBB services to be evaluated.

- References

RP-251881, “New SID: Study on 6G Radio”, 3GPP TSG RAN meeting #108, Prague, Czech Republic, June 9-13, 2025.

3GPP TR 38.865, Study on further NR RedCap UE complexity reduction (Release 18)

https://iotbusinessnews.com/download/white-papers/TSR-2024-2025-Cellular-Broadband-Device-Module-Market-Summary.pdf