3GPP TSG RAN meeting #109									RP-252148

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.2

Source:	PML

Title:	Proposals on Integrated Sensing And Communication (ISAC) for 6GR Stduy Rel-20

Document for:	  Discussion and Decision

- Introduction and outcome of RAN#108 [1]

3GPP TSG RAN #108 approved SID: Study on 6G Radio[2].

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914].

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance.

Sensing/ISAC is highlighted as a key enabler for 6G, which mainly strives for functionality and performance for diverse sensing use scenarios. The basic components for 6GR Sensing Rel-20 are shown as below.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

We would like to provide our views on Sensing/ISAC study based on mTRP/massiveTRP, for discussions on the agenda of RAN #109 [4].

- Motivations for 6G multi/massive TRP Joint Sensing/ISAC study

① Motivation #1 -Sensing Service Driven

5G-Adv Rel-19 delivered only an ISAC channel model, and Rel-20 focuses on gNB-only mono-static UAV detection with restricted scenarios, node types and KPIs.
The consensus is sensing in 5G-Adv and 6G are independent,so it is essential from 6G Rel-20 onward to take a holistic view of diverse use case/scenario requirements, align sensing modalities, and ensure the soundness of architectural design so that network functionality and performance objectives are fully met.

The full scope defined in TR 22.837 / TS 22.137—three use-case families (object detection & tracking, environment monitoring, motion monitoring) and all node combinations (gNB-gNB, gNB-UE, UE-UE) .

Emerging services such as unmanned-traffic management, low-altitude economy, holographic XR, digital twins and smart cities demand “communication-sensing-compute” convergence with positioning accuracy ≤0.1 m, refresh rate ≥100 Hz and latency ≤1 ms.
Spectrum scarcity: 6G targets a 2~3× spectral-efficiency boost, making multi /massive TRP for sensing and communication mandatory.

The sensing targets and existing communication scenarios as a starting point for NR ISAC agreed as following table[6]:

Sensing Targets; scenarios. UAVs; RMa-AV, UMa-AV, UMi-AV (TR 36.777). Humans indoors; InF, Indoor Office, [Indoor Room (TR 38.808)], [UMi, UMa]. Humans outdoors; UMi, UMa, [RMa]. Automotive vehicles (at least outdoors); Highway, Urban grid, UMa, UMi, RMa. Automated guided vehicles (e.g. in indoor factories); InF. Objects creating hazards on roads/railways (examples defined in TR 22.837); Highway, Urban grid, HST.

Observation 1a: 6G will support a far broader array of sensing services, and the diversity of use cases demands an equally diverse set of sensing modalities to deliver the required precision.

② Motivation #2 -Sensing Performance Driven

TR 22.137 [3] Performance requirements for 5G Wireless sensing merely serve as the baseline; many vendors are already demanding far more stringent sensing capabilities.

The following Sensing-related capabilities Definition could be referred as lower bound for the 6GR sensing study before 6G TPR settled.

[TS 22.137] Table 6.2-1:Performance requirements for 5G Wireless sensing

Multi-static mTRP sensing offers the following decisive advantages over monostatic sensing:

- From the coverage perspective
Multiple, geographically separated TRPs create overlapping fields of view, eliminating single-point blind zones and extending the effective sensing area without increasing per-node transmit power.

- From the accuracy perspective
Diverse bistatic/multistatic geometries provide extra angular and range diversity, enabling centimetre- to millimetre-level positioning and velocity estimation that monostatic radars cannot achieve with the same aperture.

- From the reliability perspective
If any TRP is blocked, jammed, or fails, the remaining nodes maintain full functionality, giving the system built-in redundancy and graceful degradation—critical for safety-of-life applications.

- From the resolution perspective
Synthetic aperture effects formed by distributed TRPs yield finer cross-range resolution than a single collocated array of comparable physical size.

- From the multipath & clutter suppression perspective
Multi-static links observe targets from different aspect angles; fusion algorithms can coherently enhance the target while incoherently averaging out clutter and multipath ghosts that plague monostatic systems.

- From the cost & deployment flexibility perspective
Low-power, low-complexity TRPs can be sprinkled throughout an environment (street furniture, UAVs, satellites), leveraging existing 6G infrastructure instead of deploying expensive high-power monostatic radars.

- From the scalability perspective
Adding more TRPs linearly increases the number of independent sensing links, whereas scaling a monostatic system typically demands a disproportionate increase in antenna elements and processing load.

Observation 1b:

Monostatic sensing mode suffers from significant limitations from many perspectives compared to Multistatic with mTRP/massiveTRP sensing mode.

Consequently, 6GR must treat the sensing performance on par with communications as a core 6G capability starting in Rel-20 , so that Rel-21 can deliver a richer, more comprehensive set of use cases. Without end-to-end ISAC natively specified in Rel-21, 6GR will miss its flagship feature and delay industry adoption.

③ Motivation #3 - Sensing with Cell-free Mobility Driven

In the network, sensing-capable nodes—such as base stations and UEs—can conduct sensing/ISAC services in two ways: by transmitting or receiving sensing signals between different nodes, or by having a single node transmit and receive its own signals. Because the state of the sensing target (spatial position, orientation, velocity, etc.) and the sensing environment may change over time, the network may need to perform sensing-node handovers to maintain ISAC service continuity and to sustain the required ISAC QoS. Moreover, given that different sensing nodes have varying capabilities and are unevenly distributed, the handover process may also involve a switch between distinct sensing modalities.

Multi-TRP cooperative sensing significantly reduces sensing handover latency and enhances service continuity and user experience through redundancy, parallelism, modality fusion, and localized processing.

Observation 1c:

Sensing mobility is as critical as communication mobility, both directly shape the user experience, and the diversity of sensing modalities is a key driver of service experience.

=========================================================================

Proposal 1: Based on above motivation oberservations, we propose that 6GR Sensing study should consider the following aspects:

- 	build ISAC native architecture and key technologies upon the 5G NR sensing baseline;

- 	extend sensing modalities—especially the convergence of communications and sensing under mTRP deployment patterns;

-	co-design waveform, frame structure and resource scheduling with 6GR communications.

- Detailed discussions of mTRP joint sensing

  - Item #1 - 6GR ISAC sensing modalities definition

5G-Adv Sensing mode defined in TS22.137 [3], shown as following.

Sensing operation can be implemented in a couple of different ways, from radar like sensing where the sensing transmitter and sensing receiver are co-located in the same entity (figure 4.2-1), called Monostatic sensing, to have the sensing receiver and sensing transmitter in different entities (figure 4.2-2), also called Bistatic sensing. A more advanced scenario with multiple sensing transmitters and receivers is also possible, called Multistatic sensing. The reflections of the sensing signal sent from the sensing transmitter are received by the sensing receiver and processed to obtain characteristics of the sensed object and its environment (e.g., location). Below figure 4.2-1 and figure 4.2-2 show examples of the sensing operation and the nomenclature used in this specification. Figure 3-1: Example of sensing with co-located sensing receiver and sensing transmitter Figure 3-2: Example of sensing with separated sensing receiver and sensing transmitter Sensing related terminology agreements[7] - Sensing transmitter: the TRP or a UE that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be located in the same or different TRP or a UE as the sensing receiver. - Sensing receiver: the TRP or a UE that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be located in the same or different TRP or a UE as the sensing transmitter. - Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the sensing signal. - Background environment: background (clutter and/or environmental objects) that are not the sensing target(s). - Mono-static sensing: sensing where a sensing transmitter that transmits a sensing signal and a sensing receiver that receives the sensing signal are co-located in the same TRP or UE. - Bi-static sensing: sensing where a sensing transmitter that transmits a sensing signal and a sensing receiver that receives the sensing signal are not co-located in the same TRP or UE. - Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers, for a sensing target. - Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

Observation 2: For 6G ISAC, the fundamental definitions of monostatic and bistatic, multistatic should be adapted to the specific requirements of the 6G communication system. Considering both communication and sensing, these modes must address synchronization, signal processing, resource management, and interference management（between communication and sensing, transmitter and receiver）issues that are tightly coupled with the Cell (PCI) . The 5G-Advanced definition of sensing modalities doesn’t adequately capture Cell and TRP relationship clearly, which impacts the 6GR study items breakdown and timeline planning for Sensing/ISAC research.

Proposal 2:Building upon the established sensing paradigms of monostatic, bistatic, and multistatic sensing, we propose to extend the taxonomy with well-defined sub-modes. These sub-modes will be explicitly mapped to mTRP and Cell associations pattern, enabling a co-design approach that natively supports ISAC, and will also serve as the basis for sensing roadmap and study plan in well-defined phases. Futhermore, massive-static mode may be necessary for the case of large scale TRPs.

  - Item #2 - Multistatic with sTRP transmitter - mTRP receiver sensing mode for Rel-20 6GR study

In the case of 6G cell-free RAN, in addition to the single TPR sensing approach as depicted in Figure 3-3, sensing with multiple TRPs, as illustrated in Figure 3-4, becomes feasible and advantageous due to its architectural support for distributed coordination.

Figure 3-3: Example of sensing with single TX and RX pair (TRP)

Figure 3-4: Example of sensing with multiple TX and RX pairs (TRPs)

mTRP sensing features multiple advantages including but not limited to the following:

- Observing from multiple perspectives (or view angles) effectively solves the problem of irregular and fluctuating RCS (namely, radar cross section), which improves the reliability and accuracy of sensing.

- Observing from multiple perspectives allows antenna arrays with reduced scale and complexity. Single TRP requires any single AAU to collect radio signals for target range, velocity, and DOA estimation, which usually requires a large array aperture for delivering sound angular resolution. While in the case of mTRPs, DOA can be solved with relatively smaller antenna arrays, which would contribute to early adoption of 6G ISAC use cases.

- Observing from multiple perspectives allows better positioning accuracy. In the case of mTRP, target localization with time-synchronized triangulation leverages high ranging resolution (and accuracy) bought about by improved radio signal bandwidth, which in practice is more reliable and accurate than many DOA-based methods using either DBF or hybrid-BF in the case of single TRP.

- Last but not the least, mTRP allows graceful performance degradation, meaning that a trade-off strategy can possibly be planned between the sensing performance and Cell-free RAN resource allocation based on real-time system needs.

Figure 3-5 Overview of PML Outdoor Cell-free RAN ISAC Testbed

Figure 3-6 Tests showed that AAUs feature different blind zones even when the UAV is their theoretic sensing range

Figure 3-7 Tests show that mTRP signal fusion improves positioning accuracy compared with sTRP under the condition of identical waveform

Supporting Evidences:

We have carried out both experimental and simulation study to evaluate the performance of sTRP and mTRP configurations. Figure 3-5 shows an overview of the PML 6G ISAC testbed which currently includes 12 mmWave AAU devices deployed at four sites(TRP) (21,22,23,24) with different orientations, visible and IR cameras for ground truth recording, dual surveillance UAV airports, and a distributed time synchronization, signal processing, data acquisition and storage system.

Figure 3-6 shows recorded empirical data from one fly test in which the system was configured as an mTRP scenario (1TX3RX). Figure 3-6 tells that there exist “blind zones” for all three Rx AAUs co-located in the same site (TRP), where target disappeared from their radar scopes with broken lines.

Figure 3-7 reports a study about sTRP (i.e., the blue curve for single BS) and mTRP (i.e., the gold curve for signal fusion) under the condition of identical ISAC waveform setup. Figure 3-7 tells that mTRP features the potential of reducing the positioning error by one order of magnitude from sTRP.

To evaluate mTRP at scale, we also conducted simulation to explore the impact of TRPs to the positioning error in the 2D scenario, as reported by Figures. In Figure 3-8, it can be observed that the RMSE (i.e., root mean square error) decreases with an increasing number of RX sites in the mTRP case. Figure 3-9 reveals that increasing TRPs would be mostly effective for “2 to 3” and “3 to 4” Rx sites, but offer diminishing returns after 5 TRPs.

Figure 3-8: Simulation Results: Positioning Error vs Rx Counts

Figure 3-9: Simulation Results: Error Reduction vs Rx Counts

Observation 3: The sTRP transmitter - mTRP receiver sensing mode demonstrates clear advantages in performance dimensions such as sensing accuracy and missed-detection rate, enabling 6G to deliver commercially valuable services right from its first release.

Proposal 3: It is recommended that the sTRP transmitter - mTRP receiver sensing mode be prioritized as a high-priority study item and co-designed with the 6GR MIMO communication framework, with RAN1 taking the lead. Channel modelling evaluation[5] with the corresponding Sensing transmitters and receivers properties is the prerequisite for the study.

  - Item #3  Sensing with Random Signals

In general, sensing in 6G ISAC can be carried out via two approaches:

- Sensing With Dedicated Signals (SWDS) utilizes predefined waveforms and exclusively allocated resources (e.g., slots, REs), analogous to 4G/5G reference signals (CSI-RS, SRS, PRS) for object detection, target positioning, environment mapping, etc. While experimentally validated, SWDS introduces a​resource competition problem: sensing consumes air- interface resources that would otherwise support communication, inherently limiting the data throughput.

Figure 3-10: Sensing With Dedicated Signals (SWDS): dedicated resources (waveforms, time slots, OFDM symbols, subcarriers) are allocated for sensing, decoupling it from communication via TDD or FDD multiplexing

- Sensing with Random Signals (SWRS): This approach reuses existing communication signals (reference signals and encoded data) for sensing, making it a by-product of communication. Though SWRS avoids resource competition, it suffers from impaired sensing quality of service (QoS)—such as range, accuracy and latency—due to the lack of optimized waveforms or coverage guarantees.

Figure 3-12 illustrates an example scenario of SWRS, in which RU A is communicating with the UE. A UAV in a nearby area is illuminated by the communication beam, and the scattered signal is collected by RU A/B/C.

In this case, EDU(Edge Distributed Unit) A and UCDU(User-Centered Distributed Unit)/SCDU(Service-Centered Distributed Unit) can both execute SWRS, provided that the transmitted random bits are unloaded correctly as shown in Figure 3-11 after data exchange over fiber connections among EDU A, EDU B, and UCDU/SCDU.

The SWRS approach leverages randomized communication data for sensing tasks, eliminating the need for dedicated signal resources (e.g., waveforms, time slots, OFDM symbols, subcarriers). Consequently, sensing operations are intrinsically coupled with the communication system.

Figure 3-11: Sensing With Random Signals (SWRS)

Figure 3-12: Showcase: Data Convergence Enables mTRP SWRS.

Communication signals between RU A and UE can be utilized for UAV sensing. Computation for SWRS is executed at EDU A and UCDU/SCDU, where scattered signals received by RU A/B/C converge

SWDS and SWRS are complementary to each other in real deployments. In urban areas with dense wireless connection and rich data traffic, ISAC is preferred to be achieved with SWRS with minimal impacts to communication; while in rural areas with relatively low communication data traffic, SWDS can be used to provided offer sensing range and accuracy.

To support SWRS, a high-bandwidth middlehaul/backhaul must be available for data exchange (used for “Random Waveform Unloading” as depicted in Figure 3-11) between the EDU and its associated UCDU/SCDU, as well as among geographically nearby UCDU/SCDUs.

To alleviate the load pressure on fiber connections among DU entities, real-time lossless compression shall be applied before data exchange. For example, each RU may produce peak throughput of 10Gbps, therefor lossless compression is highly desirable for connections among DUs.

Observation 4: “Sensing With Random Signals”, combined with “Sensing With Dedicated Signals”, may deliver excellent performance and adaptive capacity. SWRS dramatically simplifies the joint design of communication and sensing in 6G systems, as well as the complexity of physical-layer elements such as waveforms, reference signals, and frame structures, offering high application value.

Proposal 4: It is recommended to include Sensing With Random Signals as a key candidate technology on the Rel-20 study agenda, RAN1 will take the lead. 6G sensing-architecture studies need to incorporate data-compression mechanisms, algorithms, and procedures. RAN3 will take the lead and work closely with RAN1, RAN2, SA2, and other relevant WGs.

  - Item #4 - TRP Sensing Cases for Study Roadmap

To facilitate understanding and to kick-start the study of the multi-static sensing mode and its further decomposition, we have summarized the 6G sensing scenarios together with their optimal sensing modes in the table below.

Table - Sensing Scenario and preferred mode

| # | Sensing Scenario | Preferred Mode | Key Rationale (scenario pain-points + mTRP/multi-static strengths) |
| --- | --- | --- | --- |
| 1 | 6G UAV low-altitude / urban-canyon flight | mTRP-based multi-static | Eliminates NLOS blind zones, cm-level real-time 3D positioning for fast UAVs, natively supported by 6G without extra hardware. |
| 2 | Large-scale object detection & tracking (traffic / factory) | same | Occlusion & crossing motions cause missed detections; multi-TRP raises detection probability >99 % and network-slicing guarantees ms-level latency. |
| 3 | Collapse monitoring of critical infrastructure (bridges, dams, high-rises) | same | mm-level displacement, 99.999 % long-term reliability via redundancy; seamless fusion with InSAR/UAV data for space-air-ground monitoring. |
| 4 | Structural deformation detection (tunnels, subways, large buildings) | same | Sub-mm accuracy requires multi-baseline interferometry; suppresses multipath; 6G high-frequency & wideband enables 3-D deformation imaging. |
| 5 | Maritime search-and-rescue & oil-spill monitoring | same | No fixed infrastructure at sea; temporary 6G TRPs on buoys/USVs/LEO satellites deliver multi-angle echoes that mitigate wave clutter and enhance small-target/thin-oil detection. |
| 6 | Emergency integrated comms-sensing after disasters (earthquake, flood) | same | Post-disaster terrestrial networks down; airdropped/vehicle-mounted TRPs deploy in 10 min, restoring connectivity while simultaneously locating survivors and landslides. |
| 7 | Smart agriculture (crop growth & pest monitoring) | same | Single-UAV endurance limited; multi-TRP on towers, tractors, HAPS provides 24 h seamless coverage with cm-level plant-height/LAI inversion for variable-rate fertilization. |
| 8 | Indoor high-precision positioning & gesture recognition (XR / industrial metaverse) | same | Severe multipath & human blockage indoors; distributed 6G/Wi-Fi TRPs achieve sub-cm positioning and micro-gesture tracking for controller-free XR. |
| 9 | Railway perimeter intrusion detection (high-speed lines) | same | 100 km linear corridor; single radar too costly; chain of TRPs uses joint Doppler-angle estimation to distinguish human/animal/train with <1 % false alarm. |
| 10 | Low-altitude airspace management for eVTOL fleets | same | Hundreds of eVTOLs need 4-D trajectory sharing; multi-TRP fuses 6G sensing beacons with inter-aircraft links, giving air-traffic control one interface for 10 cm-level 4-D tracks. |
| 11 | Underground utility-tunnel inspection (gas/power) | same | GPS unavailable; ceiling-mounted micro-TRPs co-located with 6G repeaters exploit multipath fingerprints & multi-angle imaging for mm-level settlement and crack detection. |
| 12 | Polar research station environment monitoring (glacier movement, auroral interference) | same | Long satellite backhaul; local low-power TRP arrays use coherent integration against ionospheric scintillation, delivering <3 mm daily glacier displacement continuity. |

Observation 5: Across air, space, sea, indoor, underground, polar, emergency, agriculture, transport and XR domains, the common pain-points—occlusion, accuracy, coverage, reliability, rapid deployment and cost—point to one solution: in the 6G integrated sensing & communication architecture, mTRP-based multi-static sensing is the only general-purpose technology that simultaneously satisfies “high-precision + wide-coverage + high-reliability + network-native” requirements.

Variety of sensing deployment modes is essential to meet these evolving requirements effectively, the sensing mode needs to be further decomposed so as to accelerate the key-technology study of 6G ISAC.

Proposal 5: The proposed TRP sensing modes for roadmap:

Case #1: Monostatic Sensing with sTRP shared by both transmitter - receiver mapping to the same Cell(PCI).

Case # 2: Bistatic Sensing with sTRP transmitter - sTRP receiver mapping to the same Cell(PCI).

Case # 2a: Bistatic Sensing with sTRP transmitter - sTRP receiver mapping to the different Cell(PCI).

Case #3: Multistatic Sensing with sTRP transmitter - mTRP receiver mapping to the same Cell(PCI).

Case #3a: Multistatic Sensing with sTRP transmitter - mTRP receiver mapping to different Cell(PCI).

Case #4: Multistatic/Massivestatic Sensing with sTRP transmitter - massiveTRP receiver mapping to the same Cell(PCI).

Case #4a: Multistatic/Massivestatic Sensing with mTRP transmitter - massiveTRP receiver mapping to the same Cell(PCI).

Case #4b: Multistatic/Massivestatic Sensing with mTRP transmitter - massiveTRP receiver mapping to the same Cell(PCI).

Case #5: Sensing Mobility (Sensing TRP Switching).

- Conclusions

In this contribution, the proposals of 6GR sensing/ISAC study, our views are given:

Proposal 1: Based on above motivation oberservations, we propose that 6GR Sensing study should consider the following aspects:

- 	build ISAC native architecture and key technologies upon the 5G NR sensing baseline;

- 	extend sensing modalities—especially the convergence of communications and sensing under mTRP deployment patterns;

-	co-design waveform, frame structure and resource scheduling with 6GR communications.

Proposal 2:Building upon the established sensing paradigms of monostatic, bistatic, and multistatic sensing, we propose to extend the taxonomy with well-defined sub-modes. These sub-modes will be explicitly mapped to mTRP and Cell associations pattern, enabling a co-design approach that natively supports ISAC, and will also serve as the basis for sensing roadmap and study plan in well-defined phases.

Proposal 3: It is recommended that the sTRP transmitter - mTRP receiver sensing mode be prioritized as a high-priority study item and co-designed with the 6GR MIMO communication framework, with RAN1 taking the lead. Channel modelling evaluation[5] with the corresponding Sensing transmitters and receivers properties is the prerequisite for the study.

Proposal 4: It is recommended to include Sensing With Random Signals as a key candidate technology on the Rel-20 study agenda, RAN1 will take the lead. 6G sensing architecture studies need to incorporate data-compression mechanisms, algorithms, and procedures. RAN3 will take the lead and work closely with RAN1, RAN2, SA2, and other relevant WGs.

Proposal 5: The proposed TRP sensing modes for roadmap:

Case #1: Monostatic Sensing with sTRP shared by both transmitter - receiver mapping to the same Cell(PCI).

Case # 2: Bistatic Sensing with sTRP transmitter - sTRP receiver mapping to the same Cell(PCI).

Case # 2a: Bistatic Sensing with sTRP transmitter - sTRP receiver mapping to the different Cell(PCI).

Case #3: Multistatic Sensing with sTRP transmitter - mTRP receiver mapping to the same Cell(PCI).

Case #3a: Multistatic Sensing with sTRP transmitter - mTRP receiver mapping to different Cell(PCI).

Case #4: Multistatic/Massivestatic Sensing with sTRP transmitter - massiveTRP receiver mapping to the same Cell(PCI).

Case #4a: Multistatic/Massivestatic Sensing with mTRP transmitter - massiveTRP receiver mapping to the same Cell(PCI).

Case #4b: Multistatic/Massivestatic Sensing with mTRP transmitter - massiveTRP receiver mapping to the same Cell(PCI).

Case #5: Sensing Mobility (Sensing TRP Switching).

- References

- RP-251891, DRAFT Meeting Report for TSG RAN meeting #108, June 2025

- RP-251881, New SID: Study on 6G Radio, RAN#108, June 2025.

- 3GPP TS 22.137, Service requirements for Integrated Sensing and Communication; Stage 1 (Release 19), V19.1.0, 2024

- RP-251892, Revised agenda for RAN #109, Sep. 2025

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz; Release 19, V19.0.0 , 2025

- R1-2401937, Final Report of 3GPP TSG RAN WG1 #116 v1.0.0, RAN1#116, Feb. 2024

- R1-2403821, Final Report of 3GPP TSG RAN WG1 #116b v1.0.0, RAN1#116b, Apr. 2024