3GPP TSG RAN Meeting #109								RP-252149

Beijing, China, September 15-18, 2025

Agenda Item:		9.6.3.2

Source:			ZTE Corporation, NEC

Title:				WI summary: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

WI code(s):			NR_AIML_NGRAN_enh-Core

leading WG:		RAN3

Release:			Rel-19

1	Introduction

This work item [1] is to specify new AI/ML-based use cases (i.e., Network Slicing and Coverage and Capacity Optimization) and to introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

2	Description

The work item “NR_AIML_NGRAN_enh-Core” specified the following key functionalities:

AI/ML enabled Network Slicing:

The feature enables the data collection requesting and reporting between gNBs to support AI/ML enabled Network Slicing.

The predicted radio resource status per slice, predicted slice available capacity and per slice UE performance can be configured to be collected and measured by a gNB by using the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

AI/ML enabled Coverage and Capacity Optimization (CCO):

The feature enables that the gNB derives predicted CCO issues and the corresponding future coverage status for its affected cells and beams, and notify its neighbour gNBs about the future coverage state changes together with modification cause and time information through NG-RAN Node Configuration Update procedure. In case of split architecture, a gNB-DU may receive the predicted CCO assistance information including a predicted CCO issue, the predicted affected cells and beams and the time when the predicted CCO issue will happen. The gNB-DU may indicate to the gNB-CU a future coverage configuration for its cells and beams; a gNB-DU may also receive the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU a future coverage configuration for its cells and beams, and an associated time and cause.

R18 Leftovers:

Split Architecture:

The feature enables the support of data collection in split architecture.

With respect to Network Energy Saving, the gNB-DU can report the Energy Cost (EC) to gNB-CU upon request through the Resource Status Reporting Initiation procedure, and the Resource Status Reporting procedure.

Regarding the UE performance feedback, Packet Delay measurement collection for UE performance from gNB-DU is reported to gNB-CU-UP through the ASSISTANCE INFORMATION DATA frame. PDCP Average UE DL/UL packet loss from gNB-CU-UP is configured and reported through the Data Collection Reporting Initiation procedure and the Data Collection Reporting procedure.

Mobility Optimization for NR-DC:

The feature enables that the Master Node (MN) configures the collection and reporting of UE performance measurements from the Secondary Node (SN) after successful SN addition or SN change. The MN can request the UE performance feedback from the SN through the Data Collection Reporting Initiation procedure, while the actual reporting from SN is through the Data Collection Reporting procedure.

Continuous MDT Collection:

The feature enables the measurement continuity to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states, and trace correlation to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE. OAM configures management-based MDT for continuous MDT towards the NG-RAN and provides specific information to NG-RAN for NG-RAN to identify continuous MDT.

3	References

[1]	RP-251245, Revised WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2]	RP-252150, Status report for Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC