3GPP TSG RAN Meeting #109 	RP-252161

Beijing, China, September 15-18, 2025

Title:	RAN4 led NTN topics for 5G Advanced Rel-20

Source: 	Thales, Fraunhofer IIS, Fraunhofer HHI

Type:	discussion

Document for:	discussion

Agenda Item:	10.1.4.6 - REL-20 proposals led by RAN4/ NTN topics

# Introduction

At RAN#108 meeting, it was agreed (see RP-251801 “Moderator's summary for RAN4 led REL-20 topics in RAN#108”) that the RAN4 5G-Advanced Rel-20 work plan would include the following NTN enhancements. However, the exact scope and WID draft will be discussed and decided in RAN#109

  - NTN HPUE

    - Finish remaining PC1.5 (~29 dBm) work for hand-held devices for both NR NTN and IoT NTN (several studies and discussions already done in R19)

      - NR NTN with 2TX and/or FFS 1TX

  - NTN HD-FDD VSAT for Ka/Ku bands

    - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm).

    - Once the mobile VSAT with small antennas work is done,

      - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable

    - RAN1/2 impact is expected and will be further discussed in RAN#109

      - Leverage RedCap over NTN HD-FDD solution to NTN as applicable.

  - NTN DL CA

    - Intra-SAN CA limited to DL intra-band contiguous and non-contiguous with 2CC

      - Example bands will be further decided within L/S-band and Ku/Ka-band as example bands

      - Depending on the available band combinations, interested companies can propose to extend this work to intra-band with more than 2CC in the check-point in June 2026

    - RAN1/2 impact is expected and will be further discussed in RAN#109

  - Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements

    - 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3

      - Take one example band with minimum work with PC3

      - The rest cases can be considered as part of spectrum work once the requirements on the example band is completed

- Target 1TU in Q4’25 and 1.5 TU allocation including RF, RRM and demod from Q1’26

In line with the above, some of the identified topics are further detailed in terms of justification and potential objectives.

# Proposed NTN related inputs for the 5GA Rel-20 RAN4 led work item

## NTN HD-FDD VSAT for Ka/Ku band

### Justification

This activity aims at defining the requirements for the support of HD-FDD mode with a new “ultra-small Mobile VSAT” (equivalent antenna size 20 x 20 cm) operating with NGSO constellation (and as a second priority, possibly MEO, GEO/GSO) in above 10 GHz bands allocated to satellite services (i.e. for both Ka and Ku-band), as requested by the verticals in [1].

Note 1: Mobile VSAT are referred in ITU-R as Earth Station in Motion (ESIM) which is defined as “an Earth station that is placed on a moving platform, such as a ship at sea, a moving train, or an aircraft in flight” [2]

HD-FDD is an enabler of significantly reduced VSAT implementation complexity and supports both the ‘ultra small’ form-factor and simplifies operation in the Ku band when using the 12.75 – 13.25 GHz Ku band uplink [Note 2].

Note 2: The 12.75 GHz uplink (earth to space) starting frequency is adjacent to the Ku band downlink (space to earth) range 10.7 - 12.75 GHz.

Ultra-small Mobile VSAT is intended to be installed on a wide variety of moving platforms, including aircraft, ships, and different types of land vehicles (commercial and domestic), with stringent size, weight and power constraints to support data-rich and reliable connections. This VSAT type is expected to be implemented as a flat panel conformal / low profile antenna able to generate electronically steered beams to track the orbiting satellites.

Two duplexing modes are considered:

- Priority 1: HD-FDD mode: single panel providing non-simultaneous 1 Rx and 1 Tx functionality

- Priority 2: FDD mode: which would require at least 1 Rx and 1 Tx. In case there are 2 independent Rx beams, one beam can be used for connectivity with the serving cell (1Rx receiving user data) and the other Rx beam used for measurement (e.g., pre-synch with the target SSB) purposes of target satellite. (1 Rx measuring).

The maximum speed shall be up to 1500 km/h (maximum relative speed with respect to the ground for commercial aircraft) as explained per [1]. The connectivity shall cover at least LEO@600km and LEO@1200km NGSO types.

Note 2: The Release 18 NTN work items scope for operation above 10 GHz, does not support Mobile VSATs (ESIMs) connected to NGSO Satellites in 3GPP previous releases [3]. However, since the WRC-2023, such mobile VSAT can operate with NGSO in all portions of Ka band and Ku band.

Related references:

- [1] RP-242638, Market considerations on Mobile VSAT operating with NGSO/GSO from verticals point view, Source: Airbus, Thales et all.

- [2] Satellite issues: Earth Station in Motion (ESIM) https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx

- [3] 3GPP TS 38.101-5 v18.7.0 NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements.

### Objectives

#### Core part

The objectives of core part for the introduction of HD-FDD mode for an ultra-small Mobile VSAT type (equivalent aperture 20 x 20 cm) as requested in [1] UEs for NR_NTN (Non-Terrestrial Networks) operating with NGSO in satellite service allocated bands in above 10 GHz:

- Specify the support of HD-FDD mode with NR-NTN at VSAT UE level. Assess whether RedCap over NTN HD-FDD solution to NTN mechanisms can be leveraged (RAN1, RAN2)

- Specify RF requirements for the “ultra-small” Mobile VSAT for NR NTN in all Ka-bands (e.g. n512, n511, n510) and all Ku-bands (e.g. n5xx, n2xx) allocated to NTN satellite services

  - Necessary mobile VSAT Tx/Rx requirements (if any)

    - to define a new VSAT Type for the ultra-small Mobile VSAT communicating with LEO (and possibly MEO, GEO/GSO), in both Ku and Ka-band, based on the RF characteristics of VSAT types defined in Rel-19 (See table 9.2.1.0-1 in TS 38.101-5)

      - 2 duplex modes may be considered: HD-FDD (Priority 1) and FDD (Priority 2) mode

    - Note 1: The minimum performance values from existing VSAT Types are including, but are not limited to:

        - Minimum peak EIRP

        - Minimum OTA Rx sensitivity

        - Minimum Pointing/Tracking accuracy

  - Note 2: Other mobile VSAT UE types (if any) and their respective characteristics to be discussed in RAN4

- Specify RRM Requirements as appropriate

  - Ultra-small Mobile VSAT configuration assumption with 1Tx & 1Rx (e.g. for land/maritime vehicle applications) as well as 1Tx & 2Rx (e.g. for aeronautical applications)

#### Performance part

Upgrade performance part for ultra-small Mobile VSAT and HD-FDD mode as necessary.

### Working groups

- Lead WG: RAN4

## Intra SAN Carrier aggregation (at both SAN and UE level)

### Justification

When NR support for Non-Terrestrial Networks (NTN) was introduced in Release 17, support for Carrier Aggregation (CA) was not included in the work scope.

As a result of that, this feature is not available for new NR NTN planned for the near future. As we are nearing deployment of new constellations, the criticality of this features is becoming more and more apparent.

Furthermore, the lack of Carrier Aggregation support for NTN creates a significant feature gap and disparity with the current NR terrestrial solution, and may thus potentially result in a disparity with some satellite systems being planned or deployed.

There are also a number of other technical aspects that strongly justify the introduction of CA.  For example, initial channel allocations for NTN may in certain regions be constrained by existing/legacy services, whereby channels may become available in non-contiguous areas of spectrum.

### Objectives

The Core Part objectives are as follows:

- Specify the introduction of Carrier Aggregation (CA) for NR NTN on DL and UL for all NR NTN bands (below 7.125 GHz and above 10 GHz)

  - Focus on the intra-SAN scenario, where component carriers are co-located in the same Satellite Access Node (SAN) and supported by the same payload.

  - First priority: Intra-SAN, DL only, 2 Component Carrier, contiguous for an example band

  - Second Priority: Support intra-SAN contiguous and non-contiguous DL and UL CA for bands required by satellite network operators

- Specify the necessary RF and RRM requirements

### Working groups

Lead WG: RAN4 (addressing both RF/OTA, Demod/RRM)

Contributing WG: RAN1, RAN2

## Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements

### Justification

When NR NTN was first specified in Release 17, certain channel bandwidths, namely CHBW above 20 MHz, were not specified, primarily due to a concern regarding lack of time during Release 17, and due to the mistaken assumption that NTN operators would not have access to blocks of spectrum larger than 20 MHz.

The latter statement, in particular, is not true, since:

- Some NTN operators in S-band have access to the full 30 MHz of band n256 in certain regions, and some potential allocations of additional S-band elsewhere in the world (e.g. South/Central America) are leading to up to 35 MHz of total available spectrum, which in some instances may be allocated to a single operator.

- In Release 19, there was an operator demand from several operators to introduce new L-band definitions to combine the Extended L-band (n253) with the Standard L-band (n255), giving rise to bands with a total combined DL of up to 41 MHz (i.e. n251, n250).  Some NTN operators have access to the full 41 MHz of L-band spectrum in certain regions and are wishing to utilize it.

- In some instances, operators may opt to pool the spectrum beyond the allocation of any given single operator in a given geographical area, by operating a common shared network, thus improving flexibility and spectral efficiency.

Lastly, even when an operator may have access to interrupted blocks of spectrum across the full band (e.g. 41 MHz), the deployment of a larger channel encompassing the full band would allow an operator to flexibly schedule users across the block, even in sub-blocks less than the minimum 3 MHz required for a single standalone channel to be deployed.  This also helps improved spectrum efficiency by minimizing guard bands and giving more flexibility in the dynamic allocation of resources.

Later on in Rel-17 and early Rel-18, in order to comply with the submission requirements for ITU-R IMT-2020 Satellite Self-Evaluation, support for 30 MHz channel BW was introduced, but with a reduced and incomplete set of requirements, such that some companies requested that a NOTE be added, clarifying that further requirements were needed in order to make the 30 MHz CHBW deployable.

Similarly, now that the new n251 and n250 bands have been introduced in Rel-19, the lack of Core requirements for all CHBW from 25 MHz and up has created significant issues in the WI procedures.  During the Ku band Work Item definition, great pushback was received in introducing CHBW above 20 MHz for Ku band in its FR1 configuration, however, CHBW up to at least 50 MHz are critical for Ku band operators.

It is thus very urgent to complete the Core requirements for the missing CHBWs, namely 30, 40 and possibly 25, 35, 45 and 50 MHz, wherever they are missing, to allow in particular UE implementations to be completed and be able to support such operator deployments.

### Objectives

The objectives of the core part are as follows:

Specify the missing core UE and SAN for the following FR1 Channel BWs: 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3 (Note 1)

  - Specify RF, RRM, Performance/Demodulation and Conformance requirements as needed

  - Take the work in NR_NTN_Ku_bands into account, to avoid any duplication.

Note 1:

- Take one example band with minimum work with PC3

- The rest cases can be considered as part of spectrum work once the requirements on the example band is completed

### Working groups

Lead WG: RAN4

# Conclusion and Way forward

Proposal 1: RAN to discuss at next plenary candidate RAN4 led NR-NTN topics as part of the 5G Advanced Rel-20 work plan in decreasing order of priority:

- Support of HD-FDD with ultra-small Mobile VSAT: definition of additional type in above 10 GHz band

- Intra SAN Carrier aggregation (at both SAN and UE level)

- Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements

END