3GPP TSG RAN Meeting #109								RP-252165

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	Xiaomi

Title:	Discussion on 6G communication deployment scenarios

Document for:	Discussion and Decision

- Introduction

In RAN#107 meeting, the following objectives were added in the updated SID [1]:

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

In RAN#108 meeting, deployment scenarios for 6GR were discussed, with several communication deployment scenarios agreed and captured in TR 38.914 section 4 [2]. While the structure of the deployment scenarios has been established, the details of some deployment scenarios remain under investigation. Notably, the deployment scenario for Non-Terrestrial Network lacks descriptions.

In this contribution, our views on the issues of the 6GR deployment scenarios are given.

- Deployment scenarios for TN

In RAN plenary meeting #108, 6G deployment scenarios e.g., indoor hotspot (InH), dense urban (DU), rural, urban macro (UMa), sub-urban macro (SMa), etc. were discussed, and attributes including carrier frequency, aggregated system bandwidth, layout, ISD, user distribution and speed, etc. has been captured in TR38.914. However, attributes on BS antenna elements and UE antenna elements are still under investigation.

BS antenna elements

Beamforming gain is positively proportional to the number of antennas. As an evolution and a further enhancement of 5G NR, more antennas are expected for larger beamforming gain to compensate the severe propagation loss in 6G. At BS side, for carrier frequencies around 700MHz, e.g., in rural and SMa scenarios, up to 64 antennas can be considered; for carrier frequencies around 2GHz and 4GHz, up to 256 antennas can be considered; for carrier frequencies around 7GHz and 30GHz, up to 1024 antennas can be considered.

Proposal 1: At BS side,

- for carrier frequencies around 700MHz, up to 64 antennas can be considered;

- for carrier frequencies around 2GHz and 4GHz, up to 256 antennas can be considered;

- for carrier frequencies around 7GHz and 30GHz, up to 1024 antennas can be considered.

UE antenna elements

For UE antenna elements, as different device types may have different UE antenna configurations, considering only the maximum number of UE antenna number of all types of devices may not be a reasonable assumption. Instead, the minimum technique performance requirements shall be able to be satisfied by the most popular 6GR user devices, i.e. handheld UE. Therefore, the assumption on the maximum number of UE antenna elements shall consider the antenna configuration of handheld UEs as the baseline.

Proposal 2: The assumption of maximum number of UE antenna elements shall consider the antenna configuration of handheld UEs as the baseline.

Different from the BS, only a few antennas can be equipped at the handheld UE due to its limited size and power. Scaling up the number of TX antennas requires additional PA and RF chains, and will introduce issues such as mutual coupling interference, thermal management, power efficiency degradation, and hardware costs. Furthermore, more spectrum bands need to be supported by 6GR handheld UEs, which will inevitably increase the difficulty of RF design considering constraints of size, cost,  and antenna isolation, etc. Finally, owing to the size limitation, the channel matrix corresponding to a single UE (SU) is still not at all independently and identically distributed (i.i.d.) and the number of spatially multiplexable layers is limited.

Comprehensively considering the necessity of multiantenna deployment for a handheld UE and its restrictions in respects of physical size and power consumption, it is preferred to consider 1 Tx and 2 Rx antennas (i.e., 1T2R) for carrier frequencies around 700MHz and 2GHz, and consider 2 Tx and 4 Rx antennas (i.e., 2T4R) for carrier frequencies around 4GHz and 7GHz.

Proposal 3: For handheld UE,

- consider 1T2R for carrier frequencies around 700MHz and 2GHz,

- consider 2T4R for carrier frequencies around 4GHz and 7GHz.

- Deployment scenarios for NTN

As per outcome of the current ongoing RAN plenary study on 6G scenarios and requirements, TR 38.914 lacks input on NTN deployment scenario. As per footnote under section 4, subsection 4.10 covering NTN deployment scenario needs further input in aspects including carrier frequency, aggregated system bandwidth, orbit/payload information, UE distribution and service profile.

In RAN1#122 meeting, RAN WG-level SI on 6G radio, namely 6GR, study was kicked off. The following agreements were made on NTN regarding evaluation aspects.

Conclusion

- Template in R1-2506582 is to be used for collecting inputs from companies.

  - Additional NTN or TN assumptions, if any, or any necessary change of the parameters, are to be incorporated into the updated one of R1-2506582.

Since RAN1 has scheduled a post meeting discussion on left over evaluation assumptions including those specific to NTN, the NTN carrier frequency had better be settled from RAN plenary to facilitate the evaluation assumption discussion to be carried out in Q4.

From RAN1 evaluation perspective, once carrier frequency is settled, the follow up evaluation parameters on satellite/UE antenna setting, EIPR setting and aperture setting and channel model assumption will be determined based on TR 38.821 and TR 38.811

Observation 1: From both RAN SI study on usage scenario and RAN1 evaluation perspective, NTN carrier frequency had better be settled during this plenary meeting.

Carrier frequencies around 2GHz for both DL and UL as well as around 20 GHz DL and 30GHz UL have been widely evaluated since 5G and deployed across the world for handheld UE and VSAT UE respectively. From carrier frequency perspective, these values are likely to be re-used for 6G. The Duplex mode is FDD for these bands as per the legacy duplex type setting for these bands.

For system bandwidth, throughout the RAN1 evaluations in both system level calibration and link budget analysis, the bandwidth of up to 2*30MHz has been widely used in 2GHz for both DL and UL and 2*400MHz has been widely used in 20GHz DL and 30GHz UL. Thus we propose re-using the typical evaluation assumption values as system bandwidth settings for the relevant carrier frequencies.

For other aspects including satellite orbits, UE distribution, UE mobility, satellite architecture, we think re-using the TR 38.913 settings are sufficient.

For service profile, since video streaming can be categorized as delay tolerant services, we suggest re-using the TR 38.913 6.1.12 service profile texts for the time being. Depending on further input from ongoing study from ITU-R WP4B on IMT-2030 Future Technology Trends or  Framework as well as RAN plenary further study, revision based on this version can be made as needed.

Proposal 4: At least consider two carrier frequencies including around 2GHz DL/UL and 20GHz DL/30GHz UL for NTN. The accompanying UE types are handheld and VSAT respectively. The duplex mode is FDD.

Proposal 5: Adopt the following TP to section 4.10 in TR 38.914

4.10 Non-Terrestrial Network ***Unchanged Omitted*** This deployment scenario is defined to allow for the provision of services for those areas where the terrestrial service is not available and also for those services that can be more efficiently supported by the satellite systems such as broadcasting service. Satellite acts as a fill-in especially on roadways and rural areas where the terrestrial service isn’t available. The supported services via the Satellite system are not limited to just data and voice, but also for others such as machine type communications, broadcast and other delay tolerant services. Some of its attributes are listed in Table 4.10-1. Table 4.10-1: Examples for Satellite Deployment Attributes; Deployment-1; Deployment-2. Carrier Frequency NOTE1; Around 2 GHz for both DL and UL; Around 20 GHz for DL Around 30 GHz for UL. Duplexing; FDD; FDD. Satellite architecture; Bent-pipe On-Board Processing; Bent-pipe, On-Board Processing. System Bandwidth (DL + UL); Up to 2*30 MHz; Up to 2*400 MHz. Satellite Orbit; GEO, LEO; LEO, MEO, GEO. UE Distribution; 100% Outdoors; 100% Outdoors. UE Mobility; Fixed, Portable, Mobile NOTE3; Fixed, Portable, Mobile. ***Unchanged Omitted***

- Conclusion

In this contribution, the deployment scenarios for 6GR TN and NTN are discussed, and the following observation and proposals are made:

Observation 1: From both RAN SI study on usage scenario and RAN1 evaluation perspective, NTN carrier frequency had better be settled during this plenary meeting.

Proposal 1: At BS side,

- for carrier frequencies around 700MHz, up to 64 antennas can be considered;

- for carrier frequencies around 2GHz and 4GHz, up to 256 antennas can be considered;

- for carrier frequencies around 7GHz and 30GHz, up to 1024 antennas can be considered.

Proposal 2: The assumption of maximum number of UE antenna elements shall consider the antenna configuration of handheld UEs as the baseline.

Proposal 3: For handheld UE's,

- consider 1T2R for carrier frequencies around 700MHz and 2GHz,

- consider 2T4R for carrier frequencies around 4GHz and 7GHz.

Proposal 4: At least consider two carrier frequencies including around 2GHz DL/UL and 20GHz DL/30GHz UL for NTN. The accompanying UE types are handheld and VSAT respectively. The duplex mode is FDD.

Proposal 5: Adopt the following TP to section 4.10 in TR 38.914

4.10 Non-Terrestrial Network Editor note: orbit type and payload type will be captured in section 4.10 ***Unchanged Omitted*** This deployment scenario is defined to allow for the provision of services for those areas where the terrestrial service is not available and also for those services that can be more efficiently supported by the satellite systems such as broadcasting service. Satellite acts as a fill-in especially on roadways and rural areas where the terrestrial service isn’t available. The supported services via the Satellite system are not limited to just data and voice, but also for others such as machine type communications, broadcast and other delay tolerant services. Some of its attributes are listed in Table 4.10-1. Table 4.10-1: Examples for Satellite Deployment Attributes; Deployment-1; Deployment-2. Carrier Frequency NOTE1; Around 2 GHz for both DL and UL; Around 20 GHz for DL Around 30 GHz for UL. Duplexing; FDD; FDD. Satellite architecture; Bent-pipe On-Board Processing; Bent-pipe, On-Board Processing. System Bandwidth (DL + UL); Up to 2*30 MHz; Up to 2*400 MHz. Satellite Orbit; GEO, LEO; LEO, MEO, GEO. UE Distribution; 100% Outdoors; 100% Outdoors. UE Mobility; Fixed, Portable, Mobile NOTE3; Fixed, Portable, Mobile. ***Unchanged Omitted***

- References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, Inc., Deutsche Telecom, Mar. 12-14, 2025

- 3GPP TR 38.914 v 0.1.0, Study on 6G Scenarios and requirements, Jun. 2025