3GPP TSG RAN #109	RP-252172

Beijing, China, 15th – 18th September, 2025

Agenda item:			8.2.1.2

Source:	MediaTek Inc.

Title:		IMT-2030 proposals and TR 38.914 revisions

Document for:					Discussion and Decision

# Introduction

This contribution makes proposals for the remaining outstanding IMT-2030 TPRs. It also proposes some additions/modifications to existing text for KPI clauses agreed at RAN#108.

# Follow up for ITU-R TPR definitions/target values

At RAN#108, progress was made on recommended TPRs and target values towards ITU-R. In the tables 1 and 2 below, we make proposals on the pending aspects.

Table 1: Conventional TPRs

| TPR | Proposed Requirement | Evaluation Assumptions used to achieve targets |
| --- | --- | --- |
| Peak data rate (Gbps) | 1.5 x IMT-2020 | 7GHz. DL: 8Rx + 1024QAM, UL: 4Tx + 256QAM |
| Peak spectral efficiency (bps/Hz) | 1.5 x IMT-2020 | 7GHz with practical Bandwidths, not 5MHz. DL: 8Rx + 1024QAM, UL: 4Tx + 256QAM |
| Average spectral efficiency (bps/Hz) | Dense Urban/InH: 2.8 x IMT2020 UL Rural: TBD | For DU/InH: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx |
| 5th percentile user spectral efficiency (bps/Hz) | Dense Urban/InH: 2 x IMT2020 UL Rural: TBD | For DU/InH: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx |
| User experienced data rate (Mbps) – change title? | 3 x IMT2020 for DL 2 x IMT2020 for UL in up to 40MHz | For DU: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx (assuming no BW increase) |
| Area traffic capacity (Mbps/m2) | 3 x IMT2020 DL (for InH) 2.8 x IMT-2020 UL (for InH) | For InH: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx (assuming no BW increase) |
| Mobility Interruption Time | 10ms If 0ms is agreed, restrict to multi-carrier/TRP operation | Handover with single TRP/carrier operation |
| NOTE: Applicable device antenna assumptions may be limited by form-factor, complexity and operational constraints of the device. |  |  |

Table 2: New TPRs

| TPR | Proposed Definition |
| --- | --- |
| Sustainability (EE) | Network EE: “Quantify the relative improvement in energy consumption for a given system loading (unloaded, partially loaded) vs the energy consumption in fully loaded scenario.” Device EE: “Quantify the relative improvement in Device EE for a given traffic loading (zero load, partial load) vs energy consumption in fully loaded scenario. Proponents to report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving operation.” No target value for ITU-R, just report what is achieved. |
| Composite Requirement for Immersive Communication | Add “…for Immersive Communication” to the TPR name. “The connections per TRxP, determined by 95% of satisfied users determined in DL and UL independently, where each satisfied user fulfils the requirements of the following traffic profiles: - For DL: 30Mbps data rate (single stream), 60 frames/second, 62.5kB packet size, 10ms packet delay budget, 1% PER - For UL: 10Mbps data rate (single stream), 60 frames/second, 20.833kB packet size, 30ms packet delay budget, 1% PER” |
| Positioning | “The RIT shall support the following basic performance: - Horizontal accuracy of <50 metres for 80% of users and Vertical accuracy of <5 metres for 80% of users. Proponents shall also describe how the RIT [is forwards compatible / may be evolved in future to support the following improved performance requirements: - For indoor deployments: Horizontal accuracy of 1 metre for 90% of users and vertical accuracy of 3 metres for 90% of users. - For outdoor deployments: Horizontal accuracy of 10 metres for 80% of users and vertical accuracy of 3 metres for 80% of users.” Scenarios: Reuse evaluation scenarios from TR 38.855 and Rel-17 TR 38.857. |
| Coverage | Link Budget evaluation – same as IMT2020. No target value. |
| Sensing capabilities | Qualitative (note that 3GPP could provide quantitative performance results based on its own target metrics in addition to description). “Describe the ability of the RIT to support Sensing capability, considering design metrics such as Detection Probability, Object Location Accuracy and False Alarm Probability for a selected use case/environment.” |

Proposal 1: Agree to the proposals in Table 1 and 2 in this document for the still undecided IMT-2030 TPRs.

# 3	Modifications to agreed text in TR38.914

At RAN#108, text was agreed in relation to a subset of KPIs that are relevant for IMT-2030. However, we believe further clarifications are required for some of those clauses, articulating around the guiding principles of a) simplexity and b) practical user experience for the 6G studies [2, 3].

- Average spectral efficiency: 3GPP drafting rules do not allow footnotes, so footnote is modified to a NOTE.

- User Plane latency: In our view, while the target requirement is acceptable for IMT-2030 purposes, we believe that more importance needs to be placed on aspects such as robust performance for latency-bound traffic and the realistic impacts and constraints in relation to scheduling.

- Control Plane latency: The sentence about “encouraging to target 10ms” seems to have been copied from ITU-R documentation (given that it refers to “proponents”). It is not a requirement and we have seen no justification in 3GPP to agree on this, so we propose to remove it.

- Reliability: The text in the last sentence seems to have been copied from ITU-R documentation (given that it refers to “proponents”). It is not a requirement and not has been justified in 3GPP. Therefore, we propose to remove it. For clarification, we do believe high reliability for larger packet sizes would be practically useful, but not tied to a 1ms latency requirement.

Proposal 2: Agree to the text proposals to clauses 5.1.5, 5.1.7, 5.1.8 and 5.1.10 of the 3GPP TR 38.914 in the ANNEX of this document to address the issues raised in this document.

# 4	Proposals

The following is proposed:

Proposal 1: Agree to the proposals in Table 1 and 2 in this document for the still undecided IMT-2030 TPRs.

Proposal 2: Agree to the text proposals to clauses 5.1.5, 5.1.7, 5.1.8 and 5.1.10 of the 3GPP TR 38.914 in the ANNEX of this document.

# 5	References

[1]	3GPP TR 38.914 v0.1.0 “Study on 6G Scenarios and Requirements”.

# ANNEX		: Text Proposal to TR38.914

Below we propose a TP to 38.914 to incorporate the proposals made in section 3 of this document.

----------------------------- START OF CHANGES -----------------------------------

### 5.1.5	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

NOTE:	Average spectral efficiency corresponds to “spectrum efficiency” in Recommendation ITU R M.2160.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user I (downlink) or from user I (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency for various [test environments] are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – IC | 27 | TBD |
| Dense Urban – IC | 23.4 | TBD |
| Rural – IC | TBD | TBD |

----------------------------- NEXT CHANGE -----------------------------------

### 5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

–	4ms for IC

–	1ms for HRLLC

The above assumes unloaded conditions (i.e. a single user) for small IP packets (e.g. 0 byte payload + IP header), for both downlink and uplink.

The following practical aspects shall be key focus areas for 6G latency design:

-	Ability to guarantee robust latency performance for practical packet sizes including for users at the 5% point of the CDF for latency (to be evaluated via the Composite requirement).

-	The impact of components such as HARQ, scheduling delays and support (e.g. SR/BSR) for uplink scheduling under typical assumptions (duplex mode constraints, dynamic scheduling) on latency shall be taken into account in the design.

### 5.1.8	Control plane latency

Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE).

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is 20 ms.

----------------------------- NEXT CHANGE -----------------------------------

### 5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario.

The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Urban Macro-HRLLC [test environment], assuming small application data (e.g. 20 bytes application data + protocol overhead).

----------------------------- END OF CHANGES -----------------------------------

-