3GPP TSG RAN Meeting #109	                               RP-252178

Beijing, China, September 15-18, 2025

Source: 	Pengcheng Laboratory

Title:	Requirements for fully-decoupled 6G RAN architecture

Agenda item:	8.2.1.3

Document for:	Discussion

- Introduction

In TR 38.913[1], the requirements of architecture and migration of 5G RAT are captured, including the interworking between 5G and LTE, and functions to be supported by 5G RAN.

In this contribution we provide text proposals for the 3GPP TR 38.914 on the architecture and migration of 6G, with special focus on a flexible 6G RAN architecture design to address potential new scenarios and emerging requirements.

- Discussion

The 6G RAN should support flexible RAN architecture deployments and enable flexible designs to enhance spectrum utilization and provide more efficient control and data transmission methods. It should also prioritize energy efficiency for both the network and user equipment (UE), incorporate flexible mechanisms to efficiently utilize diverse and fragmented spectrum resources, support diverse devices, improve mobility management, and accommodate to various deployment scenarios (e.g., converged TN and NTN) and use cases.

The design concept of Fully-Decoupled RAN (FD-RAN) architecture, which incorporates decoupled control and (user-plane) data TRPs as well as decoupled uplink and downlink TRPs, can provide more flexible deployments, improve spectrum utilization, enable more flexible control and data transmission methods, and significantly reduce network cost and energy consumption [2][3].

Observation 1: Decoupling control link and data link is beneficial for:

- network and UE energy saving

Data link can independently enter into sleep mode when data traffic is low, thus saving network energy.

Various mechanisms can be adopted to optimize control link for UE, e.g., using dedicated low power RF and fewer antennas, and operating on a low frequency band (e.g., 700 MHz), such that UE energy can be saved.

- large coverage of network

Control link can operate on low frequency band to enhance coverage.

- mobility

Control link can help assist the handover of data link.

- separate optimization of control signalling and data transmission

Basically, they have diverse objectives: control requires low latency and high reliability, and user-plane data requires high data rate. Optimization can be done on both PHY and L2/L3 technologies.

- distributed MIMO

Control link can help alleviate the overheads between the UE and multiple TRPs.

Observation 2: Decoupling uplink and downlink is beneficial for:

- network and UE energy saving

Uplink and downlink can enter into sleep mode separately, thereby providing more opportunities for saving energy.

UE can associate with a more efficient uplink (not the same as downlink), e.g., a closer TRP or a lower frequency carrier.

- UL coverage enhancement

Free association of UL and free deployment of UL TRPs/carriers can help enhance UL coverage.

- lower deployment cost

The cost of UL-only TRPs is lower than a full-function TRP and a DL-only TRP.

- asymmetric UL/DL traffic

For IoT/LPWA devices and UAVs (as UE), uplink traffic is usually dominant. Deploying more UL TRPs/carriers can better support these scenarios.

- spectrum utilization

Fragmented spectrum can be aggregated and flexibly switched for uplink or downlink.

- distributed MIMO and MU-MIMO

For uplink, joint reception can be considered with multiple UL TRPs at different locations.

For downlink, the effective coverage of DL TRP becomes larger without the limitations from uplink. Thus, there will be more joint transmission and MU-MIMO opportunities.

- TN and NTN integration

For example, for low-altitude UAVs at around 1km height, uplink can use TN and downlink can use NTN.

Observation 3: The above decoupled design can be based on NR CA/DC mechanisms for easy transition purpose.

Proposal 1: 6G RAN architecture should support decoupled control link and data link.

Proposal 2: 6G RAN architecture should support decoupled uplink and downlink.

- Text Proposal for TR 38.914

Based on [1], we propose the following text proposal for TR 38.914 for Requirements for architecture and migration of 6G Radio Access Technologies.

# X	Requirements for architecture and migration of 6G Radio Access Technologies

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The RAN architecture shall support decoupled control link and data link.

-	The RAN architecture shall support decoupled uplink and downlink.

-	The RAN architecture shall support flexible spectrum utilization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall allow integrated sensing and communication function(s).

-	The RAN architecture shall support connection through TN or NTN.

-	The design of the RAN architecture shall enable lower CAPEX/OPEX.

- Conclusion

In this contribution, we propose to add the text proposal to TR 38.914.

- References

- 3GPP TR 38.913 V18.0.0 (2024-03) Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 18).

- FuTURE FORUM, Fully-decoupled Radio Access Network Architecture White Paper（2025）

- J. Chen, X. Liang, J. Xue, Y. Sun, H. Zhou and X. Shen, "Evolution of RAN Architectures Toward 6G: Motivation, Development, and Enabling Technologies," in IEEE Communications Surveys & Tutorials, vol. 26, no. 3, pp. 1950-1988, thirdquarter 2024, doi: 10.1109/COMST.2024.3388511.