3GPP TSG RAN Meeting #109								     RP-252197

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Xiaomi

Title:	Discussion on the requirements for IoT towards 6G

Document for:	Discussion and Decision

- Introduction

In RAN#106 meeting, RAN plenary study item on “Study on 6G scenarios and requirements” [1] was approved. In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [2] was approved.

This contribution discusses about the views on IoT design towards 6G requirements of new and existing services.

- 6G IoT design

Internet-of-Things (IoT) is developed for many years and the relative branches of technologies have been evolving for several releases in 3GPP. In 4G LTE and 5G NR, IoT technologies includes NB-IoT, LTE MTC, LTE Cat. 1/1b/4, NR RedCap, NR eRedCap. In 4G and 5G, IoT technologies were designed from Day-2, which means that the basic system is firstly designed to support normal UE, and then the enhancement, such as complexity reduction, coverage enhancement and power consumption reduction, were designed based on the basic system to support IoT UE. This procedure can cause potential fragmentation and complication of the system design, and limitation of IoT design. According to the SID [2], 6GR targets scalable and forward compatible design for diverse device types. It is important to take IoT device into account in 6G Day-1 design, targeting a common system design for both IoT and non-IoT devices.

Figure 1: Support of IoT devices

Proposal 1: 6GR should support diverse device types, including IoT devices, with single framework from Day 1.

The requirement on 6G IoT can consider several aspects including bandwidth, duplex operation, peak data rate and energy aspect.

Bandwidth

It was agreed in RAN1#122 to study a scalable 6GR design for diverse device types. The bandwidth of low-tier 6GR device type(s) could potentially impact 6GR physical design e.g. common channel/signal designs.

In RAN1#122 meeting, the following agreements was achieved.

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

The bandwidths of NB-IoT and eMTC are 180kHz and 1.4MHz respectively. For NR RedCap and eRedCap, the UE RF bandwidth is 20 MHz while the BB bandwidths are 20MHz and 5MHz respectively.

For 6GR low-tier IoT device types, among the above options, we think the smallest maximum supported UE BW should be no smaller than 5MHz, i.e. at least Option 1 should not be considered. First of all, 3MHz compared with 5MHz is not expected to provide meaningful UE complexity reduction and/or power saving. In addition, 3MHz UE BW would unnecessarily restrict the physical layer design of channels and signals leading to performance degradation of common channels/signals.

Regarding whether the UL BW can be different from DL BW, and whether RF and BB UE BW are same or different can be left to RAN1 for further study.

Duplex mode

FDD spectrum is suitable for IoT deployment considering the lower frequency with better coverage performance. From UE complexity point of view, half-duplex FDD can further reduce the UE complexity compared with FD-FDD and the reduced data rate is expected to be acceptable to IoT devices. TDD can also be considered if low-tier 6G IoT devices are also expected to operate in TDD bands.

Peak data rate

The peak data rate of eRedCap is 10 Mbps for both UL and DL. Considering the reduced bandwidth of 6G IoT of 5 MHz, a reasonable lower peak data rate can be expected. Higher data rate can provide beneficial performance of IoT system, too low data rate may not satisfy the 6G expectation and requirement on diverse scenarios. 5~10 Mbps can be studied for 6G IoT design, and an even higher data rate can also be considered.

Energy aspect

5G works on the power saving for UE and network in several releases. Power consumption relative aspects are always popular topic in IoT. 6G should also consider about how to minimize the power consumption and prolong battery life of IoT device. Regarding power saving, 6G can consider to reuse existing mechanism or study new mechanism for IoT device. Furthermore, energy charging, such as RF resource, can be considered and studied as a supplementary method to extend the battery life of IoT devices.

Proposal 2: To satisfy diverse IoT requirements, following aspects need to be considered:

- The smallest maximum bandwidth for the 6GR lowest-tier IoT device should be no less than 5MHz

  - It is up to RAN1 to further discuss whether the RF and BB UE BW are same or different.

- Half-duplex FDD should be considered for 6G IoT devices and TDD can also be considered

- Peak data rate, e.g., 5~10 Mbps can be studied for 6G IoT design to satisfy diverse requirements from industry, and avoid the impact on system performance

- Energy efficiency, such as power saving mechanism and energy charging based on RF resource, can be considered and studied as a supplementary method to extend the battery life of IoT devices.

- Conclusion

This contribution discusses about the views on IoT design towards 6G requirements of new and existing services, and the following proposals are concluded:

Proposal 1: 6GR should support diverse device types, including IoT devices, with single framework from Day 1.

Proposal 2: To satisfy diverse IoT requirements, following aspects need to be considered:

- The smallest maximum bandwidth for the 6GR lowest-tier IoT device should be no less than 5MHz

  - It is up to RAN1 to further discuss whether the RF and BB UE BW are same or different.

- Half-duplex FDD should be considered for 6G IoT devices and TDD can also be considered

- Peak data rate, e.g., 5~10 Mbps can be studied for 6G IoT design to satisfy diverse requirements from industry, and avoid the impact on system performance

- Energy efficiency, such as power saving mechanism and energy charging based on RF resource, can be considered and studied as a supplementary method to extend the battery life of IoT devices.

- References

[1] RP-243327, “New SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

[2] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)