3GPP TSG RAN #109	RP-252209

Beijing, China, Sep 15th – 18th, 2025

Agenda item:	8.2.1.3

Source:	CSCN

Title:	Motivation for Introducing Multi-Satellite and Satellite-Terrestrial Collaborative Services for NTN

Document for:	Discussion

Introduction

To realize the 6G vision of a seamlessly integrated Terrestrial and Non-Terrestrial Network (TN/NTN), it is crucial to move beyond single-link dependency and address its inherent limitations in reliability and performance. This contribution introduces Dual Connectivity (DC) as a foundational architectural principle, proposing its application in both multi-satellite (NTN-NTN) and hybrid satellite-terrestrial (NTN-TN) service configurations. These deployment scenarios are beneficial to providing resilient, high-performance connectivity. However, such configurations introduce challenges that conventional DC mechanisms are not optimized. We therefore aim to initiate the discussion on defining these critical collaborative service and studying the essential enhancements required for the 6G system.

Discussion

To achieve the 6G vision of ubiquitous connectivity and the seamless integration of Terrestrial and Non-Terrestrial Networks (TN/NTN), the inherent limitations of a single-satellite link must be addressed. A single link, from a single satellite access node including (NGSO and GSO), represents a fundamental bottleneck and a single point of failure. It is constrained by several factors, including:

- limited visibility windows;

- lower stability of satellite-to-ground link, particularly susceptibility to weather or terrain blockage;

- finite link capacity;

- limited capability of satellite platforms, including power supply from solar panels, thermal dissipation capacity, and the processing capability of space-grade processors.

Collectively, these constraints render a single link insufficient for guaranteeing the Quality of Service (QoS) required by demanding 6G applications.

Observation 1: A single-satellite link, constrained by limited capacity, reliability, and on-board resources, is a fundamental bottleneck that cannot alone meet the high-performance demands of 6G NTN services.

Therefore, to ensure the required levels of service robustness, continuity, and peak throughput, it is imperative to enable a single UE to leverage the resources of more than one access node concurrently. This dual-connectivity (DC) capability, encompassing both multi-satellite and hybrid satellite-terrestrial configurations, is not merely an enhancement but a foundational requirement for building a resilient and high-performance 6G architecture.

In the multi-satellite case, an instance is to use a lower frequency band as a robust primary link for coverage, signaling and control, while a higher frequency band acts as a secondary link for high data throughput. For satellite-terrestrial cooperation, the satellite can act as a Master Node to provide a wide, continuous coverage layer ensuring service availability, while a terrestrial node serves as a Secondary Node to opportunistically boost data rates, thus significantly balancing coverage and capacity and enhancing mobility robustness by reducing handover interruption probability and delay. Furthermore, this satellite-terrestrial cooperation should support inter-RAT dual connectivity, enabling NTN devices to leverage the vast and mature deployments of terrestrial 4G/5G networks.

Observation 2: The introduction of collaborative service supporting both NTN-NTN and NTN-TN dual connectivity is a fundamental necessity for building a resilient, high-performance, and truly integrated 6G architecture.

Proposal 1: It is proposed to define a 6G NTN collaborative service scenario in TR 38.914 characterized by a single UE being served cooperatively by more than one satellite nodes. This service scenario shall encompass various inter-orbit combinations (e.g., LEO-MEO, LEO-GSO, or LEOs in identical/different orbital planes).

Proposal 2: It is proposed to define a Satellite-Terrestrial (NTN-TN) Dual Connectivity service scenario in TR 38.914, characterized by a single UE being served concurrently by both a satellite node (acting as a wide-coverage anchor) and a terrestrial node (acting as a high-capacity booster), where the scenario shall include support for inter-RAT connectivity.

Defining NTN-involved dual-connectivity scenarios has profound implications for the required technical design of the 6G system. The primary benefit is achieving higher reliability and throughput via link diversity and resource aggregation, ensuring service continuity during handovers and mitigating link degradation.

However, these service scenarios introduce significant design challenges that must be characterized to guide future 6G development. Both concurrent links will often be highly asymmetric. In the NTN-NTN case, links will differ significantly in propagation delay, Doppler shift, and path loss. This asymmetry becomes even more extreme in the NTN-TN case, with vast differences in latency and jitter between the terrestrial and satellite links. This dynamic and heterogeneous nature implies that the 6G system must support mobility and cross-link resource management capabilities substantially enhanced beyond those used in terrestrial DC. Characterizing this asymmetry within the deployment scenarios is critical for driving the subsequent study of enhanced RRC, MAC schedulers, and data splitting functionalities tailored for these high-dynamics environments.

Observation 3: Conventional Dual Connectivity mechanisms, designed for the relatively symmetric and stable link conditions of Terrestrial Networks, are insufficient to manage the significant and highly dynamic link asymmetry inherent to both multi-satellite and satellite-terrestrial scenarios.

Proposal 3: It is proposed that essential enhancements to the dual-connectivity framework be studied and specified to address the unique challenges of NTN-involved connectivity, particularly concerning the management of high-mobility and asymmetric link characteristics.

Conclusion

Observation 1: A single-satellite link, constrained by limited capacity, reliability, and on-board resources, is a fundamental bottleneck that cannot alone meet the high-performance demands of 6G NTN services.

Observation 2: The introduction of collaborative service supporting both NTN-NTN and NTN-TN dual connectivity is a fundamental necessity for building a resilient, high-performance, and truly integrated 6G architecture.

Observation 3: Conventional Dual Connectivity mechanisms, designed for the relatively symmetric and stable link conditions of Terrestrial Networks, are insufficient to manage the significant and highly dynamic link asymmetry inherent to both multi-satellite and satellite-terrestrial scenarios.

Proposal 1: It is proposed to define a 6G NTN collaborative service scenario in TR 38.914 characterized by a single UE being served cooperatively by more than one satellite nodes. This service scenario shall encompass various inter-orbit combinations (e.g., LEO-MEO, LEO-GSO, or LEOs in identical/different orbital planes).

Proposal 2: It is proposed to define a Satellite-Terrestrial (NTN-TN) Dual Connectivity service scenario in TR 38.914, characterized by a single UE being served concurrently by both a satellite node (acting as a wide-coverage anchor) and a terrestrial node (acting as a high-capacity booster), where the scenario shall include support for inter-RAT connectivity.

Proposal 3: It is proposed that essential enhancements to the dual-connectivity framework be studied and specified to address the unique challenges of NTN-involved connectivity, particularly concerning the management of high-mobility and asymmetric link characteristics.

- Annex 1

=======================Start of the changes======================

8	Requirements for architecture and migration

-	The RAN architecture shall support deployment scenarios for inter-satellite cooperative dual connectivity.

-	The RAN architecture shall support dual connectivity between two satellite nodes, both of which are based on a regenerative payload (on-board processing) architecture.

-	The RAN architecture shall support an interface enabling effective interaction between the two satellite nodes.

-	The RAN architecture shall support dual connectivity cooperation between satellite nodes in intra-orbit, inter-orbit, and different altitude orbits (e.g., LEO-MEO, LEO-GEO).

-	The RAN architecture shall, for inter-satellite cooperative scenarios, only support intra-6G RAT dual connectivity.

-	The RAN architecture shall support deployment scenarios for satellite-terrestrial cooperative dual connectivity.

-	The RAN architecture shall support dual connectivity where the satellite node is based on a regenerative payload (on-board processing) architecture.

-	The RAN architecture shall support an interface enabling effective interaction between the satellite node and the terrestrial node.

-	The RAN architecture shall support dual connectivity between LEO/MEO/GEO satellite nodes and terrestrial nodes.

-	The RAN architecture shall, for satellite-terrestrial cooperative scenarios, support intra- and inter-RAT dual connectivity, where the satellite node is based on 6G RAN, and the terrestrial node may be based on LTE, NR, or 6G RAN.

=======================End of the changes=======================