# 3GPP TSG RAN Meeting #109																		RP-252216

# Beijing, China, September 15-18, 2025

Source:	Qualcomm

Title:	pCR on RAN Deployment Scenarios

Document for:	Approval

Agenda item:	8.2.1.1

Spec:	3GPP TR 38.914

Abstract

This document proposes changes to TR 38.914, including:

- Introductory text / remark about Deployment Scenarios (sec. 4)

- Changes to the Deployment scenarios tables, including

  - adding 15GHz Carrier Frequency, for Indoor, Dense Urban and Urban Macro

  - removing some multi-band scenarios, considered unnecessary

  - removing pointers to missing “NOTEx”

Proposed Changes

* * * First Change * * * *

# 4.	Deployment scenarios

Editor note: Sensing related aspects will be included in section 4

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD/ Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in the new RAT study item.

The deployment scenarios in this section are defined for the sole purpose of evaluating RAN performance requirements or features, not intended to mandate actual deployments or network/device implementation.

Note: Regarding the various carrier frequency configurations and options listed in the tables below, only a sub-set of them will be required to be evaluated for ITU IMT-2030 self-evaluation, others may be optionally used for specific RAN features or requirements evaluation.

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) Around 15 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) NOTE: co-located layout is assumed for Around 4GHz+Around 7GHz |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | TBD |
| UE antenna elements | TBD |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz |
| Aggregated system bandwidth | Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 15 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid NOTE: co-located layout is assumed for Around 4GHz+Around 7GHz Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | TBD |
| UE antenna elements | TBD |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

* * * Next Change * * * *

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz |
| Aggregated system bandwidth | Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 15 GHz: Up to [400] MHz (DL+UL) NOTE3 Around 30 GHz: [1]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid NOTE: co-located layout is assumed for Around 4GHz+Around 7GHz Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | TBD |
| UE antenna elements | TBD |
| User distribution and UE speed | TBD [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz |
| System bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) |
| Layout | Single layer - Hex. Grid NOTE: co-located layout is assumed for Around 4GHz+Around 7GHz |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| TRP antenna elements | TBD |
| UE antenna elements | TBD |
| User distribution and UE speed | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h] [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |