3GPP TSG RAN Meeting #109	RP-252232

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	Futurewei

Title:	Urban Coverage for Massive Connection

Document for:	Discussion and Decision

# Introduction

TR38.914 [1] contains high-level descriptions of deployment scenarios, with more detailed attributes deferred to the WG study. This contribution contains a proposal for the “Urban Coverage for Massive Connection” deployment scenario.

# Discussion

Many of the deployment scenarios from TR38.914 have been carried over from TR38.913 [2], including “Urban coverage for massive connection”. This scenario is designed for the evaluation of the KPI of connection density, and the (already agreed) KPI text is defined for this scenario:

### 5.1.9	Connection density

Connection density is the number of connections fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is 106 connections per km2.

Similarly to the KPI, the scenario description should be copied over (and potentially simplified) from TR38.913. Aspects unrelated to the connection density evaluation should not be included, maintaining a clear connection between the scenario, the KPI, and the ITU evaluation of mMTC connection density.

Proposal 1: TR38.914 section 4.8 Urban coverage for massive communication is for the evaluation of the KPI of connection density, with baseline text taken from TR38.913 section 6.1.7.

### 6.1.7	Urban coverage for massive connection

The urban coverage for massive connection scenario focuses on large cells and continuous coverage to provide mMTC. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas, with very high connection density of mMTC devices. This deployment scenario is for the evaluation of the KPI of connection density.

Some of its attributes are listed in Table 6.1.7-1.

Table 6.1.7-1: Attributes of urban coverage for massive connection

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700MHz, 2100 MHz as an option |
| Network deployment including ISD | Macro only, ISD = 1732m, 500m |
| Device deployment | Indoor, and outdoor in-car devices |
| Maximum mobility speed | 20% of users are outdoor in cars (100km/h) or 20% of users are outdoors (3km/h) 80% of users are indoor (3km/h) Users dropped uniformly in entire cell |
| Service profile | Non-full buffer with small packets |
| BS antenna elements | 2 and 4 Rx ports (8 Rx ports as optional) |
| UE antenna elements | 1Tx |

One observation is that the TR38.913 text only contains attributes of the UE antenna elements on the TX side. One addition, still in line with the clear purpose of evaluating connection density, would be to add UE antenna attributes on the RX side as well.

Proposal 2: Consider adding UE antenna attributes on the RX side for urban coverage for massive connection.

# Conclusion

The following observations and proposals were discussed in this contribution.

Proposal 1: TR38.914 section 4.8 Urban coverage for massive communication is for the evaluation of the KPI of connection density, with baseline text taken from TR38.913 section 6.1.7.

Proposal 2: Consider adding UE antenna attributes on the RX side for urban coverage for massive connection.

# References

TR38.914, “Study on 6G Scenarios and Requirements.”

TR38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies.”