3GPP TSG-RAN5 Meeting #108	R5-254478

# Bengaluru, India, August 25-29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 1063 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test point analysis for Rel-16 n71 related EN-DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Several Rel-16 n71 related EN-DC configurations are to be introduced into test specification. This CR is to add test point analysis for these EN-DC configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding TP analysis for spurious emissions and reference sensitivity for these EN-DC configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testing for these EN-DC configurations can’t be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1, 4.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Add attached .zip file to TR 38.905: “38.521-3_TpAnalysisSpur(DC_1A_n71A).zip” “38.521-3_TpAnalysisSpur(DC_3A_n71A).zip” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 EN-DC test cases in TS 38.521-3 [4] clause 7. The analyses are performed per EN-DC configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration | Single UL | Refsens exception, victim band (NOTE 2) | Requirement coverage (NOTE 2) | Comments |
| --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 | Only single UL requirements defined | 5 | Exception | RAN5#92-e |
| 12 | Only single UL requirements defined | 12 | Exception | RAN5#92-e |
| 38 | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| 41 | Yes | N/A | NE | RAN5#88-e |
| 48 | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| 71 | Yes | 71 | Exception | RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_3A_n3A | Yes | 3 | Exception | RAN5#88-e |
| DC_5A_n5A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_7A_n7A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_41A_n41A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_48A_n48A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_66A_n66A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_71A_n71A | Only single UL requirements defined | N/A | NE | RAN5#92-e |
| DC_XA_nYA (NOTE 1) |  |  |  |  |
| DC_1A_n3A | Yes | 1 (IMD3), CBI | NE, IMD, CBI, CBI avoid | RAN5#89-e |
| DC_1A_n7A | No | N/A | NE | RAN5#94-e |
| DC_1A_n8A | No | 1 (IMD4) | NE, IMD | RAN5#95-e |
| DC_1A_n28A | No | 1 (HD3) | NE, HD, HD avoid | RAN5#98 |
| DC_1A_n41A | No | 1 (CBI, PC2&PC3), n41 (CBI) | NE, CBI | RAN5#98 |
| DC_1A_n71A | No | n71 (HM) | NE, HM, HM avoid | RAN5#108 |
| DC_1A_n77A | No | n77 (HD2), 1 (IMD2, PC2&PC3), 1 (IMD4, PC2&PC3) | NE, HD, IMD | RAN5#104 |
| DC_1A_n78A | No | 1 (IMD4) | NE, IMD | RAN5#88-e |
| DC_1A_n79A | No | N/A | NE | RAN5#103 |
| DC_2A_n41A | No | 2 (CBI) | NE, CBI (PC2&PC3) | RAN5#106 |
| DC_2A_n66A | No | 2 (IMD3), n66 (IMD5) | NE, IMD | RAN5#94-e |
| DC_2A_n71A | No | 2 (HD3), n71 (HM) | NE, HD, HM | RAN5#93-e |
| DC_2A_n77A | No | n77 (HD), 2 (HM), 2 (IMD2, PC2&PC3), 2 (IMD4, PC2&PC3), 2 (IMD5), 2 (CBI, PC2) | NE, HD, HD avoid, HM, HM avoid, IMD, CBI | RAN5#103 |
| DC_2A_n78A | No | n78 (HD2), 2 (IMD2), 2 (IMD4) | NE, HD, IMD | RAN5#94-e |
| DC_3A_n1A | Yes | n1 (IMD3), 3 (CBI) | NE, IMD, CBI, CBI avoid | RAN5#107 |
| DC_3A_n5A | Yes | 3 (IMD4), n5 (IMD2) | NE, IMD | RAN5#94-e |
| DC_3A_n8A | No | 3 (IMD5), n8 (IMD4) | NE, IMD | RAN5#96-e |
| DC_3A_n28A | No | N/A | NE | RAN5#95-e |
| DC_3A_n71A | No | N/A | NE | RAN5#108 |
| DC_3A_n77A | No | n77 (HD2), 3 (HM, PC2&PC3), 3 (IMD2), 3 (IMD4) | NE, HD, HD avoid HM, HM avoid, IMD | RAN5#106 |
| DC_3A_n78A | Yes | 3 (IMD2, PC2&PC3), 3 (IMD4, PC2&PC3), 3 (HM), n78 (HD2) | NE, IMD, HD, HD avoid, HM, HM avoid | RAN5#106 |
| DC_3A_n79A | No | N/A | NE | RAN5#103 |
| DC_5A_n66A | Yes | 5 (IMD2) | NE (anchor agnostic), IMD | RAN5#93-e |
| DC_5A_n77A | No | n77 (HD), 5 (IMD4, PC2&PC3), 5 (IMD5) | NE, HD, HD avoid, IMD | RAN5#94-e |
| DC_5A_n78A | No | n78 (HD4), 5 (IMD4) 5 (HM) | NE, HD, HD avoid, IMD HM, HM avoid | RAN5#106 |
| DC_7A_n1A | No | N/A | NE | RAN5#90-e |
| DC_7A_n3A | No | 7 (IMD4¦3*fB3-1*fB7¦) | NE, IMD | RAN5#90-e |
| DC_7A_n5A | Yes | n5 (IMD3) | NE, IMD | RAN5#94-e |
| DC_7A_n8A | No | N/A | NE | RAN5#95-e |
| DC_7A_n28A | No | N/A | NE | RAN5#94-e |
| DC_7A_n66A | No | 7 (IMD4) | NE, IMD | RAN5#99 |
| DC_7A_n71A | No | 7 (HD) | NE, HD, HD avoid | RAN5#99 |
| DC_7A_n77A | No | 7 (IMD4) | NE, IMD | RAN5#99 |
| DC_7A_n78A | No | CBI | NE, CBI | RAN5#94-e |
| DC_8A_n1A | No | n1 (IMD4 ¦2*fB1-2*fB8¦) | NE, IMD | RAN5#90-e |
| DC_8A_n3A | No | 8 (IMD4 ¦3*fB8-1*fB3¦), n3 (IMD5 ¦4*fB8-1*fB3¦) | NE, IMD | RAN5#90-e |
| DC_8A_n20A | Yes | 8 (IMD3), n20 (IMD3) | NE, IMD | RAN5#94-e |
| DC_8A_n28A | No | N/A | NE | RAN5#95-e |
| DC_8A_n41A | No | n41 (HD3), 8 (IMD3) | NE HD, ,HD avoid IMD | RAN5#98 |
| DC_8A_n77A | No | 8 (HD4), 8 (IMD4) | NE, HD, HD avoid, IMD | RAN5#92-e |
| DC_8A_n78A | No | n78 (HD4), 8 (IMD4) | NE, HD, HD avoid, IMD | RAN5#96-e |
| DC_8A_n79A | No | n79 (HD5), 8 (IMD5) | NE, HD, HD avoid, IMD | RAN5#96-e |
| DC_11A_n77A | No | N/A | NE | RAN5#91-e |
| DC_11A_n78A | No | N/A | NE | RAN5#91-e |
| DC_11A_n79A | No | 11 (HM) | NE, HM, HM avoid | RAN5#92-e |
| DC_12A_n2A | No | N/A | NE | RAN5#98 |
| DC_12A_n66A | No | n66 (HD3) | NE, HD, HD avoid | RAN5#97 |
| DC_12A_n78A | Yes (DC_12_n78) | n78 (HD5), 12 (IMD5) | NE, HD, HD avoid, IMD | RAN5#98 |
| DC_13A_n77A | No | n77 (HD), 13 (HM), 13 (IMD5, PC2&PC3) | NE, HD, HD avoid, HM, HM avoid, IMD | RAN5#94-e |
| DC_14A_n2A | No | N/A | NE | RAN5#98 |
| DC_18A_n77A | No | n77 (HD5), 18 (IMD4), 18 (IMD5) | NE, HD, IMD | RAN5#104 |
| DC_18A_n78A | No | 18 (IMD4) | NE, IMD | RAN5#104 |
| DC_19A_n77A | No | n77 (HD5), n77 (HD4) | NE, HD | RAN5#99 |
| DC_19A_n78A | No | n78(HD4), 19(HM, PC2&PC3), 19(IMD4, PC2&PC3) | NE, HD, HM, IMD | RAN5#102 |
| DC_19A_n79A | No | 19 (HM ¦5*fB19DL-1*fB79¦, PC2&PC3) | NE, HM, HM avoid | RAN5#101 |
| DC_20A_n1A | No | N/A | NE | RAN5#90-e |
| DC_20A_n3A | No | n3 (IMD4), 20 (IMD4) | NE, IMD | RAN5#89-e |
| DC_20A_n7A | DC_20_n7 | 20 (IMD3) | NE, IMD | RAN5#94-e |
| DC_20A_n8A | DC_20_n8 | 20 (IMD3), n8 (IMD3) | NE, IMD | RAN5#95-e |
| DC_20A_n78A | No | 20 (IMD4), n78 (HD) | NE, IMD, HD, HD avoid | RAN5#88-e |
| DC_21A_n28A | Yes | 21 (IMD5) | NE, IMD | RAN5#99 |
| DC_21A_n78A | No | N/A | NE | RAN5#103 |
| DC_21A_n79A | No | 21 (HM, PC2&PC3), 21 (IMD3, PC2&PC3) | NE, HM, HM avoid, IMD | RAN5#101 |
| DC_25A_n41A | No | 25 | CBI | RAN5#98 |
| DC_26A_n41A | No | n41 (HD3), 26 (IMD3), 26 (HM ¦3*fB26DL-1*fB41¦) | NE, HD, HD avoid, IMD, HM | RAN5#106 |
| DC_26A_n77A | No | n77 (HD4), 26 (IMD4) | NE, HD, HD avoid, IMD | RAN5#92-e |
| DC_26A_n78A | No | n78 (HD4), 26 (IMD4) | NE, HD, HD avoid, IMD | RAN5#92-e |
| DC_26A_n79A | No | 26 (HM ¦5*fB26DL-1*fB79¦) | NE, HM, HM avoid | RAN5#106 |
| DC_28A_n5A | No | 28 (CBI) | NE, CBI, CBI avoid | RAN5#107 |
| DC_28A_n7A | No | N/A | NE | RAN5#94-e |
| DC_28A_n77A | No | n77 (HD), n77 (HM), 28 (IMD5, PC2&PC3) | NE, HD, HD avoid, HM, IMD | RAN5#106 |
| DC_28A_n78A | No | n78 (HD), 28 (IMD5) | NE, HD, HD avoid, IMD | RAN5#106 |
| DC_30A_n66A | No | n66 (CBI) | NE, CBI | RAN5#97 |
| DC_38A_n78A | No | 38 (CBI) | NE, CBI | RAN5#99 |
| DC_39A_n41A4 | No | n41 (CBI) | NE, CBI | RAN5#104 |
| DC_40A_n1A | No | N/A | NE | RAN5#89-e |
| DC_40A_n41A4 | No | 40 (CBI) | NE, CBI | RAN5#107 |
| DC_40A_n78A | No | 40 (HM) | NE, HM, HM avoid | RAN5#89-e |
| DC_40A_n79A | No | N/A | NE | RAN5#94-e |
| DC_41A_n28A | No | N/A | NE | RAN5#98 |
| DC_41A_n77A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦, PC2&PC3), 41 (CBI, PC2&PC3), n77 (CBI) | NE, HM, CBI | RAN5#106 |
| DC_41A_n78A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB78¦), 41 (CBI), n78 (CBI) | NE, HM, CBI | RAN5#106 |
| DC_42A_n77A | Only single UL requirements defined | N/A | NE | RAN5#91-e |
| DC_48A_n66A | No | 48 (HD2), 66 (IMD5) | NE (anchor agnostic), HD, IMD | RAN5#92-e |
| DC_66A_n2A | DC_66_n2 | n2 (IMD3), 66 (IMD5) | NE (anchor agnostic), IMD | RAN5#93-e |
| DC_66A_n5A | No | n5 (IMD2) | NE, IMD | RAN5#97 |
| DC_66A_n25A | No | 66, n25 (IMD3), 66 (IMD5) | NE, IMD | RAN5#99 |
| DC_66A_n41A | No | 66 (CBI) | NE, CBI | RAN5#98 |
| DC_66A_n71A | No | 66 (IMD4) | NE (anchor agnostic), IMD | RAN5#93-e |
| DC_66A_n77A | No | n77 (HD), 66 (IMD2, PC2&PC3), 66 (IMD5, PC2&PC3), 66 (CBI, PC2) | NE, HD, HD avoid, IMD, CBI | RAN5#103 |
| DC_66A_n78A | No | n78 (HD2), 66 (IMD5) | NE (anchor agnostic), HD, IMD | RAN5#93-e |
| DC_71A_n2A | No | n2 (HD3), 71 (HM¦3*fB71DL-1*fB2¦) | NE (anchor agnostic), HD, HM, HM avoid | RAN5#107 |
| DC_71A_n66A | No | n66 (IMD4) | NE (anchor agnostic), IMD | RAN5#98 |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 HD avoid: single carrier requirements apply with aggressor of UL Harmonic Distortion active. HM avoid: single carrier requirements apply with aggressor of RX Harmonic Mixing active. CBI avoid: single carrier requirements apply with aggressor of Cross Band Isolation active. NOTE 3: HM cannot be avoided with aggressor still active since CBI exception always exists. Therefore, no HM avoid test point is added. NOTE 4: Exception for this configuration is tested only when the UE reports supporting Simultaneous Rx/Tx capability. |  |  |  |  |

Table 4.3.3.1-2: Reference Sensitivity test cases per EN-DC configuration (3CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (NOTE 2) | Fallback DC configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | NE | DC_(n)41AA DC_41A_n41A | RAN5#89-e |
| 41 | Yes | DC_41A_n41A | N/A | NE | DC_(n)41AA DC_41A_n41A | RAN5#89-e |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | NE | DC_(n)48AA DC_48A_n48A | RAN5#89-e |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | NE | DC_(n)48AA DC_48A_n48A | RAN5#89-e |
| DC_XA_nYC |  |  |  |  |  |  |
| DC_3A_n78C | Yes | DC_3A_n78A | N/A | NE |  | RAN5#93-e |
| DC_XA_nY(2A) |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nYA-nZA |  |  |  |  |  |  |
| DC_1A_n3A-n78A | Yes | DC_1A_n3A | n78 (3 band IMD2) | NE, IMD |  | RAN5#107 |
|  | No | DC_1A_n78A | n3 (3 band IMD2) | NE, IMD |  | RAN5#107 |
| DC_1A_n28A-n78A | No | DC_1A_n28A | n78 (3 band IMD3) | NE, IMD |  | RAN5#92-e |
|  | No | DC_1A_n78A | n28 (3 band IMD5) | NE, IMD |  | RAN5#92-e |
| DC_1A_n78A-n79A | No | DC_1A_n78A | n79 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#103 |
|  | No | DC_1A_n79A | n78 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#103 |
| DC_3A_n28A-n78A | No | DC_3A_n28A | n78 (3 band IMD5) | NE, IMD |  | RAN5#92-e |
|  | Yes | DC_3A_n78A | No 3CC exception | No test required |  |  |
| DC_3A_n78A-n79A | Yes | DC_3A_n78A | n79 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#106 |
|  | No | DC_3A_n79A | n78 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#106 |
| DC_7A_n5A-n78A | Yes | DC_7A_n5A | No 3CC exception | No test required |  | RAN5#94-e |
|  | No | DC_7A_n78A | No 3CC exception | No test required |  | RAN5#94-e |
| DC_7A_n28A-n78A | No | DC_7A_n28A | n78 (3 band IMD2) | NE, IMD |  | RAN5#92-e |
|  | No | DC_7A_n78A | n28 (3 band IMD2) | NE, IMD |  |  |
| DC_19A_n78A-n79A | No | DC_19A_n78A | n79 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#106 |
|  | No | DC_19A_n79A | n78 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#106 |
| DC_20A_n28A-n78A | No | DC_20A_n28A | n78 (3 band IMD4) | NE, IMD |  | RAN5#93-e |
|  | No | DC_20A_n78A | n28 (3 band IMD4) | NE, IMD |  | RAN5#93-e |
| DC_21A_n78A-n79A | No | DC_21A_n78A | n79 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#103 |
|  | No | DC_21A_n79A | n78 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#103 |
| DC_28A_n7A-n78A | No | DC_28A_n7A | n78 (3 band IMD2) | NE, IMD |  | RAN5#94-e |
|  | No | DC_28A_n78A | n7 (3 band IMD2) | NE, IMD |  | RAN5#94-e |
| DC_XA-YA_nZA |  |  |  |  |  |  |
| DC_1A-3A_n28A | No | DC_1A_n28A | 3 (3 band IMD5) | IMD |  | RAN5#92-e |
|  | No | DC_3A_n28A | 1 (3 band IMD4) | IMD |  | RAN5#92-e |
| DC_1A-3A_n41A | No | DC_1A_n41A | No 3CC exception | No test required |  | RAN5#98 |
|  | No | DC_3A_n41A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-3A_n71A | No | DC_3A_n71A | 1 (3 band IMD4) | IMD |  | RAN5#108 |
| DC_1A-3A_n77A | Yes | DC_1A_n77A | 3 (3 band IMD2, PC2&PC3) 3 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#104 |
|  | Yes | DC_3A_n77A | 1 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#104 |
| DC_1A-3A_n78A | No | DC_1A_n78A | 3 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#102 |
|  | Yes | DC_3A_n78A | 1 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-3A_n79A | No | DC_1A_n79A | No 3CC exception | No test required |  | RAN5#101 |
|  | No | DC_3A_n79A | 1(3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-7A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_7A_n3A |  |  |  |  |
| DC_1A-7A_n28A | No | DC_1A_n28A | 7 (3 band IMD2) | IMD |  | RAN5#92-e |
|  | No | DC_7A_n28A | No 3 band exception | No test required |  | RAN5#92-e |
| DC_1A-7A_n78A | No | DC_1A_n78A | 7 (IMD4 ¦3*fB1-1*fB78¦) | IMD |  |  |
|  | No | DC_7A_n78A | 1 (IMD4 ¦2*fB7-2*fB78¦) | IMD |  |  |
| DC_1A-8A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_8A_n3A |  |  |  |  |
| DC_1A-18A_n77A | No | DC_1A_n77A | 18 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#105 |
|  | No | DC_18A_n77A | 1 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#105 |
| DC_1A-19A_n78A | No | DC_1A_n78A | 19 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_19A_n78A | 1 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-19A_n79A | No | DC_1A_n79A | 19 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_19A_n79A | 1 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-20A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_20A_n3A |  |  |  |  |
| DC_1A-20A_n8A | No | DC_1A_n8A | 20 (3 band IMD4) | IMD |  | RAN5#95-e |
|  | Yes | DC_20A_n8A | No 3CC exception | - |  | RAN5#95-e |
| DC_1A-20A_n28A | No | DC_1A_n28A | No 3CC exception | No test required |  | RAN5#93-e |
|  | No | DC_20A_n28A | No 3CC exception | No test required |  | RAN5#93-e |
| DC_1A-20A_n78A | No | DC_1A_n78A | 20 (IMD5) | IMD |  |  |
|  | No | DC_20A_n78A | 1 (IMD3) | IMD |  |  |
| DC_1A-21A_n78A | No | DC_1A_n78A | 21 (3 band IMD2, PC2&PC3) 21 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_21A_n78A | 1 (3 band IMD2, PC2&PC3) 1 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-21A_n79A | No | DC_1A_n79A | No 3CC exception | NE |  | RAN5#103 |
|  | No | DC_21A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_1A-28A_n3A | Yes | DC_1A_n3A | 1 (3 band IMD4) | IMD | Yes | RAN5#90-e |
|  | No | DC_28A_n3A | No 3CC exception | - | No |  |
| DC_1A-28A_n5A | No | DC_1A_n5A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_28A_n5A |  |  |  |  |
| DC_1A-28A_n78A | No | DC_1A_n78A | 28 (3 band IMD3) | IMD |  | RAN5#92-e |
|  | No | DC_28A_n78A | 1 (3 band IMD5) | IMD | DC_28A_n78A |  |
| DC_1A-41A_n28A | No | DC_1A_n28A | 41 (IMD2 ¦1*fB1+1*fn28¦) | IMD |  | RAN5#98 |
|  | No | DC_41A_n28A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-41A_n41A | No | DC_1A_n41A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-41A_n77A | No | DC_1A_n77A | 41 (IMD4 ¦1*fn77-3*fB1¦, PC2&PC3) 41 (IMD5 ¦3*fB1-2*fn77¦, PC2&PC3) | IMD |  | RAN5#104 |
|  | No | DC_41A_n77A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-42A_n78A | No | DC_1A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_1A-42A_n79A | No | DC_1A_n79A | 42 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_2A-66A_n2A | No | DC_2A_n2A | No 3CC exception | No test required |  | RAN5#104 |
|  | No | DC_66A_n2A | 2 (3 band IMD3) | NE, IMD |  |  |
| DC_2A-66A_n5A | No | DC_2A_n5A | 66 (3 band IMD4) | IMD |  | RAN5#98 |
|  | Yes (DC_66_n5) | DC_66A_n5A | No 3CC exception | No test required |  |  |
| DC_2A_n5A-n77A | No | DC_2A_n5A | n77 (3 band IMD3) | NE, IMD |  | RAN5#101 |
|  | No | DC_2A_n77A | No 3CC exception | NE |  |  |
| DC_2A-13A_n77A | No | DC_2A_n77A | No 3CC exception | NE |  | RAN5#103 |
|  | No | DC_13A_n77A | 2 (3 band IMD3, PC2&PC3) | NE, IMD |  |  |
| DC_2A-66A_n41A | No | DC_2A_n41A | 2 (3 band IMD4) | IMD |  | RAN5#96-e |
|  | No | DC_66A_n41A | No 3CC exception |  |  |  |
| DC_2A-66A_n77A | No | DC_2A_n77A | 66 (3 band IMD2, PC2&PC3), 66 (3 band IMD4, PC2&PC3), 66 (3 band IMD5) | NE, IMD |  | RAN5#103 |
|  | No | DC_66A_n77A | 2(3 band IMD2, PC2&PC3) | NE, IMD |  |  |
| DC_3A-7A_n1A | Yes | DC_3A_n1A | No 3CC exception | No test required |  | RAN5#90-e |
|  | No | DC_7A_n1A | No 3CC exception |  |  |  |
| DC_3A-7A_n5A | Yes | DC_3A_n5A | 7 (3 band IMD2) | IMD |  | RAN5#95-e |
|  | Yes | DC_7A_n5A | No 3CC exception | - |  | RAN5#95-e |
| DC_3A-7A_n8A | No | DC_3A_n8A | 7 (3 band IMD2, IMD3) | IMD |  | RAN5#96-e |
|  | No | DC_7A_n8A | No 3CC exception | No test required |  | RAN5#96-e |
| DC_3A-7A_n28A | No | DC_3A_n28A | 7 (3 band IMD3) | IMD |  | RAN5#92-e |
|  | No | DC_7A_n28A | 3 (3 band IMD2) | IMD |  | RAN5#92-e |
| DC_3A-8A_n28A | No | DC_3A_n28A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_8A_n28A |  |  |  |  |
| DC_3A-18A_n77A | Yes | DC_3A_n77A | No 3CC exception | No test required |  | RAN5#98 |
|  | No | DC_18A_n77A | 3 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#105 |
| DC_3A-19A_n78A | Yes | DC_3A_n78A | No 3CC exception | No test required |  | RAN5#102 |
|  | No | DC_19A_n78A | 3 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
| DC_3A-19A_n79A | No | DC_3A_n79A | 19 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#101 |
|  | No | DC_19A_n79A | 3 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#101 |
| DC_3A-20A_n1 | Yes | DC_3A_n1A | No 3CC exception | No test required |  |  |
|  | No | DC_20A_n1A |  |  |  |  |
| DC_3A-20A_n8A | No | DC_3A_n8A | No 3CC exception | No test required |  | RAN5#96-e |
|  | Yes (DC_20_n8) | DC_20A_n8A | 3 (3 band IMD4) | IMD |  | RAN5#96-e |
| DC_3A-20A_n28A | No | DC_3A_n28A | No 3CC exception | No test required |  | RAN5#92-e |
|  | No | DC_20A_n28A | 3 (3 band IMD4) | IMD |  | RAN5#92-e |
| DC_3A-20A_n78 | Yes | DC_3A_n78A | No 3CC exception |  |  |  |
|  | No | DC_20A_n78A | 3 (3 band IMD3) | IMD |  |  |
| DC_3A-21A_n78 | Yes | DC_3A_n78A | 21 (3 band IMD4, PC2&PC3) 21 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_21A_n78A | 3 (3 band IMD2) | IMD |  | RAN5#101 |
| DC_3A-21A_n79 | No | DC_3A_n79A | No 3CC exception | No test required |  | RAN5#101 |
|  | No | DC_21A_n79A | 3 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
| DC_3A-28A_n78A | Yes | DC_3A_n78A | No 3CC exception |  | DC_3A_n78A | RAN5#92-e |
|  | No | DC_28A_n78A | 3 (3 band IMD3) | IMD | DC_28A_n78A |  |
| DC_3A-40A_n1A | Yes | DC_3A_n1A | 40 (3 band IMD5) | IMD5 if UE supporting dual UL |  |  |
|  | No | DC_40A_n1A | No 3CC exception |  |  |  |
| DC_3A-41A_n28A | No | DC_3A_n28A | 41 (IMD2 ¦1*fB3+1*fn28¦), 41 (IMD3 ¦2*fB3-fn28¦) | IMD |  | RAN5#98 |
|  | No | DC_41A_n28A | 3 (IMD2 ¦1*fB41-1*fn28¦) | IMD |  | RAN5#98 |
| DC_3A-41A_n41A | No | DC_3A_n41A | No 3CC exception | No test required |  | RAN5#98 |
| DC_3A-41A_n77A | Yes | DC_3A_n77A | 41 (IMD5 ¦3*fB3-2*fB77¦, PC2&PC3) | IMD |  | RAN5#105 |
|  | No | DC_41A_n77A | 3 (IMD3 ¦2*fB41-1*fB77¦) | IMD |  | RAN5#105 |
| DC_3A-42A_n78A | Yes | DC_3A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_3A-42A_n79A | No | DC_3A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_7A-8A_n3A | No | DC_7A_n3A | 8 (3 band IMD3) | IMD |  | RAN5#95-e |
|  | No | DC_8A_n3A | 7 (3 band IMD2) 7 (3 band IMD3) | IMD |  | RAN5#95-e |
| DC_7A-20A_n1A | No | DC_20A_n1A | 20 (3 band IMD5) | IMD |  | RAN5#90-e |
|  | No | DC_7A_n1A | No 3CC exception | _ |  |  |
| DC_7A-20A_n3A | No | DC_7A_n3A | 20 (IMD2 ¦1*fB7-1*fB3) | NE, IMD |  |  |
|  | No | DC_20A_n3A | 7 (IMD2 ¦1*fB3+1*fB20) | NE, IMD |  |  |
| DC_7A-20A_n8A | No | DC_7A_n8A | 20 (3 band IMD3) | IMD |  | RAN5#95-e |
|  | Yes | DC_20A_n8A | 7 (3 band IMD3) | IMD |  | RAN5#95-e |
| DC_7A-20A_n28A | No | DC_7A_n28A | No 3CC exception | No test required |  | RAN5#92-e |
|  | No | DC_20A_n28A | 7 (3 band IMD5) | IMD |  |  |
| DC_7A-20A_n78A | No | DC_7A_n78A | 20 (3 band IMD2) 20 (3 band IMD5) | IMD |  |  |
|  | No | DC_20A_n78A | 7 (3 band IMD2) | IMD |  |  |
| DC_7A-28A_n3A | No | DC_7A_n3A | 28 (3 band IMD2) | IMD |  | RAN5#90-e |
|  | No | DC_28A_n3A | 7 (3 band IMD3) | IMD |  |  |
| DC_7A-28A_n5A | Yes | DC_7A_n5A | 28 (3 band IMD5) | IMD |  | RAN5#95-e |
|  | No | DC_28A_n5A | 7 (3 band IMD5) | IMD |  | RAN5#95-e |
| DC_7A-28A_n78A | No | DC_7A_n78A | 28 (3 band IMD2) 28 (3 band IMD5) | IMD |  | RAN5#92-e |
|  | No | DC_28A_n78A | 7 (3 band IMD2) | IMD | DC_28A_n78A |  |
| DC_13A_n2A-n77A | No | DC_13A_n2A | n77 (3 band IMD3) | NE, IMD |  | RAN5#103 |
|  | No | DC_13A_n77A | n2 (3 band IMD3, PC2&PC3) | NE, IMD |  |  |
| DC_13A-66A_n2A | No | DC_13A_n2A | 66 (3 band IMD4) | NE, IMD |  | RAN5#101 |
|  | No | DC_66A_n2A | No 3CC exception | NE |  |  |
| DC_13A-66A_n77A | No | DC_13A_n77A | 66 (3 band IMD3, PC2&PC3) | NE, IMD |  | RAN5#103 |
|  | No | DC_66A_n77A | No 3CC exception | NE |  |  |
| DC_18A-41A_n77A | No | DC_18A_n77A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_41A_n77A | 18 (IMD5 ¦2*fB77-3*fB41¦) | IMD |  |  |
| DC_18A-41A_n78A | No | DC_18A_n78A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_41A_n78A | 18 (IMD5 ¦2*fB78-3*fB41¦) | IMD |  |  |
| DC_19A-21A_n78A | No | DC_19A_n78A | No 3CC exception | No test required |  | RAN5#102 |
|  | No | DC_21A_n78A | 19 (3 band IMD3, PC2&PC3) 19 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#102 |
| DC_19A-21A_n79A | No | DC_19A_n79A | 21 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_21A_n79A | No 3CC exception | No test required |  | RAN5#101 |
| DC_19A-42A_n78A | No | DC_19A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_19A-42A_n79A | No | DC_19A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_21A-42A_n78A | No | DC_21A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_21A-42A_n79A | No | DC_21A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_66A_n2A-n77A | No | DC_66A_n2A | No 3CC exception | NE |  | RAN5#103 |
|  | No | DC_66A_n77A | n2 (3 band IMD2, PC2&PC3) | NE, IMD |  |  |
| DC_66A_n5A-n77A | No | DC_66A_n5A | n77 (3 band IMD3) | NE, IMD |  | RAN5#101 |
|  | No | DC_66A_n77A | No 3CC exception | NE |  |  |
| DC_(n)XAA-nYA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nXA_nYA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

Table 4.3.3.1-3: Reference Sensitivity test cases per EN-DC configuration (4CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (NOTEote 2) | Fallback DC configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XDA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | NE | DC_(n)41CA DC_41C_n41A | RAN5#89-e |
| 41 | Yes | DC_41A_n41A | N/A | NE | N/A | RAN5#89-e |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | NE | DC_(n)48CA DC_48C_n48A | RAN5#89-e |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | NE | N/A | RAN5#89-e |
| DC_XA_nY(2A)-nZA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nYA-nZC |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

For EN-DC configurations affected by exceptions, the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement. This is indicated in Table 4.3.3.1-4 following the frequency relation formulas defined in clause D.2.3. Exceptions in TS 38.101-3 clause 7.3B.2.3.4 (cross band isolation) and 7.3B.2.3.5 (2UL intermodulation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-3.

Table 4.3.3.1-4: Test frequency selection per band pair for bands and test points with exceptions avoidable by test frequency setting (UL harmonics, Rx mixing)

| Band set (Note 2) | Frequency [MHz] | Channel BW [MHz] | Exception type | Comments |
| --- | --- | --- | --- | --- |
| 1A / n3A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | Low/High | 20 / 40 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 1A / n28A | DL 2140 / UL 713 MHz | 20/10 | UL Harmonic Interference HD3 | Exception Note 9 of TS 38.101-3 Table 7.3B.2.3.1-1 |
|  | DL 2160 / UL 708 MHz | 20/10 | UL Harmonic avoiding | Exception avoiding. Not overlapping interference since BWINT=30 MHz, ¦FINT¦ = 36, ¦FINT¦ > (30+20)/2 |
| 1A/71A | DL High / UL 1948.5 | 5/5 | Receiver Harmonic Mixing | Exception Note 4 of TS 38.101-3 Table 7.3B.2.3.2-1 |
|  | DL High / UL 1948.5 | 20/5 | Receiver Harmonic Mixing | Exception Note 4 of TS 38.101-3 Table 7.3B.2.3.2-1 |
|  | DL High / UL Low | 5/5 | HM avoid | Exception avoiding. |
| 1A/77A | UL 1950/ DL 3900 | 20/100 | UL Harmonic Interference | Exception Note 2 and 13 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1950/ DL 3870 | 20/20 | UL Harmonic Interference | Exception Note 3 and 13 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1950/UL 4090.005 | 5/5 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
|  | UL 1950/ UL3709.005 | 5/5 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 2A / n41A | High/Low | 20/100 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 2A/71A | DL1980 MHz/ UL665.5 MHz | 20/5 | UL Harmonic Interference | Exception Note 11 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL1980 MHz/ UL673 MHz | 20/10 | UL Harmonic Interference | Exception Note 12 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL1881 MHz/UL Low | 20/20 | Receiver Harmonic Mixing | Exception Note 4 in TS 38.101-3 Table 7.3B.2.3.2-1 |
| 2A/n77A | UL 1860 MHz / DL 3720 MHz | 20/100 | UL Harmonic Interference | Exception Note 2, 13 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1860 MHz / DL 3690 MHz | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL Mid (1960) / UL 3920 | 20/100 | Receiver harmonic mixing HM2 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | UL 1860 MHz / DL 3850 MHz | 20/100 |  | HD avoid |
|  | DL Mid / UL Mid | 20/100 |  | HM avoid |
|  | DL 1987.5 MHz / UL 3350 MHz | 5/100 | CBI avoid (PC2 only) | CBI avoid |
| 2A/78A | DL 3740 MHz / UL 1870 MHZ | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3740 MHz / UL 1885 MHz | 20 MHz) / Highest (20 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| 3A / n1A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | High/Low | 20 / 20 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 3A / n41A |  |  |  | CMCC CR R5-205929 |
| 3A / n77A 3A / n78A | Mid/High | Highest/Highest | - | Non-Exception. |
|  | Mid/3495 | Highest/Highest | UL Harmonic Interference | Exception Note 2 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid/3525 | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid /3685.005 | 20/20 | Receiver Harmonic Mixing | Receiver Harmonic Mixing Exception in TS 38.101-3 Table 7.3B.2.3.2-1 |
|  | Low/High | 20/20 |  | Receiver Harmonic Mixing avoided |
|  | UL 1740/ UL3574.995 | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
|  | UL 1765/ UL 3435 | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 5A/n77A | UL 837.5 MHz / DL 3350.01 MHz | 10/100 | UL Harmonic Interference | Exception Note 6, 7, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 829 MHz / DL 4145.01 MHz | 10/100 | UL Harmonic Interference | Exception Note 4, 5, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL Mid/DL Mid | 10/100 |  | HD avoid |
|  | UL 844 MHz/DL 3421.005 MHz | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
|  | UL 826.5 MHz/DL 4177.5 MHz | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 5A/n78A | UL 840 MHz / DL 3360 MHz | 10/100 | UL Harmonic Interference | Exception Note 6, 7, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 840 MHz / DL 3560.01 MHz | 10/100 |  | HD avoid |
|  | UL 844 MHz/UL 3421.005 MHz | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 7A / n66A | UL2535 MHz/ UL 1730 MHz | 10/5 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 7A / n71A | UL 668 MHz/ DL 2672 MHz | 20/5 | UL Harmonic Interference | Exception Note 6, 7, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 668 MHz/ DL 2620 MHz | 20/5 | UL Harmonic avoiding | HD avoid |
| 7A / n77A | UL2540 MHz/ UL 3870 MHz | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 7A / n78A | High/Low | 20/100 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 8A / n41A | UL 885 / DL 2655 MHz | 10/100 | UL Harmonic Interference HD3 | Exception Note 9 of TS 38.101-3 Table 7.3B.2.3.1-1 |
|  | UL 885 / DL 2546 MHz | 10/100 | UL Harmonic avoiding | Exception avoiding. Not overlapping interference since BWINT=30 MHz, ¦FINT¦ = 109, ¦FINT¦ > (30+100)/2 |
| 8A /n77A 8A/n78A | Mid (UL 897.5 MHz) / 3590.01 | Highest (10 MHz) / Highest (100 MHz) | UL Harmonic Interference HD4 | Exception Note 7 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | Mid (UL 897.5 MHz) / 3520.005 | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=40 MHz, FINT = (40+100)/2 = 70 |
|  | UL 897.5/ UL 3634.995 | 5/40 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 8A / n79A | UL 900/DL 4500 | 10/100 | UL Harmonic Interference | Exception Note 4 nad 5 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL 900/DL 4299.99 | 10/100 |  | HD avoid |
|  | UL 897.5/ UL 4532.505 | 5/40 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 11A / n79A | Mid (DL 1485.9 MHz) / 4457.7 | Highest (10 MHz) / Highest (100 MHz) | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | Mid (DL 1485.9 MHz) / 4512.7 MHz | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=100 MHz, FINT = (100+10)/2 = 55 |
| 12A/n66A | UL 710/DL 2130 | 10/40 | UL Harmonic Interference | Exception Note 4 nad 5 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL 710/DL 2165 | 10/40 | UL Harmonic avoid | Non-Exception. Not overlapping interference since BWINT=30 MHz, FINT = (30+40)/2 = 35 |
| 12A / n78A | UL 707 / DL 3535 MHz | 10/100 | UL Harmonic Interference HD5 | Exception Note 5 of TS 38.101-3 Table 7.3B.2.3.1-1 |
|  | UL 707 / DL 3750 MHz | 10/100 | UL Harmonic avoiding | Exception avoiding. Not overlapping interference since BWINT=50 MHz, ¦FINT¦ = 215, ¦FINT¦ > (50+100)/2 |
| 13A/n77A | UL 782 MHz / DL 3910 MHz | 10/100 | UL Harmonic Interference | Exception Note 4, 5 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL Mid (751) / UL 3755 | 10/100 | Receiver harmonic mixing | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | UL 782 MHz /DL Mid | 10/100 |  | HD avoid |
|  | DL Mid / UL 3850 | 10/100 |  | HM avoid |
| 18A/ n77A | UL 820MHz/ DL 4100 MHz | 10/40 | UL Harmonic Interference | Exception Note 4, 5 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 820MHz/ DL 4000 MHz | 10/40 | UL Harmonic avoiding | HD avoid |
| 19A/ n77A | UL 835MHz/ DL 4175 MHz | 10/40 | UL Harmonic Interference | Exception Note 4, 5 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 835MHz/ DL 4100 MHz | 10/40 | UL Harmonic avoiding | HD avoid |
| 19A / n79A | DL 884 MHz / Low (4420.02 MHz) | 10 MHz / 40 MHz | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | DL 884 MHz / Low (4445.02 MHz) | 10 MHz / 40 MHz |  | Non-Exception. Not overlapping interference since BWINT=40 MHz, FINT = (40+10)/2 = 25 |
| 20A / n78A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | Mid/3388 | Highest/Highest | UL Harmonic Interference | Exception Note 7 of TS 38.101-3 table 7.3B.2.3.1-1 |
| 21A / n79A | DL 1506/UL4518 | 20/100 | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | DL 1506/ UL 4800 | 20/100 |  | Receiver Harmonic Mixing avoid |
|  | UL 1457.5/UL 4420.5 | 5/40 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 26A / n41A | High/Low | 5/100, 5/10 | UL Harmonic Interference HD3 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | High/High | 5/10 |  | HD avoid |
|  | UL 839 / 2562 | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
|  | DL 864 / UL MID Low / Mid | 5/10 15/15 | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | High / Mid | 15/15 |  | Receiver Harmonic Mixing avoid |
| 26A / n77A 26A / n78A | High (UL 841.5 MHz) / 3366 MHz | Highest (15 MHz) / Highest (100 MHz) | UL Harmonic Interference HD4 | Exception Note 7 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | High (UL 841.5 MHz) / 3446 MHz | Highest (15 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=60 MHz, FINT = (60+100)/2 = 80 |
| 26A / n79A | High (DL 886.5 MHz) / 4432.5 MHz) | Highest (15 MHz) / 60 MHz | Receiver Harmonic Mixing HM5 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | High (DL 886.5 MHz) / 4470.5 MHz) | Highest (15 MHz) / 60 MHz |  | Non-Exception. Not overlapping interference since BWINT=60 MHz, FINT = (60+15)/2 = 37.5 |
| 30A/n66A | Low/High | 10/40 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 38A/n78A | DL High/UL Low | 10/20 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 40A / n78A | Low/Mid | Highest/Highest | - | Non-Exception. |
|  | Mid/3525 | 20/20 | Receiver Harmonic Mixing | Receiver Harmonic Mixing Exception Note 8 in TS38.101-3 Table 7.3B.2.3.2-1 |
| 41A / n77A 41A / n78A | Low (DL 2593 MHz) / 3750 MHz) | Highest (20 MHz) / Highest (100 MHz) | Receiver Harmonic Mixing HM4 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | Mid (DL 2593 MHz) / 3974.5 MHz) | Highest (20 MHz) / Highest (100 MHz) | Cross band isolation | Not overlapping HM interference since BWINT=240 MHz, FINT = (240+100)/2 = 170 |
|  | Mid / Mid | Highest / Highest | - | Non-Exception only possible with 1UL in band 41 |
| 48A/n66A | DL 3555 MHz/ UL 1777.5 MHz | 10/20 | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3565 MHz/ UL 1770 MHz | 10/20 | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| 66A/n25A | UL 1775 MHz / UL 1855 MHz | 5/5 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 66A/n25A | UL 1712.5 MHz / UL 1912.5 MHz | 5/5 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 66A/n25A | UL 1750 MHz / UL 1883.3 MHz | 5/5 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 66A/n77A | UL 1720 MHz / DL 3440 MHz | 20/100 | UL Harmonic Interference | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1720 MHz / DL 3420 MHz | 20/20 | UL Harmonic Interference | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL Mid / DL Mid | 20/100 |  | HD avoid |
|  | DL 2197.5 MHZ / UL 3350 MHz | 5/100 | CBI avoid (PC2 only) | CBI avoid |
| 66A/n78A | DL 3510 MHz/ UL 1755 MHZ | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3480 MHz/ UL 1755 MHz | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| 66A/n41A (PC2 and PC3) | High/Low | 20/100 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 71A/n2A | UL665.5 MHz/ DL1987.5 MHz | 5/5 | UL Harmonic Interference | Exception Note 9 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL Low / UL1881 MHz | 5/5 | Receiver Harmonic Mixing | Exception Note 4 in TS 38.101-3 Table 7.3B.2.3.2-1 |
|  | DL Low / UL1881 MHz | 5/20 | Receiver Harmonic Mixing | Exception Note 4 in TS 38.101-3 Table 7.3B.2.3.2-1 |
|  | DL High / UL1881 MHz | 20/20 | HM avoid | Exception avoiding. Not overlapping interference since BWINT=20 MHz, ¦FINT¦ = 45, ¦FINT¦ > (20+20)/2 |
| NOTE 1: This selection is used in test case 7.3B.2.3 unless otherwise stated NOTE 2: Aggressor band in bold NOTE 3: Test frequencies defined in this table are required to be used in test cases 7.3B.2.3 and 7.3B.2.3_1.1 of TS38.521-1 [2] |  |  |  |  |

4.3.3.2	Spurious emissions test cases for EN-DC

In this case, it is sufficient to verify the minimum requirements in frequency ranges affected by 2nd and 3rd order intermodulation products. The frequency ranges and UL RB allocations used in the test are calculated here.

The analyses are performed per EN-DC configuration and are stored as zip-files as defined in annex A.

Table 4.3.3.2-1: Frequency range analysis availability per EN-DC configuration

| EN-DC config | Justification | Comments |
| --- | --- | --- |
| DC_1A_n3A | 38.521-3_TpAnalysisSpur(DC_1A_n3A)_v1.zip | RAN5#107 |
| DC_1A_n5A | 38.521-3_TpAnalysisSpur(DC_1A_n5A).zip | RAN5#94-e |
| DC_1A_n7A | 38.521-3_TpAnalysisSpur(DC_1A_n7A).zip | RAN5#94-e |
| DC_1A_n8A | 38.521-3_TpAnalysisSpur(DC_1A_n8A).zip | RAN5#95-e |
| DC_1A_n28A | 38.521-3_TpAnalysisSpur(DC_1A_n28A).zip | RAN5#92-e |
| DC_1A_n41A | 38.521-3_TpAnalysisSpur(DC_1A_n41A).zip | RAN5#98 |
| DC_1A_n71A | 38.521-3_TpAnalysisSpur(DC_1A_n71A).zip | RAN5#108 |
| DC_1A_n77A | 38.521-3_TpAnalysisSpur(DC_1A_n77A).zip | RAN5#92-e |
| DC_1A_n78A | 38.521-3_TpAnalysisSpur(DC_1A_n78A)_v1.zip | RAN5#92-e |
| DC_1A_n79A | 38.521-3_TpAnalysisSpur(DC_1A_n79A).zip | RAN5#92-e |
| DC_2A_n5A | 38.521-3_TpAnalysisSpur(DC_2A_n5A)_v1.zip | RAN5#100 |
| DC_2A_n41A | 38.521-3_TpAnalysisSpur(DC_2A_n41A)_v2.zip | RAN5#100 |
| DC_2A_n48A | 38.521-3_TpAnalysisSpur(DC_2A_n48A).zip | RAN5#100 |
| DC_2A_n66A | 38.521-3_TpAnalysisSpur(DC_2A_n66A)_v2.zip | RAN5#102 |
| DC_2A_n71A | 38.521-3_TpAnalysisSpur(DC_2A_n71A)_v3.zip | RAN5#100 |
| DC_2A_n77A | 38.521-3_TpAnalysisSpur(DC_2A_n77A)_v3.zip | RAN5#100 |
| DC_2A_n78A | 38.521-3_TpAnalysisSpur(DC_2A_n78A).zip | RAN5#88-e |
| DC_3A_n1A | 38.521-3_TpAnalysisSpur(DC_3A_n1A).zip | RAN5#88-e |
| DC_3A_n5A | 38.521-3_TpAnalysisSpur(DC_3A_n5A).zip | RAN5#94-e |
| DC_3A_n7A | 38.521-3_TpAnalysisSpur(DC_3A_n7A)_v1.zip | RAN5#91-e |
| DC_3A_n8A | 38.521-3_TpAnalysisSpur(DC_3A_n8A).zip | RAN5#96-e |
| DC_3A_n28A | 38.521-3_TpAnalysisSpur(DC_3A_n28A).zip | RAN5#92-e |
| DC_3A_n41A | 38.521-3_TpAnalysisSpur(DC_3A_n41A)_v3.zip | RAN5#94-e |
| DC_3A_n71A | 38.521-3_TpAnalysisSpur(DC_3A_n71A).zip | RAN5#108 |
| DC_3A_n77A | 38.521-3_TpAnalysisSpur(DC_3A_n77A).zip | RAN5#92-e |
| DC_3A_n78A | 38.521-3_TpAnalysisSpur(DC_3A_n78A)_v1.zip | RAN5#91-e |
| DC_3A_n79A | 38.521-3_TpAnalysisSpur(DC_3A-n79A)_v1.zip | RAN5#83 |
| DC_5A_n2A | 38.521-3_TpAnalysisSpur(DC_5A_n2A)_v3.zip | RAN5#100 |
| DC_5A_n66A | 38.521-3_TpAnalysisSpur(DC_5A_n66A)_v2.zip | RAN5#92-e |
| DC_5A_n77A | 38.521-3_TpAnalysisSpur(DC_5A_n77A)_v3.zip | RAN5#100 |
| DC_5A_n78A | 38.521-3_TpAnalysisSpur(DC_5A_n78A)_v2.zip | RAN5#92-e |
| DC_7A_n1A | 38.521-3_TpAnalysisSpur(DC_7A_n1A).zip | RAN5#88-e |
| DC_7A_n3A | 38.521-3_TpAnalysisSpur(DC_7A_n3A)_v1.zip | RAN5#96-e |
| DC_7A_5A | 38.521-3_TpAnalysisSpur(DC_7A_n5A).zip | RAN5#94-e |
| DC_7A_n8A | 38.521-3_TpAnalysisSpur(DC_7A_n8A).zip | RAN5#95-e |
| DC_7A_n28A | 38.521-3_TpAnalysisSpur(DC_7A_n28A).zip | RAN5#92-e |
| DC_7A_n66A | 38.521-3_TpAnalysisSpur(DC_7A_n66A).zip | RAN5#88-e |
| DC_7A_n78A | 38.521-3_TpAnalysisSpur(DC_7A_n78A)_v1.zip | RAN5#92-e |
| DC_8A_n1A | 38.521-3_TpAnalysisSpur(DC_8A_n1A).zip | RAN5#88-e |
| DC_8A_n3A | 38.521-3_TpAnalysisSpur(DC_8A_n3A)_v2.zip | RAN5#96-e |
| DC_8A_n20A | 38.521-3_TpAnalysisSpur(DC_8A_n20A)_v1.zip | RAN5#96-e |
| DC_8A_n28A | 38.521-3_TpAnalysisSpur(DC_8A_n28A).zip | RAN5#95-e |
| DC_8A_n41A | 38.521-3_TpAnalysisSpur(DC_8A_n41A)_v4.zip | RAN5#100 |
| DC_8A_n77A | 38.521-3_TpAnalysisSpur(DC_8A_n77A)_v2.zip | RAN5#100 |
| DC_8A_n78A | 38.521-3_TpAnalysisSpur(DC_8A_n78A)_v1.zip | RAN5#93-e |
| DC_11A_n77A | 38.521-3_TpAnalysisSpur(DC_11A_n77A)_v1.zip | RAN5#92-e |
| DC_11A_n78A | 38.521-3_TpAnalysisSpur(DC_11A_n78A)_v1.zip | RAN5#92-e |
| DC_11A_n79A | 38.521-3_TpAnalysisSpur(DC_11A_n79A)_v1.zip | RAN5#92-e |
| DC_12A_n2A | 38.521-3_TpAnalysisSpur(DC_12A_n2A)_v1.zip | RAN5#100 |
| DC_12A_n66A | 38.521-3_TpAnalysisSpur(DC_12A_n66A)_v1.zip | RAN5#91-e |
| DC_12A_n77A | 38.521-3_TpAnalysisSpur(DC_12A_n77A).zip | RAN5#101 |
| DC_12A_n78A | 38.521-3_TpAnalysisSpur(DC_12A_n78A)_v1.zip | RAN5#92-e |
| DC_13A_n2A | 38.521-3_TpAnalysisSpur(DC_13A_n2A)_v4.zip | RAN5#100 |
| DC_13A_n66A | 38.521-3_TpAnalysisSpur(DC_13A_n66A).zip | RAN5#89-e |
| DC_13A_n77A | 38.521-3_TpAnalysisSpur(DC_13A_n77A)_v3.zip | RAN5#100 |
| DC_14A_n2A | 38.521-3_TpAnalysisSpur(DC_14A_n2A)_v5.zip | RAN5#100 |
| DC_14A_n66A | 38.521-3_TpAnalysisSpur(DC_14A_n66A)_v4.zip | RAN5#95-e |
| DC_14A_n77A | 38.521-3_TpAnalysisSpur(DC_14A_n77A).zip | RAN5#101 |
| DC_18A_n77A | 38.521-3_TpAnalysisSpur(DC_18A_n77A)_v1.zip | RAN5#107 |
| DC_18A_n78A | 38.521-3_TpAnalysisSpur(DC_18A_n78A)_v1.zip | RAN5#107 |
| DC_19A_n1A | 38.521-3_TpAnalysisSpur(DC_19A_n1A).zip | RAN5#94-e |
| DC_19A_n77A | 38.521-3_TpAnalysisSpur(DC_19A_n77A).zip | RAN5#92-e |
| DC_19A_n78A | 38.521-3_TpAnalysisSpur(DC_19A_n78A).zip | RAN5#92-e |
| DC_19A_n79A | 38.521-3_TpAnalysisSpur(DC_19A_n79A).zip | RAN5#92-e |
| DC_20A_n1A | 38.521-3_TpAnalysisSpur(DC_20A_n1A)_v1.zip | RAN5#96-e |
| DC_20A_n3A | 38.521-3_TpAnalysisSpur(DC_20A_n3A)_v2.zip | RAN5#96-e |
| DC_20A_n7A | 38.521-3_TpAnalysisSpur(DC_20A_n7A).zip | RAN5#94-e |
| DC_20A_n8A | 38.521-3_TpAnalysisSpur(DC_20A_n8A).zip | RAN5#95-e |
| DC_20A_n78A | 38.521-3_TpAnalysisSpur(DC_20A_n78A).zip | RAN5#92-e |
| DC_20A_n28A | 38.521-3_TpAnalysisSpur(DC_20A_n28A).zip | RAN5#92-e |
| DC_21A_n1A | 38.521-3_TpAnalysisSpur(DC_21A_n1A).zip | RAN5#94-e |
| DC_21A_n28A | 38.521-3_TpAnalysisSpur(DC_21A_n28A)_v2.zip | RAN5#99 |
| DC_21A_n77A | 38.521-3_TpAnalysisSpur(DC_21A_n77A).zip | RAN5#92-e |
| DC_21A_n78A | 38.521-3_TpAnalysisSpur(DC_21A_n78A).zip | RAN5#92-e |
| DC_21A_n79A | 38.521-3_TpAnalysisSpur(DC_21A_n79A).zip | RAN5#92-e |
| DC_25A_n41A | 38.521-3_TpAnalysisSpur(DC_25A_n41A)_v3.zip | RAN5#100 |
| DC_25A_n77A | 38.521-3_TpAnalysisSpur(DC_25A_n77A).zip | RAN5#100 |
| DC_26A_n41A | 38.521-3_TpAnalysisSpur(DC_26A_n41A)_v3.zip | RAN5#100 |
| DC_26A_n77A | 38.521-3_TpAnalysisSpur(DC_26A_n77A)_v2.zip | RAN5#100 |
| DC_26A_n78A | 38.521-3_TpAnalysisSpur(DC_26A_n78A)_v1.zip | RAN5#92-e |
| DC_26A_n79A | 38.521-3_TpAnalysisSpur(DC_26A_n79A)_v1.zip | RAN5#91-e |
| DC_28A_n3A | 38.521-3_TpAnalysisSpur(DC_28A_n3A)_v1.zip | RAN5#92-e |
| DC_28A_n5A | 38.521-3_TpAnalysisSpur(DC_28A_n5A).zip | RAN5#94-e |
| DC_28A_n7A | 38.521-3_TpAnalysisSpur(DC_28A_n7A).zip | RAN5#94-e |
| DC_28A_n78A | 38.521-3_TpAnalysisSpur(DC_28A_n78A).zip | RAN5#92-e |
| DC_28A_n77A | 38.521-3_TpAnalysisSpur(DC_28A_n77A).zip | RAN5#92-e |
| DC_28A_n79A | 38.521-3_TpAnalysisSpur(DC_28A_n79A).zip | RAN5#92-e |
| DC_30A_n2A | 38.521-3_TpAnalysisSpur(DC_30A_n2A).zip | RAN5#101 |
| DC_30A_n5A | 38.521-3_TpAnalysisSpur(DC_30A_n5A)_v2.zip | RAN5#100 |
| DC_30A_n66A | 38.521-3_TpAnalysisSpur(DC_30A_n66A).zip | RAN5#102 |
| DC_30A_n77A | 38.521-3_TpAnalysisSpur(DC_30A_n77A).zip | RAN5#100 |
| DC_39A_n41A | 38.521-3_TpAnalysisSpur(DC_39A_n41A)_v2.zip | RAN5#94-e |
| DC_39A_n79A | 38.521-3_TpAnalysisSpur(DC_39A_n79A)_v1.zip | RAN5#92-e |
| DC_40A_n1A | 38.521-3_TpAnalysisSpur(DC_40A_n1A)_v1.zip | RAN5#92-e |
| DC_40A_n41A | 38.521-3_TpAnalysisSpur(DC_40A_n41A)_v2.zip | RAN5#94-e |
| DC_40A_n78A | 38.521-3_TpAnalysisSpur(DC_40A_n78A)_v1.zip | RAN5#92-e |
| DC_40A_n79A | 38.521-3_TpAnalysisSpur(DC_40A_n79A).zip | RAN5#94-e |
| DC_41A_n28A | 38.521-3_TpAnalysisSpur(DC_41A_n28A).zip | RAN5#98 |
| DC_41A_n77A | 38.521-3_TpAnalysisSpur(DC_41A_n77A)_v2.zip | RAN5#100 |
| DC_41A_n78A | 38.521-3_TpAnalysisSpur(DC_41A_n78A)_v1.zip | RAN5#92-e |
| DC_41A_n79A | 38.521-3_TpAnalysisSpur(DC_41A_n79A)_v1.zip | RAN5#92-e |
| DC_42A_n77A | 38.521-3_TpAnalysisSpur(DC_42A_n77A).zip | RAN5#92-e |
| DC_48A_n5A | 38.521-3_TpAnalysisSpur(DC_48A_n5A)_v2.zip | RAN5#100 |
| DC_48A_n66A | 38.521-3_TpAnalysisSpur(DC_48A_n66A)_v1.zip | RAN5#96-e |
| DC_66A_n2A | 38.521-3_TpAnalysisSpur(DC_66A_n2A).zip | RAN5#88-e |
| DC_66A_n5A | 38.521-3_TpAnalysisSpur(DC_66A_n5A)_v1.zip | RAN5#92-e |
| DC_66A_n41A | 38.521-3_TpAnalysisSpur(DC_66A_n41A)_v1.zip | RAN5#96-e |
| DC_66A_n71A | 38.521-3_TpAnalysisSpur(DC_66A_n71A)_v2.zip | RAN5#92-e |
| DC_66A_n77A | 38.521-3_TpAnalysisSpur(DC_66A_n77A)_v2.zip | RAN5#94-e |
| DC_66A_n78A | 38.521-3_TpAnalysisSpur(DC_66A_n78A)_v2.zip | RAN5#92-e |
| DC_71A_n2A | 38.521-3_TpAnalysisSpur(DC_71A_n2A)_v1.zip | RAN5#100 |
| DC_71A_n66A | 38.521-3_TpAnalysisSpur(DC_71A_n66A).zip | RAN5#98 |

<End of Changes>

## Add the following attached .zip files to TR 38.905:

## See attachment “38.521-3_TpAnalysisSpur(DC_1A_n71A).zip”

## See attachment “38.521-3_TpAnalysisSpur(DC_3A_n71A).zip”