3GPP TSG-RAN5 Meeting #108	R5-254134

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3438 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General updates of TS 38.521-1 clause 5 for R17 CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Verizon, Huawei, NTT DOCOMO INC., KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some R17 NR CA combos which are already 100% completion in RAN4 and assigned to interested companies need to be added into operating bands and channel bandwidth of TS38.521-1 clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of 2CC CA combos CA_n2A-n48(A-B), CA_n5A-n48(A-B), CA_n8A-n77A, CA_28A-n71A, CA_n48(A-B)-n66A, CA_n48(2A)-n77C and CA_n48B-n77C configs in Table 5.2A.2.1-1 and Table 5.5A.3.1-1. - Addition of a new table: Table 5.5A.2-2: NR CA configurations and bandwidth combination sets defined for mixed intra-band contiguous and non-contiguous CA. - Addition of some 3CC CA combos configs in Table 5.2A.2.2-1 and Table 5.5A.3.2-1. - Addition of some 4CC CA combos configs in Table 5.5A.3.3-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The activated NR CA configurations in clause 5 will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2.1, 5.2A.2.2, 5.5A.2, 5.5A.3.1, 5.5A.3.2, 5.5A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, 5.2A.2.2-1 and Table 5.2A.2.3-1, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

Table 5.2A.2-1: Void

Table 5.2A.2-2: Void

Table 5.2A.2-3: Void

5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n3 | n1, n3 |  |
| CA_n1-n5 | n1, n5 |  |
| CA_n1-n8 | n1, n8 |  |
| CA_n1-n28 | n1, n28 |  |
| CA_n1-n411 | n1, n41 |  |
| CA_n1-n771 | n1, n77 | No |
| CA_n1-n781 | n1, n78 | No |
| CA_n1-n791 | n1, n79 | No |
| CA_n2-n5 | n2, n5 |  |
| CA_n2-n12 | n2, n12 |  |
| CA_n2-n14 | n2, n14 |  |
| CA_n2-n30 | n2, n30 |  |
| CA_n2-n48 | n2, n48 |  |
| CA_n2-n66 | n2, n66 |  |
| CA_n2-n77 | n2, n77 |  |
| CA_n3-n5 | n3, n5 |  |
| CA_n3-n8 | n3, n8 |  |
| CA_n3-n28 | n3, n28 |  |
| CA_n3-n401 | n3, n40 | No |
| CA_n3-n411 | n3, n41 | No |
| CA_n3-n771 | n3, n77 | No |
| CA_n3-n781 | n3, n78 | No |
| CA_n5-n7 | n5, n7 |  |
| CA_n5-n30 | n5, n30 |  |
| CA_n5-n48 | n5, n48 |  |
| CA_n5-n66 | n5, n66 |  |
| CA_n5-n771 | n5, n77 |  |
| CA_n5-n781 | n5, n78 | No |
| CA_n7-n781 | n7, n78 |  |
| CA_n8-n771 | n8, n77 |  |
| CA n8-n781 | n8, n78 | No |
| CA_n14-n30 | n14, n30 |  |
| CA_n14-n66 | n14, n66 |  |
| CA_n14-n77 | n14, n77 |  |
| CA_n20-n78 | n20, n78 |  |
| CA_n24-n41 | n24, n41 |  |
| CA_n24-n48 | n24, n48 |  |
| CA_n24-n77 | n24, n77 |  |
| CA_n25-n66 | n25, n66 |  |
| CA_n25-n77 | n25, n77 |  |
| CA_n25-n78 | n25, n78 |  |
| CA_n25-n466 | n25, n46 |  |
| CA_n26-n66 | n26, n66 |  |
| CA_n26-n70 | n26, n70 |  |
| CA_n28-n401 | n28, n40 |  |
| CA_n28-n411 | n28, n41 |  |
| CA_n28-n771 | n28, n77 | No |
| CA_n28-n7112 | n28, n71 |  |
| CA_n28_n781 | n28, n78 | No |
| CA_n28-n791 | n28, n79 |  |
| CA_n29-n66 | n29, n66 |  |
| CA_n29-n70 | n29, n70 |  |
| CA_n29-n71 | n29, n71 |  |
| CA_n30-n66 | n30, n66 |  |
| CA_n30-n77 | n30, n77 |  |
| CA_n39-n41 | n39, n41 | No |
| CA_n40-n771 | n40, n77 |  |
| CA_n41-n66 | n41, n66 |  |
| CA_n41-n711 | n41, n71 |  |
| CA_n41-n771 | n41, n77 |  |
| CA_n41-n791,3 | n41, n79 | No |
| CA_n46-n481,6 | n46, n48 |  |
| CA_n46-n666 | n46, n66 |  |
| CA_n48-n66 | n48, n66 |  |
| CA_n48-n70 | n48, n70 |  |
| CA_n48-n71 | n48, n71 |  |
| CA_n48-n7713,14,18 | n48, n77 |  |
| CA_n66-n70 | n66, n70 |  |
| CA_n66-n71 | n66, n71 |  |
| CA_n66-n77 | n66, n77 |  |
| CA_n66-n78 | n66, n78 |  |
| CA_n70-n71 | n70, n71 |  |
| CA_n71-n77 | n70, n71 |  |
| CA_n71-n78 | n71, n78 |  |
| CA_n77-n795,7 | n77, n79 |  |
| CA_n78-n795 | n78, n79 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: FFS. NOTE 3: The frequency range below 2506MHz for Band n41 is not used in this combination. NOTE 4: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 6: The PCell is allocated in the licensed band in this combination. NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: FFS NOTE 10 FFS NOTE 11: FFS NOTE 12: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 13: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n48 with an n77 implementation. NOTE 14: The band n48 and n77 will synchronize their uplink and downlink configurations and in commonly TDD network coordination NOTE 15: FFS NOTE 16: FFS NOTE 17: FFS NOTE 18: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. |  |  |

5.2A.2.2	Inter-band CA (three bands)

Table 5.2A.2.2-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n1-n3-n28 | n1, n3, n28 |  |
| CA_n1-n3-n77 | n1, n3, n77 |  |
| CA_n1-n3-n783 | n1, n3, n78 | No for CA_n1-n78, CA_n3-n78 |
| CA_n1-n5-n78 | n1, n5, n78 | No for CA_n1-n78, CA_n5-n78 |
| CA_n1-n8-n783 | n1, n8, n78 |  |
| CA_n1-n28-n773 | n1, n28, n77 |  |
| CA_n1-n28-n793 | n1, n28, n79 |  |
| CA_n1-n28-n783 | n1, n28, n78 |  |
| CA_n1-n41-n773 | n1, n41, n77 |  |
| CA_n1-n78-n79 | n1, n78, n79 |  |
| CA_n2-n5-n48 | n2, n5, n48 |  |
| CA_n2-n5-n66 | n2, n5, n66 |  |
| CA_n2-n5-n77 | n2, n5, n77 |  |
| CA_n2-n14-n30 | n2, n14, n30 |  |
| CA_n2-n14-n66 | n2, n14, n66 |  |
| CA_n2-n14-n77 | n2, n14, n77 |  |
| CA_n2-n30-n77 | n2, n30, n77 |  |
| CA_n2-n48-n66 | n2, n48, n66 |  |
| CA_n2-n48-n77 | n2, n48, n77 |  |
| CA_n2-n66-n77 | n2, n66, n77 |  |
| CA_n3-n5-n78 | n3, n5, n78 | No for CA_n3-n78, CA_n5-n78 |
| CA_n3-n8-n783 | n3, n8, n78 |  |
| CA_n3-n28-n403 | n3, n28, n40 |  |
| CA_n3-n28-n413 | n3, n28, n41 |  |
| CA_n3-n28-n773 | n3, n28, n77 |  |
| CA_n3-n28-n783 | n3, n28, n78 |  |
| CA_n3-n40-n77 | n3, n40, n77 |  |
| CA_n3-n41-n773 | n3, n41, n77 |  |
| CA_n5-n30-n77 | n5, n30, n77 |  |
| CA_n5-n48-n66 | n5, n48, n66 |  |
| CA_n5-n48-n77 | n5, n48, n77 |  |
| CA_n5-n66-n77 | n5, n66, n77 |  |
| CA_n14-n30-n66 | n14, n30, n66 |  |
| CA_n14-n66-n77 | n14, n66, n77 |  |
| CA_n25-n66-n77 | n25, n66, n77 |  |
| CA_n25-n66-n78 | n25, n66, n78 |  |
| CA_n26-n66-n70 | n26, n66, n70 |  |
| CA_n28-n40-n77 | n28, n40, n77 |  |
| CA_n28-n41-n773 | n28, n41, n77 |  |
| CA_n28-n41-n793 | n28, n41, n79 |  |
| CA_n28-n78-n79 | n28, n78, n79 |  |
| CA_n29-n66-n70 | n29, n66, n70 |  |
| CA_n41-n66-n71 | n41, n66, n71 |  |
| CA_n48-n66-n70 | n48, n66, n70 |  |
| CA_n48-n66-n71 | n48, n66, n71 |  |
| CA_n48-n66-n77 | n48, n66, n77 |  |
| CA_n48-n70-n71 | n48, n70, n71 |  |
| CA_n66-n70-n71 | n66, n70, n71 |  |
| CA_n66-n71-n77 | n66, n71, n77 |  |
| CA_n66-n71-n78 | n66, n71, n78 |  |
| NOTE 1: FFS NOTE 2: FFS NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

<<Unchanged sections skipped>>

5.5A.2	Configurations for intra-band non-contiguous CA

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| CA_n48(2A) |  | 10, 15, 20, 40, 50, 60 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 1402 | 0 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1402 | 1 |
| CA_n66(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  | 80 | 1 |
|  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  | 80 | 2 |
| CA_n66(3A) | - | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 80 | 0 |
| CA_n71(2A) | - | 5,10, 15, 20 | 5,10,15, 20 |  | 30 | 0 |
| CA_n77(2A) | n773 CA_n77(2A) | 20, 40, 80, 100 | 20, 40, 80, 100 |  | 200 | 0 |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 200 | 1 |
|  |  | See n77 channel bandwidths in Table 5.3.5-1 for each carrier |  |  | 200 | 4 and 5 |
| CA_n78(2A) |  | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 200 | 1 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 200 | 2 |
| NOTE 1: For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |

Table 5.5A.2-2: NR CA configurations and bandwidth combination sets defined for
mixed intra-band contiguous and non-contiguous CA

| NR CA configuration | Uplink CA configuration or single uplink carrier4 | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48(A-B) | CA_n48B | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701, 801, 901, 1001 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: FFS NOTE 3: FFS NOTE 4: Only single uplink carriers with power class other than PC3 are listed. |  |  |  |  |

5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: Void

Table 5.5A.3-2: Void

Table 5.5A.3-3: Void

5.5A.3.0	General

For the NR inter-band CA configurations in sub-clause 5.5A.3, when the capability supportedBandPairListNR-r18 is present, three or four bands can be configured in the uplink with simultaneous uplink transmission on up to two bands, and the corresponding inter-band CA requirements with uplink assigned to one or two bands shall apply. For each uplink band pair in the NR inter-band CA configurations, according to the capability uplinkTxSwitchingOptionForBandPair-r18,

–	if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with uplink assigned to one band on band X or band Y apply;

–	if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with uplink CA between the two uplink bands apply. For a UE supporting [BandOrdering1T1Tto1T1T] for parallel uplink transmission switching for a band combination consisting with four different bands, the UE is allowed to report capability [preferredBandPairs] via band-ordering approach to indicate the UE’s preferred switching band pairs for which it supports dualUL and perform the switching case configured by network.

5.5A.3.1	Configurations for inter-band CA (two bands)

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier5 | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n5A | CA_n1A-n5A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1(2A)-n3A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n5A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n8A | CA_n1A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1(2A)-n8A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n41A | CA_n1A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n78A | n14 n784 CA_n1A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 3 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n78C | CA_n1A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1(2A)-n78A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n79A | CA_n1A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A | CA_n2A-n5A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n2(2A)-n5A | CA_n2A-n5A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n2A-n12A | CA_n2A-n12A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n2A-n14A | CA_n2A-n14A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n2(2A)-n14A | CA_n2A-n14A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n2A-n30A | CA_n2A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n30A | CA_n2A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n48A | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |
| CA_n2A-n48(2A) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n48B | CA_n48B CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n48(A-B) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n66A | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n66(2A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n66(2A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n66(3A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n77A | n774,9 CA_n2A-n77A4,13,14 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n77(2A) | n774 CA_n2A-n77A4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n77A | n774 CA_n2A-n77A4 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n77(2A) | n774 CA_n2A-n77A4 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n77C | n774 CA_n77C CA_n2A-n77A4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n3A-n5A | CA_n3A-n5A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n5A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, |  |
| CA_n3A-n8A | CA_n3A-n8A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n8A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3A-n28A | CA_n3A-n28A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n40A | CA_n3A-n40A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n3A-n41A | n414 CA_n3A-n41A4 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n77A | n774,9 CA_n3A-n77A4 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n77(2A) | CA_n77(2A) CA_n3A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n78A | n34 n784 CA_n3A-n78A4 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n78(2A) | CA_n3A-n78A CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3(2A)-n78A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n79A | CA_n3A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n5A-n7A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n5A-n30A | CA_n5A-n30A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n5A-n48A | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |
| CA_n5A-n48(2A) | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n5A-n48B | CA_n48B CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n5A-n48(A-B) | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n5A-n66A | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n66(2A) | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n66(3A) | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n5A-n77A | n774,9 CA_n5A-n77A4,13,14 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n77C | n774 CA_n5A-n77A4 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n77(2A) | n774 CA_n5A-n77A4 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n78A | CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5B-n77A | CA_n5A-n77A | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5B-n77C | CA_n5A-n77A | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | CA_n5B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n7A-n78A | CA_n7A-n78A | n7 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n8A-n77A | CA_n8A-n77A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n78A | CA_n8A-n78A13 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
| CA_n14A-n30A | CA_n14A-n30A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n14A-n66A | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n66(2A) | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n66(3A) | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n14A-n77A | n774 CA_n14A-n77A4 | n14 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n77(2A) | n774 CA_n14A-n77A4 | n14 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n20A-n78A | CA_n20A-n78A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n41A | CA_n24A-n41A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
| CA_n24A-n48A | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |
| CA_n24A-n48B | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n24A-n77A | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n77C | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n25A-n46A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
| CA_n25A-n66A | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77A | CA_n25A-n77A4,13,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77(2A) | CA_n25A-n77A4 CA_n77(2A) | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n78A | CA_n25A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n78(2A) | CA_n25A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n26A-n66A | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n70A | CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n28A-n40A | CA_n28A-n40A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n28A-n41A | n414 CA_n28A-n41A4,13 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20,30 | 1 |
|  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n41C | CA_n41C CA_n28A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
| CA_n28A-n71A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n28A-n77A | n774,9 CA_n28A-n77A4 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77(2A) | CA_n77(2A) CA_n28A-n77A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n78A | CA_n28A-n78A4,13 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 30 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n78(2A) | CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n28A-n79A | n794 CA_n28A-n79A4 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n66A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n29A-n66B | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n29A-n66(2A) | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n29A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n71A | - | n29 | 5, 10 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n30A-n66A | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n30A-n66(2A) | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n30A-n66(3A) | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n30A-n77A | n77A4, CA_n30A-n77A4 | n30 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n77(2A) | n774 CA_n30A-n77A4 | n30 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n39A-n41A | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n40A-n77A | CA_n40A-n77A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 7015, 80, 9015, 100 |  |
| CA_n41A-n66A | - | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n41A-n71A | - | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41A-n77A | n414 n774,9 CA_n41A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n79A | n414, n794 CA_n41A-n79A4 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n79C | CA_n79C | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n41C-n79A | CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41C-n79C | CA_n41C CA_n79C CA_n41A-n79A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n46A-n48A | CA_n46A-n48A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | 20 |  |
| CA_n46B-n48A | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 20 |  |
| CA_n46C-n48A | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 20 |  |
| CA_n46D-n48A | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 20 |  |
| CA_n46A-n66A | - | n46 | 20, 40, 60, 80 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48A-n66A | CA_n48A-n66A | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48B-n66A | CA_n48B CA_n48A-n66A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66A | CA_n48A-n66A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66(2A) | CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48(A-B)-n66A | CA_n48A-n66A | n48 | CA_n48(A-B)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48A-n70A6 | CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n70A | CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n70A | CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n71A | CA_n48A-n71A | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48(2A)-n71A | CA_n48A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n71(2A) | CA_n48A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48B-n71A | CA_n48A-n71A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n77A | n774,9 | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n77C | CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48(2A)-n77A | - | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48(2A)-n77C | - | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48B-n77A | CA_n48B | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48B-n77C | - | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS2 | 3 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n66A-n70A | - | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66B-n70A | - | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66(2A)-n70A | - | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66A-n71A | CA_n66A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71(2A) | - | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  | CA_n66A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66(2A)-n71A | CA_n66A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66B-n71A | CA_n66A-n71A | n66 | CA_n66B_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n77A | n774,9 CA_n66A-n77A4,13,14 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n77(2A) | n774 CA_n66A-n77A4 CA_n77(2A) | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66(2A)-n77A | n774 CA_n66A-n77A4 | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n77(2A) | n774 CA_n66A-n77A4 | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n66(3A)-n77A | n774 CA_n66A-n77A4 | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n78A | CA_n66A-n78A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n78(2A) | CA_n66A-n78A | n66 | 5, 10, 15, 20, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n66A-n77C | n774 CA_n77C CA_n66A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n70A-n71A | CA_n70A-n71A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70 | 5, 10, 15, 201 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n71A-n77A | n774 CA_n71A-n77A4,13,14 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n77(2A) | CA_n71A-n77A | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71A-n78A | CA_n71A-n78A | n71 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n78(2A) | CA_n71A-n78A | n71 | 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n77A-n79A | - | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n78A-n79A | n784 n794 | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. NOTE 2: The minimum requirements for intra-band contiguous or non-contiguous CA apply. NOTE 3: For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs. NOTE 4: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination. NOTE 5: Only single uplink carriers with power class other than PC3 are listed. NOTE 6: The same configuration applies to corresponding NR-DC configuration in Table 5.5B.1-1. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in CA test cases. NOTE 7: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 8: Reserved NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination. NOTE 10: Reserved NOTE 11: Reserved NOTE 12: Reserved NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14: Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 15: This UE channel bandwidth is optional in release 18 of the specification. |  |  |  |  |

5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
| CA_n1A-n3A-n77A | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | - | n1 | 5, 10, 15, 20 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n28A-n77A | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n41A-n77A | CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n78A-n79A5 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
| CA_n2A-n5A-n48B | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n5A-n48(A-B) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2A-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n5A-n77A | n777, 9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77C | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2(2A)-n5A-n77A | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77(2A) | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n77A | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n77A | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n77(2A) | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n77A | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n77(2A) | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n77A | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48B-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(A-B)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48A-n77A | n777, 9 CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n77C | CA_n2A-n48A CA_n2A-n77A CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77A | n777, 9 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77C | CA_n2A-n66A CA_n2A-n77A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2(2A)-n66A-n77A | n777 CA_n2A-n66A CA_n2A-n77A CA_n66A-n77A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66(2A)-n77A | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A | n3 | 5, 10, 15, 20, 25, 30 |  |
|  | CA_3A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  | CA_n8A-n78A | n78 | 10, 15, 20,40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n41A | n417 CA_n3A-n28A CA_n3A-n41A CA_n28A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n40A | CA_n3A-n28A CA_n3A-n40A CA_n28A-n40A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n40 | 20, 40 |  |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n41B_BCS0 |  |
| CA_n3A-n28A-n78A | - | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n40A-n77A | CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 30, 35, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n77A | n417 n777 CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n77A | CA_n5A-n30A CA_n5A-n77A CA_n30A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n77(2A) | CA_n5A-n30A CA_n5A-n77A CA_n30A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48B-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(A-B)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48A-n77A | n777, 9 CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n77C | CA_n5A-n48A CA_n5A-n77A CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48B-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48B-n77C | - CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48(2A)-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A)-n77C | - CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77A | n777, 9 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n77C | CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66(2A)-n77A | n777 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n77(2A) | n777 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n30A-n66A | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n30A-n66(2A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n66A-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66(2A)-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66A-n77(2A) | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n28A-n40A-n77A | CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77A | n417 n777 CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77(2A) | n41 n77 - CA_n28A-n41A CA_n28A-n77A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n78A-n79A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0. |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n66A-n71A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48A-n66A-n77A | n777, 9 CA_n48A-n66A CA_n66A-n77A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66A-n77C | CA_n48A-n66A CA_n66A-n77A CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48B-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77C | - CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48A-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66A-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n78(2A) | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs. NOTE 4: The minimum requirements only apply for non-simultaneous Rx/Tx between all carriers for TDD combinations. NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 6: FFS NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 8: FFS NOTE 9: Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination. NOTE 10: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

5.5A.3.3	Configurations for inter-band CA (four bands)

Table 5.5A.3.3-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier4 | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
| CA_n2A-n5A-n48A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48B-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |

| CA_n2A-n5A-n48A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48A-n77C | CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n48(2A)-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | - CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48B-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | - CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66A-n77A | CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66A-n77C | CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n77A5 CA_n5A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48A-n66A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A-n77C | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48(2A)-n66A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | - CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48B-n66A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | - CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n40A-n77A | CA_n3A-n28A CA_n3A-n40A CA_n3A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n77A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n77(2A) | - CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n5A-n48A-n66A-n77A | - | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n66A-n77C | CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48(2A)-n66A-n77A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | - CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20, 25 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48B-n66A-n77A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | - CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20, 25 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20, 25 | 3 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs. NOTE 4: Only single uplink carriers with power class other than PC3 are listed. NOTE 5: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

<<End of changes>>