3GPP TSG-RAN5 Meeting #108	R5-254632

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3483 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of REFSENS for Rel17 PC3 NR CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Reference sensitivity for the following NR CA configurations are not complete. CA_n1A-n28A-n79A, CA_n1A-n78A-n79A, CA_n28A-n78A-n79A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding reference sensitivity exceptions for the following NRCA configurations in Table 7.3A.0.5-2. CA_n1A-n28A-n79A, CA_n1A-n78A-n79A - Adding reference sensitivity without exceptions for the following NRCA configurations in 7.3A.2.5-1 CA_n1A-n28A-n79A, CA_n28A-n78A-n79A - Adding requirement about reference sensitivity of following NRCA configuration into Table 7.3A.2_1.4.1-1 and Table 7.3A.2_1.5-1. CA_n1A-n28A-n79A, CA_n1A-n78A-n79A, CA_n28A-n78A-n79A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference sensitivity for the NRCA configuration will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.5, 7.3A.2, 7.3A.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR38.905 CR 1068 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

-------- Skip sections not changed -------

Table 7.3A.0.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD22 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 31.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 31.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 27.9 | FDD | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n1-n5-n78 | n1 | N/A | 5 | N/A | 2122 | 18.1 | FDD | IMD3 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 885 | 3.1 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3610 | 15.7 | TDD | IMD3 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n8-n78 | n1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3745 | 14.9 | TDD | IMD3 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 3.3 | FDD | IMD5 |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| CA_n1-n28-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD32 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 4.2 | FDD | IMD5 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 4.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD3 |
| CA_n1-n28-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4630 | 14.9 | TDD | IMD31 |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 788 | 15.2 | FDD | IMD32 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167.5 | 1.2 | FDD | IMD41 |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
| CA_n1-n41-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 19.6 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | TDD | IMD41 |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 9.3 | FDD | IMD4 |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 15.6 | FDD | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n2-n5-n30 | n2 | 1870 | 5 | 25 | 1959 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 9.7 | FDD | IMD4 |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | FDD | N/A |
| CA_n2-n5-n48 | n2 | N/A | 5 | N/A | 1962 | 15.6 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n48 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3593 | 16.6 | TDD | IMD3 |
| CA_n2-n5-n66 | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2140 | 7.2 | FDD | IMD4 |
| CA_n2-n5-n77 | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 887.5 | 3.8 | FDD | IMD55 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987 | 16.5 | FDD | IMD35 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | TDD | IMD31 |
| CA_n2-n12-n77 | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD32 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | TDD | IMD31,2 |
| CA_n2-n14-n66 | n2 | 1874 | 5 | 25 | 1954 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2162 | 7.6 | FDD | IMD4 |
|  | n2 | N/A | 5 | N/A | 1954 | 7.2 | FDD | IMD4 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
| CA_n2-n14-n77 | n2 | N/A | 5 | N/A | 1954 | 16.5 | FDD | IMD3 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3466 | 16.0 | TDD | IMD31 |
| CA_n2-n30-n77 | n2 | N/A | 5 | N/A | 1986 | 8.6 | FDD | IMD4 |
|  | n30 | 2312 | 5 | 25 | 2357 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2354 | 10.6 | FDD | IMD41 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | TDD | N/A |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2354 | 3.4 | FDD | N/A |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | TDD | IMD5 |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4180 | 29.4 | TDD | IMD22 |
| CA_n2-n48-n66 | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3625 | 32.0 | TDD | IMD2 |
|  | n66 | 1770 | 5 | 25 | 2190 | N/A | FDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n48 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2155 | 12.1 | FDD | IMD4 |
|  | n2 | N/A | 5 | N/A | 1960 | 28.3 | FDD | IMD21 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n2-n66-n77 | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD25 |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3900 | 8.9 | TDD | IMD4 |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2115 | 29.2 | FDD | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2140 | 10.4 | FDD | IMD4 |
|  | n77 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2175 | 4.0 | FDD | IMD5 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 9.1 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 2.1 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| CA_n3-n5-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3408 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3512 | 4.5 | TDD | IMD5 |
|  | n3 | N/A | 5 | N/A | 1862 | 15.7 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n3-n8-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3550 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3370 | 4.5 | TDD | IMD5 |
|  | n3 | N/A | 5 | N/A | 1820 | 15.7 | FDD | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
| CA_n3-n28-n41 | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | TDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 26.0 | FDD | IMD24 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
|  | n3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | FDD | IMD2 |
| CA_n3-n28-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | TDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |
| CA_n3-n28-n78 | n3 | 1755 | 5 | 25 | 1850 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3320 | 17.3 | TDD | IMD3 |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3764 | 4.5 | TDD | IMD5 |
| CA_n3-n40-n77 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n40 | 2320 | FFS | 25 | 2320 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4050 | 19.0 | TDD | IMD21 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n40 | N/A | FFS | N/A | 2310 | 29.4 | TDD | IMD21 |
|  | n77 | 4030 | 10 | 50 | 4030 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 29.9 | FDD | IMD22 |
|  | n40 | 2310 | FFS | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n41 | N/A | 5 | N/A | 2640 | 5.3 | TDD | IMD5 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1840 | 16.4 | FDD | IMD3 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 16.8 | TDD | IMD31 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 4.4 | FDD | IMD5 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | 4.4 | TDD | IMD5 |
| CA_n5-n14-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 4052 | 10 | 50 | 4052 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 11.6 | FDD | IMD41 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3298 | 10 | 50 | 3298 | 10.3 | TDD | IMD41 |
| CA_n5-n25-n66 | n5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1712 | 5 | 25 | 2132 | 7.2 | FDD | IMD4 |
| CA_n5-n25-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | TDD | IMD3 |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | 1907 | 5 | 25 | 1987 | 16.5 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| CA_n5-n30-n66 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n30 | 2307.5 | 5 | 25 | 2352.5 | N/A | FDD | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 4 | FDD | IMD5 |
| CA_n5-n30-n77 | n5 | N/A | 5 | N /A | 880 | 15.2 | FDD | IMD3 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | TDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3780 | 16.1 | TDD | IMD3 |
| CA_n5-n48-n66 | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | N/A | 3622 | 3.6 | TDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
| CA_n5-n66-n77 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3465 | 16.1 | TDD | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4192 | 8.2 | TDD | IMD45 |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3535 | 3.3 | TDD | IMD5 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2142 | 13.2 | FDD | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n12-n30-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3913 | 10 | 50 | 3913 | 16.0 | TDD | IMD3 |
| CA_n12-n66-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD3 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n66 | 1746 | 5 | 25 | 2146 | 13.2 | FDD | IMD3 |
|  | n77 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n12 | 704 | 5 | 25 | 734 | N/A | FDD | N/A |
|  | n66 | 1723 | 5 | 25 | 2123 | N/A | FDD | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | 16.0 | TDD | IMD31,2 |
| CA_n14-n30-n77 | n14 | 793 | 5 | 25 | 763 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3896 | 10 | 50 | 3896 | 16.0 | TDD | IMD3 |
| CA_n14-n66-n77 | n14 | N/A | 5 | N/A | 763 | 15.2 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2155 | 13.2 | FDD | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3341 | 16.0 | TDD | IMD31,2 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n66-n77 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | FDD | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD2 |
| CA_n25-n71-n77 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8.0 | TDD | IMD31,2 |
|  | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| CA_n28-n40-n77 | n28 | N/A | 5 | N/A | 800.5 | 11 | FDD | IMD31 |
|  | n40 | 2302.5 | FFS | 25 | 2302.5 | N/A | TDD | N/A |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
|  | n28 | 708 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | 2310 | FFS | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3736 | 16.0 | TDD | IMD32 |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n40 | N/A | FFS | N/A | 2134 | 15.7 | TDD | IMD3 |
|  | n77 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n77 | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD24 |
|  | n41 | 2567.5 | 10 | 50 | 2567.5 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n28 | 727.5 | 5 | 25 | 782.5 | 3.0 | FDD | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3323 | 10 | 50 | 3323 | 28.2 | TDD | IMD24 |
| CA_n28-n41-n79 | n28 | N/A | 5 | N/A | 780 | 13.0 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4600 | 10.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2645 | 10.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | N/A | n77 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A | n79 |
|  | n28 | 745 | 5 | 25 | 800 | 16.2 | IMD21,2 | n28 |
| CA_n28-n78-n79 | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 26.2 | TDD | IMD21,3,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 26.9 | TDD | IMD23,4 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 800 | 16.2 | FDD | IMD21 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
| CA_n30-n66-n77 | n30 | 2310 | 5 | 25 | 2355 | 29.2 | FDD | IMD21 |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 8.7 | FDD | IMD4 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4055 | 10 | 50 | 4055 | 28.4 | TDD | IMD21 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n66-n77 | n41 | 2560 | 5 | 25 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD31,2 |
|  | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD5 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2530 | 5 | 25 | 2530 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21 |
|  | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
|  | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2615 | 5 | 25 | 2615 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n71 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n48-n66-n70 | n48 | 3625 | 10 | 50 | 3625 | N/À | TDD | N/A |
|  | n66 | 1742.5 | 5 | 25 | 2142.5 | 2.8 | FDD | IMD5 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |
| CA_n48-n66-n71 | n48 | 3552.5 | 10 | 50 | 3552.5 | N/A | TDD | N/A |
|  | n66 | 1761.5 | 5 | 25 | 2161.5 | 14.4 | FDD | IMD3 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n48 | 3695 | 10 | 50 | 3695 | 5.2 | TDD | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n48-n70-n71 | n48 | 3694 | 10 | 50 | 3694 | 9 | TDD | IMD41 |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | TDD | IMD31,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD3 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3724 | 9 | TDD | IMD41 |
|  | n66 | N/A | 5 | N/A | 2160 | 15.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |

-------- Skip sections not changed -------

7.3A.2	Reference sensitivity power level for 3DL CA without exceptions

-------- Skip sections not changed -------

Table 7.3A.2.5-1: Reference sensitivity requirement for 3DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Intra-band non-contiguous 3DL CA | CA_n48(3A) | - |
|  | CA_n66(3A) | - |
| Inter-band 3DL CA | CA_n1A-n78C |  |
|  | CA_n1A-n78(2A) | - |
|  | CA_n1A-n8A-n78A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n78A-n79A | - |
|  | CA_n1A-n3A-n28A | - |
|  | CA_n1A-n3A-n77A |  |
|  | CA_n1A-n3A-n78A | - |
|  | CA_n1A-n5A-n78A |  |
|  | CA_n1A-n28A-n77A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n28A-n79A | - |
|  | CA_n1A-n41A-n77A |  |
|  | CA_n2A-n5A-n48A |  |
|  | CA_n2A-n5A-n66A |  |
|  | CA_n2A-n5A-n77A |  |
|  | CA_n2A-n14A-n30A | - |
|  | CA_n2A-n14A-n66A | - |
|  | CA_n2A-n14A-n77A | - |
|  | CA_n2A-n30A-n77A | - |
|  | CA_n2A-n48A-n66A |  |
|  | CA_n2A-n48A-n77A |  |
|  | CA_n2A-n66A-n77A |  |
|  | CA_n2(2A)-n5A | - |
|  | CA_n2(2A)-n14A | - |
|  | CA_n2(2A)-n30A | - |
|  | CA_n2A-n48(2A) |  |
|  | CA_n2A-n48B |  |
|  | CA_n2(2A)-n66A | - |
|  | CA_n2A-n66(2A) | - |
|  | CA_n2A-n77C |  |
|  | CA_n2A-n77(2A) | - |
|  | CA_n3A-n5A-n78A |  |
|  | CA_n3A-n8A-n78A |  |
|  | CA_n3A-n28A-n40A |  |
|  | CA_n3A-n28A-n41A |  |
|  | CA_n3A-n28A-n77A |  |
|  | CA_n3A-n28A-n78A | - |
|  | CA_n3A-n40A-n77A |  |
|  | CA_n3A-n41A-n77A |  |
|  | CA_n3A-n77(2A) |  |
|  | CA_n5A-n30A-n77A |  |
|  | CA_n5A-n48A-n66A |  |
|  | CA_n5A-n48A-n77A |  |
|  | CA_n5A-n66A-n77A |  |
|  | CA_n5A-n48(2A) |  |
|  | CA_n5A-n48B |  |
|  | CA_n5A-n66(2A) | - |
|  | CA_n5A-n77C |  |
|  | CA_n5A-n77(2A) | - |
|  | CA_n5B-n77A |  |
|  | CA_n14A-n30A-n66A |  |
|  | CA_n14A-n66(2A) | - |
|  | CA_n14A-n66A-n77A | - |
|  | CA_n14A-n77(2A) | - |
|  | CA_n25A-n66A-n77A | - |
|  | CA_n25A-n66A-n78A | - |
|  | CA_n26A-n66-n70A |  |
|  | CA_n26A-n66(2A) |  |
|  | CA_n28A-n40A-n77A |  |
|  | CA_n28A-n41A-n77A |  |
|  | CA_n28A-n41C |  |
|  | CA_n28A-n41A-n79A | - |
|  | CA_n28A-n77(2A) |  |
|  | CA_n28A-n78A-n79A |  |
|  | CA_n3A-n78A-n79A | - |
|  | CA_n30A-n66(2A) | - |
|  | CA_n30A-n77(2A) | - |
|  | CA_n41A-n66A-n71A |  |
|  | CA_n41A-n79C |  |
|  | CA_n41C-n79A |  |
|  | CA_n48A-n66(2A) |  |
|  | CA_n48A-n71(2A) |  |
|  | CA_n48A-n77C |  |
|  | CA_n48B-n66A |  |
|  | CA_n48B-n70A |  |
|  | CA_n48B-n71A |  |
|  | CA_n48B-n77A |  |
|  | CA_n48(2A)-n66A |  |
|  | CA_n48(2A)-n70A |  |
|  | CA_n48(2A)-n71A |  |
|  | CA_n48(2A)-n77A |  |
|  | CA_n48A-n66A-n70A |  |
|  | CA_n48A-n66A-n71A |  |
|  | CA_n48A-n66A-n77A |  |
|  | CA_n48A-n70A-n71A |  |
|  | CA_n66A-n70A-n71A | - |
|  | CA_n66A-n71A-n77A | - |
|  | CA_n66A-n71A-n78A | - |
|  | CA_n66A-n71(2A) |  |
|  | CA_n66(2A)-n70A | - |
|  | CA_n66(2A)-n71A | - |
|  | CA_n66B-n70A | - |
|  | CA_n66B-n71A | - |
|  | CA_n66A-n77C |  |
|  | CA_n66A-n77(2A) | - |
|  | CA_n66(2A)-n77A | - |
|  | CA_n70A-n71A(2A) |  |
| SDL configuration | CA_n29A-n66A-n70A | - |
|  | CA_n29A-n66B | - |
|  | CA_n29A-n66(2A) | - |

7.3A.2_1	Reference sensitivity power level for 3DL CA exceptions

-------- Skip sections not changed -------

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n77 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1775/ DL 1870 | n77 | 3915 | n1 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n78 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | UL 1950/ DL 2140 | n3 | UL 1770/ DL 1865 | n78 | 3360 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3780 | n3 | UL 1770/ DL 1865 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n5A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/DL 874 | n78 | 3780 | n1 | DL 2122 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1975/DL2165 | n78 | 3405 | n5 | DL 885 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1950/DL2140 | n5 | UL830/DL875 | n78 | DL 3610 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1945/ DL 2135 | n8 | UL 900/ DL 945 | n78 | DL 3745 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1940/ DL 2130 | n78 | 3380 | n8 | DL 940 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n77 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n1A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n41 | 2650 | n77 | 3330 | 5MHz | 10MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2640 | n77 | 3710 | n1 | DL 2140 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n28 | UL 733/ DL 788 | n78 | 3416 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n78 | 3352 | n28 | DL 794 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n28A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n78 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n79A_UL_n1A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n28 | UL 730/ DL 785 | n79 | 4630 | 5MHz | 5MHz | 40MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n79A_UL_n1A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1930/ DL 2120 | n79 | 4648 | n28 | DL 788 | 5MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 745.5/ DL 800.5 | n79 | 4420 | n1 | DL 2167.5 | 5MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n78A-n79A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3410 | n79 | 4870 | 5MHz | 10MHz | 40MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n78A-n79A_UL_n1A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n79 | 4670 | n78 | 3490 | 5MHz | 40MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n78A-n79A_UL_n78A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3400 | n79 | 4660 | n1 | DL 2140 | 10MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n66A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1990/ DL 1980 | n5 | UL 830/ DL 875 | n66 | DL 2140 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n5A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n48 | 3640 | n2 | DL 1962 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n5 | UL 844/ DL 1889 | n48 | DL 3593 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1907.5/ DL 1987.5 | n77 | 3305 | n5 | DL 887.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n2 | DL 1987 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n5 | UL 830/ DL 875 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n66A_UL_n2A-n14A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1874/ DL 1954 | n14 | UL 793/ DL 763 | n66 | DL 2162 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n66A_UL_n14A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793/ DL 763 | n66 | UL 1770/ DL 2170 | n2 | DL 1954 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n77A_UL_n2A-n14A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1880/ DL 1960 | n14 | UL 793/ DL 763 | n77 | DL 3466 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n77A_UL_n14A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793/ DL 763 | n77 | 3540 | n2 | DL 1954 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n30A-n77A_UL_n2A-n30A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1870/ DL 1950 | n30 | UL 2310/ DL 2355 | n77 | DL 4180 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n30A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n77 | 3361 | n30 | DL 2354 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n2 | UL 1860/ DL 1940 | n77 | 3967 | n30 | DL 2354 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n30A-n77A_UL_n30A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n30 | UL 2312/ DL 2357 | n77 | 3305 | n2 | DL 1986 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n66 | UL 1770/ DL 2190 | n48 | DL 3625 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n48 | 3560 | n66 | DL 2155 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n48A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1735/ DL 2135 | n48 | 3695 | n2 | DL 1960 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3620 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3900 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n77 | 3970 | n66 | DL 2115 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n77 | 3500 | n66 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n2 | UL 1885/ DL 1965 | n77 | 3915 | n66 | DL 2175 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1760/ DL 2160 | n77 | 3720 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n66 | UL 1760/ DL 2160 | n77 | 3350 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n66 | UL 1760/ DL 2160 | n77 | 3620 | n2 | DL 1960 | 5Mhz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n5A-n78A_UL_n3A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL1825 | n5 | UL 839/ DL 884 | n78 | DL 3408 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1730/ DL1825 | n5 | UL 839/ DL 884 | n78 | DL 3512 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n5A-n78A_UL_n5A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n78 | 3540 | n3 | DL 1862 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n3A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3550 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3370 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n8A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | UL 910/ DL 955 | n78 | 3640 | n3 | DL 1820 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2518 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 25 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2687 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n3A-n40A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL 1825 | n40 | 2320 | n77 | 4050 | 5MHz | FFS | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n77 | 4030 | n40 | 2310 | 5MHz | 10MHz | FFS | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n40A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2310 | n77 | 4130 | n3 | 1820 | FFS | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2510 | n28 | UL 735/ DL 790 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 710.5/ DL 765.5 | n41 | 2543 | n3 | UL 1737.5/ DL 1832.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n28 | UL 733/ DL 788 | n77 | 4173 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n77 | 3320 | n3 | UL 1755/ DL 1850 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1712.5/ DL 1807.5 | n77 | 4195 | n28 | UL 715/ DL 770 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n78A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1755/ DL 1850 | n28 | UL 735/ DL 790 | n78 | 3320 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1750/ DL 1845 | n28 | UL 743/ DL 798 | n78 | 3764 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2580 | n77 | 3440 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2620 | n77 | 3400 | n3 | DL 1840 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n77 | 3900 | n41 | 2640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n30A-n77A_UL_n5A-n30A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 840/ DL 885 | n30 | UL 2310/ DL 2355 | n77 | DL 3780 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n30A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 835/ DL 880 | n77 | 4025 | n30 | DL 2355 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | CP-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n30A-n77A_UL_n30A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n30 | UL 2310/ DL 2355 | n77 | 3740 | n30 | DL 880 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | CP-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n48A-n66A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/ DL 874 | n66 | UL 1760/ DL 2160 | n48 | DL 3622 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n5 | UL 845/ DL 890 | n66 | UL 1775/ DL 2175 | n77 | DL 3465 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n5 | UL 826.5/ DL 871.5 | n66 | UL 1712.5/ DL 2112.5 | n77 | DL 4192 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n5 | UL 835/ DL 880 | n66 | UL 1735/ DL 2135 | n77 | DL 3535 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 826.5/ DL 871.5 | n77 | 3795 | n66 | DL 2142 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n14A-n66A-n77A_UL_n14A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793 / DL 763 | n66 | UL 1755 / DL 2155 | n77 | DL 3341 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n14A-n66A-n77A_UL_n14A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793 / DL 763 | n77 | 3741 | n66 | DL 2155 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n14A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1712.5 / DL 2112.5 | n77 | 4188 | n14 | DL 763 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n28A-n40A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 708/ DL 2120 | n40 | 2310 | n77 | 3736 | 5MHz | FFS | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 708/ DL 763 | n77 | 3550 | n40 | 2134 | 5MHz | 10MHz | FFS | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n40A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2302.5 | n77 | 3795 | n28 | 800.5 | FFS | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 743/ DL 798 | n41 | 2580 | n77 | 3323 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 738/ DL 793 | n77 | 3380 | n41 | 2642 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2642 | n77 | 3440 | n28 | DL 798 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 25 | n41 | 2567.5 | n77 | 3460 | n28 | DL 782.5 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n41A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2600 | n79 | 4600 | n28 | UL 725/ DL 780 | 10MHz | 40 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 720/ DL 780 | n41 | 2600 | n79 | 4600 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n79 | 4850 | n41 | 2645 | 5MHz | 40 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n78A-n79A_UL_n28A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n78 | 3700 | n79 | 4440 | 5MHz | 10MHz | 40MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n78A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n79 | 4440 | n78 | 3700 | 5MHz | 40MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n78A-n79A_UL_n78A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3620 | n79 | 4420 | n28 | DL 800 | 10MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n77 | 4108 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n77 | 4080 | n71 | DL 640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n71A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 690/ DL 644 | n77 | 3530 | n66 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n78 | 3724 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n71A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 693/ DL 647 | n78 | 3546 | n66 | DL 2160 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a. Note 5: This test ID is for UL PC2 only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skip sections not changed -------

7.3A.2_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 ans A.2.3 with parameters specified in Table 7.3A.2_1.5-1 for PC3 and Table 7.3A.2_1.5-1a for PC2.

Table 7.3A.2_1.5-1: Reference sensitivity requirement for PC3 inter-band 3DL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 7, Note 8, Note 9) |
| --- | --- | --- | --- | --- |
| DL_n1A-n3A-n77A_UL_n1A-n3A | 1 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +28.4 |
| DL_n1A-n3A-n77A_UL_n3A-n77A | 1 | n1 | 5 | REF_victim +31.0 |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n1A-n3A-n78A_UL_n1A-n3A | 1 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +28.4 |
|  | 2 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +11.2 |
| DL_n1A-n3A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +27.9 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n5A-n78A_UL_n5A-n78A | 1 | n1 | 5 | REF_victim +18.1 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n5A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_victim +3.1 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n5A-n78A_UL_n1A-n5A | 1 | n1 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +15.7 |
| DL_n1A-n8A-n78A_UL_n1A-n8A | 1 | n1 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +14.9 |
| DL_n1A-n8A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_victim +3.3 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n28A-n78A_UL_n1A-n28A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +15.7 |
| DL_n1A-n28A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +4.2 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n28A-n78A_UL_n28A-n78A | 1 | n1 | 5 | REF_victim +15.7 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n28A-n77A_UL_n28A-n77A | 1 | n1 | 5 | REF_victim +15.7 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n1A-n28A-n79A_UL_n1A-n28A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +14.9 |
| DL_n1A-n28A-n79A_UL_n1A-n79A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +15.2 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n1A-n28A-n79A_UL_n28A-n79A | 1 | n1 | 5 | REF_victim +1.2 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n1A-n41A-n77A_UL_n1A-n41A | 1 | n1 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +19.6 |
| DL_n1A-n41A-n77A_UL_n41A-n77A | 1 | n1 | 5 | REF_victim +9.3 |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n1A-n78A-n79A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +15.9 |
| DL_n1A-n78A-n79A_UL_n1A-n79A | 1 | n1 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.6 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n1A-n78A-n79A_UL_n78A-n79A | 1 | n1 | 5 | REF_victim +15.6 |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n2A-n5A-n48A_UL_n5A-n48A | 1 | n2 | 5 | REF_victim +15.6 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_aggressor |
| DL_n2A-n5A-n48A_UL_n2A-n5A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_victim +16.6 |
| DL_n2A-n5A-n66A_UL_n2A-n5A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +7.2 |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_victim +3.8 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 5 | REF_victim +16.5 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n5A-n77A_UL_n2A-n5A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.0 |
| DL_n2A-n14A-n66A_UL_n2A-n14A | 1 | n2 | 5 | REF_aggressor |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +7.6 |
| DL_n2A-n14A-n66A_UL_n14A-n66A | 1 | n2 | 5 | REF_victim +7.2 |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
| DL_n2A-n14A-n77A_UL_n2A-n14A | 1 | n2 | 5 | REF_aggressor |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.0 |
| DL_n2A-n14A-n77A_UL_n14A-n77A | 1 | n2 | 5 | REF_victim +16.5 |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n30A-n77A_UL_n2A-n30A | 1 | n2 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim + 29.4 |
| DL_n2A-n30A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_victim + 10.6 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_victim + 3.4 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n30A-n77A_UL_n30A-n77A | 1 | n2 | 5 | REF_victim + 8.6 |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n48A-n66A_UL_n2A-n66A | 1 | n2 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_victim +32 |
|  |  | n66 | 5 | REF_aggressor |
| DL_n2A-n48A-n66A_UL_n2A-n48A | 1 | n2 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +12.1 |
| DL_n2A-n48A-n66A_UL_n48A-n66A | 1 | n2 | 5 | REF_victim +28.3 |
|  |  | n48 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
| DL_n2A-n66A-n77A_UL_n2A-n66A | 1 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +29.4 |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +8.9 |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +29.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +10.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +4.0 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 5 | REF_victim +32.1 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n2 | 5 | REF_victim +9.1 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n2 | 5 | REF_victim +2.1 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n5A-n78A_UL_n3A-n5A | 1 | n3 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +16.1 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.5 |
| DL_n3A-n5A-n78A_UL_n5A-n78A | 1 | n3 | 5 | REF_victim + 15.7 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n3A-n8A-n78A_UL_n3A-n8A | 1 | n3 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +16.1 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.5 |
| DL_n3A-n8A-n78A_UL_n8A-n78A | 1 | n3 | 5 | REF_victim +15.7 |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n3A-n28A-n78A_UL_n3A-n28A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +17.3 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.5 |
| DL_n3A-n28A-n41A_UL_n3A-n28A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +27.4 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +15.9 |
| DL_n3A-n28A-n41A_UL_n3A-n41A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +26.0 |
|  |  | n41 | 5 | REF_aggressor |
| DL_n3A-n28A-n41A_UL_n28A-n41A | 1 | n3 | 5 | REF_victim +26.0 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_aggressor |
| DL_n3A-n28A-n77A_UL_n3A-n28A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim 15.9 |
| DL_n3A-n28A-n77A_UL_n28A-n77A | 1 | n3 | 5 | REF_victim +17.0 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n28A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +15.3 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n40A-n77A_UL_n3A-n40A | 1 | n3 | 5 | REF_aggressor |
|  |  | n40 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim + 19.0 |
| DL_n3A-n40A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n40 | 5 | REF_victim + 29.4 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n40A-n77A_UL_n40A-n77A | 1 | n3 | 5 | REF_victim + 29.9 |
|  |  | n40 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n41A-n77A_UL_n3A-n41A | 1 | n3 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.8 |
| DL_n3A-n41A-n77A_UL_n41A-n77A | 1 | n3 | 5 | REF_victim +16.4 |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n41A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +5.3 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n30A-n77A_UL_n5A-n30A | 1 | n5 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim + 16.1 |
| DL_n5A-n30A-n77A_UL_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_victim + 13.2 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n30A-n77A_UL_n30A-n77A | 1 | n5 | 5 | REF_victim + 15.2 |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n48A-n66A_UL_n5A-n66A | 1 | n5 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_victim +3.6 |
|  |  | n66 | 5 | REF_aggressor |
| DL_n5A-n66A-n77A_UL_n5A-n66A | 1 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.1 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +8.2 |
|  | 3 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +3.3 |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +13.2 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n14A-n66A-n77A_UL_n14A-n66A | 1 | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.0 |
| DL_n14A-n66A-n77A_UL_n14A-n77A | 1 | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +13.2 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n14A-n66A-n77A_UL_n66A-n77A | 1 | n14 | 5 | REF_victim +15.2 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n28A-n41A-n77A_UL_n28A-n41A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +28.2 |
| DL_n28A-n41A-n77A_UL_n28A-n77A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +29.5 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n28A-n41A-n77A_UL_n41A-n77A | 1 | n28 | 5 | REF_victim +30.8 |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n28 | 5 | REF_victim +3.0 |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n28A-n41A-n79A_UL_n41A-n79A | 1 | n28 | 5 | REF_victim +13 |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n28A-n41A-n79A_UL_n28A-n41A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +10.1 |
| DL_n28A-n41A-n79A_UL_n28A-n79A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +10.4 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n66A-n71A-n77A_UL_n66A-n71A | 1 | n66 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +15.9 |
| DL_n28A-n78A-n79A_UL_n28A-n78A | 1 | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +26.2 |
| DL_n28A-n78A-n79A_UL_n28A-n79A | 1 | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +26.9 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n28A-n78A-n79A_UL_n78A-n79A | 1 | n28 | 5 | REF_victim +16.2 |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n66A-n71A-n77A_UL_n66A-n77A | 1 | n66 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_victim +15.3 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n66A-n71A-n78A_UL_n66A-n71A | 1 | n66 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +9 |
| DL_n66A-n71A-n78A_UL_n71A-n78A | 1 | n66 | 5 | REF_victim +15.5 |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void. Note 4: Void Note 5: Void Note 6: Void Note 7: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 9: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 10: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

[End of change]