3GPP TSG-RAN5 Meeting #108	R5-255299

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0860 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UE capability for Rel18 PC2 NR CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE capability information of the following Inter-band NRCA configuration is not complete. CA_ n1A-n79A PC2 CA_ n1A-n28A-n78A PC2 CA_ n1A-n28A-n79A PC2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the following NRCA configuration in Table A.4.3.2A.4.1-4. CA_ n1A-n79A PC2 - Adding the following NRCA configurations in Table A.4.3.2A.4.2-4 CA_ n1A-n28A-n78A PC2 CA_ n1A-n28A-n79A PC2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capabilities for these NRCA configuration will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2A.4.1, A.4.3.2A.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-255299 is the final version of R5-254535 with below change. - Removed changes to PC2 UE RF Baseline Implementation Capabilities for UL CA_n1A-n28A from Table A.4.3.2A.4.2-4, and corrected the Item No. |  |  |  |  |  |  |  |  |

[Start of change]

A.4.3.2A.4	NR Inter-band CA within FR1

A.4.3.2A.4.1	NR Inter-band CA within FR1 (two bands)

-------- Skip sections not changed -------

Table A.4.3.2A.4.1-4: Inter-band CA within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities

| Item | CA configuration | Inter-band CA within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CA_n1A-n78A | n1 band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) n78 band: 3300-3800 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n1A_n78A_PC2_Supp |  |
| A | CA_n2A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| A1 | CA_n2A-n77C | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| A2 | CA_n2A-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| A3 | CA_n2(2A)-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| A4 | CA_n2(2A)-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 1B | CA_n3A-n41A | n3 band: 1710-1785 MHz (UL),1805- 1880 MHz (DL) n41 band: 2496-2690 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n3A_n41A_PC2_Supp |  |
| 2 | CA_n3A-n78A | n3 band: 1710-1785 MHz (UL),1805- 1880 MHz (DL) n78 band: 3300-3800 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n3A_n78A_PC2_Supp |  |
| 2A | CA_n5A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 2B | CA_n5A-n77C | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 2C | CA_n5A-n77(2A) | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 2D | CA_n14A-n77(2A) | n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n14A_n77A_PC2_Supp |  |
| 3 | CA_n28A-n41A | n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n41 band: 2496-2690 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n28A_n41A_PC2_Supp |  |
| 3A | CA_n28A-n78A | n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n78 band: 3300-3800 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n28A_n78A_PC2_Supp |  |
| 4 | CA_n28A-n79A | n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n28A_n79A_PC2_Supp |  |
| 4A | CA_n30A-n77(2A) | n30 band: 2305-2315 MHz (UL), 2350-2360 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n30A_n77A_PC2_Supp |  |
| 5 | CA_n41A-n79A | n41 band: 2496-2690 MHz n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n41A_n79A_PC2_Supp |  |
| 6 | CA_n66A-n77A | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6A | CA_n66A-n77C | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6B | CA_n66A-n77(2A) | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6C | CA_n66(2A)-n77A | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6D | CA_n66(2A)-n77(2A) | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6E | CA_n66(3A)-n77A | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 7 | CA_n78A-n79A | n78 band: 3300-3800 MHz n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n78A_n79A_PC2_Supp |  |
| 8 | CA_n3A-n77A | n3 band: 1710-1785 MHz (UL), 1805- 1880 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n3A_n77A_PC2_Supp |  |
| 9 | CA_n41A-n77A | n41 band: 2496-2690 MHz n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n41A_n77A_PC2_Supp |  |
| 10 | CA_n28A-n77A | n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n28A_n77A_PC2_Supp |  |
| 11 | CA_n1A-n79A | n1 band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n1A_n79A_PC2_Supp |  |

-------- Skip sections not changed -------

A.4.3.2A.4.2	NR Inter-band CA within FR1 (three bands)

-------- Skip sections not changed -------

Table A.4.3.2A.4.2-4: Inter-band CA within FR1 (three bands) PC2 UE RF Baseline Implementation Capabilities

| Item | CA configuration | Inter-band CA within FR1 (three bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | CA_n2A-n48A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n48 band: 3550-3700 MHz n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 1 | CA_n2A-n5A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2 | CA_n2A-n5A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 2A | CA_n2(2A)-n5A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2B | CA_n2(2A)-n5A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 2C | CA_n2A-n5A-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2D | CA_n2A-n5A-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 2E | CA_n2A-n14A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2F | CA_n2A-n14A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n14A_n77A_PC2_Supp |  |
| 2G | CA_n2(2A)-n14A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2H | CA_n2(2A)-n14A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n14A_n77A_PC2_Supp |  |
| 2I | CA_n2A-n14A-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2J | CA_n2A-n14A-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n14A_n77A_PC2_Supp |  |
| 3 | CA_n2A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 4 | CA_n2A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 4A | CA_n5A-n48A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n48 band: 3550-3700 MHz n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 4B | CA_n2(2A)-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 4C | CA_n2(2A)-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 4D | CA_n2A-n66(2A)-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 4E | CA_n2A-n66(2A)-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 4F | CA_n2A-n66A-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 4G | CA_n2A-n66A-n77(2A) | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 5 | CA_n5A-n66A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 6 | CA_n5A-n66A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6A | CA_n5A-n66(2A)-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 6B | CA_n5A-n66(2A)-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6C | CA_n5A-n66A-n77(2A) | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 6D | CA_n5A-n66A-n77(2A) | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6E | CA_n14A-n66A-n77A | n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n14A_n77A_PC2_Supp |  |
| 6F | CA_n14A-n66A-n77A | n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6G | CA_n14A-n66(2A)-n77A | n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n14A_n77A_PC2_Supp |  |
| 6H | CA_n14A-n66(2A)-n77A | n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6I | CA_n14A-n66-n77(2A) | n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n14A_n77A_PC2_Supp |  |
| 6J | CA_n14A-n66A-n77(2A) | n14 band: 788-798 MHz (UL), 758-768 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 7 | CA_n48A-n66A-n77A | n48 band: 3550-3700 MHz n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 8 | CA_n1A-n28A-n78A | n1 band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n78 band: 3300-3800 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n1A_n78A_PC2_Supp |  |
| 9 | CA_n1A-n28A-n78A | n1 band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n78 band: 3300-3800 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n28A_n78A_PC2_Supp |  |
| 10 | CA_n1A-n28A-n79A | n1 band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n1A_n79A_PC2_Supp |  |
| 11 | CA_n1A-n28A-n79A | n1 band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n28A_n79A_PC2_Supp |  |

[End of change]