3GPP TSG-RAN5 Meeting #108 	R5-255221

# Bengaluru, India, 25th Aug. 2025 - 29th Aug. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3502 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity requirements for Rel-18 2DL CA for HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, NTT DOCOMO INC., Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following 2DL CA configurations are missing in 7.3A Reference sensitivity. - CA_n8A-n78A (PC2) - CA_n78A-n79A (PC1.5) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - CA_n8A-n78A was added to Tables 7.3A.0.4-4c, 7.3A.0.5-1a, 7.3A.1_1.4.1-1, and 7.3A.1_1.5-1a. - CA_n78A-n79A was added to Tables 7.3A.0.6-3b, 7.3A.1_1.4.1-1, and 7.3A.1_1.5-1b. Editorial correction to text style of the title of Table 7.3A.0.4-4c. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The missing CA configuration will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.4, 7.3A.0.5, 7.3A.0.6, 7.3A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis: TR 38.905 CR 1069 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-254752 and the outcome of 1 revision to this document. r1: Added Huawei, HiSilicon as co-source. Added CA_n8A-n78A (PC2). |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.0.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band in frequency range 1 is impacted by UL harmonic interference from another band which belongs to NR band in frequency range 1 of the same downlink CA configuration. Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for either single band uplink or PC3 or PC2 CA are specified in Table 7.3A.0.4-1. For these exceptions, only the listed test points in Table 7.3A.0.4-1 are needed to be tested.

<<Unchanged tables skipped>>

Table 7.3A.0.4-4c: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | 10 | 15 | 25 | 5 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 4.4 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n14 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 14.1 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 10.2 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n78 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n78 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 11.7 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. |  |  |  |  |  |  |  |  |

<<Unchanged table skipped>>

7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two CCs from up to two UL NR bands and three CCs from two UL NR bands given in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-1b, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-1b, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a. For these test points the reference sensitivity requirement specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b, Table 7.3.2.3-2 and Table 7.3.2.3-2a are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-1b, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a.

<<Unchanged table skipped>>

Table 7.3A.0.5-1a: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 20.0 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 19.10 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 18.4 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| CA_n3-n774 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
| CA_n5-n77 | 5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD44,13 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 34.33 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 7: Void. NOTE 8: Void. NOTE 9: Void. NOTE 10: Void. NOTE 11: Void. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: Void. NOTE 15: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

<<Unchanged table skipped>>

7.3A.0.6	Reference sensitivity exceptions due to cross band isolation for CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part which belongs to NR band of the same NR CA configuration due to cross band isolation issues. The reference sensitivity degradation for the victim band due to cross band isolation is specified only for the specific uplink and downlink test points specified in Table 7.3A.0.6-1 for either PC3 and PC2 NR CA from a PC3 aggressor NR UL band, and for PC2 NR CA, in Table 7.3A.0.6-3a from a PC2 aggressor NR UL band, and in Table 7.3A.0.6-3b from a PC1.5 aggressor NR single band uplink.

In Tables 7.3A.0.6-1, 7.3A.0.6-3a and 7.3A.0.6-3b the following terminology is used to define the source of cross-band isolation interference:

-	"ACLR1" indicates that the first adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

-	"ACLR2" indicates that the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

-	">ACLR2" indicates that neither the first, nor the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

<<Unchanged tables skipped>>

Table 7.3A.0.6-3b: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 13.3 | >ACLR2 |
| n77 | n2 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.8 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 9.0 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 9.0 | >ACLR2 |
| n77 | n66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 1.8 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 8 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 8 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 8 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 8.6 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 8.6 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.A.

<<Unchanged sections skipped>>

7.3A.1_1	Reference sensitivity power level for 2DL CA exceptions

Editor’s Note: The following aspects are either missing or not yet determined:

- Test point analysis for CA_n3A-n5A harmonic mixing is currently missing in TR 38.905.

Editor’s Note: Test configuration table and test requirements in clauses 7.3A.1_1.4.1 and 7.3A.1_1.5 alignment with TS 38.101-1 (V18.9.0) is FFS.

7.3A.1_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.1_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2DL CA

7.3A.1_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.1_1.4	Test description

7.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Refer to “PCC”and “SCC” columns |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n8 | 897.5 MHz (UL) | n78 | 3635 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 45 | n78 | 3770 | n8 | Mid | 10MHz | 5MHz | CP-OFDM QPSK | 25@0 |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 55 | n78 | 3770 | n8 | Mid | 10MHz | 20MHz | CP-OFDM QPSK | 25@0 |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n78A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3750 | n79 | 4420 | 100 | 40 | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 2 | n79 | 4450 | n78 | 3795 | 100 | 10 | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 3 | n79 | 4450 | n78 | 3750 | 100 | 100 | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 49 | n78 | 3750 | n79 | 4450 | 100 | 100 | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 5 | n78 | 3750 | n79 | 4405 | 100 | 10 | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3A.0.4-1. REFSENS_CA_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3A.0.4-4b for PC3, Table 7.3A.0.4-4c for PC2, and Table 7.3A.0.4-4d for PC1.5. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference due to 2UL CA according to table 7.3A.0.5-1 for PC3, table 7.3A.0.5-1a for PC2 and table 7.3A.0.5-1b for PC1.5. REFSENS_CA_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation for NR CA FR1 according to table 7.3A.0.6-1 for PC3, Table 7.3A.0.6-3a for PC2, and Table 7.3A.0.6-3b for PC1.5. Note 3: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 4: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2A.4.1-3 of TS 38.508-2. Note 5: This test ID is for UL PC2 only. Note 6: This test ID is for UL PC3 only. Note 7: This test ID is for UL PC1.5 only. Note 8: Test ID 7 for CA_n3A-n77A shall not be tested as BCS1 is not defined for this combo. Note 9: This test ID is for UL PC2 and UL PC1.5. |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1 for CA configuration with single uplink carrier indicating PC2 in TS 38.508-2 table A.4.3.2A.4.1-3 but without note 4 applied in table 5.5A.3.1-1.

For inter-band carrier aggregation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier with note 4 applied in table 5.5A.3.1-1 and UE indicating power class 2 in TS 38.508-2 Table A.4.3.2A.4.1-3, and Table 7.3A.1_1.5-1b for PC1.5 2UL CA configuration and CA configuration with single uplink carrier indicating power class 1.5 in TS 38.508-2 table A.4.3.2A.4.1-3.

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

<<Unchanged table skipped>>

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter-band CA with PC2 single UL carrier or PC2 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 8, Note 9, Note 10) |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | 1 | n1 | 5 | REF_victim +17.8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n2A-n77A | 4 | n2 | 5 | REF_victim +9.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n2 | 20 | REF_victim +4.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n2 | 5 | REF_victim +32.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +19.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n2 | 5 | REF_victim +20.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n2 | 5 | REF_victim +1.0 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n3A-n41A | 1 | n3 | 5 | REF_victim +18.4 |
|  |  | n41 | 10 | REF_aggressor |
|  | 2 | n3 | 5 | REF_victim +2.3 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n3A-n77A | 4 | n3 | 5 | REF_victim +31.9 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +18.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n3 | 5 | REF_victim +8.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n3 | 40 | REF_victim +1.0 |
|  |  | n77 | 40 | REF_aggressor |
| CA_n3A-n78A | 6 | n3 | 5 | REF_victim + 8.1 |
|  |  | n78 | 5 | REF_aggressor |
|  | 7 | n3 | 40 | REF_victim + 1.0 |
|  |  | n78 | 40 | REF_aggressor |
| CA_n5A-n77A | 5 | n5 | 20 | REF_victim +1.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n5 | 5 | REF_victim +8.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n5 | 5 | REF_victim +18.6 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n8A-n78A | 3 | n8 | 5 | REF_victim +15.5 |
|  |  | n78 | 10 | REF_aggressor |
|  | 4 | n8 | 5 | REF_victim +8.1 |
|  |  | n78 | 10 | REF_aggressor |
|  | 5 | n8 | 20 | REF_victim +1.5 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n14A_n77A | 4 | n14 | 5 | REF_victim + 34 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n14 | 10 | REF_victim + 31 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A_n77A | 3 | n28 | 5 | REF_victim +19.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 4 | n28 | 5 | REF_victim +18.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n28 | 5 | REF_victim +18.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n28 | 5 | REF_victim +31 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A_n78A | 3 | n28 | 5 | REF_victim + 31 |
|  |  | n78 | 10 | REF_aggressor |
|  | 4 | n28 | 30 | REF_victim + 11.7 |
|  |  | n78 | 30 | REF_aggressor |
| CA_n30A-n77A | 1 | n30 | 5 | REF_victim + 13.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n30 | 10 | REF_victim + 10.6 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n30 | 5 | REF_victim + 17.6 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n41A-n77A | 1 | n41 | 10 | REF_victim +13.2 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim +8.8 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.5 |
|  | 4 | n41 | 10 | REF_victim+6.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim+6.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n41A-n79A | 1 | n41 | 100 | REF_aggressor |
|  |  | n79 | 10 | REF_victim +3.1 |
|  | 2 | n41 | 10 | REF_victim +3.5 |
|  |  | n79 | 100 | REF_aggressor |
| CA_n66A-n77A | 4 | n66 | 5 | REF_victim +34.33 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n66 | 5 | REF_victim +11.27 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n66 | 5 | REF_victim +1.0 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n78A-n79A | 1 | n78 | 100 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +5 |
|  | 2 | n78 | 10 | REF_victim +5.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 3 | n78 | 100 | REF_victim +5.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 4 | n78 | 100 | REF_aggressor |
|  |  | n79 | 100 | REF_victim +5 |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 9: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 10: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

Table 7.3A.1_1.5-1b: Reference sensitivity requirement for inter-band CA with PC1.5 single UL carrier or PC1.5 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 3, Note 4, Note 5) |
| --- | --- | --- | --- | --- |
| CA_n2A-n77A | 4 | n2 | 5 | REF_victim +11.8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n2 | 20 | REF_victim +6.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n2 | 5 | REF_victim +35.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +26.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n2 | 5 | REF_victim +28.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n2 | 5 | REF_victim +1.8 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n3A-n77A | 8 | n3 | 5 | REF_victim +10.7 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n5A-n77A | 5 | n5 | 20 | REF_victim +2.6 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n5 | 5 | REF_victim +10.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n5 | 5 | REF_victim +25.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n5 | 5 | REF_victim +24.3 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n25A-n77A | 6 | n25 | 5 | REF_victim +37.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n25 | 5 | REF_victim +29.8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n25 | 5 | REF_victim +25.8 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A_n77A | 7 | n28 | 5 | REF_victim +37 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n41A-n77A | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +13.3 |
|  | 4 | n41 | 10 | REF_victim +9.0 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim +9.0 |
|  |  | n77 | 100 | REF_aggressor |
|  | 10 | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +14.5 |
|  | 11 | n41 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +5.5 |
|  | 12 | n41 | 10 | REF_victim +17.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 13 | n41 | 100 | REF_victim +7.9 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n66A-n77A | 4 | n66 | 5 | REF_victim +40.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n66 | 5 | REF_victim +26.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n66 | 5 | REF_victim +1.8 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n71A-n77A | 2 | n71 | 5 | REF_victim +16.0 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n78A-n79A | 1 | n78 | 100 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +8 |
|  | 2 | n78 | 10 | REF_victim +8.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 3 | n78 | 100 | REF_victim +8.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 4 | n78 | 100 | REF_aggressor |
|  |  | n79 | 100 | REF_victim +8 |
|  | 5 | n78 | 100 | REF_aggressor |
|  |  | n79 | 10 | REF_victim +8 |

Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2). Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 4: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 5: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports.

<<End of change>>