3GPP TSG-RAN5 Meeting #108	R5-255000

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5132 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to assumed PUSCH PHR MAC Test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Specific message contents for Physical layer parameters for DCI format 0_1 for Transform precoder indicator needs to be specified as per 38.212. cl 7.3.1.1.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added specific message contents for for Physical layer parameters for DCI format 0_1 for Transform precoder indicator |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test spec remains misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.3.23	UE power headroom reporting / PCMAX for assumed PUSCH reporting

7.1.1.3.23.1	Test Purpose (TP)

(1)

with { UE is in RRC_connected state. UE is configured with phr-AssumedPUSCH-Reporting and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 is set to disabled in active BWP }

ensure that {

when { phr-PeriodicTimer is configured in UE }

then { UE doesn't transmit a MAC PDU containing Power Headroom with assumed PUSCH MAC Control Element }

}

(2)

with { UE is in RRC_connected state. UE is configured with phr-Tx-PowerFactorChange, phr-AssumedPUSCH-Reporting and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 is set to enabled in active BWP }

ensure that {

when { phr-PeriodicTimer is configured in UE }

then { UE transmits a MAC PDU containing Power Headroom with assumed PUSCH MAC Control Element }

}

(3)

with { UE is in RRC_connected state. UE is configured with phr-AssumedPUSCH-Reporting and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 is set to enabled in active BWP }

ensure that {

when { phr-PeriodicTimer expires and UL resources allocated for new transmission }

then { UE transmits a MAC PDU containing Power Headroom with assumed PUSCH MAC Control Element }

}

(4)

with { UE is in RRC_connected state. UE is configured with phr-AssumedPUSCH-Reporting and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 is set to enabled in active BWP }

ensure that {

when { phr-ProhibitTimer expires and power headroom report is triggered due to DL Pathloss change}

then { UE transmits a MAC PDU containing Power Headroom with assumed PUSCH MAC Control Element }

}

7.1.1.3.23.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.321 clause 5.4.6, 6.1.3.78 and 6.1.3.79. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.4.6]

RRC controls Power Headroom reporting by configuring the following parameters:

-	dpc-Reporting-FR1;

-	phr-AssumedPUSCH-Reporting;

…

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

NOTE 1:	The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink of which firstActiveDownlinkBWP-Id is not set to dormant BWP;

-	activation of an SCG;

-	addition of the PSCell except if the SCG is deactivated (i.e. PSCell is newly added or changed);

-	phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

-	Upon switching of activated BWP from dormant BWP to non-dormant DL BWP of an SCell of any MAC entity with configured uplink;

-	if dpc-Reporting-FR1 is configured, ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance, as specified in TS 38.101-1 [14] and TS 38.101-3 [16]).

-	if mpe-Reporting-FR2 is configured, and mpe-ProhibitTimer is not running:

-	the measured P-MPR applied to meet FR2 MPE requirements as specified in TS 38.101-2 [15] is equal to or larger than mpe-Threshold for at least one activated FR2 Serving Cell since the last transmission of a PHR in this MAC entity; or

-	the measured P-MPR applied to meet FR2 MPE requirements as specified in TS 38.101-2 [15] has changed more than phr-Tx-PowerFactorChange dB for at least one activated FR2 Serving Cell since the last transmission of a PHR due to the measured P-MPR applied to meet MPE requirements being equal to or larger than mpe-Threshold in this MAC entity.

in which case the PHR is referred below to as 'MPE P-MPR report'.

NOTE 2:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,f,c/PH when a PHR is triggered by other triggering conditions.

NOTE 3:	If a HARQ process is configured with cg-RetransmissionTimer and if the PHR is already included in a MAC PDU for transmission on configured grant by this HARQ process, but not yet transmitted by lower layers, it is up to UE implementation how to handle the PHR content.

If the MAC entity has UL resources allocated for a new transmission the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start phr-PeriodicTimer.

1>	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in clause 5.4.3.1:

2>	if multiplePHR with value true is configured:

3>	for each activated Serving Cell with configured uplink associated with any MAC entity of which the active DL BWP is not dormant BWP; and

3>	for each activated Serving Cell with configured uplink associated with E-UTRA MAC entity:

4>	if this MAC entity is configured with twoPHRMode:

5>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN and the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	if the UE supports mTRP-PUSCH-PHR-Type1-Reporting-r17:

7>	obtain two values of the Type 1 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell.

6>	else:

7>	obtain two values of the Type 1 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell.

5>	else if this Serving Cell is configured with multiple TRP PUSCH repetition (i.e., not configured with multipanelSchemeSDM or multipanelSchemeSFN) and the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	obtain two values of the Type 1 or the value of Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell.

5>	else:

6>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell.

4>	else (i.e. this MAC entity is not configured with twoPHRMode):

5>	if this Serving Cell is configured with multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN and if the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	if there is at least one real PUSCH transmission at the slot where the PHR MAC CE is transmitted:

7>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

8>	if the first TCI-State or TCI-UL-State is applied for a real PUSCH transmission:

9>	obtain the value of the Type 1 power headroom of the real PUSCH transmission associated with the first TCI-State or TCI-UL-State for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

8>	else:

9>	obtain the value of the Type 1 power headroom of the real PUSCH transmission associated with the second TCI-State or TCI-UL-State for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

7>	else if this Serving Cell is configured with multiple TRP PUSCH repetition:

8>	obtain the value of the Type 1 power headroom of the first real transmission of the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

6>	else if there is no real PUSCH transmission at the slot where the PHR MAC CE is transmitted:

7>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

8>	obtain the value of the Type 1 power headroom of the reference PUSCH transmission associated with the first TCI-State or TCI-UL-State for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

7>	else if this Serving Cell is configured with multiple TRP PUSCH repetition:

8>	if the UE supports mTRP-PUSCH-PHR-Type1-Reporting-r17:

9>	obtain the value of the Type 1 power headroom of the reference PUSCH transmission associated with the SRS-ResourceSet with a lower SRS-resourceSetID for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

8>	else:

9>	obtain the value of the Type 1 power headroom of the reference PUSCH transmission associated with the SRS-ResourceSet with a lower SRS-resourceSetID or the value of the Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

5>	else:

6>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell.

4>	if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

5>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

5>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

6>	if dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 or dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 is set to enabled in the active BWP of this Serving Cell:

7>	obtain the value for the corresponding PCMAX,f,c field for assumed PUSCH from the physical layer if available, as specified in clause 7.7 of TS 38.213 [6].

6>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

6>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

7>	obtain the value for the corresponding MPE field from the physical layer.

…

3>	if phr-Type2OtherCell with value true is configured:

4>	if the other MAC entity is E-UTRA MAC entity:

5>	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity);

5>	if phr-ModeOtherCG is set to real by upper layers:

6>	obtain the value for the corresponding PCMAX,f,c field for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity) from the physical layer.

3>	if this MAC entity is configured with mpe-Reporting-FR2-r17:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple entry PHR as defined in clause 6.1.3.49 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and any Serving Cell belonging to this MAC entity is configured with multipanelSchemeSDM or multipanelSchemeSFN:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE as defined in clause 6.1.3.82 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and any Serving Cell belonging to this MAC entity is configured with multiple TRP PUSCH repetition:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP MAC CE as defined in clause 6.1.3.51 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR with assumed PUSCH MAC CE as defined in clause 6.1.3.79 based on the values reported by the physical layer.

3>	else:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR MAC CE as defined in clause 6.1.3.9 based on the values reported by the physical layer.

2>	else (i.e. Single Entry PHR format is used):

3>	if this MAC entity is configured with twoPHRMode for multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN:

4>	obtain two values of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell.

3>	else:

4>	obtain the value of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell.

3>	if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

4>	if dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 or dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 is set to enabled in the active BWP of this Serving Cell:

5>	obtain the value for the corresponding PCMAX,f,c field for assumed PUSCH from the physical layer, if available, as specified in clause 7.7 of TS 38.213 [6].

3>	if this MAC entity is configured with twoPHRMode and if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

4>	obtain two values for the corresponding PCMAX,f,c,k fields from the physical layer.

4>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

5>	obtain two values for the corresponding MPEk fields from the physical layer.

3>	else:

4>	obtain the value for the corresponding PCMAX,f,c field from the physical layer;

4>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2:

5>	obtain the value for the corresponding MPE field from the physical layer.

4>	if mpe-Reporting-FR2-r17 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

5>	obtain the value for the corresponding MPEi field from the physical layer;

5>	obtain the value for the corresponding Resourcei field from the physical layer.

4>	if dpc-Reporting-FR1 is configured and this Serving Cell operates on FR1:

5>	obtain the value for the corresponding DPC field from the physical layer.

3>	if this MAC entity is configured with mpe-Reporting-FR2-r17:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Single entry PHR as defined in clause 6.1.3.48 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Single Entry PHR for multiple TRP STx2P MAC CE as defined in clause 6.1.3.81 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and this Serving Cell is configured with multiple TRP PUSCH repetition:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Single Entry PHR for multiple TRP MAC CE as defined in clause 6.1.3.50 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR with assumed PUSCH MAC CE as defined in clause 6.1.3.78 based on the values reported by the physical layer.

3>	else:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR MAC CE as defined in clause 6.1.3.8 based on the values reported by the physical layer.

2>	if this PHR report is an MPE P-MPR report:

3>	start or restart the mpe-ProhibitTimer;

3>	cancel triggered MPE P-MPR reporting for Serving Cells included in the PHR MAC CE.

2>	start or restart phr-PeriodicTimer;

2>	start or restart phr-ProhibitTimer;

2>	cancel all triggered PHR(s).

All triggered PHRs shall be cancelled when there is an ongoing SDT procedure as in clause 5.27 and the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the PHR MAC CE plus its subheader.

[TS 38.321, clause 6.1.3.78]

The Single Entry PHR with assumed PUSCH MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-2b.

It has a variable size and consists of two or three octets defined as follows (Figure 6.1.3.78-1):

-	R: Reserved bit, set to 0;

-	Power Headroom (PH): This field indicates the power headroom level. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1 (the corresponding measured values in dB are specified in TS 38.133 [11]);

-	E: This field indicates the presence of a PCMAX,f,c for assumed PUSCH field for PCell. The E field set to 1 indicates that a PCMAX,f,c for assumed PUSCH field for PCell is reported. The E field set to 0 indicates that a PCMAX,f,c for assumed PUSCH field for PCell is not reported;

-	P: If mpe-Reporting-FR2 is configured and the Serving Cell operates on FR2, the MAC entity shall set this field to 0 if the applied P-MPR value, to meet MPE requirements, as specified in TS 38.101-2 [15], is less than P-MPR_00 as specified in TS 38.133 [11] and to 1 otherwise. If mpe-Reporting-FR2 is not configured or the Serving Cell operates on FR1, this field indicates whether power backoff is applied due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]). The MAC entity shall set the P field to 1 if the corresponding PCMAX,f,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,f,c: This field indicates the PCMAX,f,c (as specified in TS 38.213 [6]) used for calculation of the preceding PH field. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in Table 6.1.3.8-2 (the corresponding measured values in dBm are specified in TS 38.133 [11]);

-	PCMAX,f,c for assumed PUSCH: This field indicates the PCMAX,f,c for assumed PUSCH(as specified in TS 38.213 [6]). The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in [Table 6.1.3.8-2] (the corresponding measured values in dBm are specified in TS 38.133 [11]);

-	MPE: If mpe-Reporting-FR2 is configured, and the Serving Cell operates on FR2, and if the P field is set to 1, this field indicates the applied power backoff to meet MPE requirements, as specified in TS 38.101-2 [15]. This field indicates an index to Table 6.1.3.8-3 and the corresponding measured values of P-MPR levels in dB are specified in TS 38.133 [11]. The length of the field is 2 bits. If mpe-Reporting-FR2 is not configured, or if the Serving Cell operates on FR1, or if the P field is set to 0, R bits are present instead.

Figure 6.1.3.78-1: Single Entry PHR with assumed PUSCH MAC CE

[TS 38.321, clause 6.1.3.79]

The Multiple Entry PHR with assumed PUSCH MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-2b.

It has a variable size, and includes the bitmap, a Type 2 PH field , an octet containing the associated PCMAX,f,c field (if reported) and an octet containing the associated PCMAX,f,c field for assumed PUSCH (if reported) for SpCell of the other MAC entity; a Type 1 PH field, an octet containing the associated PCMAX,f,c field (if reported) and an octet containing the associated PCMAX,f,c field for assumed PUSCH (if reported) for the PCell. It further includes, in ascending order based on the ServCellIndex, one or multiple of Type X PH fields, octets containing the associated PCMAX,f,c fields (if reported) and octets containing the associated PCMAX,f,c fields for assumed PUSCH (if reported) for Serving Cells other than PCell indicated in the bitmap. X is either 1 or 3 according to TS 38.213 [6] and TS 36.213 [17].

The presence of Type 2 PH field for SpCell of the other MAC entity is configured by phr-Type2OtherCell with value true.

A single octet Ci bitmap is used for indicating the presence of PH per Serving Cell when the highest ServCellIndex of Serving Cell with configured uplink is less than 8, otherwise four octets are used.

The length of the Ek bitmap is based on the total number of Serving Cells, i.e. Serving Cells for which Ci field set to 1 and PCell. A single octet Ek bitmap is included if the total number of Serving Cells is greater than 0 and less than 9; a two octets Ek bitmap is included if the total number of Serving Cells is greater than 8 and less than 17; a three octets Ek bitmap is included if the total number of Serving Cells is greater than 16 and less than 25; a four octets Ek bitmap is included if the total number of Serving Cells is greater than 24; Ek bitmap is not included if the total number of Serving Cells is zero.

The MAC entity determines whether PH value for an activated Serving Cell is based on real transmission or a reference format by considering the configured grant(s) and downlink control information which has been received until and including the PDCCH occasion in which the first UL grant for a new transmission that can accommodate the MAC CE for PHR as a result of LCP as defined in clause 5.4.3.1 is received since a PHR has been triggered if the PHR MAC CE is reported on an uplink grant received on the PDCCH or until the first uplink symbol of PUSCH transmission minus PUSCH preparation time as defined in clause 7.7 of TS 38.213 [6] if the PHR MAC CE is reported on a configured grant.

For a band combination in which the UE does not support dynamic power sharing, the UE may omit the octets containing Power Headroom field and PCMAX,f,c field for Serving Cells in the other MAC entity except for the PCell in the other MAC entity and the reported values of Power Headroom and PCMAX,f,c for the PCell are up to UE implementation.

The PHR MAC CEs are defined as follows:

-	Ci: This field indicates the presence of a PH field for the Serving Cell with ServCellIndex i as specified in TS 38.331 [5]. The Ci field set to 1 indicates that a PH field for the Serving Cell with ServCellIndex i is reported. The Ci field set to 0 indicates that a PH field for the Serving Cell with ServCellIndex i is not reported;

-	Ek: This field indicates the presence of a PCMAX,f,c for assumed PUSCH field of the kth Serving Cell. The Serving Cells for which Ci field is set to 1 and PCell are indexed sequentially starting with PCell and followed by other Serving cells in ascending order of ServCellIndex i as specified in TS 38.331 [5]. The Ek field set to 1 indicates that a PCMAX,f,c for assumed PUSCH field for the kth Serving Cell is reported. The Ek field set to 0 indicates that a PCMAX,f,c for assumed PUSCH field for the kth Serving Cell is not reported. For the E-UTRA Serving Cell, the corresponding Ek field is set to 0;

-	R: Reserved bit, set to 0;

-	V: This field indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, the V field set to 0 indicates real transmission on PUSCH and the V field set to 1 indicates that a PUSCH reference format is used. For Type 2 PH, the V field set to 0 indicates real transmission on PUCCH and the V field set to 1 indicates that a PUCCH reference format is used. For Type 3 PH, the V field set to 0 indicates real transmission on SRS and the V field set to 1 indicates that an SRS reference format is used. Furthermore, for Type 1, Type 2, and Type 3 PH, the V field set to 0 indicates the presence of the octet containing the associated PCMAX,f,c field and the MPE field, and the V field set to 1 indicates that the octet containing the associated PCMAX,f,c field and the MPE field is omitted;

-	Power Headroom (PH): This field indicates the power headroom level. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1 (the corresponding measured values in dB for the NR Serving Cell are specified in TS 38.133 [11] while the corresponding measured values in dB for the E-UTRA Serving Cell are specified in TS 36.133 [12]);

-	P: If mpe-Reporting-FR2 is configured and the Serving Cell operates on FR2, the MAC entity shall set this field to 0 if the applied P-MPR value, to meet MPE requirements, as specified in TS 38.101-2 [15], is less than P-MPR_00 as specified in TS 38.133 [11] and to 1 otherwise. If mpe-Reporting-FR2 is not configured or the Serving Cell operates on FR1, this field indicates whether power backoff is applied due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]). The MAC entity shall set the P field to 1 if the corresponding PCMAX,f,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,f,c: If present, this field indicates the PCMAX,f,c (as specified in TS 38.213 [6]) for the NR Serving Cell and the PCMAX,c or P̃CMAX,c (as specified in TS 36.213 [17]) for the E-UTRA Serving Cell used for calculation of the preceding PH field. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in Table 6.1.3.8-2 (the corresponding measured values in dBm for the NR Serving Cell are specified in TS 38.133 [11] while the corresponding measured values in dBm for the E-UTRA Serving Cell are specified in TS 36.133 [12]);

-	PCMAX,f,c for assumed PUSCH: If present, this field indicates the PCMAX,f,c for assumed PUSCH(as specified in TS 38.213 [6]) for the NR Serving Cell. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in [Table 6.1.3.8-2] (the corresponding measured values in dBm for the NR Serving Cell are specified in TS 38.133 [11];

-	MPE: If mpe-Reporting-FR2 is configured, and the Serving Cell operates on FR2, and if the P field is set to 1, this field indicates the applied power backoff to meet MPE requirements, as specified in TS 38.101-2 [15]. This field indicates an index to Table 6.1.3.8-3 and the corresponding measured values of P-MPR levels in dB are specified in TS 38.133 [11]. The length of the field is 2 bits. If mpe-Reporting-FR2 is not configured, or if the Serving Cell operates on FR1, or if the P field is set to 0, R bits are present instead.

Figure 6.1.3.79-1: Multiple Entry PHR with assumed PUSCH MAC CE with the highest ServCellIndex of Serving Cell with configured uplink is less than 8

Figure 6.1.3.79-2: Multiple Entry PHR with assumed PUSCH MAC CE with the highest ServCellIndex of Serving Cell with configured uplink is equal to or higher than 8

7.1.1.3.23.3	Test description

7.1.1.3.23.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.3.23.3.2	Test procedure sequence

Table 7.1.1.3.23.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. | <-- | - | - | - |
| 2 | SS transmits NR RRCReconfiguration message to configure specific Power Headroom parameters for NR Cell(Note 1). | <-- | (RRCReconfiguration) | - | - |
| 3 | Check: does the UE transmit a MAC PDU containing Power Headroom MAC CE? (Note 2, 5) | --> | MAC PDU | 1 | P |
| 4 | The UE transmits an NR RRCReconfigurationComplete message to confirm the setup of Power Headroom parameters. (Note 2,3) | --> | (RRCReconfigurationComplete) | - | - |
| 5 | Check: does the UE transmit a MAC PDU containing Power Headroom MAC CE 500ms after step 3? (Note 5) | --> | MAC PDU | 1 | P |
| 6 | SS transmits NR RRCReconfigurationmessage to configure specific Power Headroom parameters for NR Cell(Note 1). | <-- | (RRCReconfiguration) | - | - |
| 7 | Check: does the UE transmit a MAC PDU containing Power Headroom MAC CE including assumed PUSCH? (Note 2, 6) | --> | MAC PDU | 2 | P |
| 8 | The UE transmits an NR RRCReconfigurationComplete message to confirm the setup of Power Headroom parameters. (Note 2,3) | --> | (RRCReconfigurationComplete) | - | - |
| 9 | Check: does the UE transmit a MAC PDU containing Power Headroom MAC CE including assumed PUSCH 500ms after step 7? (Note 6) | --> | MAC PDU | 3 | P |
| 10 | SS transmits NR RRCReconfigurationmessage to configure specific Power Headroom parameters for NR Cell.(Note 1) | <-- | (RRCReconfiguration) | - | - |
| 11 | Check: does the UE transmit a MAC PDU containing Power Headroom MAC CE including assumed PUSCH? (Note 4, 6) | --> | MAC PDU | 2 | P |
| 12 | The UE transmits an NR RRCReconfigurationComplete message to confirm the setup of Power Headroom parameters. (Note 3,4) | --> | (RRCReconfigurationComplete) | - | - |
| 13 | Reduce SS power level for NR Cell by 5 dB so as to cause a DL_Pathloss change at UE by 5 dB. | - | - | - | - |
| 14 | Check: after prohibitPHR-Timer after step 11, does the UE transmit a MAC PDU containing Power Headroom MAC CE including assumed PUSCH? (Note 6) | --> | MAC PDU | 4 | P |
| 15 | Increase SS power level for NR Cell by 5 dB so as to cause a DL_Pathloss change at UE by 5 dB. | - | - | - | - |
| 16 | Check: after prohibitPHR-Timer after step 14, does the UE transmit a MAC PDU containing Power Headroom MAC CE including assumed PUSCH? (Note 6) | --> | MAC PDU | 4 | P |
| 17 | The SS transmits an NR RRCReconfiguration message to disable Power Headroom reporting.(Note 1) | <-- | (RRCReconfiguration) | - | - |
| 18 | The UE transmits an NR RRCReconfigurationComplete message to confirm the disabling of Power Headroom parameters.(Note 3) | --> | (RRCReconfigurationComplete) | - | - |
| Note 1: for EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: Steps 3 and 4/7 and 8 can happen in any order. Note 3: for EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 4: Steps 11 and 12 can happen in any order. Note 5: For NR5GC the received MAC PDU will contain Single-entry PHR MAC CE. For EN-DC the received MAC PDU will contain Multiple-Entry PHR MAC CE. Note 6: For NR5GC the received MAC PDU will contain Single-entry PHR with assumed PUSCH MAC CE. For EN-DC/ the received MAC PDU will contain Multiple-Entry PHR with assumed PUSCH MAC CE. |  |  |  |  |  |

7.1.1.3.23.3.3	Specific message contents

Table 7.1.1.3.23.3.3-1: RRCReconfiguration (step 2 Table 7.1.1.3.23.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-2: CellGroupConfig (Table 7.1.1.3.23.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | sf500 |  |  |
| phr-ProhibitTimer | sf1000 |  |  |
| phr-Tx-PowerFactorChange | infinity |  |  |
| multiplePHR | false |  |  |
| multiplePHR | true |  | EN-DC |
| phr-Type2PCell | false |  |  |
| phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| phr-AssumedPUSCH-Reporting-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-3: RRCReconfiguration (step 6 Table 7.1.1.3.23.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-4: CellGroupConfig (Table 7.1.1.3.23.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | sf500 |  |  |
| phr-ProhibitTimer | sf1000 |  |  |
| phr-Tx-PowerFactorChange | infinity |  |  |
| multiplePHR | false |  |  |
| multiplePHR | true |  | EN-DC |
| phr-Type2PCell | false |  |  |
| phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| phr-AssumedPUSCH-Reporting-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| servingCellConfig | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-5: ServingCellConfig (Table 7.1.1.3.23.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-6: BWP-UplinkDedicated (Table 7.1.1.3.23.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| Setup | PUSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-7: PUSCH-Config (Table 7.1.1.3.23.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 | enabled |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-8: RRCReconfiguration (step 10 Table 7.1.1.3.23.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-9: CellGroupConfig (Table 7.1.1.3.23.3.3-8)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | infinity |  |  |
| phr-ProhibitTimer | sf1000 |  |  |
| phr-Tx-PowerFactorChange | 3dB |  |  |
| multiplePHR | false |  |  |
| multiplePHR | true |  | EN-DC |
| phr-Type2PCell | false |  |  |
| phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| phr-AssumedPUSCH-Reporting-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| servingCellConfig | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-10: ServingCellConfig (Table 7.1.1.3.23.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-11: BWP-UplinkDedicated (Table 7.1.1.3.23.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| Setup | PUSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-12: PUSCH-Config (Table 7.1.1.3.23.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 | enabled |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-13: RRCReconfiguration (step 17 Table 7.1.1.3.23.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-14: CellGroupConfig (Table 7.1.1.3.23.3.3-13)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.23.3.3-15: Physical layer parameters for DCI format 0_1 (Step 7 to 16 of Table 7.1.1.3.23.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.1.2-1 with condition DWS |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Transform precoder indicator | A value of "0" indicates that transform precoder is enabled, i.e. DFT-s-OFDM waveform is used. | “0” |  |