3GPP TSG-RAN5 Meeting #108	R5-255088

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0676 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 11.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is no need to release the emergency PDU session immediately after release of the emergency call and the PDU session is kept in the generic IPCAN implementation in TTCN. In addition according to TS 24.229 clause 5.1.6.6 an emergency registration cannot be deregisterred but exists until it expires. But the emergency registration is useless without associated PDU session. - Editorial issues. - RRC connection needs to be released after the emergency call to bring the UE to idle mode. - Specific message contents refer to C.11 which is not applicable for 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition "keep emergency PDU session" added to the reference to TS 38.508-1 clause 4.9.12B. - Editorial corrections. - RRC connection release added as step 15A. - References in specific message contents corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 2 revision of R5-253980. |  |  |  |  |  |  |  |  |

## 11.18	eCall capable / Automatic initiation / Emergency registration / 200 OK with ACK / PSAP Callback / SIP INFO request for MSD Update / Success

11.18.1	Test Purpose (TP)

(1)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall type of emergency service }

ensure that {

when { UE receives INVITE for an incoming IMS call for PSAP callback }

then { UE responds with 200 OK including correctly composed header fields }

}

(2)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall type of emergency service }

ensure that {

when { SS requests for transfer of MSD during an incoming IMS call }

then { UE responds with 200 OK and performs MSD transfer }

}

11.18.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[Rel-19, TS 24.229, clause 5.1.6B]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. The procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD shall be supported by a UE during an incoming IMS call when the following conditions are fulfilled:

1)	the UE previously established or attempted to establish an emergency call either over IMS or in the CS domain of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service;

2)	the incoming IMS call occurred within an implementation dependent time period of at least one hour;

3)	the INVITE for the incoming IMS call contains a Priority header field with header field value "psap-callback";

4)	the INVITE for the incoming IMS call contains an Accept header field that includes the value “application/EmergencyCallData.Control+xml”; and

5)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

NOTE 1:	When there are multiple attempts to establish the emergency call, the emergency call in condition 2) refers to the last attempt.

When conditions 1) to 5) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

When conditions 1) and 2) and, as an implementation option, condition 3) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 2:	A PSAP can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the PSAP, the serving PLMN, the HPLMN or by some other intermediate network.

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6B.

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6B.

11.18.3	Test description

11.18.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.18.3.2	Test procedure sequence

Table 11.18.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start an automatic eCall | - | - | - | - |
| 2-14 | Steps 1-13 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. (Steps 1-4 of Annex A.3 and Steps 1-3 of Annex A.23) | - | - | - | - |
| 15 | The Test procedure for IMS MT Emergency call release as specified in TS 38.508-1 [21], subclause 4.9.12B takes place using condition 'keep emergency PDU session'. | - |  | - | - |
| 15A | The SS releases the RRC connection. | - | - | - | - |
| 16-27 | Steps 1-12 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [21] Table 4.9.16.2.2-1 are performed. (Steps 1-5 of Annex A.5.1a for UE configured with usage of preconditions else A.5.2a) | - | - | - | - |
| - | EXCEPTION: Steps 28a1-28b6 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 28a1-28a6 | IF the UE is configured to use preconditions THEN steps 6-10A of Annex A.5.1a take place. | - | - | - | - |
| 28a7 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 11 of annex A.5.1a) | --> | 200 OK | 1 | P |
| 28a8 | SS sends ACK | <-- | ACK | - | - |
| 28b1-39b4 | ELSE steps 6-8A of Annex A.5.2a of take place | - | - | - | - |
| 28b5 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 9 of annex A.5.2a) | --> | 200 OK | 1 | P |
| 28b6 | SS send ACK | <-- | ACK | - | - |
| 29-30 | Steps 4-5, as defined in Annex A.23 is performed. | - | - | - | - |
| 31 | Check: Does UE send SIP INFO message with MSD update? (Step 6 of annex A.23) | --> | SIP INFO | 2 | P |
| 32 | SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 33 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.18.3.3	Specific message contents

Table 11.18.3.3-1: INVITE (Step 1 of annex A.23, in parallel to steps 11-13, Table 11.18.3.2-1)

Derivation path: Step 1 in Annex A.23 with condition A21

Table 11.18.3.3-2: INVITE (Step 24, Table 11.18.3.2-1)

Derivation path: Step 1 in Annex A.5.1a or Annex A.5.2a with condition A12

Table 11.18.3.3-3: 200 OK for INVITE (Steps 28a7, 28b5, Table 11.18.3.2-1)

Derivation path: Step 11 in Annex A.5.1a or step 9 in Annex A.5.2a with condition A24

Table 11.18.3.3-4: BYE (Steps 15, 33, Table 11.18.3.2-1)

Derivation path: Step 1 in Annex A.8 with condition A9