3GPP TSG-RAN5 Meeting #108	R5-255233

# Bengaluru, India, 25-29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3516 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of AMPR NS_18 testing for n28 PC2 UL-MIMO test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n28_PC2_40MHz_CBW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 38.101-1 specified the AMPR/ASE NS_18 requirements for n28 PC2. The relevant test needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding minimum requirement, test configuration, test requirement for NS_18 with n28 PC2 into UL-MIMO test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The additional requirements for n28 PC2 can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3, 6.5D.3_1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 1077 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-255233 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2D.3	UE additional maximum output power reduction for UL MIMO

-	TP is FFS for NS_47 for Power Class 1.5.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04, NS_06, NS_07 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 1.5, Power Class 2 and 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

The requirements of this test apply in test case 6.5D.3_1.3 Additional Spurious Emissions for network signalling value NS_04, NS_07, NS_12, NS_13, NS_14_NS_15, NS_47, NS_21, NS_43, NS_43U, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3_1.3 are executed.

The requirements of UL full power transmission apply in the test cases 6.2D.1, 6.2D.2 and 6.2D.4 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of UL full power transmission apply in the test cases 6.5D.2.3 Additional spectrum emission mask for UL MIMO, 6.5D.2.4.2 Adjacent channel leakage ratio for UL MIMO and 6.5D.3_1.3 Additional spurious emissions for UL MIMO for corresponding network signalling value to all types of NR UE release 16 and for forward that support UL MIMO.NOTE:	This requirements of ULFPTx in this test case is not tested.

6.2D.3.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the A-MPR values specified in clause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.

6.2D.3.4	Test description

6.2D.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2D.3.4.1-1 to 6.2D.3.4.1-11. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

<Unchanged Text Skipped>

Table 6.2D.3.4.1-22: Test Configuration table for NS_47 power class 1.5

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 30 MHz |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK |  | Edge_1RB_Left (A1) |  |  |
| 2 | Default | 30 | Default |  |  | QPSK |  | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 3 | Default | 30 | Deafult |  |  | QPSK |  | Edge 1RB_Right (A3) |  |  |
| 4 | Default | 30 | Default |  |  | QPSK |  | Outer_Full (A2) |  |  |
| 5 | Default | 30 | Default |  |  | QPSK |  | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 6 | Default | 30 | Default |  |  | QPSK |  | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 7 | Default | 30 | Default |  |  | QPSK |  | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 8 | Default | 30 | Default |  |  | QPSK |  | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| 9 | Default | 30 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A1) |  |  |
| 10 | Default | 30 | Default |  |  | 16 QAM |  | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 11 | Default | 30 | Default |  |  | 16 QAM |  | Edge 1RB_Right (A3) |  |  |
| 12 | Default | 30 | Default |  |  | 16 QAM |  | Outer_Full (A2) |  |  |
| 13 | Default | 30 | Default |  |  | 16 QAM |  | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 14 | Default | 30 | Default |  |  | 16 QAM |  | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 15 | Default | 30 | Default |  |  | 16 QAM |  | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 16 | Default | 30 | Default |  |  | 16 QAM |  | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| 17 | Default | 30 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A1) |  |  |
| 18 | Default | 30 | Default |  |  | 64 QAM |  | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 19 | Default | 30 | Default |  |  | 64 QAM |  | Edge 1RB_Right (A3) |  |  |
| 20 | Default | 30 | Default |  |  | 64 QAM |  | Outer_Full (A2) |  |  |
| 21 | Default | 30 | Default |  |  | 64 QAM |  | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 22 | Default | 30 | Default |  |  | 64 QAM |  | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 23 | Default | 30 | Default |  |  | 64 QAM |  | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 24 | Default | 30 | Default |  |  | 64 QAM |  | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| 25 | Default | 30 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A1) |  |  |
| 26 | Default | 30 | Default |  |  | 256 QAM |  | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 27 | Default | 30 | Default |  |  | 256 QAM |  | Edge 1RB_Right (A3) |  |  |
| 28 | Default | 30 | Default |  |  | 256 QAM |  | Outer_Full (A2) |  |  |
| 29 | Default | 30 | Default |  |  | 256 QAM |  | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 30 | Default | 30 | Default |  |  | 256 QAM |  | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 31 | Default | 30 | Default |  |  | 256 QAM |  | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 32 | Default | 30 | Default |  |  | 256 QAM |  | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-24: Test Configuration table for NS_18 (power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Maximum supported channel bandwidth in each group: {3MHz, 5MHz}, {10MHz, 15MHz, 20MHz}, {25MHz, 30MHz, 40MHz} |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | Low | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz | Default | N/A | CP-OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  | 25MHz, 30MHz, 40MHz | 15kHz |  | CP-OFDM | 256 QAM | 75@58 (25MHz) 87@73 (30MHz) 112@104 (40MHz) | A3 |
| 10 |  |  | 30kHz |  |  | 256 QAM | 36@29 (25MHz) 42@36 (30MHz) 54@52 (40MHz) | A3 |
| 11 |  |  | Default |  |  | 256 QAM | Edge_1RB_Right | A3 |
| 12 |  |  | Default |  |  | 256 QAM | Outer_Full | A4 |
| 13 |  |  | Default |  |  | 16QAM | Edge_1RB_Left | A5 |
| 14 |  |  | Default |  |  | 64QAM | Edge_1RB_Left | A5 |
| 15 |  |  | Default |  |  | 256QAM | Edge_1RB_Left | A5 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-25: Test Configuration table for NS_18 (power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Maximum supported channel bandwidth in each group: {3MHz, 5MHz}, {10MHz, 15MHz, 20MHz}, {25MHz, 30MHz, 40MHz} |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | Low | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz | Default | N/A | CP-OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  | 25MHz, 30MHz, 40MHz | 15kHz |  | CP-OFDM | 256 QAM | 75@58 (25MHz) 87@73 (30MHz) 112@104 (40MHz) | A3 |
| 10 |  |  | 30kHz |  |  | 256 QAM | 36@29 (25MHz) 42@36 (30MHz) 54@52 (40MHz) | A3 |
| 11 |  |  | Default |  |  | 256 QAM | Edge_1RB_Right | A3 |
| 12 |  |  | 15kHz |  |  | 256 QAM | Outer_Full | A4 |
| 13 |  |  | 30kHz |  |  | 64QAM | Edge_1RB_Left | A5 |
| 14 |  |  | Default |  |  | 256QAM | Edge_1RB_Left | A5 |
| 15 |  |  | Default |  |  | QPSK | 75@58 (25MHz) 87@73 (30MHz) 112@104 (40MHz) | A6 |
| 16 |  |  | Default |  |  | QPSK | 36@29 (25MHz) 42@36 (30MHz) 54@52 (40MHz) | A6 |
| 17 |  |  | Default |  |  | 64QAM | 75@58 (25MHz) 87@73 (30MHz) 112@104 (40MHz) | A6 |
| 18 |  |  | Default |  |  | 64QAM | 36@29 (25MHz) 42@36 (30MHz) 54@52 (40MHz) | A6 |
| 19 |  |  | Default |  |  | 256QAM | 75@58 (25MHz) 87@73 (30MHz) 112@104 (40MHz) | A6 |
| 20 |  |  | 15kHz |  |  | 256QAM | 36@29 (25MHz) 42@36 (30MHz) 54@52 (40MHz) | A6 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable Table in clause 6.2D.3.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3.4.3.

6.2D.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to applicable table in clause 6.2D.3.4.1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

4.	Void

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in applicable table in clause 6.2D.3.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2D.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

6.2D.3.4.3.1	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.2D.3.4.3.2	Message contents exceptions for network signalling value "NS_35"

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

6.2D.3.4.3.3	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3.4.3.4	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3.4.3.5	Message contents exceptions for network signalling value "NS_48"

1.	Information element additionalSpectrumEmission is set to NS_48. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_48"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_48) |  |  |

6.2D.3.4.3.6	Message contents exceptions for network signalling value "NS_49"

1.	Information element additionalSpectrumEmission is set to NS_49. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_49"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_49) |  |  |

6.2D.3.4.3.7	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2D.3.4.3.8	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 6.2D.3.4.3.8-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

6.2D.3.4.3.9	Message contents exceptions for network signalling value "NS_03U"

1.	Information element additionalSpectrumEmission is set to NS_03U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.9-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03U) |  | for band n2, n25, n66 |

6.2D.3.4.3.10	Message contents exceptions for network signalling value "NS_46"

1.	Information element additionalSpectrumEmission is set to NS_46. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.10-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_46"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_46) |  | for band n7 |

6.2D.3.4.3.11	Message contents exceptions for network signalling value "NS_21"

1.	Information element additionalSpectrumEmission is set to NS_21. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.11-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

6.2D.3.4.3.12	Message contents exceptions for network signalling value "NS_44"

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.12-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_44"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_44) |  |  |

6.2D.3.4.3.13	Message contents exceptions for network signalling value "NS_27"

1.	Information element additionalSpectrumEmission is set to NS_27. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.13-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

6.2D.3.4.3.14	Message contents exceptions for network signalling value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.14-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2D.3.4.3.15	Message contents exceptions for network signalling value "NS_50"

1.	Information element additionalSpectrumEmission is set to NS_50. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.15-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_50"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_50) |  |  |

6.2D.3.4.3.16	Message contents exceptions for network signalling value "NS_43"

1.	Information element additionalSpectrumEmission is set to NS_43. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.16-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2( NS_43) |  |  |

6.2D.3.4.3.17	Message contents exceptions for network signalling value "NS_43U"

1.	Information element additionalSpectrumEmission is set to NS_43U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.17-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_43U) |  |  |

6.2D.3.4.3.18	Message contents exceptions for network signalling value "NS_06"

1.	Information element additionalSpectrumEmission is set to NS_06. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.18-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_06" and NR band n13

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_06) |  |  |

6.2D.3.4.3.19	Message contents exceptions for network signalling value "NS_07"

1.	Information element additionalSpectrumEmission is set to NS_07. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.19-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_07"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_07) |  |  |

6.2D.3.4.3.20	Message contents exceptions for network signalling value "NS_12"

1.	Information element additionalSpectrumEmission is set to NS_12. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.20-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_12"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_12) |  |  |

6.2D.3.4.3.21	Message contents exceptions for network signalling value "NS_13"

1.	Information element additionalSpectrumEmission is set to NS_13. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.21-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_13"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_13) |  |  |

6.2D.3.4.3.22	Message contents exceptions for network signalling value "NS_14"

1.	Information element additionalSpectrumEmission is set to NS_14. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.22-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_14"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_14) |  |  |

6.2D.3.4.3.23	Message contents exceptions for network signalling value "NS_15"

1.	Information element additionalSpectrumEmission is set to NS_15. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.23-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_15"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_15) |  |  |

6.2D.3.4.3.24	Message contents exceptions for network signalling value "NS_47"

1.	Information element additionalSpectrumEmission is set to NS_47. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.24-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_47) |  |  |

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-19. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

<Unchanged Text Skipped>

Table 6.2D.3.5-27: UE Power Class 1.5 test requirements (NS_47) for n41 (large FWA form factor with 20 dB or above antenna isolation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 2 | 29 | 0 | 1.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 3 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 4 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 5 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 6 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 7 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 8 | 29 | 0 | 4 | 4 | 0 | 25 | 3 | 3 | 31+TT | 22.0-TT |
| 9 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 10 | 29 | 0 | 2 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 11 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 12 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 13 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 16 | 29 | 0 | 4 | 4 | 0 | 25 | 3 | 3 | 31+TT | 22.0-TT |
| 17 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 18 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 19 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 20 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 21 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 24 | 29 | 0 | 4.5 | 0 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 25 | 29 | 0 | 7.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 26 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 27 | 29 | 0 | 7.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 28 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 29 | 29 | 0 | 7.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 30 | 29 | 0 | 7.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 31 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 32 | 29 | 0 | 7.5 | 0 | 0 | 21.5 | 5 | 3 | 31+TT | 16.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-30: UE Power Class 3 test requirements (NS_18) for n28

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 3, 5 | 23 | 3 | 5 | 0 | 18 | 5 | 2.5 | 25+TT | 13-TT |
|  | 10, 15, 20 | 23 | 3 | 6.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 3, 4 | 3, 5 | 23 | 3 | 5 | 0 | 18 | 5 | 2.5 | 25+TT | 13-TT |
|  | 10, 15, 20 | 23 | 3 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 5, 6 | 3, 5 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 10, 15, 20 | 23 | 3.5 | 8.5 | 0 | 14.5 | 6 | 2.5 | 25+TT | 8.5-TT |
| 7, 8 | 3, 5 | 23 | 6.5 | 8.5 | 0 | 14.5 | 6 | 2.5 | 25+TT | 8.5-TT |
|  | 10, 15, 20 | 23 | 6.5 | 11.5 | 0 | 11.5 | 6 | 2.5 | 25+TT | 5.5-TT |
| 9, 10, 11 | 25 | 23 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5–TT |
|  | 30, 40 | 23 | 7 | 4.5 | 0 | 16 | 5 | 2.5 | 25+TT | 11–TT |
| 12 | 25 | 23 | 6.5 | 9.5 | 0 | 13.5 | 6 | 2.5 | 25+TT | 7.5–TT |
|  | 30, 40 | 23 | 7 | 9.5 | 0 | 13.5 | 6 | 2.5 | 25+TT | 7.5–TT |
| 13 | 25 | 23 | 3 | 5 | 0 | 18 | 5 | 2.5 | 25+TT | 13-TT |
|  | 30, 40 | 23 | 3.5 | 5 | 0 | 18 | 5 | 2.5 | 25+TT | 13-TT |
| 14 | 25 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 30, 40 | 23 | 4 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
| 15 | 25 | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 2.5 | 25+TT | 9.5-TT |
|  | 30, 40 | 23 | 7 | 7.5 | 0 | 15.5 | 6 | 2.5 | 25+TT | 9.5-TT |

Table 6.2D.3.5-31: UE Power Class 2 test requirements (NS_18)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 3, 5 | 26 | 4.0 | 5.5 | 0 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
|  | 10, 15, 20 | 26 | 4.0 | 9.5 | 0 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 3, 4 | 3, 5 | 26 | 4.0 | 5.5 | 0 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
|  | 10, 15, 20 | 26 | 4.0 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 5, 6 | 3, 5 | 26 | 4.5 | 6 | 0 | 20 | 5 | 3 | 28+TT | 15-TT |
|  | 10, 15, 20 | 26 | 4.5 | 11.5 | 0 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 7, 8 | 3, 5 | 26 | 8.0 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
|  | 10, 15, 20 | 26 | 8.0 | 11.5 | 0 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 9, 10, 11 | 25 | 26 | 8.0 | 5.5 | 0 | 18 | 5 | 3 | 28+TT | 13-TT |
|  | 30, 40 | 26 | 8.5 | 5.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 12 | 25 | 26 | 8 | 12.5 | 0 | 13.5 | 6 | 3 | 28+TT | 7.5-TT |
|  | 30, 40 | 26 | 8.5 | 12.5 | 0 | 13.5 | 6 | 3 | 28+TT | 7.5-TT |
| 13 | 25 | 26 | 4.5 | 7 | 0 | 19 | 5 | 3 | 28+TT | 14-TT |
|  | 30, 40 | 26 | 5 | 7 | 0 | 19 | 5 | 3 | 28+TT | 14-TT |
| 14 | 25 | 26 | 8 | 8 | 0 | 18 | 5 | 3 | 28+TT | 13-TT |
|  | 30, 40 | 26 | 8.5 | 8 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 15, 16 | 25 | 26 | 3.5 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
|  | 30, 40 | 26 | 4 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 17, 18 | 25 | 26 | 4.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16-TT |
|  | 30, 40 | 26 | 5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16-TT |
| 19, 20 | 25 | 26 | 8 | 6.5 | 0 | 18 | 5 | 3 | 28+TT | 13-TT |
|  | 30, 40 | 26 | 8.5 | 6.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |

<Unchanged Text Skipped>

6.5D.3_1.3	Additional spurious emissions for UL MIMO (Rel-16 onward)

6.5D.3_1.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3_1.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support 2-layer codebook based UL MIMO.

6.5D.3_1.3.3 Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The additional spurious emission requirements specified in clause 6.5.3.3.3 apply. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1. For UEs supporting ULFPTx for UL MIMO, the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3_1.3.4	Test description

6.5D.3_1.3.4.1	Initial conditions

Same initial conditions as in clause 6.5D.3.3.4.1 together with tables 6.5D.3_1.3.4-1 to 6.5D.3_1.3.4-8.

Table 6.5D.3_1.3.4.1-1: Test Configuration table for NS_05

Same test configuration as listed in Table 6.2D.3.4.1-3 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-2: Test Configuration table for NS_48

Same test configuration as listed in Table 6.2D.3.4.1-5 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-3: Test Configuration table for NS_49

Same test configuration as listed in Table 6.2D.3.4.1-6 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-4: Test Configuration table for NS_46

Same test configuration as listed in Table 6.2D.3.4.1-8 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-5: Test Configuration table for NS_21

Same test configuration as listed in Table 6.2D.3.4.1-9 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-6: Test Configuration table for NS_44

Same test configuration as listed in Table 6.2D.3.4.1-10 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-7: Test Configuration table for NS_27

Same test configuration as listed in Table 6.2D.3.4.1-11 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-8: Test Configuration table for NS_07 and NS_50

Same test configuration as listed in Table 6.2D.3.4.1-13 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-9: Test Configuration table for NS_12

Same test configuration as listed in Table 6.2D.3.4.1-18 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-10: Test Configuration table for NS_13

Same test configuration as listed in Table 6.2D.3.4.1-19 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-11: Test Configuration table for NS_14

Same test configuration as listed in Table 6.2D.3.4.1-20 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-12: Test Configuration table for NS_15

Same test configuration as listed in Table 6.2D.3.4.1-21 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

Table 6.5D.3_1.3.4.1-13: Test Configuration table for NS_18

Same test configuration as listed in Table 6.2D.3.4.1-23 for power class 3 and Table 6.2D.3.4.1-24 for power class 2 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

6.5D.3_1.3.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to tables in clause 6.5D.3_1.3.4.1 as appropriate. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each UE antenna connector in the channel bandwidth of the radio access mode according to the test configurations in clause 6.5D.3_1.3.4.1, which shall meet the requirements described in clauses 6.2D.3.5 as appropriate for each network signalling. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of transmitted power at each UE antenna connector with a measurement filter of bandwidths according to clauses 6.5.3.3.3.1 to 6.5.3.3.3.25 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

5.	Void.

6.5D.3_1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and same exceptions listed in clause 6.5.3.3.4.3.

6.5D.3_1.3.5	Test requirement

The measured power from both UE antenna connector derived in step 4 shall meet the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated in clause 6.5.3.3.5.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

< End of Changes >