3GPP TSG-RAN5 Meeting #108	R5-254093

# Bengaluru, India, 25 – 29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3436 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding receiver requirements for band n110 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R19-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing receiver requirements for band n110 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added receiver requirements for band n110 into clauses 7.3.2, 7.6.2, 7.6.3, 7.6.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Receiver requirements for band n110 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 7.6.2, 7.6.3, 7.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 7.3.2	Reference sensitivity power level

Editor’s note: The following aspects are either missing or not yet determined:

- MU and TT for >6GHz are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

7.3.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that don’t support RedCap.

7.3.2.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a, 7.3.2.3-1b, Table 7.3.2.3-1c, Table 7.3.2.3-1d, Table 7.3.2.3-2 and Table 7.3.2.3-2a.

Table 7.3.2.3-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

|  | Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
|  |  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 | -90.2 | -89.7 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 | -90.3 | -89.7 |
| n2 |  | 15 |  | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 |  | -81.5 |  |  |
|  |  | 30 |  |  | -95.1 | -93.1 | -92 | -90.8 | -84.2 |  | -81.6 |  |  |
|  |  | 60 |  |  | -95.5 | -93.4 | -92.2 | -90.9 | -84.3 |  | -81.7 |  |  |
| n3 |  | 15 |  | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
|  |  | 30 |  |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.3 | -82.4 | -81.4 | -79.8 |
|  |  | 60 |  |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.4 | -82.6 | -81.5 | -79.9 |
| n5 |  | 15 |  | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  |  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n71 |  | 15 |  | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 |  | -88.6 |  | -81.5 |
|  |  | 30 |  |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 |  | -88.7 |  | -81.5 |
|  |  | 60 |  |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 |  | -88.9 |  | -81.5 |
| n8 |  | 15 |  | -97.0 | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |  |  |  |
|  |  | 30 |  |  | -94.1 | -91.7 | -87.2 | -84.7 | -81.4 | -78.5 |  |  |  |
| n12 |  | 15 |  | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n13 |  | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n14 |  | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n20 |  | 15 |  | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 |  | 15 |  | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -97.1 |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 |  | 15 |  | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -81.7 | -79.5 | -77.6 |  |
|  |  | 30 |  |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -81.8 | -79.6 | -77.7 |  |
|  |  | 60 |  |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 | -81.9 | -79.7 | -77.8 |  |
| n26 |  | 15 | -99.7 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 |  | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  | -66.3 |  |  |
|  |  | 30 |  |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  | -66.3 |  |  |
| n30 |  | 15 |  | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -96.1 |  |  |  |  |  |  |  |  |
| n31 |  | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n65 |  | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  |  | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  |  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 |  | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 |  | -90.1 | -89.6 |  |
|  |  | 30 |  |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 |  | -90.2 | -89.7 |  |
|  |  | 60 |  |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 |  | -90.4 | -89.8 |  |
|  |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n70 |  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 |  | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.18 -74.89 | -82.58 -67.19 | -80.78 -64.09 |  |  |  |
|  |  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.28 -74.99 | -82.68 -67.29 | -80.88 -64.19 |  |  |  |
| n72 |  | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n74 |  | 15 |  | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  |  | 30 |  |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  |  | 60 |  |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| n85 |  | 15 | -99.2 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n100 |  | 15 | -102.2 | -100 |  |  |  |  |  |  |  |  |  |
| n105 |  | 15 |  | -97.210 | -94.0 | -91.6 | -86.9 | -85.1 | -83.8 | -82.5 |  |  |  |
|  |  | 30 |  |  | -94.3 | -91.9 | -87.9 | -85.5 | -84.3 | -82.6 |  |  |  |
| n106 |  | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |
| n110 |  | 15 | -102.2 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 9: Applies to UEs that support optional symmetric UL/DL for this BW. NOTE 10: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS. |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 7.3.2.3-3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n1 | 15 |  | 25 | 501 | 751 | 1001 | 1281 | 1281 |  | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 501 | 641 | 641 |  | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 24 | 301 | 301 |  | 301 | 301 | 301 |  |  |  |  |  |  |
| n2 | 15 |  | 25 | 501 | 501 | 501 | 501 | 481 |  | 401 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 241 | 241 | 241 | 241 |  | 201 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| n3 | 15 |  | 25 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |
| n5 | 15 |  | 25 | 251 | 251 | 251 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 |  | 25 | 501 | 751 | 751 | 721 | 641 |  | 451 |  | 451 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 361 | 361 | 321 |  | 201 |  | 201 |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 181 | 181 | 161 |  | 101 |  | 101 |  |  |  |  |  |  |
| n8 | 15 |  | 25 | 251 | 251 | 251 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | 25 | 201 | 202 | 202 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 102 | 102 |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | 25 | 501 | 501 | 501 | 501 | 481 | 401 | 401 | Note 5 |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 241 | 241 | 241 | 241 | 201 | 201 | Note 5 |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |
| n26 | 15 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 121 | 121 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 |  | 251 |  | 251 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 |  | 101 |  | 101 |  |  |  |  |  |  |  |  |
| n30 | 15 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | 58 | 58 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n34 | 15 |  | 25 | 50 | 75 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 |  | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n39 | 15 |  | 25 | 50 | 75 | 100 | 128 | 160 | 180 | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 | 90 | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 | 40 | 50 |  |  |  |  |  |  |  |  |
| n40 | 15 |  | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  | 128 | 162 |  | 216 |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  | 64 | 75 |  | 100 |  |  |  |
| n41 | 15 |  | 25 | 50 | 75 | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  | 75 |  | 100 |  | 128 | 162 | 180 | 216f | 243 | 270 |  |
|  | 60 |  |  | 10 | 18 | 24 |  | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n48 | 15 |  | 25 | 50 | 75 | 100 |  | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 |  | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n50 | 15 |  | 25 | 50 | 75 | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  | 75 |  | 100 |  | 128 | 162 |  | Note 3 |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 |  | 36 |  | 50 |  | 64 | 75 |  | Note 3 |  |  |  |
| n51 | 15 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
| n53 | 15 |  | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n54 | 15 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
| n65 | 15 |  | 25 | 501 | 751 | 1001 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 501 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 |  | 25 | 501 | 751 | 1001 | 1281 | 160 |  | 216 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 501 | 641 | 751 |  | 1001 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 24 | 301 | 361 |  | 501 |  |  |  |  |  |  |  |  |
| n70 | 15 |  | 25 | 501 | 751 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201,6 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101,6 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| n72 | 15 | 58 | 58 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n74 | 15 |  | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 51 | 51 | 51 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  |  | 50 | 75 | 100 |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  |  |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  |  | 10 | 18 | 24 |  |  |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n78 | 15 |  |  | 50 | 75 | 100 | 128 | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n79 | 15 |  |  | 50 |  | 100 |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 |  | 50 |  |  |  | 100 |  | 128 | 162 |  | 216 |  | 270 |  |
|  | 60 |  |  | 10 |  | 24 |  |  |  | 50 |  | 64 | 75 |  | 100 |  | 135 |  |
| n85 | 15 | 15 | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n91 | 15 |  | 254 | 201,4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 |  | 25 | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 |  | 254 | 251,4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 |  | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 15 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n101 | 15 |  | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n104 | 15 |  |  |  |  | 100 |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  | 50 |  |  |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  |  |  |  | 24 |  |  |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n105 | 15 |  | 25 | 251 | 201 | 201 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n109 | 15 |  | 25 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 |  | Note 7 |  | Note 7 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | Note 7 | Note 7 | Note 7 | Note 7 |  | Note 7 |  | Note 7 |  |  |  |  |  |  |
| n110 | 15 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For band n20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =8; for 60kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =3 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =4. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. Note 7: For this DL channel bandwidth, the UL configuration of 5MHz for 15kHz SCS and 10MHz for 30kHz shall be used. Note 8: In the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Unless given by Table 7.3.2.3-4, the minimum requirements specified in Tables 7.3.2.3-1a, Tables 7.3.2.3-1b, Tables 7.3.2.3-1c, Tables 7.3.2.3-1d shall be verified with the network signalling value NS_01 (Table 6.2.3.3-1) configured.

Table 7.3.2.3-4: Network signalling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n2 | NS_03 |
| n12 | NS_06 |
| n13 | NS_06 |
| n14 | NS_06 |
| n24 | NS_56 |
| n25 | NS_03 |
| n30 | NS_21 |
| n48 | NS_27 |
| n53 | NS_45 |
| n54 | NS_62 |
| n66 | NS_03 |
| n70 | NS_03 |
| n71 | NS_35 |
| n85 | NS_06 |
| n110 | NS_06 |

For the UE which supports CA, SUL or DC band combination, the minimum requirement for reference sensitivity in Table 7.3.2.3-1a and Table 7.3.2.3-1b shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.3 for the applicable operating bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.2.

-------- Skipped unchanged clauses --------

Table 7.3.2.4.1-3: Uplink configuration for reference sensitivity, LCRB @ RBstart format

| Operating Band | SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 |  | 25@0 | 50@21 | 75@41 | 100@61 | 128@51 | 128@321 |  | 128@881 | 128@1141 | 128@1421 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 50@11 | 64@11 | 64@141 |  | 64@421 | 64@551 | 64@691 |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 24@0 | 30@11 | 30@81 |  | 30@211 | 30@281 | 30@351 |  |  |  |  |  |  |
| n2 | 15 |  | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 48@1121 |  | 40@1761 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 |  | 20@861 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 |  | 10@411 |  |  |  |  |  |  |  |  |
| n3 | 15 |  | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 50@1101 | 50@1381 | 50@1661 | 50@1921 | 50@2201 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 | 24@681 | 24@821 | 24@951 | 24@1091 |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 | 10@341 | 10@411 | 10@481 | 10@551 |  |  |  |  |  |  |
| n5 | 15 |  | 25@0 | 25@271 | 25@541 | 25@811 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 |  | 25@0 | 50@21 | 75@41 | 75@311 | 72@611 | 64@961 |  | 45@1711 |  | 45@2251 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 36@151 | 36@291 | 32@461 |  | 20@861 |  | 20@1131 |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 18@61 | 18@131 | 16@221 |  | 10@411 |  | 10@551 |  |  |  |  |  |  |
| n8 | 15 |  | 25@0 | 25@271 | 25@541 | 25@811 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | 20@51 | 20@321 | 20@591 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | 20@01 | 20@01 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | 20@01 | 20@01 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | 25@0 | 20@01 | 20@112 | 20@162 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@01 | 10@62 | 10@82 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 48@1121 | 40@1481 | 40@1761 | Note 5 |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 | 20@721 | 20@861 | Note 5 |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 | 10@341 | 10@411 | Note 5 |  |  |  |  |  |  |  |
| n26 | 15 | 15@0 | 25@0 | 25@271 | 25@541 | 25@811 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 12@121 | 12@261 | 12@391 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 15@0 | 25@0 | 25@271 | 25@541 | 25@811 |  | 25@1351 |  | 25@1911 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 |  | 10@681 |  | 10@961 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | 20@51 | 20@321 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | 5@9 | 5@10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n34 | 15 |  | 25@0 | 50@0 | 75@0 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n39 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 | 180@0 | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 | 90@0 | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 | 40@0 | 50@0 |  |  |  |  |  |  |  |  |
| n40 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  | 128@0 | 162@0 |  | 216@0 |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  | 64@0 | 75@0 |  | 100@0 |  |  |  |
| n41 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 |  | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 |  | 75@0 |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 |  | 36@0 |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n48 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 |  | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 |  | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 |  | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n50 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 |  |  |  | 100@0 |  | 128@0 | 162@0 |  | NOTE 3 |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 |  |  |  | 50@0 |  | 64@0 | 75@0 |  | NOTE 3 |  |  |  |
| n51 | 15 |  | 25@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 |  | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n54 | 15 |  | 25@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 |  | 25@0 | 50@21 | 75@41 | 100@61 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 50@11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 |  | 25@0 | 50@21 | 75@41 | 100@61 | 128@51 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 50@11 | 64@11 | 75@31 |  | 100@61 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 24@0 | 30@11 | 36@21 |  | 50@11 |  |  |  |  |  |  |  |  |
| n70 | 15 |  | 25@0 | 50@21 | 75@41 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | 25@0 | 25@01 | 20@01 | 20@01 | 20@01,6 | 20@01,6 | 20@01,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 12@01 | 10@01 | 10@01 | 10@01,6 | 10@01,6 | 10@01,6 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n72 | 15 | 5@9 | 5@10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n74 | 15 |  | 25@0 | 25@271 | 25@541 | 25@811 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 5@61 | 5@131 | 5@191 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  |  | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  | - | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n78 | 15 |  |  | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n79 | 15 |  |  | 50@0 |  | 100@0 |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 |  | 50@0 |  |  |  | 100@0 |  | 128@0 | 162@0 |  | 216@0 |  | 270@0 |  |
|  | 60 |  |  | 10@0 |  | 24@0 |  |  |  | 50@0 |  | 64@0 | 75@0 |  | 100@0 |  | 135@0 |  |
| n85 | 15 | 15@0 | 20@51 | 20@321 | 20@591 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n91 | 15 |  | 25@04 | 20@321, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 |  | 25@0 | 20@321 | 20@591 | 20@861 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 |  | 25@04 | 25@271, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 |  | 25@0 | 25@271 | 20@591 | 20@861 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 12@121 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 15 | 15@0 | 25@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n101 | 15 |  | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n104 | 15 |  |  |  |  | 100@0 | 160@0 |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  | 50@0 | 75@0 |  |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  |  |  |  | 24@0 | 36@0 |  |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
|  | 15 |  | 25@0 | 25@01 | 20@01 | 20@01 |  |  |  |  |  |  |  |  |  |  |  |  |
| n105 | 30 |  |  | 12@01 | 10@01 | 10@01 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | 15@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n109 | 15 |  | 25@0 | 25@0 | 25@0 | 25@0 | 25@0 | 25@0 |  | 25@0 |  | 25@0 |  |  |  |  |  |  |
|  | 30 |  |  | 24@0 | 24@0 | 24@0 | 24@0 | 24@0 |  | 24@0 |  | 24@0 |  |  |  |  |  |  |
| n110 | 15 | 15@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For band n20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =8; for 60kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =3 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =4. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3. does not apply. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for 2 Rx antenna port, Tables 7.3.2.5-2 a and Tables 7.3.2.5-2b for 4 Rx antenna port, Table 7.3.2.5-2c and Table 7.3.2.5-2d for PC2 UE on FDD bands, Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8 Rx antenna ports, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3.2.4.1-3.

Table 7.3.2.5-1a: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands for PC3

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n1 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT |  | -90.6 +TT | -90.1 +TT | -89.6 +TT | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT |  | -90.7 +TT | -90.2 +TT | -89.7 +TT |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT |  | -90.9 +TT | -90.3 +TT | -89.7 +TT |  |
| n2 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -84.1 +TT |  | -81.5 +TT |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -84.2 +TT |  | -81.6 +TT |  |  |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -90.9 +TT | -84.3 +TT |  | -81.7 +TT |  |  |  |
| n3 | 15 |  | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -86.2+TT | -82.3 +TT | -81.3+TT | -79.7 +TT | FDD |
|  | 30 |  |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -86.3+TT | -82.4 +TT | -81.4+TT | -79.8 +TT |  |
|  | 60 |  |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -86.4+TT | -82.6 +TT | -81.5+TT | -79.9 +TT |  |
| n5 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -86.8 +TT | -84.8 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -88.6 +TT | -84.9 +TT |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -89.9 +TT |  | -88.6 +TT |  | -81.5 +TT | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -90.0 +TT |  | -88.7 +TT |  | -81.5 +TT |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -91.0 +TT | -90.1 +TT |  | -88.9 +TT |  | -81.5 +TT |  |
| n8 | 15 |  | -97.0 +TT | -93.8 +TT | -91.4 +TT | -85.8 +TT | -83.6+TT | -81.3+TT | -78.4+TT |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.7 +TT | -87.2 +TT | -84.7+TT | -81.4+TT | -78.5+TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | -89.3 +TT | -82.2 +TT | -81.7 +TT | -79.5 +TT | -77.6+TT |  | FDD |
|  | 30 |  |  | -93.6 +TT | -91.6 +TT | -90.5 +TT | -89.4 +TT | -82.3 +TT | -81.8 +TT | -79.6 +TT | -77.7 +TT |  |  |
|  | 60 |  |  | -94.0 +TT | -91.9 +TT | -90.7 +TT | -89.6 +TT | -82.4 +TT | -81.9 +TT | -79.7 +TT | -77.8 +TT |  |  |
| n26 | 15 | -99.7 +TT | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |
| n28 | 15 | -100.2 +TT | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  | -66.3 + TT |  |  | FDD |
|  | 30 |  |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  | -66.3 + TT |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | -95.7 + TT | -93.5 + TT |  |  |  |  |  |  |  |  |  | FDD |
| n65 | 15 |  | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |
| n66 | 15 |  | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT |  | -90.1 +TT |  |  | FDD |
|  | 30 |  |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT |  | -90.2 +TT |  |  |  |
|  | 60 |  |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT |  | -90.4 +TT |  |  |  |
| n70 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |
| n71 | 15 |  | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT | -84.17 +TT -74.88+TT | -82.57 +TT -67.18+TT | -80.77 +TT -64.08 +TT |  |  |  | FDD |
|  | 30 |  |  | -94.3 +TT | -91.9 +TT | -87.4 +TT | -84.27 +TT -74.98 +TT | -82.67 +TT -67.28 +TT | -80.87 +TT -64.18 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n72 | 15 | -95.7 + TT | -93.5 + TT |  |  |  |  |  |  |  |  |  | FDD |
| n74 | 15 |  | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |
| n85 | 15 | -99.2 +TT | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
| n100 | 15 | -102.2 +TT | -100.0 +TT |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | -97.29 +TT | -94.0 +TT | -91.6 +TT | -86.9 +TT |  |  |  |  |  |  |  |
| n105 | 30 |  |  | -94.3 +TT | -91.9 +TT | -87.9 +TT |  |  |  |  |  |  | FDD |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | -99.2 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
| n110 | 15 | -102.2 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 7: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 8: Applies to UEs that support optional symmetric UL/DL for this BW. NOTE 9: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS. |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### 7.6.2	In-band blocking

Editor’s note: The following aspects are either missing or not yet determined:

- The test is incomplete for NR Band n105

- Testability, MU and TT are pending for NR Band n105.

7.6.2.1	Test purpose

In band blocking is defined for an unwanted interfering signal falling into the range from 15 MHz below to 15 MHz above the UE receive band, with FDL_high < 2700 MHz and FUL_high < 2700 MHz, or into an immediately adjacent frequency range up 3*BWChannel below or above the UE receive band, with FDL_low ≥3300 MHz and FUL_low ≥ 3300 MHz, at which the relative throughput shall meet or exceed the requirement for the specified measurement channel.

7.6.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

7.6.2.3	Minimum conformance requirements

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, in-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band. The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL signal as described in Annex A.5) with parameters specified in Table 7.6.2.3-1 and Table 7.6.2.3-2. The relative throughput shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.2.3-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB |  | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.6.2.3-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 | -225 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |  |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n31, n34, n38, n39, n40, n41, n483, n50, n51, n53, n54, n65, n66, n67, n70, n72, n74, n75, n76, n85, n91, n92, n93, n94, n100, n101, n106, n110 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |  |
| n105 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |  | FDL_low – 7 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. NOTE 4: For SDL bands, requirements shall be applied only for CA band combination cases. NOTE 5: For Band n105 channels overlapping the 612 - 617 MHz frequency range, Pinterferer is modified to -34 dBm. |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

7.6.2.5	Test requirement

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex in Annexes A.2.2, A.2.3 and A.3.2 with parameters specified in Tables 7.6.2.5-1 and 7.6.2.5-2.

Table 7.6.2.5-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB |  | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.6.2.5-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 | -224 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |  |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n31, n34, n38, n39, n40, n41, n483, n50, n51, n53, n54, n65, n66, n67, n70, n72, n74, n85, n91, n92, n93, n94, n100, n101, n106, n110 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |  |
| n105 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |  | FDL_low – 7 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. NOTE 4: For Band n105 channels overlapping the 612 - 617 MHz frequency range, Pinterferer is modified to -34 dBm. |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### 7.6.3	Out-of-band blocking

7.6.3.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band, with FDL_high < 2700 MHz and FUL_high < 2700 MHz, or falling outside a frequency range up to 3*BWChannel below or from 3*BWChannel above the UE receive band, with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

7.6.3.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

7.6.3.3	Minimum Conformance Requirements

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3.3-1 and Table 7.6.3.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.3.3-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in transmission bandwidth configuration [dBm] |
| --- | --- |
| 3 MHz | REFSENS + 6.0 dB |
| 5 MHz | REFSENS + 6.0 dB |
| 10 MHz | REFSENS + 6.0 dB |
| 15 MHz | REFSENS + 7.0 dB |
| 20 MHz | REFSENS + 9.0 dB |
| 25 MHz | REFSENS + 10.0 dB |
| 30 MHz | REFSENS + 11.0 dB |
| 35 MHz | REFSENS + 11.5 dB |
| 40 MHz | REFSENS + 12.0 dB |
| 45 MHz | REFSENS + 12.5 dB |
| 50 MHz | REFSENS + 13.0 dB |
| 60 MHz | REFSENS + 14.0 dB |
| 70 MHz | REFSENS + 14.5 dB |
| 80 MHz | REFSENS + 15.0 dB |
| 90 MHz | REFSENS + 15.5 dB |
| 100 MHz | REFSENS + 16.0 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |

Table 7.6.3.3-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n485, n50, n51, n538, n54, n65, n66, n70, n71, n72, n74, n75, n76, n85, n91, n92, n93, n94, n100, n101, n105, n106, n110 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: Void. NOTE 7: For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE 8: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer ≥ 2580 MHz and FInterferer < 2775 MHz. NOTE 9: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |

-------- Skipped unchanged clauses --------

7.6.3.5	Test Requirement

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters specified in Tables 7.6.3.5-1 and 7.6.3.5-2.

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the number of spurious response frequencies recorded in the final step of test procedure shall not exceed  in each assigned frequency channel when measured using a  MHz step size. For these exceptions the requirements of clause 7.7 Spurious Response are applicable.

Table 7.6.3.5-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in transmission bandwidth configuration [dBm] |
| --- | --- |
| 3 MHz | REFSENS + 6.0 dB |
| 5 MHz | REFSENS + 6.0 dB |
| 10 MHz | REFSENS + 6.0 dB |
| 15 MHz | REFSENS + 7.0 dB |
| 20 MHz | REFSENS + 9.0 dB |
| 25 MHz | REFSENS + 10.0 dB |
| 30 MHz | REFSENS + 11.0 dB |
| 35 MHz | REFSENS + 11.5 dB |
| 40 MHz | REFSENS + 12.0 dB |
| 45 MHz | REFSENS + 12.5 dB |
| 50 MHz | REFSENS + 13.0 dB |
| 60 MHz | REFSENS + 14.0 dB |
| 70 MHz | REFSENS + 14.5 dB |
| 80 MHz | REFSENS + 15.0 dB |
| 90 MHz | REFSENS + 15.5 dB |
| 100 MHz | REFSENS + 16.0 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |

Table 7.6.3.5-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n24, n25, n26, n28, n30, n31, n34, n38, n39, n40, n41, n485, n50, n51, n538, n54, n65, n66, n70, n71, n72, n74, n85, n91, n92, n93, n94, n100, n101, n105, n106, n110 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: Void. NOTE 7: For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE 8: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer ≥ 2580 MHz and FInterferer < 2775 MHz. NOTE 9: Range 3 shall be tested only with the highest channel bandwidth. |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### 7.6.4	Narrow band blocking

7.6.4.1	Test Purpose

Verifies a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other NR Node B transmitters exist (except in the adjacent channels and spurious response).

7.6.4.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

7.6.4.3	Minimum Conformance Requirements

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.4.3-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.4.3-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n72, n74, n75, n76, n85, n100, n101, n106, n110 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |
|  |  | dB | 18 | 16 | 13 | 14 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b, Table 7.3.2.3-2 and Table 7.3.2.3-2a for two, four and eight antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. NOTE 5: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

7.6.4.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters specified in Table 7.6.4.5-1.

Table 7.6.4.5-1: Narrow-band blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n72, n74, n75, n76, n85, n100, n101, n106, n110 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |
|  |  | dB | 18 | 16 | 13 | 14 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b, Table 7.3.2.3-2 and Table 7.3.2.3-2a for two, four and eight antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. NOTE 5: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |  |  |  |  |

-------- End of changes --------