3GPP TSG-RAN5 Meeting #108	R5-254497

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5483 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of logged MDT test case 8.6.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on AP#107.01, Inter-RAT logged MDT test case 8.6.3.4 needs update to cover UEs supporting/not supporting protection against improper reselection to GERAN/UTRAN) feature. In preamble of test case 8.6.3.4, UTRA cell is assigned a higher priority (5) than the serving E-UTRA cell 1(4) in SIB6 and UE is expected to perform inter RAT reselection to UTRA cell at step 5 which is not feasible for a UE supporting this feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configured UTRA neighbor as high priority via dedicated signaling (RRC Connection Release message at step 2) by adding specific message content table 8.6.3.4.3.3-3A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.6.3.4	Logged MDT / Logging and reporting / Reporting at UTRAN Inter-RAT handover / PLMN list

8.6.3.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state camping normally on an E-UTRA cell where logged measurement is configured with the RPLMN included in plmn-IdentityList and the UE is able to detect a UTRA cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and UTRA neighbouring cell measurements }

}

(2)

with { UE in UTRA RRC_CONNECTED state and UE has logged E-UTRA measurements and UTRA neighbouring cell measurements available from earlier measurements performed while in E-UTRA }

ensure that {

when { UE handovers from UTRA to E-UTRA cell successfully and T330 is running and the current RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCConnectionReconfigurationComplete message with logMeasAvailable IE set to true }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and has logged measurements available for E-UTRA and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { UE receives UEInformationRequest message with logMeasReportReq present }

then { UE transmits UEInformationResponse messages with logMeasReport included }

}

8.6.3.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.304, clause 8; TS 36.331, clauses 5.4.2.3, 5.6.5.3, 5.6.6.3, 5.6.8.2, 6.2.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.304, clause 8 (TP1, TP2)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception, or is present in the plmn-IdentityList (see TS 36.331 [3]) if configured;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.4.2.3 (TP2)]

The UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

…

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

NOTE 2:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.6.6.3 (TP1)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 5.6.5.3 (TP3)]

Upon receiving the UEInformationRequest message, the UE shall

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 6.2.2 (TP1)]

-	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			LoggedMeasurementConfiguration-v1080-IEs	OPTIONAL	-- Need OP

}

LoggedMeasurementConfiguration-v1080-IEs ::= SEQUENCE {

lateNonCriticalExtension-r10	OCTET STRING						OPTIONAL,	-- Need OP

nonCriticalExtension			LoggedMeasurementConfiguration-v1130-IEs	OPTIONAL	-- Need OP

}

LoggedMeasurementConfiguration-v1130-IEs ::= SEQUENCE {

plmn-IdentityList-r11			PLMN-IdentityList3-r11		OPTIONAL,	-- Need OR

areaConfiguration-v1130			AreaConfiguration-v1130		OPTIONAL,	-- Need OR

nonCriticalExtension			SEQUENCE {}					OPTIONAL	-- Need OP

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ identities.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

8.6.3.4.3	Test description

8.6.3.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 9 and Cell 12 - Cell 1 and Cell 12 are E-UTRAN cell, Cell 9 is a UTRA cell.

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of UTRA carrier frequency list in SIB6 set as defined in TS 36.508 [18] table 6.3.1.3-1.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.3.4.3.1-1.

Table 8.6.3.4.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

Preamble:

- The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.3.4.3.3-7

- The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.3.4.3.2	Test procedure sequence

Table 8.6.3.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" and "T2" are applied at the point indicated in the Main behaviour description in Table 8.6.3.4.3.2-2.

Table 8.6.3.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 9 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | - | SnonServingCell, Cell9 > Thresh Cell9,high |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | -65 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -67 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | -100 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -100 |  |
| NOTE 1: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.3.4.3.2-2: Main behaviour

| St | Procedure |  | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in E-UTRA RRC_IDLE. |  | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. |  | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging |  | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 9 levels according to the row "T1" in table 8.6.3.4.3.2-1. (Note 1) |  | - | - | - | - |
| 5 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 9. NOTE: The UE performs an RAU procedure and the RRC connection is released. |  | - | - | - | - |
| - | Exception: Steps 5Aa1 to 5Aa1b1 describe optional behaviour that depend on the UE capability. |  | - | - | - | - |
| 5Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer |  | - | - | - | - |
| 5Aa1a1-5Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed |  | - | - | - | - |
| 5Aa1a4 | SS releases RRC Connection |  | - | - | - | - |
| 5Aa1a5 | Stop 6s timer |  | - | - | - | - |
| 5Aa1b1 | The 6s timer expires |  | - | - | - | - |
| 6-10 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 9 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. |  | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 11a1 to 11a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, goto step 12. |  | - | - | - | - |
| 11a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 9 including the DPCH compressed mode info. |  | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 11a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 9. |  | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 12 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 9. |  | <-- | MEASUREMENT CONTROL | - | - |
| 13 | The SS changes Cell 12 and Cell 9 levels according to the row "T2" in table 8.6.3.4.3.2-1. |  | - | - | - | - |
| 14 | The UE transmits a MEASUREMENT REPORT message on Cell 9 including the E-UTRA event results. |  | --> | MEASUREMENT REPORT | - | - |
| 15 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 9. |  | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with the IE logMeasAvailable set to true on Cell 12? |  | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 17 | Void |  | - | - | - | - |
| 17A | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 12. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN |  | - | - | - | - |
|  | EXCEPTION: Steps 17Ba1 to 17Ba2b1 describe optional behaviour that depend on the UE capability |  | - | - | - | - |
| 17Ba1-17Ba2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed |  | - | - | - | - |
| 18 | Void |  |  |  |  |  |
| 19 | Void |  |  |  |  |  |
| 20 | The SS transmits a UEInformationRequest message on Cell 12 with logMeasReportReq present. | <-- |  | UEInformationRequest | - | - |
| 21 | Check: Does the UE transmit a UEInformationResponse message with logMeasReport included? | --> |  | UEInformationResponse | 1, 3 | P |
| Note 1: The change of power levels is to trigger an inter-RAT cell re-selection procedure to make sure that the UE is logging inter-RAT neighbouring cell measurements (logging interval=2.56 seconds) of Cell 9 while t-ReselectionUTRA timer is running (7 seconds). |  |  |  |  |  |  |

8.6.3.4.3.3	Specific message contents

Table 8.6.3.4.3.3-1: SystemInformationBlockType6 for Cell 1 (preamble, 8.6.3.4.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Downlink UARFCN of Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Downlink UARFCN of Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| t-ReselectionUTRA | 7 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.3.4.3.3-2: System Information Block type 19 for Cell 9 (preamble, Table 8.6.3.4.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.3.4.3.3-3: LoggedMeasurementConfiguration (step 1, Table 8.6.3.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |
| loggingInterval-r10 |  |  |  |
| nonCriticalExtension SEQUENCE { | ms2560 | 2.56 seconds |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| plmn-IdentityList-r11 SEQUENCE (SIZE (1..16)) OF { |  |  |  |
| PLMN-Identity [1] | PLMN1 |  |  |
| PLMN-Identity [2] | PLMN2 |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.3.4.3.3-3A: RRCConnectionRelease (step 2, Table 8.6.3.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| freqPriorityListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 8.6.3.4.3.3-4: RRCConnectionReconfigurationComplete (step 16, Table 8.6.3.4.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-9 RRCConnectionReconfigurationComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| nonCritriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | true |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.3.4.3.3-5: UEInformationRequest (step 20, Table 8.6.3.4.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.4.3.3-6: UEInformationResponse (step 21, Table 8.6.3.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  |  |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 |  | Not present |  |  |
| measResultListUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r9[1] |  | Same as Cell 9 |  |  |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId [1] |  | Same as Cell 9 |  |  |
| cgi-Info [1] |  | Not checked |  |  |
| measResult [1] SEQUENCE { |  |  |  |  |
| utra-RSCP |  | (-5..91) |  |  |
| utra-EcN0 |  | (0..49) |  |  |
| additionalSI-Info-r9 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.3.4.3.3-7: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.3.4.3.3-8: TRACKING AREA UPDATE ACCEPT for Cell 12 (step 17A, Table 8.6.3.4.3.2-2) (step 2 in TS 36.508 subclause 6.4.2.10)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  | Cell 28 |

<End of modified section>