3GPP TSG-RAN5 Meeting #108	R5-254917

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1545 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add generic test procedure for Emergency call and RTT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add generic test procedure for Emergency call and RTT |  |  |  |  |  |  |  |  |
| Summary of change: |  | A generic test procedure has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for RTT according to EU directive 2019/882 remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 4.5A.n |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1626 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-254154 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

<Start of first modified section>

### 4.5A.36	Generic Test Procedure for IMS Emergency call and RTT establishment in EUTRA: Normal Service

#### 4.5A.36.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.36.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.36.3	Procedure

The establishment of IMS emergency call is assumed to always be mobile originated.

Table 4.5A.36.3-1: EUTRA/EPS signalling for IMS Emergency Call and RTT

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS emergency call and RTT. | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B )'. | --> | PDN CONNECTIVITY REQUEST |
| 10 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 11 to 15 below, the behaviour in table 4.5A.36.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and IMS emergency speech call establishment) | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 13 | The SS configure new RLC-UM data radio bearers with condition DRB (0,2), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST messages. EPS bearer context #4 (QCI 1) according to table 6.6.2-1 and EPS bearer context #11 (QCI 1) according to table 6.6.2-1B: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT messages. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.36.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [35]. Emergency registration procedure is performed. | - | - |
| 6-10 | Steps 1-5 defined in annex C.54 of TS 34.229-1 [35]. IMS Emergency call and RTT for EPS is established. | - | - |

#### 4.5A.36.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

<End of modified section>