3GPP TSG-RAN5 Meeting #108	R5-255401

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3451 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Rel-16 UL Tx switching test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Minimum requirements in clause 6.3A.3.0.3.2 mention uplinkTxSwitching-PowerBoosting. However, it is not clear how to consider uplinkTxSwitching-PowerBoosting in the test case. The minimum requirements in clause 6.3A.3.0.3 are not aligned with TS 38.101-1. In test case 6.3C.3.2, PUSCH on SUL needs to be scheduled on NUL via PDCCH DCI format 0_1 as SUL cell does not have DL. The conditions in the message exceptions need to be clarified with respect to SUL and NUL carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor's note on uplinkTxSwitchingPowerBoosting has been added. The minimum requirement in clause 6.3A.3.0.3.2 has been aligned to TS 38.101-1 V19.2.0 (2025-06) The test procedure in test case 6.3C.3.2 has been updated to clarify on which carrier the UL scheduling information is to be sent. The conditions in the message exceptions have been clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement will not be tested according to core requirement specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.0.3.2, 6.3A.3.2, 6.3C.3.2.4.2, 6.3C.3.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-254387. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.3A.3.0.3.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.0.3.1 and the maximum output power requirement specified in Table 6.2A.1.0.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of an inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [12], where NR UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power for CA power class 3 when the capability uplinkTxSwitching-PowerBoosting is present and the IE uplinkTxSwitchingPowerBoosting is enabled, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 as specified in 38.306 [26].

The switching periods described in Figure 6.3A.3.0.3.2-1a and Figure 6.3A.3.0.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [6], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.0.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.0.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The following applies for the uplink switching cases specified in clause 6.1.6.2 of [12] with uplinkTxSwitchingOption set to either switchedUL or dualUL when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [12] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of both non-co-located and co-located and synchronized network deployment for the two uplink carriers.

The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.

< Unchanged sections omitted >

#### 6.3A.3.2	Time mask for switching between two uplink carriers

Editor's note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	How to consider uplinkTxSwitching-PowerBoosting is FFS

6.3A.3.2.1	Test purpose

To verify that the time mask for switching between two uplink carriers meets the requirements given in 6.3A.3.0.3.2.

The time mask for switching between two uplink carriers defines the transient period(s) allowed between two uplink carriers for an uplink band pair of an inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present.

6.3A.3.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support 2UL inter-band CA and dynamic UL Tx switching.

6.3A.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.3.2.

6.3A.3.2.4	Test description

6.3A.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.2.4.1-1: Test Configuration Table for inter-band CA Uplink switching

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.1.3 for inter band CA in FR1 |  | Mid range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 2) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | CP-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: PCC is the component carrier capable of one transmit antenna connector, and is configured as Carrier 1. SCC is the component carrier capable of two transmit antenna connectors, and is configured as Carrier 2. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3A.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.3.2.4.3.

6.3A.3.2.4.2	Test procedure

1.	Sub test 1: Switching period located in Carrier 1

1.1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

1.2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.2.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured TRUE on carrier1 and FALSE on carrier 2.

1.3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

1.3a.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on PCC only. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on PCC.

1.3b.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on SCC only. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on SCC.

1.4.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.5.	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on carrier 2 starting on slot n and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.6.	Measure the output power of UE PUSCH transmission for carrier 1 during slot n-1 excluding a transient period of 10 µs and a Switching period X µs in the end of slot n-1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

1.7.	Measure the sum of output power of UE PUSCH transmission on carrier 2 over all antenna connectors during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n and in the end of slot m

1.8.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.9.	Measure the output power of UE PUSCH transmission for carrier 1 during slot m+1 excluding a switching period X and a transient period of 10 µs in the beginning of slot m+1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

2.	Sub test 2: Switching period located in Carrier 2

2.1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.2.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured TRUE on carrier 2 and FALSE on carrier 1.

2.3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

2.3a.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on PCC only. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on PCC.

2.3b.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on SCC only. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on SCC.

2.4.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.5.The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on carrier 2 on slot n (n ≥1) and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.6.	Measure the output power of UE PUSCH transmission for carrier 1 during slot n-1, for carrier 1 excluding a transient period of 10 µs in the end of slot n-1.

2.7.	Measure the sum of output power of UE PUSCH transmission on carrier 2 over all antenna connectors during during slot n and slot m excluding a switching period X and a transient period of 10 µs in the beginning of slot n and in the end of slot m. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

2.8.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.2.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.9.	Measure the output power of UE PUSCH transmission for carrier 1 during slot m+1 excluding a transient period of 10 µs in the beginning of slot m+1.

6.3A.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions.

Table 6.3A.3.2.4.3-1: CellGroupConfig

| Derivation Path: TS 38.508-1[5], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| uplinkTxSwitchingOption-r16 | switchedUL |  | switchedUL OR Both |
|  | dualUL |  | dualUL |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| switchedUL | UE indicated supporting of switchedUL in uplinkTxSwitching-OptionSupport-r16 |
| dualUL | UE indicated supporting of dualUL in uplinkTxSwitching-OptionSupport-r16 |
| Both | UE indicated supporting of both in uplinkTxSwitching-OptionSupport-r16 |

Table 6.3A.3.2.4.3-2: Void

Table 6.3A.3.2.4.3-3: ServingCellConfig

| Derivation Path: 38.508-1[5], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkTxSwitching-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingPeriodLocation-r16 | TRUE |  | PL |
|  | FALSE |  | noPL |
| uplinkTxSwitchingCarrier-r16 | carrier1 |  | 1TxCC |
|  | carrier2 |  | 2TxCC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PL | The location of UL Tx switching period is configured in this carrier |
| noPL | The location of UL Tx switching period is not configured in this carrier |
| 1TxCC | The carrier is capable of one transmit antenna connector |
| 2TxCC | The carrier is capable of two transmit antenna connectors |

Table 6.3A.3.2.4.3-4: Void

Table 6.3A.3.2.4.3-5: Void

Table 6.3A.3.2.4.3-6: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  |  |

Table 6.3A.3.2.4.3-7: FrequencyInfoUL for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  |  |

6.3A.3.2.5	Test requirement

The requirement for the power of carrier 1 measured in step 1.6, 1.9, 2.6, 2.9 of the test procedure and the power of carrier 2 measured in step 1.7 and 2.7 shall not exceed the values specified in table 6.3A.3.2.5-1.

Table 6.3A.3.2.5-1: Time mask for switching between two uplink carriers (On power)

|  | Measured output power |
| --- | --- |
| Transmit ON power | PCC: Same as Test ID 23 of Table 6.2.2.5-1 to Table 6.2.2.5-3 as appropriate SCC: Same as Test ID 1 of Table 6.2D.2.5-1 as appropriate |
| NOTE1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2A.2.1.5-3 |  |

Table 6.3A.3.2.5-2: Void

< Unchanged sections omitted >

#### 6.3C.3.2	General transmit ON/OFF time mask for switching between two uplink carriers

6.3C.3.2.1	Test purpose

To verify that the time mask for switching between two uplink carriers meets the requirements given in 6.3C.3.0.2.

The time mask for switching between two uplink carriers defines the transient period(s) allowed between two uplink carriers for an uplink band pair of an SUL configuration when the capability uplinkTxSwitchingPeriod is present

6.3C.3.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support SUL configuration and dynamic UL Tx switching.

6.3C.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3C.3.0.2.

6.3C.3.2.4	Test description

6.3C.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR SUL configuration specified in 5.5C. All of these configurations shall be tested with applicable test parameters for each SUL configuration, and are shown in table 6.3C.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3C.3.2.4.1-1: Test Configuration Table for SUL Tx switching

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for both carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Highest for SUL carrier and Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier, lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  | SUL Configuration |  |
| 1 | N/A | Modulation | RB allocation (NOTE 2) |  | Modulation | RB allocation (NOTE 2) |
|  |  | DFT-s-OFDM QPSK | Inner Full |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: SUL carrier is configured as Carrier 1 and Non-SUL carrier is configured as Carrier2. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

-	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3C.3.2.4.3.

6.3C.3.2.4.2	Test procedure

1.	Sub test 1: Switching period located in Carrier 1 (SUL carrier)

1.1	SS send an NR RRCReconfiguration message according to 6.3C.3.2.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured TRUE on carrier1 and FALSE on carrier 2.1.2 SS send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

1.3	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI on carrier 2 to schedule the UL RMC according to Table 6.3C.3.2.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.4	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.2.4.1-1 on carrier 2 starting on slot n and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.5	Measure the output power of UE PUSCH transmission for carrier 1 during slot n-1 excluding a transient period of 10 µs and a Switching period X µs in the end of slot n-1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

1.6	Measure the sum of output power of UE PUSCH transmission on carrier 2 over all antenna connectors during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n and in the end of slot m.

1.7	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI on carrier 2 to schedule the UL RMC according to Table 6.3C.3.2.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.8	Measure the output power of UE PUSCH transmission for carrier 1 during slot m+1 excluding a switching period X and a transient period of 10 µs in the beginning of slot m+1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

2.	Sub test 2: Switching period located in Carrier 2 (Non-SUL carrier)

2.1	SS send an NR RRCReconfiguration message according to 6.3C.3.2.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured FALSE on carrier1 and TRUE on carrier 2.

2.2	SS send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

2.3	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI on carrier 2 to schedule the UL RMC according to Table 6.3C.3.2.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.4	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.2.4.1-1 on carrier 2 starting on slot n and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.5	Measure the output power of UE PUSCH transmission for carrier 1 during slot n-1 excluding a transient period of 10 µs in the end of slot n-1.

2.6	Measure the sum of output power of UE PUSCH transmission on carrier 2 over all antenna connectors during slot n and slot m excluding a transient period of 10 µs and a Switching period X µs in the beginning of slot n and in the end of slot m. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

2.7	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI on carrier 2 to schedule the UL RMC according to Table 6.3C.3.2.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.8	Measure the output power of UE PUSCH transmission for carrier 1 during slot m+1 excluding a transient period of 10 µs in the beginning of slot m+1.

6.3C.3.2.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and following exceptions:

Table 6.3C.3.2.4.3-2: ServingCellConfig

| Derivation Path: 38.508-1[5], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkTxSwitching-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingPeriodLocation-r16 | TRUE |  | PL |
|  | FALSE |  | noPL |
| uplinkTxSwitchingCarrier-r16 | carrier1 |  | SUL |
|  | carrier2 |  | NUL |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PL | The location of UL Tx switching period is configured in this carrier |
| noPL | The location of UL Tx switching period is not configured in this carrier |
| SUL | For the SUL carrier |
| NUL | For the NUL carrier |

Table 6.3C.3.2.4.3-4: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.3C.3.2.4.3-5: P-Max

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  |  |

6.3C.3.2.5	Test requirement

The requirement for the power of carrier 1 measured in step 1.5, 1.8, 2.5, 2.8 of the test procedure and the power of carrier 2 measured in step 1.6 and 2.6 shall not exceed the values specified in table 6.3C.3.2.5-1.

Table 6.3C.3.2.5-1: General SUL Time mask for switching between two uplink carriers (On power)

|  | Measured output power |
| --- | --- |
| Transmit ON power | Same as table 6.2.1.5-1 for NUL carrier and table 6.2C.3.5-1 for SUL carrier |
| NOTE 1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2.1.5-3 |  |

< End of changes >