3GPP TSG-RAN5 Meeting #108	R5-254960

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1632 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to P-Access-Network-Info header for RedCap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CP-240098 introduced “3GPP-NR-REDCAP” parameter for "access-type" and "access-class" in the P-Access-Network-Info header field to identify traffic to/from UEs accessing over NR RedCap. Currently this is not handled in default messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “3GPP-NR-REDCAP” in the P-Access-Network-Info header field parameters for Redcap UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant RedCap UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.1, A.1.4, A.2.1, A.2.4, A.2.5, A.2.6, A.2.8, A.2.10, A.3.1, A.4.3, A.5.1, A.6.1, A.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254960 is the final version of R5-254501 with below changes: 1.Updated A.1.1,A.2.1 and A.3.1 condition numbers to resolve conflict. 2.Updated text for conditions and Keysight added as co-source. |  |  |  |  |  |  |  |  |

<Start of modified section>

# A.1	Default messages for IMS Registration

## A.1.1	REGISTER

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | REGISTER |  |  |
| Request-URI |  | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or SIP URI formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
|  | A14,A15 | SIP URI formed from home domain name as preconfigured in the UE |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Route |  | not present |  | RFC 3261 [15] |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1,A3, A14,A15 | IP address or FQDN, port (optional) and not checked |  |  |
|  | A2 | IP address or FQDN and, when using UDP, protected server port of the UE |  |  |
| response-port | A1,A3 | rport (when using UDP) |  | RFC 3581 [96] |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 3) or public user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A2,A15 | same public user identity as in initial REGISTER |  |  |
|  | A3 AND NOT A7 | public user identity derived from IMSI |  |  |
|  | A7 | emergency public user identity (NOTE 4) |  |  |
|  | A14 | IMPU preconfigured in the UE |  |  |
|  | A17 | same public user identity as in initial REGISTER |  |  |
| tag |  | must be present, value not checked |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 3) or public user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A2,A15 | same public user identity as in initial REGISTER |  |  |
|  | A3 AND NOT A7 | public user identity derived from IMSI |  |  |
|  | A7 | emergency public user identity (NOTE 4) |  |  |
|  | A14 | IMPU preconfigured in the UE |  |  |
|  | A17 | same public user identity as in initial REGISTER |  |  |
| tag |  | not present |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1,A3, A14,A15 | SIP URI with IP address or FQDN and indicating either an unprotected port selected by the UE or no port at all |  |  |
|  | A2 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A7 | The SIP URI shall contain the sos URI parameter |  |  |
| feature-param | A4 AND A34 | +g.3gpp.icsi-ref="(comma-separated list of tag-values)” with comma-separated list of tag-values containing at least tag-value urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2,5) |  |  |
|  | A6 AND NOT A7 AND A34 | +g.3gpp.smsip |  |  |
|  | A10 AND A34 | video |  |  |
|  | A28 AND (A29 OR A30 OR A32) AND A34 | audio |  |  |
|  | A11 AND NOT A7 AND A34 | +g.3gpp.cs2ps-srvcc | Rel-11 |  |
|  | A12 AND NOT A7 AND A34 | +g.3gpp.cs2ps-srvcc-alerting | Rel-11 |  |
|  | A13 AND NOT A16 AND A34 | +g.3gpp.accesstype=”cellular2” | Rel-11 | RFC 3840 [63] |
|  | A13 AND A16 AND A34 | +g.3gpp.accesstype=”wlan1” | Rel-11 | RFC 3840 [63] |
|  | A35 AND A34 | +g.3gpp.ps-data-off=”inactive” | Rel-15 |  |
|  | A36 | +sip.app-subtype="webrtc-datachannel" | Rel-18 | RFC 5688 [160] |
| c-p-instance | A5 | +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in RFC 7254 [122], without optional parameters | Rel-10 | RFC 5627 [61] RFC 7254 [122] |
| expires |  | 600000 (if present) |  |  |
| Expires |  | present if no expires parameter in Contact header |  | RFC 3261 [15] |
| delta-seconds |  | 600000 |  |  |
| Require | A1,A2 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Proxy-Require | A1,A2 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Supported |  |  |  | RFC 3261 [15] TS 24.229 [10] |
| option-tag | A5 | gruu |  |  |
|  |  | path |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A1,A3 A14 | must be present, value not checked |  |  |
|  | A2,A15 | must be incremented from the previous REGISTER |  |  |
| method |  | REGISTER |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | value not checked |  |  |
| Security-Client | A1,A2 |  |  | RFC 3329 [21] RFC 4835 [124] |
| mechanism-name |  | ipsec-3gpp |  |  |
| algorithm |  | hmac-sha-1-96 |  |  |
| protocol |  | esp (if present) |  |  |
| mode |  | trans (if present) |  |  |
| encrypt-algorithm |  | des-ede3-cbc or aes-cbc or null |  |  |
| spi-c |  | SPI number of the inbound SA at the protected client port |  |  |
| spi-s |  | SPI number of the inbound SA at the protected server port |  |  |
| port-c |  | protected client port |  |  |
| port-s |  | protected server port |  |  |
| Security-Client | A14,A15 | not present |  |  |
| Security-Verify | A2 |  |  | RFC 3329 [21] |
| sec-mechanism | A2 | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A1,A3 A14,A15 | not present |  |  |
| Authorization | A1 | Digest |  | RFC 2617 [16] RFC 3310 [17] |
| username | A1 | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
| realm | A1 | home domain name as stored in EFDOMAIN (when using ISIM) or home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
| nonce | A1 | set to an empty value |  |  |
| digest-uri | A1 | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
| response | A1 | set to an empty value |  |  |
| Authorization | A14(o) | Header optional |  |  |
| username | A14 | user identity as preconfigured in the UE |  |  |
| realm | A14 | home domain name as preconfigured in the UE |  |  |
| nonce | A14 | set to an empty value |  |  |
| digest-uri | A14 | preconfigured in the UE |  |  |
| response | A14 | set to an empty value |  |  |
| Authorization | A2,A15 | Digest |  | RFC 2617 [16] RFC 3310 [17] |
| username | A2 | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A15 | user identity as preconfigured in the UE |  |  |
| realm | A2,A15 | same value as received in the realm directive in the WWW Authenticate header sent by SS |  |  |
| nonce | A2,A15 | same value as in WWW-Authenticate header sent by SS |  |  |
| opaque | A2,A15 | same value as sent by the server in “401 Unauthorized for REGISTER” |  |  |
| digest-uri | A2 | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
|  | A15 | SIP URI formed from home domain name as preconfigured in the UE |  |  |
| qop-value | A2,A15 | auth |  |  |
| cnonce-value | A2,A15 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A2,A15 | counter to indicate how many times UE has sent the same value of nonce within successive REGISTER requests, initial value shall be 1 |  |  |
|  | A17 | value incremented by one for each re-registration request |  | RFC 2617 [16] |
| response | A2 | response calculated by UE |  |  |
|  | A15 | response calculated by UE using password px_DigestPasswordForSIP |  |  |
| algorithm | A2 | AKAv1-MD5 |  |  |
|  | A15 | MD5 |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A2,A15, A14(o), A16(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A2 AND A31 | access network information for E-UTRAN and, if applicable, the cell ID |  |  |
|  | A14,A15 | access network information for Fixed Broadband with access-type field matching “*DLS*” and a "dsl-location" parameter (value not checked) |  |  |
|  | A16 | access network information, containing any of “IEEE-802.11”, “IEEE-802.11a”, “IEEE-802.11b”, “IEEE-802.11g” or “IEEE-802-11n”, and i-wlan-node-id parameter containing a MAC address according to TS 24.229 [10], 7.2A.4.2. Value of MAC address not to be checked |  |  |
|  | A2 AND A32 AND NOT A38 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A2 AND A33 | access network information for UTRAN and, if applicable, the cell ID |  |  |
|  | A2 AND A38 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Content-Length |  | header shall be present if UE uses TCP to send this message and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of request body, if such is present |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Initial unprotected REGISTER (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | Subsequent REGISTER sent over security associations (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A3 | REGISTER using GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A4 | UE supports IMS Multimedia Telephony (MTSI) (A.3A/50 3GPP TS 34.229-2 [5]) |
| A5 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]). Mandatory from Rel-10 onwards. |
| A6 | UE supports SM-over-IP receiver (A.3A/62 3GPP TS 34.229-2 [5]) |
| A7 | Initial unprotected or subsequent REGISTER for emergency registration |
| A8 | Void |
| A10 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A11 | UE supports CS to PS SRVCC (A.12/40 3GPP TS 34.229-2 [5]) |
| A12 | UE supports CS to PS SRVCC in alerting state (A.12/41 3GPP TS 34.229-2 [5]) |
| A13 | UE indicates g.3gpp.accesstype media feature tag in REGISTER (A.12/46 3GPP TS 34.229-2 [5]) |
| A14 | Initial REGISTER SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A15 | Subsequent REGISTER SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A16 | IMS registration over WLAN |
| A17 | UE initiated IMS re-registration or de-registration (A.12/51 3GPP TS 34.229-2 [5]) |
| A18-A27 | Void |
| A28 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A29 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A30 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A31 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A32 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A33 | UE uses UTRAN access (A.18/2 3GPP TS 34.229-2 [5]) |
| A34 | Feature tags in Contact header to be checked. By default this condition is true. |
| A35 | UE supports PS data off (A.12/69 3GPP TS 34.229-2 [5]) |
| A36 | UE supports IMS data channel (A.15/14 3GPP TS 34.229-2 [5]) |
| A38 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	Public user identity shall be the same for ‘From’ and ‘To’.

NOTE 4:	According to TS 24.229 clause 5.1.1.1A and 5.1.6.2 [10] when the UE is using ISIM the emergency public user identity is the first public user identity in the list stored in the ISIM; when there is no ISIM it is the default public user id if the UE successfully performed IMS registration with the IM CN subsystem before, and the temporary user id (derived from IMSI) in all other cases.

NOTE 5:	URN is the outcome of the  URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

<End of modified section>

<Start of modified section>

## A.1.4	SUBSCRIBE for reg-event package

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | SUBSCRIBE |  |  |
| Request-URI |  | Public user identity used for subscription (NOTE 1) |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| route-param | A1 | <sip:SS P-CSCF address:protected server port of P-CSCF;lr>, <sip:scscf.3gpp.org;lr> |  |  |
| route-param | A2 | <sip:SS P-CSCF address: unprotected server port of P-CSCF (optional);lr>, <sip:scscf.3gpp.org;lr> |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by | A1 | IP address or FQDN and protected server port of the UE |  |  |
|  | A2 | IP address or FQDN, port (optional) and not checked |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | Public user identity used for subscription (NOTE 1) |  |  |
| tag |  | must be present, value not checked but stored for later reference |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | Public user identity used for subscription (NOTE 1) |  |  |
| tag |  | not present |  |  |
| Contact |  |  |  | RFC 3261 [15] RFC 5627 [61] |
| addr-spec | A1 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
| addr-spec | A2 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
| addr-spec | A4 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
| Expires |  |  |  | RFC 3261 [15] |
| delta-seconds |  | 600000 |  |  |
| Security-Verify | A1 |  |  | RFC 3329 [21] |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A5 | not present |  |  |
| Require | A1 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Require | A5 | not present |  |  |
| Proxy-Require | A1 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Proxy-Require | A5 | not present |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value not checked |  |  |
| method |  | SUBSCRIBE |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | value not checked, but stored for later reference |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1,A5 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A1 AND A6 | access network information and, if applicable, the cell ID |  |  |
| access-net-spec | A5 | access network information for Fixed Broadband and if applicable DSL Location Parameter |  |  |
|  | A7 AND NOT A8 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A8 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Accept |  | (if present) |  | RFC 3261 [15] RFC 3680 [22] |
| media-range |  | application/reginfo+xml |  |  |
| Event |  |  |  | RFC 6665 [140] RFC 3680 [22] |
| event-type |  | reg |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this message and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of request body, if such is present |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A3 | Void |
| A4 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A5 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A6 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A7 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A8 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	According to TS 24.229 clause 5.1.1.3 the public user identity used for subscription is:
a) when the UE has an ISIM the default public user identity or the public user identity used for initial registration
b) when the UE does not have an ISIM the default public user identity

NOTE 2:	When the UE supports IMS security then condition A1 is applied unless condition A2 is specified explicitly by the test case.

<End of modified section>

<Start of modified section>

# A.2	Default messages for Call Setup

## A.2.1	INVITE for MO Call Setup

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |  |  |  |  |  |
| Method |  |  |  | INVITE |  |  |  | RFC 3261 [15] |  |
| Request-URI |  | A4 |  | px_IMS_CalleeUri px_IMS_CalleeURI may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_IMS_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting |  |  |  | TS 24.229 [10] cl 5.1.2A.1.3, 5.1.2A.1.5, 7.2A.10 |  |
|  |  | A5 |  | MO call has been established: px_IMS_CalleeContactUri MT call has been established: address of the remote UE as sent in the Contact header field of the dialog creating MT INVITE in the preamble |  |  |  |  |  |
|  |  | A6, A7 |  | emergency service URN beginning with urn:service:sos |  |  |  | RFC 5031 [97] |  |
|  |  | A20 |  | urn:service:sos.ecall.manual |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A21 |  | urn:service:sos.ecall.automatic |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A25 |  | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94] |  | Rel-14 |  | RFC 8147 [149] |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| sent-protocol |  |  |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |  |  |  |
| sent-by |  | A1,A7 |  | IP address or FQDN and protected server port of the UE |  |  |  |  |  |
|  |  | A2,A19 |  | IP address or FQDN, port (optional) and not checked |  |  |  |  |  |
|  |  | A6 |  | IP address and, when using UDP, unprotected server port of the UE |  |  |  |  |  |
|  |  | A17 |  | IP address and unprotected server port of the UE |  |  |  |  |  |
| response-port |  | A6 |  | rport (when using UDP) |  |  |  | RFC 3581 [96] |  |
| via-branch |  |  |  | value starting with ‘z9hG4bK’ |  |  |  |  |  |
| Route |  |  |  | order of the parameters in this header must be like in the respective rows |  |  |  | RFC 3261 [15] |  |
| route-param |  | A1 |  | <sip:SS P-CSCF address: protected server port of SS;lr>, <sip:scscf.3gpp.org;lr> |  |  |  |  |  |
|  |  | A2,A17 |  | <sip:SS P-CSCF address: unprotected server port of SS (optional);lr>, <sip:scscf.3gpp.orgf;lr |  |  |  |  |  |
|  |  | A5 |  | MO call has been established: URIs of the Record-Route header of 183 response in reverse order (or any other response creating the dialog according to RFC 3261 clause 12.1 [15]) MT call has been established: same value as defined for the Record-Route header in A.2.9 |  |  |  |  |  |
|  |  | A6,A19 |  | <sip:SS P-CSCF address: unprotected server port of SS;lr> |  |  |  |  |  |
|  |  | A7 |  | <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  | A4 |  | any SIP URI being subscribed and registered as listed in the XML body of the NOTIFY request; additionally when there is a P-Preferred-Identity header within the INVITE request the SIP URI shall match the URI within the P-Preferred-Identity header |  |  |  |  |  |
|  |  | A5 |  | local SIP URI of the UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |  |  |  |
|  |  | A6 |  | Any SIP URI with display name as “Anonymous” or anonymous |  |  |  |  |  |
|  |  | A7,A19 |  | emergency public user identity (NOTE 3) |  |  |  |  |  |
| tag |  | A4 |  | must be present, value not checked |  |  |  |  |  |
|  |  | A5 |  | local tag of the dialog ID (In the earlier requests within the same dialog this tag appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  | A4 |  | px_IMS_CalleeUri |  |  |  |  |  |
|  |  | A5 |  | remote SIP URI of SS (i.e. the remote UE) as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |  |  |  |
|  |  | A6,A7 |  | emergency service URN beginning with urn:service:sos |  |  |  | RFC 5031 [97] |  |
|  |  | A20 |  | urn:service:sos.ecall.manual |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A21 |  | urn:service:sos.ecall.automatic |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A25 |  | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94] |  | Rel-14 |  | RFC 8147 [149] |  |
| tag |  | A4 |  | not present |  |  |  |  |  |
|  |  | A5 |  | remote tag of the dialog ID (In the earlier requests within the same dialog this tag appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  | A4 |  | value different to that received in REGISTER message |  |  |  |  |  |
|  |  | A5 |  | value of Call-ID as in any previous request in the same dialog |  |  |  |  |  |
| Call-Info |  | A20,A21,A25 |  |  |  | Rel-14 |  | RFC 8147 [149] |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | any cid URL |  |  |  |  |  |
| info-param |  |  |  | purpose=EmergencyCallData.eCall.MSD |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  | A4 |  | must be present, value not checked |  |  |  |  |  |
|  |  | A5 |  | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |  |  |  |
| method |  |  |  | INVITE |  |  |  |  |  |
| Supported |  |  |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  |  |  | RFC 3261 [15] |  |
| option-tag |  | A4 OR A5 |  | 100rel |  |  |  |  |  |
|  |  | A13 OR A14 |  | norefersub |  | Rel-11 |  | RFC 4488 [126] |  |
|  |  | A4 AND A26 AND NOT (A6 OR A7 OR A19 OR A20 OR A21) |  | timer |  |  |  | RFC 4028 [146] |  |
| P-Early-Media |  | A16 AND NOT A5 |  |  |  |  |  | RFC 5009 [138] IR.92 [133] |  |
| em-param |  |  |  | supported |  |  |  |  |  |
| Geolocation |  | A8 |  |  |  | Rel-9 |  | RFC 6442 [98] |  |
| locationURI |  |  |  | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |  |  |  |  |  |
| Geolocation |  | NOT A8 |  | not present |  | Rel-9 |  | RFC 6442 [98] |  |
| Geolocation-Routing |  | A8 |  | “yes” |  | Rel-9 |  | RFC 6442 [98] |  |
| Geolocation-Routing |  | NOT A8 |  | not present |  | Rel-9 |  | RFC 6442 [98] |  |
| Require |  | A1,A7 |  |  |  |  |  | RFC 3261 [15] |  |
|  |  | A6 |  | not present |  |  |  |  |  |
| option-tag |  | A1,A7 |  | sec-agree |  |  |  | RFC 3329 [21] |  |
| Proxy-Require |  | A1,A7 |  |  |  |  |  | RFC 3261 [15] RFC 3329 [21] |  |
|  |  | A6 |  | not present |  |  |  |  |  |
| option-tag |  | A1,A7 |  | sec-agree |  |  |  |  |  |
| Security-Verify |  | A1,A7 |  |  |  |  |  | RFC 3329 [21] |  |
|  |  | A2,A6 |  | not present |  |  |  |  |  |
| sec-mechanism |  | A1,A7 |  | same value as Security-Server header sent by SS |  |  |  |  |  |
| Contact |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  | (A1 OR A7) AND NOT A15 |  | SIP URI with IP address or FQDN and protected server port of UE |  |  |  |  |  |
|  |  | (A2 OR A19) AND NOT A15 |  | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |  |  |  |
|  |  | A15 AND NOT A6 |  | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |  | RFC 5627 [61] |  |
|  |  | A6 |  | SIP URI with IP address and unprotected server port of UE |  |  |  |  |  |
| c-p-instance |  | A6 |  | +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in RFC 7254 [122], without optional parameters |  | Rel-10 |  | RFC 5626 [109] RFC 7254 [122] |  |
|  |  | A3,A17 |  | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (NOTE 2, 4) |  |  |  |  |  |
| feature-param |  | A10 |  | video |  |  |  | RFC 3840 [63] |  |
|  |  | A12 |  | +g.3gpp.srvcc-alerting |  |  |  | RFC 3840 [63] |  |
|  |  | A22 AND (A23 OR A24) |  | audio |  |  |  | RFC 3840 [63] |  |
|  |  | A18 AND NOT A5 |  | +g.3gpp.ps2cs-srvcc-orig-pre-alerting |  |  |  | RFC 3840 [63] |  |
| Max-Forwards |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | non-zero value |  |  |  |  |  |
| P-Access-Network-Info |  | NOT A2 |  |  |  |  |  | RFC 7315 [132] RFC 7913 [154] |  |
| access-net-spec |  | A1 AND A27 |  | access network information and, if applicable, the cell ID |  |  |  |  |  |
|  |  | A28 AND NOT A34 |  | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID |  | Rel-15 |  |  |  |
|  |  | A1 AND A34 |  | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID |  | Rel-17 |  | TS 24.229 [10] |  |
| P-Access-Network-Info |  | A2(o) |  | header optional |  |  |  |  |  |
| access-net-spec |  |  |  | access network information and, if applicable, the cell ID |  |  |  |  |  |
| Accept |  | NOT A5 |  | header mandatory |  | Rel-7 |  | RFC 3261 [15] |  |
|  |  | A5(o) |  | header optional |  |  |  |  |  |
| media-range |  | A4 |  | application/sdp,application/3gpp-ims+xml (additional medias can be added in any order) |  |  |  |  |  |
|  |  | A13 |  | application/vnd.3gpp.mid-call+xml |  | Rel-11 |  |  |  |
|  |  | A14 |  | application/vnd.3gpp.state-and-event-info+xml |  | Rel-11 |  |  |  |
|  |  | A20,A21,A25 |  | application/EmergencyCallData.Control+xml |  | Rel-14 |  | RFC 8147 [149] |  |
| P-Preferred-Service |  |  |  |  |  |  |  | RFC 6050 [68] |  |
| Service-ID |  | A3 AND A4 |  | urn:urn-7:3gpp-service.ims.icsi.mmtel |  |  |  |  |  |
| P-Preferred-Identity |  |  |  |  |  |  |  | RFC 3325 [89] |  |
| PPreferredID-value |  | A7 |  | emergency public user identity (NOTE 3) |  |  |  |  |  |
| Recv-Info |  |  |  |  |  |  |  | RFC 6086 [139] |  |
| Info-package-type |  | A14 |  | g.3gpp.state-and-event |  |  |  |  |  |
|  |  | A20,A21 |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
| Accept-Contact |  |  |  |  |  |  |  | RFC 3841 [64] |  |
| ac-value |  | A3 AND A4 |  | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (NOTE 2, 4) |  |  |  | TS 24.173 [65] |  |
|  |  | A10 AND A11 |  | video |  |  |  | IR.94 [134] NG.114 [157] |  |
| Proxy-Authorization |  | A17 |  |  |  |  |  | RFC 2617 [16] RFC 3310 [17] |  |
| username |  | A17 |  | preconfigured in the UE |  |  |  |  |  |
| realm |  | A17 |  | same value as received in the realm directive in the Proxy-Authorization header sent by SS |  |  |  |  |  |
| nonce |  | A17 |  | same value as in Proxy-Authorization header sent by SS |  |  |  |  |  |
| digest-uri |  | A17 |  | preconfigured in the UE |  |  |  |  |  |
| qop-value |  | A17 |  | auth |  |  |  |  |  |
| cnonce-value |  | A17 |  | value assigned by UE affecting the response calculation |  |  |  |  |  |
| nonce-count |  | A17 |  | counter to indicate how many times UE has sent the same value of nonce within successive INVITE requests, initial value shall be 1 |  |  |  |  |  |
| response |  | A17 |  | response calculated by UE |  |  |  |  |  |
| algorithm |  | A17 |  | MD5 |  |  |  |  |  |
| Content-Type |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| media-type |  | NOT A8 AND NOT A20 AND NOT A21 AND NOT A25 |  | application/sdp |  |  |  |  |  |
|  |  | A8, A20, A21, A25 |  | multipart/mixed;boundary=any value |  |  |  | RFC 6442 [98] RFC 8147 [149] |  |
| Content-Length |  |  |  | header shall be present if UE uses TCP to send this message and if there is a message body |  |  |  | RFC 3261 [15] |  |
| Value |  |  |  | length of message-body |  |  |  |  |  |
| Message-body |  |  |  | consists of one or several parts as indicated by Content-Type, and each part having actual contents as follows (SDP contents, if any, is specified in dedicated sections) |  |  |  |  |  |
|  |  | A8 |  | "application/pidf+xml" MIME body part containing a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more 'geopriv' elements (Note), each containing: - At least one 'location-info' element describing the location of the UE. Note: PIDF-LO shall contain at least one <geopriv> element in a <tuple> element according to RFC 4119 or in a <device> or <person> element according to RFC 4119 and RFC 5491. |  |  |  |  |  |
|  |  | A20 OR A21 OR A25 |  | --boundary value (as provided in SIP hdr Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: converted cid URL from Call-Info header. Note that this is done by removing the cid: prefix. Content-Disposition: by-reference;handling=optional MSD in ASN.1 PER encoding --boundary value (as provided in SIP hdr Content-Type) |  | Rel-14 |  | RFC 8147 [149] |  |

| Condition |  |  | Explanation |  |  |
| --- | --- | --- | --- | --- | --- |
| A1 |  |  | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 5) |  |  |
| A2 |  |  | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 5) |  |  |
| A3 |  |  | UE supports MTSI (A.3A/50 3GPP TS 34.229-2 [5]) |  |  |
| A4 |  |  | INVITE creating a dialog (NOTE 6) |  |  |
| A5 |  |  | re-INVITE within a dialog |  |  |
| A6 |  |  | INVITE for creating an emergency session in case of no registration |  |  |
| A7 |  |  | INVITE for creating an emergency session within an emergency registration using IMS security |  |  |
| A8 |  |  | UE is capable of obtaining location information, has obtained its location and is setting up an emergency session |  |  |
| A9 |  |  | Void |  |  |
| A10 |  |  | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |  |  |
| A11 |  |  | INVITE for creating a video stream (establishment of a video call or adding a video stream to a voice call) |  |  |
| A12 |  |  | INVITE for creating a voice or video call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |  |  |
| A13 |  |  | INVITE for creating a voice call during rSRVCC and UE supports CS to PS SRVCC with the MSC assisted mid-call feature (A.12/42 3GPP TS 34.229-2 [5]) |  |  |
| A14 |  |  | INVITE for creating a voice call and UE supports CS to PS SRVCC for calls in alerting phase (A.12/41 3GPP TS 34.229-2 [5]) |  |  |
| A15 |  |  | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |  |  |
| A16 |  |  | UE supports early media (A.12/45 3GPP TS 34.229-2 [5]) |  |  |
| A17 |  |  | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |  |  |
| A18 |  |  | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request (A.12/36 3GPP TS 34.229-2 [5]) |  |  |
| A19 |  |  | INVITE for creating an emergency session within an emergency registration using GIBA |  |  |
| A20 |  |  | INVITE for creating an eCall over IMS session manually |  |  |
| A21 |  |  | INVITE for creating an eCall over IMS session automatically |  |  |
| A22 |  |  | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |  |  |
| A23 |  |  | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |  |  |
| A24 |  |  | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |  |  |
| A25 |  |  | INVITE for creating a test eCall over IMS session |  |  |
| A26 |  |  | UE supports Session Timer (A.12/57 3GPP TS 34.229-2 [5]) |  |  |
| A27 |  |  | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |  |  |
| A28 |  |  | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |  |  |
| A29 |  |  | Void |  |  |
| A30 |  |  | Void |  |  |
| A31 |  |  | Void |  |  |
| A32 |  |  | Void |  |  |
| A34 |  |  | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |  |  |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	According to TS 24.229 clause 5.1.1.1A and 5.1.6.2 [10] when the UE is using ISIM the emergency public user identity is the first public user identity in the list stored in the ISIM; when there is no ISIM it is the default public user id if the UE registered or the temporary user id (derived from IMSI) else.

NOTE 4:	URN is the outcome of URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

NOTE 5:	When the UE supports IMS security then condition A1 is applied unless condition A2 is specified explicitly by the test case.

NOTE 6:	Condition A4 is applied per default, i.e. unless overruled by another condition explicitly specified by the test case.

<End of modified section>

<Start of modified section>

## A.2.4	PRACK

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | PRACK |  |  |
| Request-URI |  | same URI value as the recipient of PRACK has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | same value as in preceding INVITE message |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| Route | A1,A2 |  |  | RFC 3261 [15] |
| route-param |  | MO call (dialog creating INVITE sent by the UE): URIs of the Record-Route header field as sent to the UE in the 183 Session Progress in reverse order (or in any other response creating the dialog according to RFC 3261 clause 12.1 [15]). MT call (dialog creating INVITE sent by the SS): URIs of the Route header field as sent to the UE in the dialog creating INVITE. |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE when PRACK is sent by the UE, but SIP URI of the SS when PRACK is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS when PRACK is sent by the UE, but SIP URI of the UE when PRACK is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |
| method |  | PRACK |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| RAck |  |  |  | RFC 3262 [33] |
| response-num |  | same value as in RSeq header of the reliable response |  |  |
| cseq-num |  | same value as in CSeq of the reliable response |  |  |
| method |  | same value as in CSeq of the reliable response |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A6 | access network information and, if applicable, the cell ID |  |  |
|  | A7 AND NOT A8 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A8 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Proxy-Authorization | A5 |  |  | RFC 2617 [16] RFC 3310 [17] |
| username | A5 | preconfigured in the UE |  |  |
| realm | A5 | same value as received in the realm directive in the Proxy-Authorization header sent by SS |  |  |
| nonce | A5 | same value as in Proxy-Authorization header sent by SS |  |  |
| digest-uri | A5 | preconfigured in the UE |  |  |
| qop-value | A5 | auth |  |  |
| cnonce-value | A5 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A5 | counter to indicate how many times UE has sent the same value of nonce within successive INVITEs, initial value shall be 1 |  |  |
| response | A5 | response calculated by UE |  |  |
| algorithm | A5 | MD5 |  |  |
| Content-Type |  | header shall be present only if there is SDP in Message-body |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of Message-body |  |  |
| Message-body |  | Optional SDP body. If included then the contents of the SDP shall be checked as described in the Test requirements section of the test case. |  | RFC 4566 [27] RFC 3264 [30] RFC 3312 [31] |

| Condition | Explanation |
| --- | --- |
| A1 | PRACK sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A2 | PRACK sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A3 | PRACK sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A4 | PRACK sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A5 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A6 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A7 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A8 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

NOTE 2:	When the UE supports IMS security then condition A1 or A3 is applied unless condition A2 or A4 is specified explicitly by the test case.

## A.2.5	UPDATE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | UPDATE |  |  |
| Request-URI |  | same URI value as the recipient of UPDATE has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1,A2 | MO call: same value as in INVITE message MT call: as defined in A.2.1 |  |  |
|  | A3,A4 | as defined in A.2.9 (There is more than one value) |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 2) |  |  |
| Route | A1,A2 |  |  | RFC 3261 [15] |
| route-param |  | MO call (dialog creating INVITE sent by the UE): URIs of the Record-Route header field as sent to the UE in the 183 Session Progress in reverse order (or in any other response creating the dialog according to RFC 3261 clause 12.1 [15]). MT call (dialog creating INVITE sent by the SS): URIs of the Route header field as sent to the UE in the dialog creating INVITE. |  |  |
| From |  |  |  |  |
| addr-spec |  | SIP URI of the UE when UPDATE is sent by the UE, but SIP URI of the SS when UPDATE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  | RFC 3261 [15] |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS when UPDATE is sent by the UE, but SIP URI of the UE when UPDATE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| Contact | A1,A2 | Must be present, value not checked |  | RFC 3261 [15] RFC 3311 [137] |
|  | A3,A4 | MO call: same Contact header as sent by SS in this dialog (e.g. in a 183 or 180 response) MT call: same value as used in MT INVITE |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |
| method |  | UPDATE |  |  |
| Require | A1 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1 | sec-agree |  |  |
| Proxy-Require | A1 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1 | sec-agree |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| Security-Verify | A1 |  |  | RFC 3329 [21] |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A2 | not present |  | RFC 3329 [21] |
| sec-mechanism |  |  |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A5 | access network information and, if applicable, the cell ID |  |  |
|  | A6 AND NOT A7 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A7 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length | A3,A4 |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |
| Message-body |  | Contents of the SDP body shall be checked as described in the Test requirements section of the test case. |  | RFC 4566 [27] RFC 3264 [30] RFC 3312 [31] |

| Condition | Explanation |
| --- | --- |
| A1 | UPDATE sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A2 | UPDATE sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A3 | UPDATE sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A4 | UPDATE sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A5 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A6 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A7 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	Branch parameter values sent by SS are different within a test case execution.

NOTE 3:	When the UE supports IMS security then condition A1 or A3 is applied unless condition A2 or A4 is specified explicitly by the test case.

## A.2.6	180 Ringing for INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 180 |  |  |
| Reason-Phrase |  | Ringing |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route |  | as defined for the common 183 response, see A.2.3 |  |  |
|  | A7 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |  |  |
|  | A8 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in INVITE message |  |  |
| Require |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag | A3 | 100rel |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | same value as received in INVITE message |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | as defined for the common 183 response, see A.2.3 |  |  |
| P-Asserted-Identity | A4 |  |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec |  | as defined for the common 183 response, see A.2.3 |  |  |
| feature-param | A2 AND A5 | +g.3gpp.srvcc-alerting |  |  |
|  | A1 | audio |  |  |
|  | A2 AND A9 AND (A10 OR A11) | audio |  |  |
| RSeq |  |  |  | RFC 3262 [33] |
| response-num | A3 AND NOT A12 | previous RSeq number sent in the same direction incremented by one |  |  |
|  | A1 AND A12 | 122 |  |  |
|  | A2 AND A12 | any value |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in INVITE message |  |  |
| P-Access-Network-Info | A2 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A13 | access network information and, if applicable, the cell ID |  |  |
|  | A14 AND NOT A15 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A15 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| P-Access-Network-Info | A1 | not present |  |  |
| Feature-Caps |  |  |  |  |
| feature-param | A1 AND A5 | +g.3gpp.srvcc-alerting |  |  |
| Content-Length | A1 |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 180 sent by the SS |
| A2 | 180 sent by the UE |
| A3 | Response sent reliably (e.g. always when it contains an SDP body) |
| A4 | 180 sent by the SS when setting up an emergency call or a non-UE detectable emergency call |
| A5 | UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |
| A6 | Void |
| A7 | Response sent by SS for emergency call without emergency registration |
| A8 | Response sent by SS for emergency call with emergency registration or a non-UE detectable emergency call |
| A9 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A10 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A. |
| A11 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23. |
| A12 | 180 Ringing is first provisional response sent reliably in this dialog |
| A13 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A14 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A15 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

<End of modified section>

<Start of modified section>

## A.2.8	BYE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | BYE |  |  |
| Request-URI |  | same URI value as the recipient of BYE has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via | A1,A2 |  |  | RFC 3261 [15] |
| via-parm1: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | MO Call has been established: same value as in INVITE message MT Call has been established: as defined in A.2.1 |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| Via | (A3 OR A4) AND NOT A9 | as defined in A.2.9 (There is more than one value) |  | RFC 3261 [15] |
| Via | A9 |  |  | RFC 3261 [15] |
| via-parm1: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A3 | The SS P-CSCF address and the SS protected server port |  |  |
|  | A4 | The SS P-CSCF address and the SS unprotected server port |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| via-parm2 |  | SIP/2.0/UDP ecscf.3gpp.org ;branch=z9hG4bK… (NOTE 1) |  |  |
| via-parm3 |  | SIP/2.0/UDP psap.3gpp.org ;branch=z9hG4bK… (NOTE 1) |  |  |
| Route |  |  |  | RFC 3261 [15] |
| route-param | A1,A2 | MO Call has been established: URIs of the Record-Route header of 183 response in reverse order (or any other response creating the dialog according to RFC 3261 clause 12.1 [15]) MT Call has been established: value of Record-Route header as defined in A.2.9 |  |  |
| Route | A3,A4, A9 | not present |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE when BYE is sent by the UE. SIP URI of the SS when BYE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS when BYE is sent by the UE. SIP URI of the UE when BYE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as sent or received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |
| method |  | BYE |  |  |
| Require | (A1 OR A5) AND NOT A6 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1,A5 | sec-agree |  |  |
| Require | A2, A6 | not present |  |  |
| Proxy-Require | (A1 OR A5) AND NOT A6 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1,A5 | sec-agree |  |  |
| Proxy-Require | A2,A6 | not present |  |  |
| Security-Verify | (A1 OR A5) AND NOT A6 |  |  | RFC 3329 [21] |
| sec-mechanism | A1,A5 | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A2,A6 | not present |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A7 | access network information and, if applicable, the cell ID |  |  |
|  | A8 AND NOT A10 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A10 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Proxy-Authorization | A5 |  |  | RFC 2617 [16] RFC 3310 [17] |
| username | A5 | preconfigured in the UE |  |  |
| realm | A5 | same value as received in the realm directive in the Proxy-Authorization header sent by SS |  |  |
| nonce | A5 | same value as in Proxy-Authorization header sent by SS |  |  |
| digest-uri | A5 | preconfigured in the UE |  |  |
| qop-value | A5 | auth |  |  |
| cnonce-value | A5 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A5 | counter to indicate how many times UE has sent the same value of nonce within successive INVITESs, initial value shall be 1 |  |  |
| response | A5 | response calculated by UE |  |  |
| algorithm | A5 | MD5 |  |  |
| Content-Length | A3,A4, A9 |  |  | RFC 3261 [15] |
| value |  | length of message body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | BYE sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A2 | BYE sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A3 | BYE sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A4 | BYE sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A5 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A6 | BYE sent by the UE for emergency call with no registration |
| A7 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A8 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A9 | BYE sent by the SS for emergency call or eCall |
| A10 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

NOTE 2:	When the UE supports IMS security then condition A1 or A3 is applied unless condition A2 or A4 is specified explicitly by the test case.

<End of modified section>

<Start of modified section>

## A.2.10	MO REFER

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | REFER |  |  |
| Request-URI |  | same URI value as the SS has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1 | IP address or FQDN and protected server port of the UE |  |  |
|  | A2 | IP address or FQDN and unprotected server port of the UE |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| Route |  | order of the parameters in this header must be like in this table |  | RFC 3261 [15] |
| route-param | A1 | URIs of the Record-Route header of 183 response in reverse order |  |  |
|  | A2 | URIs of the Record-Route header of 183 response in reverse order |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | local SIP URI of the UE which must be the same URI as used for the UE in the earlier requests within the dialog |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS which must be the same URI as used by the UE in earlier requests within the dialog |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as in the first INVITE during the call setup |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |
| method |  | REFER |  |  |
| Require | A1 |  |  | RFC 3261 [15] RFC 3312 [31] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| Proxy-Require | A1 |  |  | RFC 3261 [15] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| Security-Verify | A1 |  |  | RFC 3329 [21] |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A2 | not present |  | RFC 3329 [21] |
| Contact |  |  |  | RFC 3261 [15] RFC 5627 [61] |
| addr-spec | A1 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A2 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A3 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
| Refer-To |  |  |  | RFC 3515 [72] |
| addr-spec |  | SIP or Tel URI of the transfer target (Note 1) |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A4 | access network information and, if applicable, the cell ID |  |  |
|  | A5 AND NOT A6 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A6 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Content-Length |  | header shall be present if UE uses TCP to send this request and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A3 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A4 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A5 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A6 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	The SIP URI may contain a "Replaces" header referring to the dialog ID which has been established before.

NOTE 2:	When the UE supports IMS security then condition A1 is applied unless condition A2 is specified explicitly by the test case.

<End of modified section>

<Start of modified section>

# A.3	Generic Common Messages

## A.3.1	200 OK for other requests than REGISTER or SUBSCRIBE

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Status-Code |  |  |  | 200 |  |  |  |  |  |
| Reason-Phrase |  |  |  | OK |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| via-parm |  |  |  | same value as received in request |  |  |  |  |  |
| Record-Route |  |  |  | order of the parameters in this header must be like in the respective rows |  |  |  | RFC 3261 [15] |  |
| rec-route |  | A1 AND NOT A23 |  | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |  |  |  |
|  |  | A3 AND NOT A23 |  | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |  |  |  |
|  |  | A20 AND NOT A23 |  | same value as received in the request (initial INVITE from the SS) Note: for requests other than INVITE it is not regulated if and what the UE writes into this header in a response. |  |  |  |  |  |
|  |  | A6 |  | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |  |  |  |
|  |  | A7 |  | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request or any value added if missing from request |  |  |  |  |  |
| P-Asserted-Identity |  | A6,A7 |  |  |  |  |  | RFC 3325 [89] |  |
| addr-spec |  |  |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |  |  |  |
| uri-parameter |  |  |  | lr |  |  |  |  |  |
| Contact |  | A1,A2, A3,A4, A6,A7 |  |  |  |  |  | RFC 3261 [15] RFC 5627 [61] |  |
| addr-spec |  | A1,A3, A6,A7 |  | MO call: px_IMS_CalleeContactUri MT call: Contact address provided by SS in dialog creating INVITE |  |  |  |  |  |
|  |  | A2,A4 |  | MO call: Contact address provided by UE in dialog creating INVITE MT call: Contact address provided by UE in 1xx response establishing the dialog according to RFC 3261 [15] clause 12. (NOTE: The 'transport' url parameter may change, url parameters are not checked) If there has been no 1xx response establishing the dialog (i.e. the 200 OK establishes the dialog) then the Contact address is as specified for the 183 Session Progress in A.2.3. |  |  |  |  |  |
| feature-param |  | A10 |  | audio |  |  |  |  |  |
|  |  | A18 AND (A19 OR A20) |  | video |  |  |  | IR.94 [134] |  |
|  |  | A11 AND A15 AND (A16 OR A17 OR A22) |  | audio |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same value as received in request |  |  |  |  |  |
| Call-Info |  | A12 |  |  |  |  |  | RFC 8147 [149] |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | cid:psap@3gpp.org |  | Rel-14 |  |  |  |
| info-param |  |  |  | purpose=EmergencyCallData.Control |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | same value as received in request |  |  |  |  |  |
| P-Access-Network-Info |  | A8 |  |  |  |  |  | RFC 7315 [132] RFC 7913 [154] |  |
| access-net-spec |  | A21 |  | access network information and, if applicable, the cell ID |  |  |  |  |  |
|  |  | A22 AND NOT A26 |  | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID |  |  |  |  |  |
|  |  | A26 |  | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID |  | Rel-17 |  | TS 24.229 [10] |  |
| Accept |  | A12,A24 |  |  |  |  |  | RFC 8147 [149] |  |
| media-range |  | A12 |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/EmergencyCallData.eCall.MSD |  | Rel-14 |  |  |  |
|  |  | A24 |  | application/EmergencyCallData.Control+xml |  | Rel-19 |  | TS 24.229 [10] |  |
| Recv-Info |  | A12,A24 |  |  |  |  |  | RFC 8147 [149] |  |
| Info-package-type |  |  |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  |  |  |
| Content-Type |  | A12 |  |  |  |  |  | RFC 8147 [149] |  |
| media-type |  |  |  | multipart/mixed; boundary=boundary1 |  | Rel-14 |  |  |  |
| Content-Length |  | A10 |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message body |  |  |  |  |  |
| Message-body |  | A12 |  |  |  |  |  |  |  |
|  |  | A13 |  | --boundary1 Content-Type: application/EmergencyCallData.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.Control> --boundary1 |  |  |  | RFC 8147 [149] |  |
|  |  | A14 |  | --boundary1 Content-Type: application/EmergencyCallData.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="false" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE "/> </EmergencyCallData.Control> --boundary1 |  |  |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE/UPDATE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A2 | Response sent by UE for INVITE/UPDATE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A3 | Response sent by SS for INVITE/UPDATE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A4 | Response sent by UE for INVITE/UPDATE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A5 | Void |
| A6 | Response sent by SS for INVITE for emergency call with emergency registration or for INVITE/UPDATE for non-UE detectable emergency call |
| A7 | Response sent by SS for INVITE for emergency call without emergency registration |
| A8 | Any response sent by the UE within a dialog, except for CANCEL requests (NOTE 2) |
| A9 | Void |
| A10 | Response sent by SS |
| A11 | Response sent by UE |
| A12 | Response sent by SS for INVITE for eCall over IMS session with either ACK or NACK |
| A13 | Response sent by SS for INVITE for eCall over IMS session with ACK element = TRUE |
| A14 | Response sent by SS for INVITE for eCall over IMS session with ACK element = FALSE |
| A15 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A16 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A17 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A18 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A19 | Response sent by SS for INVITE |
| A20 | Response sent by UE for INVITE |
| A21 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A22 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A23 | Response sent for re-INVITE within an established dialog |
| A24 | Response sent by UE subjected to national regulations in CEN EN 17184:2024 (A.4.5/67 3GPP TS 34.229-2 [5] or A.4.5/68 3GPP TS 34.229-2 [5]) for INVITE for PSAP Callback |
| A26 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	When the UE supports IMS security then for establishment of a normal call condition A1 or A2 is applied unless condition A3 or A4 is specified explicitly by the test case. For emergency call condition A6 or A7 shall be specified by the test case.

NOTE 2:	As the 200 OK for SIP MESSAGE is the only case where a 200 OK for other requests than REGISTER or SUBSCRIBE is not sent within a dialog, in fact condition A8 is always applied except for the 200 OK responses for CANCEL and SIP MESSAGE.

<End of modified section>

<Start of modified section>

# A.4	Other Default Messages

<skipped>

## A.4.3	PUBLISH

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | PUBLISH |  | RFC 3903 [60] |
| Request-URI |  | any IMPU within the set of IMPUs on ISIM |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| route-param | A1 | <sip:SS P-CSCF address:protected server port of P-CSCF;lr>, <sip:scscf.3gpp.org;lr> |  |  |
|  | A2 | <sip:SS P-CSCF address: unprotected server port of P-CSCF (optional);lr>, <sip:scscf.3gpp.org;lr> |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by | A1 | IP address or FQDN and protected server port of the UE |  |  |
| sent-by | A2 | IP address or FQDN, port (optional) and not checked |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | any IMPU within the set of IMPUs on ISIM |  |  |
| tag |  | must be present, value not checked but stored for later reference |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | any IMPU within the set of IMPUs on ISIM |  |  |
| tag |  | not present |  |  |
| Expires |  | optional |  | RFC 3261 [15] |
| delta-seconds |  | same as registration timer |  |  |
| Security-Verify | A1 |  |  |  |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Require | A1 | optional |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | value not checked |  |  |
| Proxy-Require | A1 | optional |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | value not checked |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | must be present, value not checked |  |  |
| method |  | PUBLISH |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | value not checked, but stored for later reference |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A4 | access network information and, if applicable, the cell ID |  |  |
|  | A5 AND NOT A6 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID |  |  |
|  | A6 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Event |  |  |  | RFC 6665 [140] RFC 3680 [22] RFC 3903 [60] |
| event-type |  | value not checked |  |  |
| SIP-If-Match |  | optional |  | RFC 3903 [60] |
| entity-tag |  |  |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this request and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of request body, if such is present |  |  |
| Message-body |  | optional |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A3 | Void |
| A4 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A5 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A6 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	When the UE supports IMS security then condition A1 is applied unless condition A2 is specified explicitly by the test case.

<End of modified section>

<Start of modified section>

# A.5	Default messages for Conferencing

## A.5.1	SUBSCRIBE for conference event package

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | SUBSCRIBE |  |  |
| Request-URI |  | sip:final@conf-factory. appended with px_IMS_HomeDomainName |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| route-param | A1 | <sip:SS P-CSCF address:protected server port of P-CSCF;lr>, <sip:scscf.3gpp.org;lr> |  |  |
|  | A2 | <sip:SS P-CSCF address: unprotected server port of P-CSCF (optional);lr>, <sip:scscf.3gpp.org;lr> |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by | A1 | IP address or FQDN and protected server port of the UE |  |  |
| sent-by | A2 | IP address or FQDN and unprotected server port of the UE |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | any IMPU within the set of IMPUs on ISIM |  |  |
| tag |  | must be present, value not checked but stored for later reference |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | sip:final@conf-factory. appended with px_IMS_HomeDomainName |  |  |
| tag |  | not present |  |  |
| Contact |  |  |  | RFC 3261 [15] RFC 5627 [61] |
| addr-spec | A1 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A2 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A4 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
| Expires |  |  |  | RFC 3261 [15] |
| delta-seconds |  | must be present but value not checked |  |  |
| Security-Verify | A1 |  |  | RFC 3329 [21] |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Require | A1 |  |  | RFC 3261 [15] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| Proxy-Require | A1 |  |  | RFC 3261 [15] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | must be present, value not checked |  |  |
| method |  | SUBSCRIBE |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | value not checked, but stored for later reference |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A6 | access network information and, if applicable, the cell ID |  |  |
|  | A7 AND NOT A8 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A8 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Accept |  |  |  | RFC 3261 [15] |
| media-range |  | application/conference-info+xml |  | RFC 3680 [22] |
| Event |  |  |  | RFC 6665 [140] RFC 3680 [22] |
| event-type |  | conference |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this request and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of request body, if such is present |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A3 | Void |
| A4 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A5 | Void. |
| A6 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A7 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A8 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	When the UE supports IMS security then condition A1 is applied unless condition A2 is specified explicitly by the test case.

<End of modified section>

<Start of modified section>

# A.6	Default messages for Message Waiting Indication

<skipped>

## A.6.1	SUBSCRIBE for message-summary event package

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | SUBSCRIBE |  |  |
| Request-URI |  | any IMPU within the set of IMPUs on ISIM or px_IMS_MessageAccountIdentity. UE shall use px_IMS_MessageAccountIdentity when that is configured to the phone as public service identity of the message account. |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| route-param | A1 | <sip:SS P-CSCF address:protected server port of P-CSCF;lr>, <sip:scscf.3gpp.org;lr> |  |  |
|  | A2 | <sip:SS P-CSCF address: unprotected server port of P-CSCF (optional);lr>, <sip:scscf.3gpp.org;lr> |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by | A1 | IP address or FQDN and protected server port of the UE |  |  |
| sent-by | A2 | IP address or FQDN and unprotected server port of the UE |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | any IMPU within the set of IMPUs on ISIM |  |  |
| tag |  | must be present, value not checked but stored for later reference |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | any IMPU within the set of IMPUs on ISIM or px_IMS_MessageAccountIdentity. UE shall use px_IMS_MessageAccountIdentity when that is configured to the phone as public service identity of the message account. |  |  |
| tag |  | not present |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A2 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A4 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  | RFC 5627 [61] |
| Expires |  |  |  | RFC 3261 [15] |
| delta-seconds |  | must be present but value not checked |  |  |
| Security-Verify | A1 |  |  | RFC 3329 [21] |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Require | A1 |  |  | RFC 3261 [15] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| Proxy-Require | A1 |  |  | RFC 3261 [15] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | must be present, value not checked |  |  |
| method |  | SUBSCRIBE |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | value not checked, but stored for later reference |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A5 | access network information and, if applicable, the cell ID |  |  |
|  | A6 AND NOT A7 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A7 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Accept |  |  |  | RFC 3261 [15] |
| media-range |  | application/simple-message-summary |  | RFC 3842 [88] |
| Event |  |  |  | RFC 6665 [140] |
| event-type |  | message-summary |  | RFC 3842 [88] |
| Content-Length |  | header shall be present if UE uses TCP to send this request and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of request body, if such is present |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A3 | Void |
| A4 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A5 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A6 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A7 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	When the UE supports IMS security then condition A1 is applied unless condition A2 is specified explicitly by the test case.

<End of modified section>

<Start of modified section>

# A.7	Default messages for SMS

<skipped>

## A.7.3	MESSAGE for MO SMS

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | MESSAGE |  | RFC 3428 [91] |
| Request-URI |  | Public Service Identity of the SM-SC: value not checked Note: value as provided by the user or taking default value as defined in E.3.2.15 |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | IP address or FQDN and protected server port of the UE |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE |  |  |
| tag |  | any value |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same as Request-URI. |  |  |
| tag |  | not present |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | must be present, value not checked |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | any value |  |  |
| method |  | MESSAGE |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A2 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A4 | access network information and, if applicable, the cell ID |  |  |
|  | A5 AND NOT A6 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A6 | access network information for NR RedCap, containing access-class parameter with value "3GPP-NR-REDCAP" or “3GPP-NR” or access-type parameter with value "3GPP-NR-REDCAP" or “3GPP-NR-TDD” or “3GPP-NR-FDD” and also containing the cell ID | Rel-17 | TS 24.229 [10] |
| Route |  |  |  | RFC 3261 [15] |
| route-param |  | <sip:SS P-CSCF address: protected server port of SS ;lr>, <sip:scscf.3gpp.org;lr> |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/vnd.3gpp.sms |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this request |  | RFC 3261 [15] |
| Value |  | length of message-body |  |  |
| Message-body |  | RP-DATA message as specified in A.7.7 with RP-User Data set to SMS-SUBMIT type equal to - TP-MTI=’ 01’B (SMS-SUBMIT) - TP-RD=any allowed value - TP-VPF=any allowed value - TP-RP=any allowed value - TP-MR=any allowed value - TP-DA=any allowed value - TP-PID=any allowed value - TP-DCS=any allowed value - TP-VP=any allowed value if TP-VPF indicates TP-VP field present; TP-VP=not present otherwise - TP-UDL=set according to length of TP-UD field - TP-UD=must be present and non-empty |  | TS 24.011 [92] TS 23.040 [93] |

| Condition | Explanation |
| --- | --- |
| A1 | Void |
| A2 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A3 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A4 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A5 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A6 | UE supports NR RedCap (A.4.3.12-1/2 3GPP TS 38.508-2 [158]) |

NOTE 1:	When the UE supports IMS security then condition A2 is applied unless condition A3 is specified explicitly by the test case.

<End of modified section>