3GPP TSG RAN Meeting #109						           RP-252312

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	Apple

Title:	Views on deployment scenarios

Document for:	Discussion/Decision

# Introduction

The latest RAN 6G SID is approved in RP-251395 [1], was approved. The SID covers mainly two aspects of the 6G study, i.e., (1) study on ITU IMT-2030 TPR (Technical Performance Requirement), (2) study of 6G in general. Regarding the study of 6G in general, the objectives from RP-251395 can be found below:

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

In this contribution, we provide our views on deployment scenarios for 6G

# Deployment scenarios for 5G and 6G

Before we discuss the deployment scenarios for 6G, it is worthwhile to recall the relevant studies during 5G at RAN and working groups. The RAN level study was captured in [2], and identified deployment scenarios in [2] are included below:

Figure 1 Deployment scenarios in TR 38.913 (5G)

6	Scenarios	9

6.0	General	9

6.1	Deployment scenarios	9

6.1.1	Indoor hotspot	10

6.1.2	Dense urban	11

6.1.3	Rural	12

6.1.4	Urban macro	13

6.1.5	High speed	14

6.1.6	Extreme long distance coverage in low density areas	16

6.1.7	Urban coverage for massive connection	16

6.1.8	Highway Scenario	17

6.1.9	Urban Grid for Connected Car	19

6.1.10	Commercial Air to Ground scenario	21

6.1.11	Light aircraft scenario	21

6.1.12	Satellite extension to Terrestrial	21

A first observation is the 5G deployment scenarios are extensive already. It can be recalled not all those deployment scenarios directly drove working group study in Rel-14 and normative work in Rel-15 NR.

We see some a strong resemblance between the deployment scenarios for 5G and 6G, but also with importance differences.

Figure 2 Deployment Scenarios in TR 38.914 draft (6G)

4.	Deployment scenarios	2

4.1	Indoor hotspot	2

4.2	Dense urban	3

4.3	Rural	4

4.4	Urban macro	6

4.5	Sub-Urban macro	8

4.6	High speed	10

4.7	Extreme long distance coverage in low density area	10

4.8	Urban coverage for massive connection	10

4.9	Air-to-Ground Scenario	11

4.10	Non-Terrestrial Network	11

4.11	[Urban grid]	11

4.12	[Highway Scenario]	11

To facilitate working group discussions—such as managing workload and providing clear direction for technology development—it would be helpful to streamline deployment scenarios. For example, the highway and high-speed scenarios could potentially be merged. When possible, prioritizing deployment scenarios can be really helpful.

Given the limited time and capacity of working groups, such prioritization often happens de facto during working group discussions. However, without clear guidance from RAN, this process can become prolonged and consume valuable time in less effective ways.

We have

Proposal 1: Streamline the deployment scenarios and/or prioritizing deployment scenarios to facilitate working group discussions when possible.

One key difference between 5G and 6G scenario definitions are the explicit carrier aggregation consideration for 6G. For example, for the “Dense Urban” scenario, TR 38.913 identifies 2 carrier frequencies: around 4 GHz and around 30 GHz. In contrast, for the “Dense Urban” scenario in TR 38.914, the following are identified:

- Around 2GHz

- Around 4 GHz

- Around 7 GHz

- Around 4 GHz + Around 30GHz

- Around 4 GHz + Around 7 GHz

- [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz]

In our view, for the sub-scenarios “around 7 GHz” and “around 4 GHz + around 7 GHz,” co-site deployment with 4 GHz may be an important design consideration. To achieve comparable coverage at 4 GHz and 7 GHz, additional antenna elements and additional TXRUs, as well as higher transmit power or lower noise figure in the 7 GHz TXRUs, may be required.

In the Rel-19 FR3 channel model study, the number of antenna elements considered at 7 GHz or 14 GHz can be quite large. Inputs from operators on practical assumptions for such configurations would be highly valuable. If we assume that the noise figure for uplink reception at 7 GHz TXRUs remains the same as for 4 GHz TXRUs, then—beyond what is already provided in the current TR—it is essential to identify key parameters such as the number of antenna elements, number of TXRUs, and transmit power levels at 4 GHz and 7 GHz TXRU. These details are critical to inform technical discussions, including those related to MIMO and NES. We have

Proposal 2: In TR 38.914, capture the following for each sub-scenario (e.g., Around 4 GHz + Around 7 GHz):

- the number of antenna elements at a TRP

- the number of TXRUs at a TRP

- the Tx power at a 4GHz/7GHz TXRU

mTRP technology can be an important enabler of ubiquitous coverage for desired services; however, there are different variants of mTRP technology, such as coherent joint transmission, non-coherent joint transmission, and dynamic point switching. Practical considerations may influence the relative usefulness of each, so decisions on whether one or multiple technologies should be supported in 6G from day one need to be well justified by feasible deployment scenarios.

In our view, the network backhaul assumptions should be clearly stated for each scenario—for example, whether ideal backhaul can be assumed in dense urban or urban macro environments. Similarly, timing, frequency, and phase synchronization assumptions—which may be loosely referred to as fronthaul assumptions—also have a major impact on mTRP design.

We understand that different mobile operators may face varying practical conditions, such as access to dark fiber. Even so, enumerating the prevailing scenarios would be more valuable than leaving these assumptions entirely to a working group. While providing information for scenarios with little or no current deployment can be challenging, offering practical or at least plausible assumptions—particularly for well-studied environments like dense urban and urban macro—can be extremely useful. Thus, we have

Proposal 3: In TR 38.914, capture network backhaul assumptions (e.g., ideal, non-ideal with 10 ms latency) and TRP fronthaul synchronization assumptions such as timing/frequency/phase at least for some scenarios including dense urban and urban macro.

# Conclusion

In this contribution, we provide our views on 6G deployment scenarios, with the following proposals regarding

Proposal 1: Streamline the deployment scenarios and/or prioritizing deployment scenarios to facilitate working group discussions when possible.

Proposal 2: In TR 38.914, capture the following for each sub-scenario (e.g., Around 4 GHz + Around 7 GHz):

- the number of antenna elements at a TRP

- the number of TXRUs at a TRP

- the Tx power at a 4GHz/7GHz TXRU

Proposal 3: In TR 38.914, capture network backhaul assumptions (e.g., ideal, non-ideal with 10 ms latency) and TRP synchronization assumptions such as timing/frequency/phase at least for some scenarios including dense macro and urban macro.

# Reference

[1] RP-251395, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2] 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies (Release 14)