3GPP TSG RAN Meeting #109								                                       		       RP-252317

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.2

Source:	Apple

Title:	On IMT-2030 Technical Performance Requirements

Document for:	Discussion

# Introduction

RAN#108 approved the LS to ITU-R WP5D in [2] on IMT-2030 TPRs. That LS, however, did not cover some TPRs which were marked “TBD”. In this contribution we provide our considerations on those remaining TPRs, while considering the progress in ITU-R WP5D.

# Discussion

The LS to ITU-R WP5D on IMT-2030 TPRs approved by RAN#108 in [2] had the following TPR values marked as TBD:

- Peak Data Rate

- Peak Spectral Efficiency

- User Experienced Data Rate

- Energy Efficiency

- Area Traffic Capacity

- Mobility Interruption Time

- Positioning

- Sensing

- Joint/composite requirement

Furthermore, there were a few open points in the LS which affect the values, specifically:

- Number of UE antenna elements

Since that LS was sent, ITU-R WP5D has made some progress (which should be considered with the understanding that till WP5D approves the report, all the agreements are tentative and subject to change), however when it comes to TPR values, there aren’t any agreements yet and so 3GPP input from this meeting would be valuable.

Below we provide our views on these TPRs under discussion in RAN and ITU-R WP5D.

## Peak Data Rate

Arguably, the most visible TPRs are the ones related to data rate, i.e. Peak Data Rate, User Experienced Data Rate and Spectrum Efficiency. We observe that in real deployments in the field the gap between what a 3GPP technology can deliver (on paper) and what is actually being used has grown larger and larger. This is an indication that market demand has not caught up with IMT requirements and 3GPP technologies capabilities. We believe that driving the requirements much further will widen the gap and may have negative consequences.

Observation 1: the gap between what 5G/IMT-2020 can deliver “on paper” and what is actually being deployed is significant; such gap is unhealthy for the ecosystem.

With this in mind, many companies provided the opinion in RAN conference call on TPRs, that perhaps in IMT-2030/6G it would be beneficial not to define a peak data rate requirement, but rather leave this to the technology proponents (e.g. 3GPP) to decide. The following two alternatives have been discussed in the conference call:

- Alternative 1: Not to define a requirement, e.g. leave the value for peak data rate for the technology proponent to provide

- Alternative 2: Define a requirement with reasonable value

Speaking of alternative 2, it is rather unclear how one can possibly define “reasonable value” for IMT-2030 peak data rate, especially considering the fact that even the IMT-2020 values are anything but reasonable.

Proposal 1: not to define IMT-2030 peak data rate requirement or keep the IMT-2020 numbers.

## UE Antenna Elements

RAN#108 made agreements on DL 5th percentile and average user spectral efficiency, however there was no agreement for UL as there was now agreement on the number of UE antenna elements.

According to [6], the IMT-2020 number of antenna requirements were:

| Parameters | Values |
| --- | --- |
| Number of BS antenna elements | 700 MHz: Up to 64 Tx/Rx 4 GHz / 30 GHz: Up to 256 Tx /Rx 70 GHz: Up to 1024 Tx/Rx |
| Number of UE antenna elements | 700 MHz / 4 GHz: Up to 8 Tx /Rx 30 GHz: Up to 32 Tx /Rx 70 GHz: Up to 64 Tx /Rx |

For DL, RAN#108 have already agreed to increase the number of BS antenna elements in 7GHz up to 1024. This increase can already provide significant gains (if needed) and further increase of the UE antenna elements isn’t justified in our view. We should bear in mind that IMT-2030 requirements are minimum requirements and if in the future there would be a need for more, 3GPP can do this (as indeed was the case with some IMT-2020 values which went beyond the ITU-R requirements).

Proposal 2: for 700 MHz, 4 GHz and 7 GHz, the number of UE antenna elements is up to 8 Tx /Rx.

## Mobility interruption time

Regarding the mobility interruption time, our observations are quite similar to what we described above in the context of peak data rate.

Even though IMT-2020 required (and NR supported) 0ms mobility interruption time, this feature was never deployed. As is known, the technology developed to support this requirement (DAPS) turned out to be rather complex and costly (and there is no reason to believe that in 5G we will come up with a simpler and cheaper solution).

Furthermore, as it turns out, there is not much demand for 0ms mobility interruption time – others (if there was some demand), we would have seen at least some deployments by now.

Therefore, similarly to our proposal on peak data rate, we propose not to define such a requirement for IMT-2030. As mentioned above, this doesn’t necessarily mean 3GPP will not develop such a feature (if there is consensus to) – just there will be no such ITU-R requirement.

Proposal 3: not to define IMT-2030 mobility interruption time requirement.

## Sensing

According to the current draft of the ITU-R WP5D TPR report [2], WP5D seems to have settled on the following TPRs for sensing:

- Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. - Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. - Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. - [Sensing Resolution: TBD]

We suggest using the values from TS 22.137 [4], which includes ISAC performance requirements, when communicating with ITU-R, provided below for reference:

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 [3] | 5 [3] | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 0.5 | 0.5 x 0.5 for factories | 100 to 5000 | 0.1 | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |
| Environment monitoring | 5 | 95 | 10 | 0.2 (NOTE 4) | N/A | N/A | N/A | N/A | 60000 | 60 to 600 | 0.1 to 5 | 3 | Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring) |
| Motion monitoring | 6 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 5) |
|  | 7 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 to 50 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing (NOTE 6) |
| NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. NOTE 6: Category 7 has more stringent requirements (e.g., for KPIs such as positioning accuracy and sensing resolution) compared to other categories and typically requires more radio resources. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The TS 22.137 [4] provides a range of numbers for different Sensing Service Categories. Some of the scenarios considered in [4] are quite specific with rather extreme requirements (e.g. gesture recognition with 0.2m accuracy) and can hardly be considered “minimum” requirements (which is what the ITU-R report defines).

With this in mind, for minimum performance requirements, we should consider categories 1 and/or 2.

Additionally, and considering that RAN1 haven’t yet validated the numbers from 22.137 and in fact there are more conservative numbers being discussed in RAN1 [5]:

### [FL1][M] Proposal 4.8-1 - The following KPI values are considered as starting point for the evaluation of NR ISAC. Metric; Requirements. Missed detection Probability; 5%. False Alarm Rate; 2%. Horizontal Positioning Accuracy; 10 m @ 90%. Vertical Positioning Accuracy; 10 m @ 90%. Radial Velocity Accuracy; 1 m/s.

Note: these haven’t been agreed yet, the numbers above are just a proposal from the FL in the last RAN1#122 meeting.

Furthermore, ITU-R WP5D [3] are considering to use two Test Environments for ISAC:

- Urban Macro

- Indoor Factory

And so we may need to consider different values for these TEs. Therefore, we suggest to proceed with caution and recommend the following values.

On sensing resolution, out view that it can be calculated from system parameters (e.g. bandwidth) and therefore doesn’t add much in terms of technology evaluation.

Proposal 4: adopt the following IMT-2030 ISAC TPRs:

|  | Urban Macro | Indoor Factory |
| --- | --- | --- |
| Missed detection probability | 5% | 5% |
| False alarm rate | 2% | 2% |
| Horizontal position accuracy | 10m@95% | 2m@95% |
| Vertical position accuracy | 10m@95% | 2m@95% |
| Velocity accuracy | 1 m/s | 1 m/s |

## Positioning

Positioning is a new TPR as far as ITU-R evaluation is concerned, but it’s definitely not new to 3GPP technologies. This makes the discussion easier, as we can refer to the positioning capabilities of 5G. Going beyond the 5G capabilities for positioning should be justified with clear market use cases and requirements.

Observation 2: IMT-2030 positioning accuracy requirements can be based on 5G positioning requirements; going beyond those should be justified with clear market use cases and requirements.

The bulk of the work on positioning was carried out in 3GPP Release-16 and Release-17. According to the Release-17 TR 38.857 [5], the target positioning accuracy requirements were:

- For commercial use cases

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

- For Industrial-IoT use cases

- Horizontal position accuracy (< 0.2 m) for 90% of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

Considering that ITU-R requirements are minimum performance requirements (with the understanding that a technology may, and often does, do better), we suggest to use Rel-17 numbers as a recommendation for IMT-2030 requirements.

According to the ITU-R WP5D draft report [3] on Evaluation, the positioning requirements are likely to be evaluated in two Test Environments (TE):

- Indoor Factory, and

- Urban Macro

Hence the proposal:

Proposal 5: positioning accuracy requirements for the evaluation in the Urban Macro TE are 1m (horizontal) and 3m (vertical), whereas for Factor TE it is 0.2m horizontal and 1m vertical.

# Conclusion and Proposals

Observation 1: the gap between what 5G/IMT-2020 can deliver “on paper” and what is actually being deployed is significant; such gap is unhealthy for the ecosystem.

Proposal 1: not to define IMT-2030 peak data rate requirement or keep the IMT-2020 numbers.

Proposal 2: for 700 MHz, 4 GHz and 7 GHz, the number of UE antenna elements is up to 8 Tx /Rx.

Proposal 3: not to define IMT-2030 mobility interruption time requirement.

Proposal 4: adopt the following IMT-2030 ISAC TPRs:

|  | Urban Macro | Indoor Factory |
| --- | --- | --- |
| Missed detection probability | 5% | 5% |
| False alarm rate | 2% | 2% |
| Horizontal position accuracy | 10m@95% | 2m@95% |
| Vertical position accuracy | 10m@95% | 2m@95% |
| Velocity accuracy | 1 m/s | 1 m/s |

Observation 2: IMT-2030 positioning accuracy requirements can be based on 5G positioning requirements; going beyond those should be justified with clear market use cases and requirements.

Proposal 5: positioning accuracy requirements for the evaluation in the Urban Macro TE are 1m (horizontal) and 3m (vertical), whereas for Factor TE it is 0.2m horizontal and 1m vertical.

# References

[1]	RP-251825	Reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s)

[2]	5D/792-E	Annex 5.14 to Working Party 5D Chair’s Report WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[IMT-2030.TECH PERF REQ]

[3]	5D/792-E	Annex 5.12 to Working Party 5D Chair’s Report WORKING DOCUMENT TOWARDS PRELIMINARY DRAFT NEW REPORT ITU-R M.[IMT-2030.EVAL]

[4]	TS 22.137	Service requirements for Integrated Sensing and Communication

[5]	R1-2506476	Summary #1 on evaluations for NR ISAC