3GPP TSG RAN Meeting #109								RP-252318

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.3

Source:	Vodafone

Title:		Need time after June checkpoint for bilateral discussions on migration and
          architecture options for 6G

Document for:	Agreement

# 1	Introduction

The agreed RAN Working Group Study on 6G Radio (RP-251881) contains the following objective on migration and the SID’s interim milestone relates to migration aspects:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]
NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

#### 4.1.1	Interim Milestone

Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810].

TSG#112 (June/2026):

RAN1 to provide interim assessment on the following areas:

- Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c)

- Channel bandwidth (min and max), frame structure, numerology ((2) b, d)

- Basic sync signal structure and associated periodicity(ies) ((2) h)

For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026.

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

RAN plenary to make a decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

## 2	Discussion

For 4G to 5G migration, multiple architecture options were considered; a large subset of these architecture options were specified; and only a small subset have been widely commercialised.

For migration, the 6G RAN Working Group study is starting by only looking at dynamic spectrum sharing between NR and 6GR.

The need for additional migration options is likely to depend upon things like:

- RAN vendor implementations,

- the RAN equipment that the operator has deployed,

- the RAN equipment that the operator is currently planning to deploy,

- SA2’s progress on the architecture for 6G,

- etc.

Clearly, these topics are not suitable for discussion in 3GPP meetings but should be handled in separate bilateral operator-vendor meetings.

The June 2026 checkpoint on the above RAN 1 areas ought to enable the RAN vendors to assess the impact of 6G on their currently deployed and future implementations and therefore it can enable meaningful bilateral operator-vendor discussions on migration to start.

As many independent bilateral discussions will be needed, this process will take some time.

If those discussions have not started or are ongoing, it will be difficult for operators to remove architecture options (for example ones similar to option 3, option 5, option 6, etc) that they might need.

Hence, if the June 2026 RAN plenary initiates Working Group studies on additional migration options, it is likely that too many options will end up being studied.

This will increase the workload in the Working Groups (which might have a negative impact on the quality of the 6G radio interface studies), and might lead to some options that are unlikely to be commercialised (e.g. similar to option 5 and “Iu-mode GERAN”) gaining support.

As the technical and specification impacts of most of the architecture migration options are broadly understood from the 4G to 5G transition, it is suggested that RAN plenary #113 (September 2026) agrees which, if any, additional migration options should be studied by the RAN Working Groups. Joint discussions with TSG-SA may be needed in order to take SA2’s progress into account.

Decisions on which (if any) additional migration architecture options to specify should be taken by the TSGs at the end of the RAN Working Group study.

Proposal 1: TSG RAN #113 (September 2026) to decide on which (if any) additional migration options should be studied by 3GPP RAN Working Groups (taking progress in other TSGs into account).

# 3	Proposal

Based on the discussion in the previous sections we propose the following:

Proposal 1: TSG RAN #113 (September 2026) to decide on which (if any) additional migration options should be studied by 3GPP RAN Working Groups (taking progress in other TSGs into account).