3GPP TSG RAN Meeting #109					RP-252322

Beijing, China, September 15-19, 2025

Agenda Item:	8.2.1

Source:	VIAVI, AT&T, Telecom Italia, BT

Title:	Text Proposal on Base Station Feature Testing for 38.914

Document for:	Approval

# 1	Introduction

This contribution outlines the motivation for considering base station feature testing for 6G and proposes text to be added to section 5.6 of 38.914.

# 2	Discussion

Some of the strongest motivations for 6G in 3GPP discussions so far have revolved around the support for new services and use cases, improved energy efficiency and improved customer experience. The recently approved Study on 6G Radio emphasizes “practical user experience”, “performance excellence” and support for “diverse service classes”, and a desire for alignment with the evolving service demands. To ensure these things are delivered effectively, holistic black-box test and evaluation of the base station (BS) features under controlled conditions should be considered in addition to existing conformance tests. Drive tests alone are costly and time-consuming, and it is difficult to make such field tests extensive enough to cover all possible scenarios. Furthermore, the introduction of agentic AI that spans entire 3GPP solutions introduces an even greater possibility of unpredictable results due to complex agent and system interactions, resulting in more time and money spent in the field troubleshooting. Thus, there exists an opportunity to improve the test, evaluation and certification process to provide greater performance assurance levels for the BS equipment before it leaves the lab.

BS feature testing consists of establishing lab-based test configurations and methodologies for key use cases and scenarios that leverage UE emulation for controlled and repeatable tests (Figure 1). Such an approach provides a path towards service and use case-focused test and evaluation that can bridge the gap between existing BS conformance tests and field deployment. By also working towards consensus on minimum BS feature performance requirements for key scenarios and use cases in line with 38.914, 3GPP could help facilitate a form of carrier-grade certification.

Figure 1. BS Feature Testing with UE Emulation

# 3 	Text proposal

Based on the discussion above we propose the following text for TR 38.914.

## 5.6.x Base Station Feature Testing

Test and performance evaluation for 6G should include a top-down approach that aims to verify the actual value delivered at a service and end-user level. This is in line with the 6G focus on practical user experience, performance excellence, support for diverse service classes, and alignment with evolving service demands. To ensure these things are delivered effectively in 6G, test and evaluation of base station features should be considered in addition to existing conformance tests. Consensus on minimum BS performance requirements for key scenarios and use cases could also help facilitate carrier-grade certification.

# 4	Conclusion

In this contribution we propose to add the text in section 3 to TR 38.914.